CL-62X5 OS/2 v2.1 Driver Installation Disk

(v1.30)

OS/2 Desktop Driver Limitations

-------------------------------

No display switching between CRT/Panel/SimulSCAN in OS/2 Desktop.

WIN-OS/2 Windows Session (seamless) is not supported in the following

configurations:

OS/2

Desktop  Colors  & 	Win-OS/2   Colors  Supported

-------  ------   	---------  ------  ---------

640x480    16     	640x480     16      Yes

"       "      	640x480     256     No

"       "      	800x600     16      No

"	      "      	800x600     256	    No

"	      "	     	1024x768    16	    No

640x480   256     	640x480     256	    No

"       "      	640x480     16 	    No

"       "      	800x600     256	    No

"       "      	800x600     16	    No

"       "      	1024x768    16	    No

800x600   256     	640x480     256	    No

"       "      	640x480     16	    No

"       "      	800x600     16 	    No

"       "      	800x600     256	    No

"       "      	1024x768    16	    No

When the current configuration is set to OS/2 Desktop 800x600 and you reboot

system without a CRT connected, the display does not drop down to 640x480 in

the panel, as it does in windows.  When reconnect the CRT, the display comes

up on the CRT.

Resolution and Colors Supported in OS/2 PM Desktop

--------------------------------------------------

640x480 16 & 256 colors    (CRT/Panel/SimulSCAN display)

800x600 256 colors         (CRT display only)

Resolution and Colors Supported in WIN-OS/2 Window Session

----------------------------------------------------------

640x480 16 colors in OS/2 Desktop combined with	...

640x480 16 colors in WIN-OS/2	 (See Driver Limitations above)

Resolution and Colors Supported in WIN-OS/2 Full-Screen Session

---------------------------------------------------------------

640x480  16 & 256 colors    (CRT/Panel/SimulSCAN display)

800x600  16 & 256 colors    (CRT display only)

1024x768 16 colors	    (CRT display only)

How to Install OS/2 Desktop Driver & WIN-OS/2 Full-Screen Session

-----------------------------------------------------------------

This procedure uses the user-selected resolution & color for OS/2 Desktop

and 640x480 16 colors as the default for a WIN-OS/2 Full-Screen Session.

1.  Make sure the 62x5 OS/2 Driver disk is labeled "DISP" by using DOS

Label command (C:\DOS\LABEL A: or B:).

2.  Install OS/2 v2.1 (if not already installed) and select "VGA" as the

primary display adapter during installation. Do not select "Cirrus Logic"

because it currently only suppports Cirrus Logic desktop products.

3.  Select the proper display device, such as panel or CRT.

4.  Insert the Cirrus Logic 62x5 OS/2 Driver disk into the floppy drive.

5.  Open an OS/2 Full-Screen session.

6.  Change the prompt to the floppy drive which contains the driver disk.

7.  Invoke the OS/2 batch file CLINST21.CMD using OS/2 boot drive, without

a colon, as the parameter ([A:\]CLINST21 C).

8.  Select the primary display type and "Cirrus Logic 62x5 Panel/CRT driver".

9.  If you plan to start out on Panel or SimulSCAN display, choose 640x480

resolution.

10. Follow the applicable directions as they appear on screen.

How to Change Resolution & Colors in OS/2 Desktop

-------------------------------------------------

Follow steps 5-10 above (WIN-OS/2 Full-Screen session will be reset to

640x480 16 colors).

How to make WIN-OS/2 Window Screen Session work with OS/2 Desktop

-----------------------------------------------------------------

Only one configuration is supported (see OS/2 Desktop Driver

Limitations above).

If you are running OS/2 2.1:

1. Install the Cirrus Logic 640x480 16 color OS/2 driver.

2. Add sdisplay.drv=SWINVGA.DRV in the [boot] section in

\OS2\MDOS\WINOS2\SYSTEM.INI file.

If you are running OS/2 2.1 for Windows:

1. Install the  Cirrus Logic 640x480 16 color OS/2 driver.

2. Add sdisplay.drv=SWINVGA.DRV in the [boot] section in

\WINDOWS\SYSTEM.INI file.

How to Change Resolution & Colors in WIN-OS/2 Full-Screen Session

-----------------------------------------------------------------

If you are running OS/2 2.1:

1. Open Win-OS/2 Full-Screen session.

2. Select File from drop-down menu in Windows Program Manager.

3. Insert the GD62X5 OS/2 2.1 Driver disk into A: or B: and run

INSTALL.EXE.

4. A VGAUTIL group will be created.  Click SetRES icon to change resolution

and colors. Run CRT, PAN, SimulSCAN to switch display.

If you are running OS/2 2.1 for Windows:

1. If you have already installed the SetRES utilities from the Windows

driver disk v1.30, use SetRES to change resolution and colors.

Otherwise, go to the next step.

2. Open Windows.

3. Select File from drop-down menu in Windows Program Manager.

4. Insert the Windows driver disk v1.30 into A: or B: and run INSTALL.EXE.

5. A VGAUTIL group will be created.  Click SetRES icon to change

resolution and colors. Run CRT, PAN, SimulSCAN to switch display.

Special Case when running OS/2 2.1 for Windows

----------------------------------------------

If you are running OS/2 2.1 for Windows and have installed the Cirrus

Logic OS/2 drivers and wish to switch between OS/2 and Windows running

under DOS, the following changes must be made to the SYSTEM.INI file.

Return to Windows running under DOS from OS/2:

1. Boot to DOS from OS/2 Desktop (click Dual-boot icon).

2. While in DOS, open \WINDOWS\SYSTEM.INI and comment out the line

OS2vers=21 in [CLVGA] session (;OS2vers=2).

Return to OS/2 from Windows running under DOS:

1. While in DOS, open \WINDOWS\SYSTEM.INI and take out the ";" in

front of OS2vers=21 in [CLVGA] session.

2. Change to OS/2 directory (cd \os2).

3. Boot to OS/2 (boot /os2).