NAME

vgasync - Keep VGA sync polarities negative

SYNOPSIS

detach vgasync

DESCRIPTION

'vgasync' runs as a background process under OS/2 and periodically

(re)sets the horizontal and vertical sync polarities of an (S)VGA card to

negative sync.

Many (all?) SVGA cards change the polarities of the H and V sync

signals to indicate to an assumed multisync monitor the current video mode

(and implied scan frequencies). 'vgasync' allows older fixed-frequency

monitors that require a fixed sync polarity to be used with such SVGA

cards without requiring hardware modification of the sync signals.  With

the addition of manual sync controls on the monitor and/or a means (eg.

software drivers) of tweaking the VGA CRT control registers, a "fixed

frequency" monitor can then display most of the SVGA extended video modes.

This program requires IOPL to run.