ShrDog+ - master boot and boot viruses vapouriser. Version 1.00

Copyright (c) 1990 by Ten Plus Software. All rights reserved.

ShrDog+ is program designed to protect your  hard disks  against boot and

master boot viruses.  ShrDog+ isn't a resident program.  When  you start it,

simply checks previously saved master boot and boot records of the specified

hard disks and if differences are found between saved and  current  state of

master boot or boot records, ShrDog+ restores them and restarts computer. If

no changes are found, ShrDog+ exits to DOS.

ShrDog+ consists of two parts. First one is master boot and boot records

saver, and has the following syntax:

SHRDOG -save  [drives... datafile] ...

Option '-save' means that ShrDog+ has to save master boot and boot records

of specified drives in specified datafiles. If only '-save' is specified, it

saves all the master boot and boot records in single  file, named SHRDOG.DAT

in SHRDOG.EXE directory.  For example, if you have two  hard  disks  and you

want to save the master boot  and  boot  images of  drive C: in  file, named

FIRST.DAT type the following:

SHRDOG -save C: FIRST.DAT

ShrDog+ will save information about drive C: in current directory. If you

want to have FIRST.DAT in another directory,you must  specify the  full path

to this file.If you want to save master boot and boot images of the two hard

disks in separate files, type the following:

SHRDOG -save C: FIRST.DAT D: SECOND.DAT

Note: Only information about hard disk drives can be saved.

The second part of  ShrDog+  checks  boot  and  master  boot  records. If

differences are found  between  saved  and  current  state of  boot records,

ShrDog+ saves current boot or master boot record in files,named C_BOOT.IMG -

for the boot  record, and  C_MBOOT.IMG - for  the master  boot  record.These

files are placed in SHRDOG.EXE directory.  If you want to run  this  part of

ShrDog+ you must follow this syntax:

SHRDOG [datafile]...

If datafile is omitted SHRDOG.DAT is assumed.

Note: ShrDog+ uses special tracing technique to obtain address  of  ROM BIOS

disk I/O services.

You may include ShrDog+ in your AUTOEXEC.BAT file, but  it  will be safer

if this is the first program in your computer  startup  process. So, if  you

are using DOS 4.00  or  higher put  ShrDog+  in  your  CONFIG.SYS  with  the

following command:

INSTALL=Drv:\Directory\SHRDOG.EXE

Feel free to write or fax to Ten Plus Software if you have notes considering

ShrDog+ improvements on address:

Ten Plus Software

ShareLock+ Professional Technical Support

25 Veliko Tarnovo str.

4000 Plovdiv, Bulgaria

or fax to:  +359 32 262-474