-------------------------------------------------------------------------------

-            - SvgaTest.Doc for Mono.exe Program, Version 3.50 -              -

-------------------------------------------------------------------------------

SVGAT350.ZIP Contents:

- MONO.EXE	Main program

- SVGATEST.DOC  This file

-------------------------------------------------------------------------------

Hardware needed:

- A PC XT-AT or compatible, PS/2 with at least a VGA CARD.

- 256K Ram or more.

-------------------------------------------------------------------------------

Purpose:

The program has (Microsoft) mouse support and is meant to test Vga, SuperVga

and IBM8514 /a modes and colors. It draws a screen close to the PHILIPS TV-Test

used by most TV Broadcast. It can be used to verify the aspect ratio of the

video monitor you use, or to trim its controls before starting a session.

Short description:

It has command line interpreter, and two string of characters may be specified,

to be shown on the test, with a $ starting each string (Eg. MONO $TV $AT-486DX)

will show the message TV AT-486DX on the screen.

It is important to have a VGA video card, since the start menu is 640x480x16 as

resolution. At start, the program tests the hardware to detect extended SVGA

modes available, while the screen is cleared. Then a menu is prompted. If a

certain video mode is unavailable to your video bios, the related Button is

not active on the menu.

Modes under test are:

Vga and SuperVga

640x480 16 colors 		 640x480 256 colors  512K

800x600 16 colors  256K	 800x600 256 colors  512K

1024x768 16 colors  512K	1024x768 256 colors 1024K

IBM 8514 /AI

640x480 16 colors 		 640x480 256 colors

1024x768 16 colors		1024x768 256 colors  768K

-------------------------------------------------------------------------------

This is a FREEWARE program, and you are free to use, copy, distribute to your

friends and to upload on BBS. The only condition is that you MUST NOT modify

the program and its documentation in any way and you MUST distribute it in its

original zip file format, archived into PkWare's SVGAT350.ZIP format.

The Authors:

Marco Melandri

Claudia Guiducci