DSP BLASTER 2.0

Copyright 1998 by Brian Beezley, K6STI

All Rights Reserved

DSP Blaster 2.0................................1

Evaluation Version.............................1

Connecting the Sound Card......................1

Adjusting Audio Gain...........................2

Graphics Displays..............................3

Using DSP Blaster..............................3

Use with Other Software........................5

Use in Windows.................................6

Highpass Filter................................6

Lowpass Filter.................................6

Bandpass Filter................................7

Notch Filter...................................7

Noise Reduction................................7

Peaking Filter.................................8

Coherent Bandpass Filter.......................8

Automatic Gain Control........................10

Recommended Settings..........................11

Copyright and License.........................11

---- DSP BLASTER 2.0 -------------------------------------------

DSP Blaster(TM) provides real-time DSP filtering and

spectral analysis using your PC and sound card.  The program

uses digital signal processing to provide highpass, lowpass,

bandpass, peaking, automatic-notch, noise-reduction, and

coherent CW filters, as well as automatic gain control.  DSP

Blaster uses 100% assembly language to implement optimized,

floating-point algorithms.  The program requires a 486 or

better, math coprocessor, VGA, mouse, 16-bit Creative Labs sound

card, and DOS 3.0 or later.

---- EVALUATION VERSION ----------------------------------------

For full functionality, put your DB.LIC license file in

the directory that contains DB.EXE.  (If you rename DB.EXE to

XYZ.EXE, rename DB.LIC to XYZ.LIC.)  The unlicensed evaluation

version runs for a limited time and does not retain its state or

configuration memories when restarted.

---- CONNECTING THE SOUND CARD ---------------------------------

DSP Blaster can use any 16-bit Creative Labs sound card.

"Compatible" cards won't work.  DSP Blaster uses the SET BLASTER

statement to determine sound-card hardware settings.  You may

need to configure your card to these settings in AUTOEXEC.BAT by

executing a utility like DIAGNOSE or CTCM.  The two DMA channels

must differ.  See your sound-card manual for details.  DSP

Blaster does not require sound-card device drivers.

DSP Blaster normally uses 16-bit input and 8-bit output

resolution.  However, CT417x cards provide 16-bit output.  This

greatly reduces output quantization noise.

Here's the best way to hook up your sound card:  Feed

receive audio to the line-left and line-right inputs in

parallel.  Connect the line outputs to an amplified stereo

speaker system with volume control and headphone jack.  (You can

directly drive speakers or headphones from the sound-card

speaker outputs, but then you must control volume with the mouse

or keyboard.)

CAUTION:  Some amplified speakers do not use series

resistors in their headphone circuits.  These units can generate

very high sound-pressure levels when driving low-impedance

headphones.  To reduce output, add 100-ohm series resistors to

the left and right headphone leads.  Always do this when

connecting low-impedance headphones to the sound-card speaker

output.

You can obtain audio from your receiver's speaker

output, but generally it's better to use a fixed-level audio

output when available.  A fixed output level makes it

unnecessary to set your receiver's AF-gain control for proper

1

sound-card drive.  (However, some transceivers don't provide SSB

monitor or CW sidetone on their fixed-level output.)  If your

receiver's fixed-level output can't fully drive the line inputs

as described in the next section, try driving the mic input

instead.  Alternatively, drive the line inputs with the speaker

output and turn up the AF gain.

You can use your transceiver's relay-control line to

automatically bypass DSP during transmit.  Bypass eliminates the

processing delay that may make monitoring your transmit signal

awkward.  Connect the control signal (ground on transmit,

greater than 3 volts on receive) to CTS on a spare serial port.

You can use a direct connection as long as the voltage at CTS

never exceeds 15 volts.  CTS is pin 5 and ground is pin 7 on 25-

pin connectors.  CTS is pin 8 and ground is pin 5 on 9-pin

connectors.  Alternatively, you can connect a transmit contact-

closure to ground to pin 2 of the 15-pin game-port connector on

your Sound Blaster card.  Ground return is pin 4.  The sound-

card JYEN pins must be jumpered (factory default) to enable its

game port.  Disable any other game port in your computer.

To provide output on both channels during bypass, you

must connect the input signal to both line-left and line-right

inputs.  (The sound card provides no way to feed one channel to

the other during bypass.)  If you don't use bypass, you can

connect separate audio sources to the line inputs.  (You always

can connect another source to the mic input).  DSP Blaster

maintains a separate gain setting for each input so you can

switch sources without readjusting input gain.

---- ADJUSTING AUDIO GAIN --------------------------------------

Type DB at the DOS prompt to start DSP Blaster.  Pass

the mouse cursor over Setup and select the input source and

bypass-signal port by clicking on them.  Set your receiver's RF

gain to maximum and tune in a strong signal.  Place the mouse

cursor at Input to adjust input gain.  Click the up/down arrows

to change gain in 2-dB steps.  (Some sound cards have 8-dB steps

or no gain increase above 62 dB).  The bar graph in the lower-

center of the screen displays peak input amplitude.  The graph

turns red when the A/D converter clips.  Raise the input gain

until clipping occurs, then back it off one or two steps.

Infrequent, brief clips aren't audible so don't worry if the

graph occasionally flashes red.  However, sustained overdrive

will cause audible distortion.

NOTE:  It's easy to overdrive the sound card's high-gain

microphone preamp.  Reducing the input-gain setting won't

correct the resulting distortion because the preamp comes before

the gain-control stage.  Therefore, it's important to check for

distortion when using the mic input.  Tune your receiver

past a strong, clean sine wave (for example, the signal from your

calibrator/marker on a dead band).  Use the spectral display to

look for harmonics generated by an overdriven preamp.  You

should see just one signal pip as you tune through the 3-kHz

2

passband.  Look for flattened sine-wave tops and bottoms in the

waveform display.  If your receiver's fixed-level output

overdrives the mic input, add a series resistor (try 3300 ohms)

to form a voltage divider with the 540-ohm mic input resistance.

Place the mouse cursor at Output and click the up/down

arrows to adjust output gain (or press the # and # keys).  If

you're using amplified speakers, the output-gain setting isn't

critical.  However, it's still possible to overdrive an external

amplifier.  Listen carefully for distortion on signal peaks.

It's best to set output gain so that the speaker volume control

works within its normal range.  If you're directly driving

speakers or headphones, adjust output gain to set listening

volume.  If a setting below 30 dB or so is required with

headphones, add 1000-ohm resistors in series with the amplifier

outputs and a 10-ohm resistor across each earphone.  These

networks will attenuate residual amplifier hiss at least 40 dB.

---- GRAPHICS DISPLAYS -----------------------------------------

DSP Blaster displays the input-signal spectrum.  The

display normally shows spectral amplitude from 0 to 3000 Hz on

SSB and 0 to 1500 Hz on CW.  When you activate the BPF, peaking

filter, or CBPF, the frequency limits are set just beyond the

BPF passband edges.  (The frequency annotations apply to the

display ends, not to nearby tic marks.)  It's fascinating to

correlate spectral features with the sound of a voice.  Is it

true that female voices have more high-frequency content than

those of lower-pitched males?  You can find out with the

spectral display.  Here's another interesting use:  Raise the

input gain and watch the noise spectrum as you adjust your

receiver's passband tuning, engage its narrow IF filters, etc.

The display on the right depicts the input waveform or

its envelope.  The waveform display is like a synchronized

oscilloscope trace.  It shows 21 ms of signal on SSB and 38 ms

on CW.  Use it to check speech-compression levels, line-noise

characteristics, receiver AGC overshoot, etc.  The envelope

display shows 6 seconds of signal peaks on SSB and 11 seconds on

CW.  It can reveal noise ticks, static crashes, AGC attack and

recovery characteristics, etc.

---- USING DSP BLASTER -----------------------------------------

DSP Blaster draws a system block diagram on your screen.

Pass the mouse cursor over any block to display a menu of

parameters.  The active selection of multiple-choice items is

highlighted.  Click on an item to select it.  Click the up/down

arrows to adjust a numerical parameter.  Click anywhere else in

the block to toggle its function on or off.  (You also can type

the first letter of a block to toggle it; type R for NR).  The

block border and title highlight when the block is active and

processing signals.  You can cascade as many active blocks as

your computing power permits.

3

The leftmost, bottom-line selection of the HPF, LPF,

BPF, NOTCH, and NR menus provides a faster but less precise

filter.  Use this setting when DSP Blaster displays "Too much

calculation."  The program disables the last filter selected

when there's not enough processing power to run it.

Click Bypass, press Y, or click the right mouse button

to bypass digital processing.  Click or press again to process.

Click Squelch or press S to toggle the squelch.  Adjust

squelch threshold and speed in the AGC menu.

Click Waveform/Envelope to change the right-hand

display.

Click Exit or press Esc to quit DSP Blaster.  When you

restart the program, all settings are restored (licensed version

only).

The CW/SSB indicator reflects the digital sample rate.

The CW sample rate and unprocessed bandwidth are about half

those for SSB.  DSP Blaster sets the sample rate this way:  HPF,

LPF, and NOTCH invoke the SSB rate, while BPF, PEAK, and CBPF

invoke the CW rate.  Neither NR nor AGC alter it.

Some sound cards require compensation for interchannel

output delay.  Click Setup and select the Interchannel Delay

Comp setting that yields no shift in spatial position between

processed and bypassed signals.

It's easy to save and recall complex filter

configurations.  Set all parameters as desired, click Save, and

click one of the configuration memories, for example, M1.  Then

enter a configuration description (or nothing at all, if you

like, by moving the mouse away).  To recall a configuration,

simply click on its description.  You can save nine

configurations.  A recalled description turns blue when the

program state differs from that of the configuration memory.

(To change a description, click on it, click Save, and then

reenter the text.  To erase a description, click Save, click on

the description, and press Enter.  To reuse an erased memory,

set all filter parameters, click Save, click the blank space,

and enter a description.)

Pass the mouse cursor over Colors to change screen

colors.  (To show them all, enable the CBPF with AFT, raise the

input gain until clipping sometimes occurs, and view both the

waveform and envelope displays.)  Click the up/down item arrows

to select a screen item.  An item blinks when selected.  Then

click the up/down color arrows to select one of 64 intensities

for each of the red, green, and blue color components.

DSP Blaster saves the current parameter settings,

configuration memories, and screen colors in DB.INI in the

directory containing DB.EXE (licensed version only).  Delete

DB.INI to reset everything to factory defaults.

4

---- USE WITH OTHER SOFTWARE -----------------------------------

Instead of clicking Exit, you can click TSR.  TSR stands

for terminate and stay resident.  After you click TSR, the DOS

command line returns but DSP Blaster continues to process audio

in the background.  You can then start another program.

To access DSP Blaster from another program, press Pause.

The DSP Blaster screen will reappear and you can then change

filter configurations and observe the graphics displays.  To

return to the interrupted program, click Exit or press Esc.  To

uninstall DSP Blaster, click Uninstall.  (Normally you'll do

this after quitting the other program.)

DSP Blaster attempts to fully restore the interrupted

program's character-mode screen.  However, the BIOS function

that performs this service is notoriously inadequate on many VGA

cards.  If the screen appears abnormal after you return to the

interrupted program, try starting DSP Blaster with the R

command-line option (type DB R).  This tells it to bypass the

BIOS and use a screen-restore technique that should work for

ordinary character-mode screens on any VGA card.  (However, the

R option may not correctly restore screens that take advantage

of advanced VGA features).  To avoid hard-disk access, DSP

Blaster does not attempt to restore graphics screens.  For these

activate your program's screen-redraw function.

You may find it easier to use hotkeys than to switch

back to DSP Blaster with Pause.  Press Ctrl-Shift-key from

within your application to alter these DSP Blaster functions:

HPF H    BPF  B    NOTCH N    NR  R    Bypass  Y   Louder #

LPF L    PEAK P    CBPF  C    AGC A    Squelch S   Softer #

For example, to toggle noise reduction, press Ctrl-

Shift-R.

It's much faster to use a hotkey than to switch to the

DSP Blaster screen if your monitor blanks the screen for a long

time when switching video modes.  Hotkeys are particularly

useful when running DSP Blaster with a contest-logging program.

DSP Blaster requires about 52K bytes of memory.  Use the

LH DB command to load it in upper memory.

To install DSP Blaster from a batch file, add I to the

command line (DB I).  The program will load and TSR so your

batch file can continue.  To uninstall, use DB U.  Add A to the

command line to use Ctrl-Alt instead of Ctrl-Shift for control

functions.  Add S to use Alt-Shift.  Use A or S when the Ctrl-

Shift key combination is already used by another program.

Your mouse requires a device driver.  Some mouse drivers

(for example, Logitech) have bugs that can affect DSP Blaster.

Use a Microsoft mouse driver whenever possible.

5

---- USE IN WINDOWS --------------------------------------------

The following instructions assume that all DSP Blaster

files are in \DB.

To install DSP Blaster in Windows 3.x, select a program

group in Program Manager.  Select File | New.  In the New

Program Object menu, select Program Item and click OK.  In the

Program Item Properties menu, under Description enter DSP

Blaster.  Under Command Line, enter DB.  Under Working

Directory, enter \DB.  Under Shortcut Key, enter D.  Click

Change Icon.  Click OK when told no icons are available.  In the

Change Icon menu, under File Name enter \DB\DB.ICO.  Click OK

repeatedly to dissolve all menus.

To install DSP Blaster in Windows 95, start Explorer and

navigate to \DB.  Using the right mouse button, drag the DB.EXE

icon to the desktop.  Click on Create Shortcut(s) Here.  Next,

right-click on the dragged icon.  Click Properties, then click

the Program tab.  Click on Change icon.  Click Browse, navigate

to \DB, and double-click on DB.ICO.  Click OK twice.

To run DSP Blaster, double-click on its icon.  Set the

filter parameters the way you want them.  Press Alt-Tab to

switch to another program.  (If sound stops, try installing the

generic Windows VGA driver.  If it still stops, click TSR before

switching to another program.  You'll get a screen that says

your pop-up program is ready to run.  If you get a blank screen,

specify R as a command-line parameter.)  To switch back to DSP

Blaster, press Ctrl-Alt-D.  (When TSR you'll switch back to the

pop-up message screen.  You can then use DSP Blaster's hotkeys

or you can press Pause to bring up the DSP Blaster screen.)

---- HIGHPASS FILTER -------------------------------------------

DSP Blaster provides a highpass filter to reduce hum and

low-frequency QRM for SSB, digital, and image modes.  The 300-Hz

default corner frequency passes most of the SSB voice spectrum

while attenuating low-frequency power.  Adjust the corner

frequency as necessary.  (All corner and passband frequencies in

DSP Blaster are specified at the -6-dB points.)

---- LOWPASS FILTER --------------------------------------------

The lowpass filter is really handy for eliminating high-

frequency "monkey chatter" from adjacent SSB signals.  The

default 2.5-kHz corner can clean up a signal with little loss of

voice quality.  It's particularly useful with older receivers

whose IF passbands extend to 3 kHz or beyond.  Adjust the corner

frequency as QRM warrants.

Combine the highpass and lowpass filters to create a

bandpass filter for digital or image modes.  Set the passband

edges just beyond the signal spectrum.  (The spectrum of FSK and

6

FM signals normally extends well beyond the instantaneous-

frequency limits.)

---- BANDPASS FILTER -------------------------------------------

The bandpass filter is intended to reduce QRM during CW

reception.  You can adjust center frequency and bandwidth.  (The

passband becomes somewhat rounded and attenuated at bandwidths

below 100 Hz for fast cutoff and 200 Hz for slow.)  The center

frequencies of the BPF, peaking filter, and CBPF track.

---- NOTCH FILTER ----------------------------------------------

The automatic notch filter greatly reduces the power of

steady or intermittent tones.  The filter can suppress multiple

tones and automatically determines tone frequency.  Although

normally transparent to voice signals, you can switch to the

low-gain setting if you detect voice-quality degradation.  (Tone

suppression won't be quite as deep and the notch won't form as

quickly).  The notch filter is intended only for voice

reception.

---- NOISE REDUCTION -------------------------------------------

You'll likely use DSP Blaster's NR filter more than any

other.  This filter uses the Widrow-Hoff LMS algorithm to

adaptively converge the tap weights of an FIR filter to the

autocorrelation coefficients of the input signal.  Because its

frequency response approximates the peaks of the signal power

spectrum, the filter suppresses noise between spectral peaks

without restricting signal bandwidth.

Voice signals contain a series of spectral peaks at

harmonics of the fundamental vocal-chord pitch frequency.  Thus,

they are good candidates for the LMS noise-reduction strategy.

The NR filter can actually improve the quality of distorted

speech by filtering out intermodulation-distortion products

between spectral peaks.

The NR quieting parameter appears only in SSB mode.  The

normal setting provides lowest signal distortion.  High quieting

suppresses noise more effectively but may slightly distort vocal

dynamics.  Experiment to determine the setting you prefer under

various conditions.  NOTE:  Evaluate the high-quieting setting

only during speech.  This setting invokes dynamic elements that

have no effect on idle-channel noise.

The three background-noise settings offer different

trade-offs between background-noise level and noise artifacts.

The best setting is a matter of personal preference.

The low-order setting reduces computational load for

slow computers.  The high setting invokes a long LMS filter for

7

recovery of very noisy signals.  DSP Blaster provides a huge

setting in CW mode for maximum noise reduction.  The high and

huge settings somewhat soften code elements and may not be

suitable for high-speed CW reception.  These settings require

considerable computation and may not run on slower computers.

---- PEAKING FILTER --------------------------------------------

The peaking filter is intended to enhance the signal-to-

noise ratio of weak CW.  Unlike conventional analog peaking

filters, the impulse response and spectrum of DSP Blaster's

digital peaking filter match those of specific tone bursts (CW

dots at the speed set in the Peak menu).  When aligned to the

received-CW speed, the matched filter maximizes detection

probability for a threshold detector.  (The ear and brain are

sophisticated signal processors, so this filter isn't

necessarily optimal for human CW detection.)  Rectangular dots

are filtered to trapezoidal pulses that become triangles and

just barely touch when the CW and filter speeds match.  The dots

and dashes of faster CW begin to run together as the triangles

overlap.  Overlap occurs above the speed set in the Peak menu.

It's not necessary to exactly match the filter and

received-CW speeds to provide good copy.  Set the speed to

anything greater than that of the received signal.  You can

adjust speed in 5-wpm steps between 15 and 60 wpm.  The lower

you set it, the greater the signal enhancement but the narrower

the filter becomes.  The matched filter can really enhance weak

CW, but you must tune carefully because the filter is extremely

narrow.  When you activate the peaking filter, a dotted line

appears in the spectral display to mark the filter center

frequency.  Tune your receiver until the received-signal pip is

as close as possible to the dotted line.

The 15-wpm matched filter is 2.4 dB down just 5 Hz from

center.  If your receiver has 10-Hz tuning steps, you might not

be able to tune any closer.  DSP Blaster provides automatic fine

tuning (AFT) to solve this problem.  AFT aligns the filter and

signal frequencies by slowly seeking a nearby spectral peak.

Manually tune as close as you can and then let AFT make the

final adjustment.  DSP Blaster disables AFT when you bypass

signal processing or run the program in the background.  You can

manually disable AFT or clear its frequency offset in the Peak

menu.  The dotted line turns gray when AFT is disabled.  (AFT is

designed to lock to intermittent CW signals.  Its performance

cannot be properly evaluated with a steady carrier.)

---- COHERENT BANDPASS FILTER ----------------------------------

DSP Blaster provides a special narrowband processor for

CW reception.  The coherent bandpass filter phase-locks to a CW

signal and splits its power into in-phase (I) and quadrature (Q)

components.  A phase-stable signal appears only in the I

component.  Uncorrelated noise and QRM appear equally in I and

8

Q, so the signal-to-noise and signal-to-interference ratios are

enhanced 3 dB in I.  Select this output for quieter reception of

weak CW.  Monitor the Q output to evaluate signal coherence and

phase-lock quality.

The CBPF uses a rounded bandpass filter with slow

rolloff.  Its corners aren't sharp like those of the input BPF

because the CBPF is intended to reduce noise rather than fight

QRM.  When a passband with sharp corners is narrowed, the

corners aggravate noise perception.  You can narrow a rounded

passband much further without audible problems.  This makes the

CBPF very useful as a final noise-shaping stage following the

wideband NR filter.

BP is the ordinary, noncoherent BPF output.  Use it as a

reference when evaluating coherent S/N enhancement in I.  BP

also provides a handy rounded passband when it's too much bother

to carefully tune for phase-lock.

Sw is a special stereo output.  The I component is

applied in phase to the left and right output channels and

appears spatially centered.  Q is applied out of phase and

appears nondirectional and diffuse.  This provides striking

spatial separation of signals and noise.  Sw does not reduce

noise because it uses the full Q component, but the spatial

separation isolates the signal and greatly enhances perception.

It lets you hear transmitter and propagation anomalies that are

monophonically inaudible.  It also lets you spatially identify

signals with unstable phase characteristics (the angular

spectrum will be smeared but unique).  The stereo sound of

signals affected by polar flutter or EME scintillation is

fascinating.  Sw provides a wide stereo image.  The Sn output

attenuates Q 6 dB to narrow the image.  You may prefer Sn when

listening with headphones.  (High-quality earpieces with matched

phase characteristics enhance stereo imaging.)  Sn also provides

2 dB of noise reduction.

The CBPF provides the only way to separate signals on

the same frequency.  Sometimes it can even resolve severe

multipath, including simultaneous short-path/long-path

propagation and 'round-the-world echos.  Listen in stereo to

spatially resolve multipath components.

The CBPF must accurately phase-lock to a signal.  DSP

Blaster provides automatic fine tuning for signal acquisition

and tracking.  It works just like the peaking-filter AFT.  (If

you use the CBPF while running DSP Blaster in the background

where AFT is disabled, select BP or a stereo output.  Frequency

offsets that occur without AFT will cause phase errors that leak

signal power into Q and reduce S/N in I.)

The CBPF draws a polar I/Q scatter plot that shows a few

dozen I and Q samples from the last two seconds.  When the CBPF

is properly phase-locked to a strong, phase-stable signal, the

dots form a clean ray that extends upward from the polar center.

Frequency offset causes a phase error that tilts the ray.  Phase

9

instability or noise disperses the dots.  The scatter plot lets

you monitor signal coherence and phase-lock without losing

audible signal by selecting the Q output.  Disable phase-locking

(by selecting the BP output) and AFT to observe fascinating

phase/frequency behavior of polar signals.

---- AUTOMATIC GAIN CONTROL ------------------------------------

DSP Blaster's AGC provides benefit in several

situations, particularly during CW reception.  You should always

enable it in CW mode unless you're using a CT417x sound card

with 16-bit D/A.

First, a strong interfering signal may dominate your

receiver's AGC and reduce its gain.  Although DSP Blaster may

completely filter out the QRM, the residual audio level may be

so low in extreme cases that you're no longer able to hear the

desired signal.  DSP Blaster's AGC works after all filtering to

restore normal signal level.

Similarly, AGC can help when you zoom in on a very weak

CW signal with one or more filters.  Although the signal plus

noise may be within your receiver's AGC control range, DSP

Blaster's filters can isolate a very small portion of total

audio power.  If you don't restore the signal level with AGC

before D/A conversion, quantization noise and gritty sound can

result.

Finally, AGC can improve audio quality for strong

signals.  Receiver AGC overshoot may force you to back off input

gain to accomodate signal dynamics.  Less-than-full-scale input

levels then propagate to the output and expose D/A quantization

noise above a quiet, strong-signal background.  DSP Blaster uses

a special AGC algorithm that maximizes output dynamic range.

Enable it to minimize strong-signal quantization noise.

You can set the maximum gain enhancement to 20, 40, or

60 dB.  Use the 20-dB setting for mild quantization-noise

suppression or level stabilization of moderately strong signals.

Use the other settings for progressively higher gain

restoration.  DSP Blaster employs hang-AGC to minimize voice

distortion from syllabic waveform compression and to minimize

noise pumping between CW characters.  The AGC speed affects both

hang duration and the gain-recovery time constant.  Set the

speed according to your personal preference.

The purpose of the squelch is to suppress background

noise while you park your receiver on a calling channel awaiting

activity.  The squelch algorithm level-detects the signal just

prior to AGC.  To better detect weak signals, engage NR with low

background and high order.  (You can change settings once a

signal has been detected.)  Click Squelch to enable or disable.

Set the squelch threshold and speed in the AGC menu.  The slower

the speed, the better the squelch can differentiate weak signals

from noise fluctuations.  The squelch isn't intended to suppress

10

noise between syllables or CW characters, nor is it designed to

unsquelch rapidly.

Dither is a small amount of white noise added to the

output to break up funny, low-level sounds an 8-bit D/A may

nonlinearly amplify when only the least-significant bit is

changing.  0 is no dither; 1 through 9 add increasing amounts.

Tune in a strong signal and adjust dither to the lowest setting

that eliminates funny sounds between syllables or CW characters.

For a silent, strong-signal background, select -1 to invoke a

1-LSB D/A deadband.  -1 should not be used for weak signals.

Dither is not needed for 16-bit CT417x cards and does not appear

in the AGC menu.

---- RECOMMENDED SETTINGS --------------------------------------

For general SSB tuning, engage the NR filter with normal

quieting, background, and order.  For noisy signals, switch to

high quieting.  For very noisy signals, try high order.  AGC

usually isn't needed.  Engage the HPF, LPF, and notch filter as

needed.  Use the notch filter while monitoring a net.

For CW bandscanning, use CW mode with no filters

selected (the sound-card antialias filter provides a 1.6-kHz

LPF).  For narrow-passband reception, try this very effective

combination:  500-Hz BPF centered at your preferred CW pitch, NR

filter with low background and high order, and slow AGC with 40-

dB maximum gain.  For very noisy, QRS signals, try huge order.

Whenever you can focus your attention on just one signal, engage

the CBPF with 225-Hz bandwidth and stereo output to provide

noise shaping and spatial separation of signal and noise.

---- COPYRIGHT AND LICENSE -------------------------------------

This software is copyrighted.  It's licensed, not sold.

When you purchase a license for a software product, you're

granted the right to use the software under certain conditions;

ownership of the software remains with the copyright holder.

Licensing software is different than purchasing a tangible

object that you may use or dispose of as you wish.

This software is licensed to individuals at discount for

amateur use.  It has been provided to you on the condition that

you abide by the following:

1.  You will not use the software for professional, business,

government, military, or institutional purposes.

2.  You will not sell, rent, lend, give away, or otherwise

transfer your DB.LIC license file to others.

11

Brian Beezley, K6STI       (760) 599-4962

3532 Linda Vista Dr.        0700-1800 PT

San Marcos, CA 92069        k6sti@n2.net

12