S I M U L

v1.00

Ethan Brodsky

12/27/1995

SIMUL - Simultaneous recording and playback on the SB16

Copyright 1995 by Ethan Brodsky.  All right reserved.

***********************************************************************

A recurring topic in sound programming newsgroups is the possibility of

simultaneous recording (A/D) and playback (D/A) on Sound Blaster sound

cards.

Early in 1995, I released a package called SBProcess, which was supposed

to do this by using polled "direct" sampling.  Unfortunately, this

method only worked on certain SB models, mostly older cards, and the

sound quality was horrible.  The method only supported 8-bit mono sound,

and the highest sampling rate I could reach was about 10khz.

After the failure of that method, I realized that the SB16 supported two

DMA channels, and could possibly be used for simultaneous sound.  The

SB16 has separate command for 8-bit and 16-bit sampling, as well as for

recording and playback.  I had been recommending this method to all who

asked, but I hadn't tried it out for myself.  (Although a package using

this method has since been released, source code to it is not freely

available)

Now that I have some free time, I decided to try it out for myself.  The

result is a Borland C real-mode module supporting 16-bit input and 8-bit

output simultaneously.  I have also written an example program using the

module.  It implements an "echo", outputting whatever it hears, with a

delay.  The code only supports the SB16, but it will work all the way up

to 44khz, and the method can also be used for stereo sound.  The only

limitation is that the two types of I/O be done using different DMA

channels.  I have found no way to solve this problem.

The code may be freely distributed in its original form as long as the

copyright notice and documentation are included, preferably contained in

the original ZIP file.  The code or method explained in it may be used

for any purpose. Please give credit to Ethan Brodsky.  I would also

appreciate it if you would tell me how you are using it.  If it is being

used for commercial purposes, I would appreciate a contribution.

The demo program is configured for my SB16, at 220h/5/1/5. If you have

a different configuration, you must change the constants in the DELAY.C.

I had to use some conditional compiliation directives to get the SIMUL

module to compile in all memory models, but I was unable to get the

DELAY demo program to work in anything but the large model.

I haven't written this code to handle all errors, but it should work on

any sound card that supports this method.  I'm not sure whether the

interrupt status register is supported on all compatibles, so the code

may have to be modified for them.  The newest version will always be

available from my WWW page, as well as the simtel and x2ftp public FTP

sites.

I hope you find this useful,

Ethan Brodsky

Address:  Ethan Brodsky

4010 Cherokee Dr.

Madison, WI

53711

Phone:    (608) 238-4830

email:    ebrodsky@pobox.com

ericbrodsky@psl.wisc.edu (Temporarily)

www:      http://www.pobox.com/~ebrodsky/

http://www.xraylith.wisc.edu/~ebrodsky/