IBM AT-286 Slots

----------------

62 pin slot

-------------------------------        --------------------------------

I  pin  I    signal    I  i/o  I       I  pin  I   signal    I   i/o   I

-------------------------------        --------------------------------

I-------I--------------I-------I       I-------I-------------I---------I

I  A1   I  -I/O CH CK  I   I   I       I  B1   I    GND      I    -    I

I  A2   I     SD7      I  I/O  I       I  B2   I  RESET DRV  I    O    I

I  A3   I     SD6      I  I/O  I       I  B3   I     +5V     I    -    I

I  A4   I     SD5      I  I/O  I       I  B4   I    IRQ9     I    I    I

I  A5   I     SD4      I  I/O  I       I  B5   I     -5V     I    -    I

I  A6   I     SD3      I  I/O  I       I  B6   I    IRQ2     I    I    I

I  A7   I     SD2      I  I/O  I       I  B7   I    -12V     I    -    I

I  A8   I     SD1      I  I/O  I       I  B8   I     OWS     I    I    I

I  A9   I     SD0      I  I/O  I       I  B9   I    +12V     I    -    I

I  A10  I  I/O CH RDY  I   I   I       I  B10  I     GND     I    -    I

I  A11  I     AEN      I   O   I       I  B11  I   -SMEMW    I    O    I

I  A12  I     SA19     I  I/O  I       I  B12  I   -SMEMR    I    O    I

I  A13  I     SA18     I  I/O  I       I  B13  I    -IOW     I   I/O   I

I  A14  I     SA17     I  I/O  I       I  B14  I    -IOR     I   I/O   I

I  A15  I     SA16     I  I/O  I       I  B15  I   -DACK3    I    O    I

I  A16  I     SA15     I  I/O  I       I  B16  I     DRQ3    I    I    I

I  A17  I     SA14     I  I/O  I       I  B17  I   -DACK1    I    O    I

I  A18  I     SA13     I  I/O  I       I  B18  I     DRQ1    I    I    I

I  A19  I     SA12     I  I/O  I       I  B19  I  -Refresh   I   I/O   I

I  A20  I     SA11     I  I/O  I       I  B20  I     CLK     I    O    I

I  A21  I     SA10     I  I/O  I       I  B21  I     IRQ7    I    I    I

I  A22  I     SA9      I  I/O  I       I  B22  I     IRQ6    I    I    I

I  A23  I     SA8      I  I/O  I       I  B23  I     IRQ5    I    I    I

I  A24  I     SA7      I  I/O  I       I  B24  I     IRQ4    I    I    I

I  A25  I     SA6      I  I/O  I       I  B25  I     IRQ3    I    I    I

I  A26  I     SA5      I  I/O  I       I  B26  I   -DACK2    I    O    I

I  A27  I     SA4      I  I/O  I       I  B27  I     T/C     I    O    I

I  A28  I     SA3      I  I/O  I       I  B28  I     BALE    I    O    I

I  A29  I     SA2      I  I/O  I       I  B29  I      +5V    I    -    I

I  A30  I     SA1      I  I/O  I       I  B30  I      OSC    I    O    I

I  A31  I     SA0      I  I/O  I       I  B31  I      GND    I    -    I

--------------------------------       ---------------------------------			    IBM AT-286 slots

----------------

18-pins slot

Side A                                Side B

-------------------------------       ---------------------------------

I  pin  I   signal   I  i/o   I       I  pin  I    signal    I   i/o  I

-------------------------------       ---------------------------------

I  C1   I    SBHE    I  i/o   I       I  D1   I  -MEM CS16   I   i    I

I  C2   I    LA23    I  i/o   I       I  D2   I  -I/O CS16   I   i    I

I  C3   I    LA22    I  i/o   I       I  D3   I     IRQ10    I   i    I

I  C4   I    LA21    I  i/o   I       I  D4   I     IRQ11    I   i    I

I  C5   I    LA20    I  i/o   I       I  D5   I     IRQ12    I   i    I

I  C6   I    LA19    I  i/o   I       I  D6   I     IRQ15    I   i    I

I  C7   I    LA18    I  i/o   I       I  D7   I     IRQ14    I   i    I

I  C8   I    LA17    I  i/o   I       I  D8   I    -DACK0    I   o    I

I  C9   I   -MEMR    I  i/o   I       I  D9   I      DRQ0    I   i    I

I  C10  I   -MEMW    I  i/o   I       I  D10  I    -DACK5    I   o    I

I  C11  I    SD08    I  i/o   I       I  D11  I      DRQ5    I   i    I

I  C12  I    SD09    I  i/o   I       I  D12  I    -DACK6    I   o    I

I  C13  I    SD10    I  i/o   I       I  D13  I      DRQ6    I   i    I

I  C14  I    SD11    I  i/o   I       I  D14  I    -DACK7    I   o    I

I  C15  I    SD12    I  i/o   I       I  D15  I      DRQ7    I   i    I

I  C16  I    SD13    I  i/o   I       I  D16  I      +5 V    I   -    I

I  C17  I    SD14    I  i/o   I       I  D17  I   -MASTER    I   i    I

I  C18  I    SD15    I  i/o   I       I  D18  I       GND    I   -    I

-------------------------------       ---------------------------------

IBM AT-286 slots

----------------

Memory slot

---------------------------------        --------------------------------

I  PIN  I   signal    I   i/o   I        I  pin   I   signal    I  i/o  I

---------------------------------        --------------------------------

I  A1   I    MD0      I         I        I  B1    I     GND     I       I

I  A2   I    MD1      I         I        I  B2    I   MDPIN1    I       I

I  A3   I    MD2      I         I        I  B3    I     +5V     I       I

I  A4   I    MD3      I         I        I  B4    I   MDPOUT1   I       I

I  A5   I    NC       I         I        I  B5    I      NC     I       I

I  A6   I    MD4      I         I        I  B6    I   MDPIN0    I       I

I  A7   I    MD5      I         I        I  B7    I      NC     I       I

I  A8   I    MD6      I         I        I  B8    I   MDPOUT0   I       I

I  A9   I    MD7      I         I        I  B9    I      NC     I       I

I  A10  I    NC       I         I        I  B10   I     GND     I       I

I  A11  I    MD8      I         I        I  B11   I      NC     I       I

I  A12  I    MD9      I         I        I  B12   I     -WR     I       I

I  A13  I    MD10     I         I        I  B13   I    -RAS0    I       I

I  A14  I    MD11     I         I        I  B14   I    -RAS1    I       I

I  A15  I    NC       I         I        I  B15   I    -RAS2    I       I

I  A16  I    MD12     I         I        I  B16   I    -RAS3    I       I

I  A17  I    MD13     I         I        I  B17   I      NC     I       I

I  A18  I    MD14     I         I        I  B18   I      NC     I       I

I  A19  I    MD15     I         I        I  B19   I      NC     I       I

I  A20  I    NC       I         I        I  B20   I   -CAS0L    I       I

I  A21  I    MA0      I         I        I  B21   I   -CAS0H    I       I

I  A22  I    MA1      I         I        I  B22   I   -CAS1L    I       I

I  A23  I    MA2      I         I        I  B23   I   -CAS1H    I       I

I  A24  I    MA3      I         I        I  B24   I   -CAS2L    I       I

I  A25  I    MA4      I         I        I  B25   I   -CAS2H    I       I

I  A26  I    NC       I         I        I  B26   I   -CAS3L    I       I

I  A27  I    MA5      I         I        I  B27   I   -CAS3H    I       I

I  A28  I    MA6      I         I        I  B28   I      NC     I       I

I  A29  I    MA7      I         I        I  B29   I     +5V     I       I

I  A30  I    MA8      I         I        I  B30   I      NC     I       I

I  A31  I    MA9      I         I        I  B31   I     GND     I       I

---------------------------------        --------------------------------			   I/O signal descriptions

-----------------------

The I/O channel signal description  is  given below. All signal lines

are TTL-compatible. I/O adapters  should  be designed with a maximum  of

two low-power Shottky (LS) loads per line.

*   SA0 through SA19 (I/O):  These are the  addres bits 0 through 19  used

to addres memory and I/O devices. In  addition  to  these  addres  bits,

LA17 through LA23 allows acces up to 16 Mb of memory. SA0  through  SA19

are gated on system  bus  when 'BALE' is high are latched on the falling

edge of 'BALE'.

*   LA17 through LA23:  These signals (unlatched) are used to addres memo-

ry and I/O devices within the system. They  provide  the system with  up

to 16 Mb of addressability. These signals are valid when 'BALE' is high.

They are not latched during  microprocessor cycle and  therefore  do  no

stay valid for the whole cycle. The purpose  of these  signals is to ge-

nerate memory decode for   1 wait  state memory  cycles.  These  decodes

should be latched by I/O.

*   CLK (0):  This is the 6-Mhz system clock. The clock is  a  synchronous

microprocessor  cycle time of 167 nanoseconds. The clock  duty  cycle is

50%. It should be used for synchronization purpose only.

*   RESET DRV (O): This signal is used to reset or initialise  system  lo-

gic at power-up time or during a low line-voltage outage. It  is a acti-

ve high signal.

*   SD0 through SD15 (I/O):  These are the data bits for the  microproces-

sor, memory, and I/O devices. SD0 is the least-significant  bit and SD15

is the most-significant bit. All 8-bit devices on the I/O channal should

use SD0 - SD7 communications to the microprocessor. For  16-bit devices,

the SD0 - SD15 signals are used. To suppot 8-bit devices, the data SD8 -

SD15 will be gated to SD0 - SD7 during 8-bit transfers to these devices:

16-bit microprocessor  transfers to  8-bit devices  will be converted to

two 8-bit transfers.

*   BALE (0)(buffered): The BALE (Buffered Addres Latch Enabled) is provi-

ded by the 82288 Bus Controler and is used to latch valid addresses  and

memory decodes from the microprocessor. When used with  AEN  signal,  it

provides an indicator of valid microprocessor or DMA addres. Microproce-

ssor addresses SA0 - SA19 are latched on the  falling  edge  of  'BALE'.

This signal is forsed high during DMA cycles.

*   -I/O CH CK (I):  the '-I/O Channel Check' provides the system with pa-

rity(error) information about memory or devices on the I/O channel. When

it is active (low), it indicates an uncorrectable system error.

*   I/O CH RDY (I):  The 'I/O Channel Ready'is used to lengthen I/O or me-

mory cycle. Any slow devices should drive  this signal  low  immediately

upon detecting its valid address and a Real number of clock cycles  (167

nsec.). This signal should be held low for no more than 2.5 microsec.

*   IRQ3 - IRQ7, IRQ9 - IRQ12 and IRQ14, IRQ15 (I):   These interrupts si-

gnals are used to signal the microprocessor that an I/O needed attention.

The interrupt request are prioritized, with IRQ9 through IRQ12 and IRQ14

through IRQ15 having the  highest  priority  (IRQ9  is the highest)  and

IRQ3 through IRQ7 having the lowest priority  (IRQ7  is the lowest).  An

interrupt is activated when an  IRQ request line is raised from  low  to

high. It must remained  high until the  microprocessor  acknowledges the

interrupr request (interrupt service routine).

*   -IOR (I/O):  The '-I/O Read' requests an I/O device to put  data  onto

the data bus. It is  an  active low signal and may be driven by a micro-

processor or DMA controller resident on the I/O channel.

*   -IOW (I/O):  The '-I/O Write' requests an I/O device to read data from

the data bus. It is  an active low  signal and may be driven by a micro-

processor or DMA controller in the system.

*   -SMEMR(O), -MEMR(I/O):  These signals requests the memory  devices  to

drive data onto the data bus. '-SMEMR'  is active  only  when the memory

decode is within the low 1 Mb of memory space and  '-MEMR'is  active  on

all memory read cycles. It may  be driven by any  microprocessor or  DMA

controller in a system.'-SMEMR' is derived from  '-MEMR' and the  decode

of the low 1Mb of memory. When a microprocessor on the I/O channel requ-

ests to drive '-MEMR', the address lines must first be valid on the  bus

for one clock period before driving '-MEMR' active. Both signals are ac-

tive low.

*   -SMEMW(O), -MEMW(O):  These signals request the memory device to store

the data present on the data bus.  '-SMEMW' is activated  only when  the

memory decode is within the low 1Mb of memory space.'-MEMW' is activated

on all memory read cycles. When a microprocessor on the I/O channel  wi-

shes to drive '-MEMW', the address lines must  be valid on the  bus  for

one clock period before driving '-MEMW' active. Both signals are  active

low.

*   DRQ0-DRQ3 and DRQ5-DRQ7 (I):  DMA Requests 0 through  3 and  5 through

7 are available for the peripheral devices and the I/O channel micropro-

cessor to gain DMA service ( or control of  the system ). These  signals

are prioritized ('DRQ0' has the highest priority and  'DRQ7' having  the

lowest). A request is generated by bringing a DRQ line to an active high

state. It must be held high until the corresponding 'DMA Request Acknow-

ledge'(DACK) line goes active. 'DRQ0' through 'DRQ3' perform  8-bit  DMA

transfers while the request lines 'DRQ5' through  'DRQ7' perform  16-bit

transfers.

*   -DACK0 to -DACK3 and -DACK5 to -DACK7 (O):   They are used to  acknow-

ledge DMA requests (DRQ0 through DRQ7). They are active low signals.

*   AEN (O):  'Address Enable'is provided to degate the microprocessor and

the other  devices from the  I/O  channel to allow DMA transfers to take

place. When this line is active high, the DMA controller has control  of

the address bus, the data bus read command lines (memory and  I/O),  and

the write command lines (memory and I/O).

*   -REFRESH (I/O):  This signal is used to indicate  a refresh cycle  and

can be driven by a microprocessor on the I/O channel.

*   T/C (O):  'Terminal Count' provides a pulse when theterminal count for

any DMA channel is reached.

*   SBHE (I/O):  'Bus High Enable' indicates a transfer of data on the up-

per byte of the data bus, SD8 through SD15. It is used to condition data

bus buffers tied to SD8 through SD15 for 16-bit transfer.

*  -MASTER (I):  This signal is used in condition with a DRQ line to  gain

control of the system. A processor or DMA controller on the  I/O channel

may issue a DRQ to a DMA channel in a cascade mode and receive a '-DACK'

An I/O microprocessor upon receiving the '-DACK', may then pull '-MASTER'

low. This will allow it to control the system address, data, and control

lines (a condition commonly known as tri-state). After '-MASTER' is low,

the I/O microprocessor must wait for one clock period before  issuing  a

Read or Write command. This signal should held low for more than 15 mic-

roseconds otherwise system memory may be lost due to lack of refresh.

*   -MEM CS16 (I):   'MEM 16 Shp Select' indicates that the data  transfer

is a 16-bit 1-wait state, I/O cycle. It is derived from an address deco-

de. It is an active low signal and should be driven with an open collec-

tor or tri-state driver capable of sinking 20 mA.

*   OSC (O):     'Oscillator' is a high speed clock with  a 70- nanosecond

period ( 14. 31818 MHz ). This signal is not synchronous with the system

clock. It has a duty cycle of 50%.

*   OWS (I):   The 'Zero wait state' indicates to the microprocessor  that

it can complete the present bus cycle without inserting  any  additional

wait cycles. In order to run a memory cycle to a  16-bit device  without

wait cycles, this signal (OWS) is delivered from an address decode gated

with a Read or Write command. For an 8-bit device with a minimum  of two

wait states, 'OWS' should be driven  active one  system clock  after the

Read and Write command is active gated with the  address decode for  the

device. Memory Read and Write commands to an 8-bit device are active  on

the falling edge of the system clock. 'OWS' is active low and  should be

driven with an open collector or tri state driver capable of sinking  20

mA.

System I/O Address Map

______________________

----------------------

Port                           Address

----------------------------------------

DMA #1                            000

Interrupt Controller #1           020

Timer                             040

Keyboard                          060

Control Register                  061

Clock Calndar, NMI Enable         070

DMA Page Registers                080

Interrupt Controller #            0A0

DMA #2                            0C0

Clear 80287 Busy                  0F0

Reset 80287                       0F1

80287                             0F8

Parallel                          378

MEMORY MAP

------------

____________

640K+384K Memory Map

Address    Mode/Name        Function

-------------------------------------------------

000000     512K User       System  Board Memory

07FFFF     RAM             Bank 0

080000     128K User       System Board Memory

09FFFF     RAM             Bank 1

0A0000     128K Video      Reserved for Graphics Display

0BFFFF     RAM             Display Buffer

0C0000     128K I/O        Reserved for ROM On

0DFFFF     Expansion ROM   I/O Adapters

0E0000     96K Reserved    User ROM

0E7FFF     On System Board

0F8000     32K ROM On      System BIOS ROM

0FFFFF     System Board

100000     384K User       System Board Memory

15FFFF     RAM             Bank 1

160000     14848K User     I/O Channel Memory

FDFFFF     RAM

FE0000     64K Reserved    Duplicate Code Assigntment

FEFFFF     System Board    at Address 0E0000

FF0000     64K ROM On      Duplicate Code Assigntment

FFFFFF     System Board    at Address 0F0000

-------------------------------------------------------

-------------------------------------------------------

512K+512K Memory Map

Address    Mode/Name        Function

-------------------------------------------------

000000     512K User       System  Board Memory

07FFFF     RAM             Bank 0

080000     128K User       I/O Channel

09FFFF     RAM

0A0000     128K Video      Reserved for Graphics Display

0BFFFF     RAM             Display Buffer

0C0000     128K I/O        Reserved for ROM On

0DFFFF     System Board    I/O Adapters

0E0000     96K Reserved    User ROM

0E7FFF     On System Board

0F8000     32K ROM On      System BIOS ROM

0FFFFF     System Board

100000     512K User       System Board Memory

17FFFF     RAM             Bank 1

180000     14720K User     I/O Channel Memory

FDFFFF     RAM

FE0000     64K Reserved    Duplicate Code Assigntment

FEFFFF     System Board    at Address 0E0000

FF0000     64K ROM On      Duplicate Code Assigntment

FFFFFF     System Board    at Address 0F0000

---------------------------------------------------------

---------------------------------------------------------

I/O ADDRESS MAP

---------------

_______________

Address (hex)         Device

---------------------------------------------------------

000 - 01F          DMA Controller 1, 8337A-5

020 - 03F          Interrupt Controller 1, 8259A, Master

040 - 05F          Timer, 8254

060 - 06F          Keyboard Controller, 8042

070 - 07F          Real time clock, NMI

(non-maskable interrupt) mask

080 - 09F          DMA Page Registers 74LS612

0A0 - 0BF          Interrupt Controller 2, 8250A

0C0 - 0DF          DMA Controller 2, 8237A-5

0F0                Clear Math Co-processor 80287 Busy

0F1                Reset Math Co-processor 80287

0F8 - 0FF          Math Co-processor 80287

1F0 - 1F8          Fixed Disk

200 - 207          Game I/O

278 - 27F          Parallel Printer Port 2

2F8 - 2FF          Serial Port 2

300 - 31F          Prototype Card

360 - 36F          Reserved

378 - 37F          Parallel Printer Port 1

380 - 38F          SDLC, bisynchronous 2

3A0 - 3AF          Bisynchronous 1

3B0 - 3BF          Monochrome Display and Printer Adapter

3C0 - 3CF          Reserved

3D0 - 3DF          Color Graphics Monito Adapter

3F0 - 3F7          Diskette Controller

3F8 - 3FF          Serial Port 1

---------------------------------------------------------

=========================================================

DMA Channels

============

Channel            Function

--------------------------------------------

0              Spare (8-bit transfer)

1              SDLC  (8-bit transfer)

2              Floppy Disk (8-bit transfer)

3              Spare (8-bit transfer)

4              Cascade for DMA Controller 1

5              Spare (16-bit transfer)

6              Spare (16-bit transfer)

7              Spare (16-bit transfer)

========================================================

Page Register            I/O Hex Address

--------------------------------------------

DMA Channel 0                  0087

DMA Channel 1                  0083

DMA Channel 2                  0081

DMA Channel 3                  0082

DMA Channel 4               (not shown)

DMA Channel 5                  008B

DMA Channel 6                  0089

DMA Channel 7                  008A

Refresh                        008F

========================================================

Hex Address           Command Code

-----------------------------------------------------

0C0               CH0 base and current address

0C2               CH0 base and current word count

0C4               CH1 base and current address

0C6               CH1 base and current word count

0C8               CH2 base and current address

0CA               CH2 base and current word count

0CC               CH3 base and current address

0CE               CH3 base and current word count

0D0               Read status register

Write command register

0D2               Write request register

0D4               Write single mask register bit

0D6               Write mode register

0D8               Clear byte pointer flip-flop

0DA               Read temporary register/

Write master clear

0DC               Clear mask register

0DE               Write all mask register bits

============================================================

============================================================

Interrupts

----------

Level            Function

---------------------------------------------

0           System Timer Output 0

1           Keyboard Output Buffer Full

2           Interrupt from Controller 2(levels 8-15)

3           Serial Port 2

4           Serial Port 1

5           Parallel Port 2

6           Diskette Controller

7           Parallel Port 1

8           Real Time Clock

9           Software redirected to INT 0AH

10          Reserved

11          Reserved

12          Reserved

13          80287

14          Hard disk Controller

15          Reserved

=========================================================

Timers.

-------

Channel          Function

---------------------------------------

0             System Timer Output 0

1             Memory Refresh

2             Speaker Tone

==========================================================

Real Time Clock And CMOS RAM

-----------------------------

=============================

A CMOS RAM chip (Motorola MC146818 AP), which keeps configuration

information when the power is out, contains the real time clock and

64K of CMOS RAM. The internal clock circuirtry uses 14K for this RAM

and the rest is allocated to configuration information.

CMOS RAM Address Map

--------------------

Address           Description

00-0D            Real Time Clock Information

0E               Diagnostic status byte

0f               Shutdoun status byte

10               Diskette drive type byte

-drives A and B

11               Reserved

12               Fixed disk type byte

-drives C and D

13               Reserved

14               Equipment byte

15               Low base memory byte

16               High base memory byte

17               Low expansion memory byte

18               High expansion memory byte

19-2D            Reserved

2E-2F            2-byte CMOS checksum

30               *Low expansion memory byte

31               *High expansion memory byte

32               *Date century byte

33               Information flags (set during power-on)

34-3F            Reserved

Real Time Clock Information

---------------------------

Byte           Function           Address

0          Seconds                 00

1          Seconds alarm           01

2          Minutes                 02

3          Minute alarm            03

4          Hours                   04

5          Hour alarm              05

6          Day of week             06

7          Data of month           07

8          Month                   08

9          Year                    09

10          Status Register A       0A

11          Status Register B       0B

12          Status Register C       0C

13          Status Register D       0D

Floppy signal interface

=======================

----------------------------------------- Terminal -----

I      Signal     I   Direction    I   Signals  I   0V   I

I -------------------------------------------------------

I  Spare          I     input      I     2      I   1    I

I  In Use         I     input      I     4      I   3    I

I  Drive select 3 I     input      I     6      I   5    I

I  Index/Sector   I     output     I     8      I   7    I

I  Drive select 0 I     input      I    10      I   9    I

I  Drive select 1 I     input      I    12      I  11    I

I  Drive select 2 I     input      I    14      I  13    I

I  Motor On       I     input      I    16      I  15    I

I Direction selectI    input       I    18      I  17    I

I  Step           I     input      I    20      I  19    I

I  Write data     I     input      I    22      I  21    I

I  Write Gate     I     input      I    24      I  23    I

I  Track 00       I     output     I    26      I  25    I

I  Write Protect  I     output     I    28      I  27    I

I  Read Data      I     output     I    30      I  29    I

I  Side one selectI     input      I    32      I  31    I

I  Ready          I     output     I    34      I  33    I

I________________________________________________________I

Input signal

LOW level (true)

Output signal

LOW level (true)

Power connector

================

/------------\              1 -  DC +12 V

I *  *  *  * I              2 -  GND

-------------              3 -  GND

4  3  2  1                4 -  DC +5 V

--------------------------------------------			     IBM AT-286

power supply connector

----------------------

--------------------------------------------------------------------

Pin   Description   Connector   I     Pin   Description   Connector

I

1    Power good       PS1      I      1        GND          PS2

2    +5 V  DC         PS1      I      2        GND          PS2

3    +12V  DC         PS1      I      3      -5 V DC        PS2

4    -12V  DC         PS1      I      4      +5 V DC        PS2

5       GND           PS1      I      5      +5 V DC        PS2

6       GND           PS1      I      6      +5 V DC        PS2

--------------------------------------------------------------------