By:  Hansang Bae.       Last Modified: 07 Sept, 1995

This is a rough draft.  If you have any suggestions or comments, please

email me at hbae@primenet.com.  Thanks.

I wrote this primer of sorts in hopes that it will help out network

admins new to TCP/IP.  New to the concept of subnetting, anyway.

Subnetting a Class B address at the byte boundry is pretty simple.  So

I'll concentrate on subnetting a Class C address.  Please do not use

these addresses, since they really do exist!   Thanks to steved of IBM who

allowed me to use his IP address as an example.  Again, PLEASE DO NOT USE

HIS ADDRESS!!!  OK, ONCE MORE, DO *NOT* USE THESE ADDRESSES IF YOU ARE

CONNECTED TO THE INTERNET.

Also, here are some recommended readings.

1.  "Internetworking with TCP/IP Vol. I" By: Douglas E. Comer.

Publisher:  Pretice Hall

IMHO, it's the definitive work on TCP/IP.  HIGHLY recommended.

2.  "TCP/IP Network Administration" By: Craig Hunt.

Publisher:  O'Reilly & Associates, Inc.

Part of the Nutshell series.  Practical and a MUST-HAVE for any Unix

network admins.

3.  RFC 950 and 917.  Ftp to nic.ddn.mil /rfc/rfcxxx.txt .  RFC 950 is the

RFC that I allude to  in the following examples.

4.  RFC 1597.  Talks about using IP numbers for private networks.  I'll

eventually switch this doc over to it.

A user posted this to the net.  I'll use it as my example:

I'm trying to split it up into two different subnets.

bind ip to ip1 addr=204.32.24.13  mask=255.255.255.128

bind ip to ip2 addr=204.32.24.243 mask=255.255.255.128

It then complains that the subnet is all zeros or ones.

Couple of observations.

1.  Address in the range of 192-223 (first octet) is a class C address.

That is, addresses in 192.x.x.x to 223.x.x.x.

2.  Class-C address have a network mask of 255.255.255.0 (by default)

3.  When working with SUBnetwork masks, use binary numbers.

4.  255 in decimal is 1111 1111 in binary.

5.  You cannot have all 0's or all 1's subnetwork. Per RFC 950

6.  IP address are composed of 4 octets (i.e. 4 bytes).

Netmask means that where ever you have a 1, disregard it since it

belongs to the network portion.  The rest makes up your host portion.

What I mean is that if your network mask is 255.255.255.0, then your

software will automatically treat the first three bytes as the network

address.  Only the last byte will be treated as the host's address.

That's the purpose of a network mask.  To separate the network portion

of the IP address from the host portion.  This is a key point.

So your IP address is really  [NETWORK_PORTION][HOST_PORTION]  When

you borrow bits from the host portion, you are - as people commonly

call it - subnetting.  So once you subnet, your address becomes

[NETWORK_PORTION][SUBNETWORK_PORTION][HOST_PORTION].  Just by looking at

an IP number, you will not be albe to tell whether it's subnetted or

not.  The ONLY way that you can tell is by looking at the subnetwork

mask.  If you have a class B address, and the network mask is

255.255.0.0, then you know it's not subnetted.  Why?  Because the

default network mask of a Class B address is 255.255.0.0.  If it

has a subnetwork mask of 255.255.255.0 (for the same class B

address), then you know that it has been subnetted.  i.e the third byte

no longer belongs to the host portion.  It now belongs to the network

portion.  More specifically, it's the SUBNETWORK portion.

The RFCs state that you CANNOT have  an all zero or all ones SUBNET.

But the first three octets of the subnetwork mask are all ones,

you say! i.e. (in decimal) 255.255.255.0 for a class C address.

But they are NOT subnetworks.  These are the network MASKS.  By

definition, a class C address has three octets worth of network

portion (hence the network mask of 255.255.255.0).  At this point

you are not SUBnetting.  When you STEAL bits from the host portion,

you are subnetting.  It's easy to get confused about not being able

to have all one's or all zero SUBnet when your NETMASK is comprised

of all ones! But remember, your NETMASK is different then your

SUBNETwork.  To make it more clear, here are some examples.

So, inorder to subnet, you have to have a minimum of two bits for your

subnet MASK. i.e.

11111111.11111111.11111111.11 000000  dec=255.255.255.192

The above two values are subnet MASKS. (and are equivalent.  Ones in

decimal and the other in binary)

This subnet mask allows you to have the following SUBNETWORKS:

1)   11001100.00100000.00011000.00 000000  (204.32.24.0)

2)   11001100.00100000.00011000.01 000000  (204.32.24.64)

3)   11001100.00100000.00011000.10 000000  (204.32.24.128)

4)   11001100.00100000.00011000.11 000000  (204.32.24.192)

^^^^^^--- Host portion

^^---------- Subnetwork portion.

^^^^^^^^^^^^^^^^^^^^^^^^^^------------- Network portion.

So following the RFCs, 1) and 4) are not usuable. All zero and all one's

subnetworks are not allowed, remember?  If this is not crystal clear,

then look at the Subnetwork portion.  Notice that #1's and #4's

subnetwork portions are all zeros and and ones, respectively.  This

means that we have TWO usable subnetworks.  Both will use the

subnetwork MASK of 255.255.255.192.  The mask is same for subnetworks

204.32.24.64 and 204.32.24.128.  So both subnetworks will use the

subnetwork MASK of 255.255.255.192.

SubnetMASK of 255.255.255.128  (what the orig poster used) is:

11111111.11111111.11111111.1 0000000 in binary.

Remember, the above is subnetwork MASKs.  _NOT_ SUBNETWORKS.

This allows you to have subnetworks of:

1)   11001100.00100000.00011000.0 0000000  (204.32.24.0)

2)   11001100.00100000.00011000.1 0000000  (204.32.24.128)

^^^^^^^--- Host portion.

^----------- Subnetwork portion.

^^^^^^^^^^^^^^^^^^^^^^^^^^------------- Network portion.

Both are not usuable because one subnetwork is comprised of a "0", and the

other is made up of single "1".  i.e. both subnetworks are all zeros and

all ones subnetwork, respectively.

Using 255.255.255.192 allows you to have TWO usuable subnets 2, and 3 in

the first example.  Since you have six bits in the host portion, that

means that you have a total of 64 possible choices.  Why 64?  Because you

have 6 bits to play with for the host portion.  So in binary, this is:

000000 through 111111 or 0 through 63 in decimal.  But one is used to denote

"This network" (000000) and another is used to denote "broadcast"(111111).

Therefore, you end up with 2 sets of 62 hosts.  ie. 2 subnetworks

with 62 hosts each (with two set aside for 'this network' and

'broadcast' in each of the two subnetworks.)

To break it down:

11001100.00100000.00011000.01 000000 == 204.32.24.64

^^^^^^-- host portion.

^^--------- subnetwork portion.

This denotes the 'current' network.  This is one of the special IP

addresses that is reserved.  You cannot use this for one of your hosts.

In other words, the the host portion is all zeros.  An all zero host

portion denotes 'this current network.'

11001100.00100000.00011000.01 000001 == 204.32.24.65

This is the FIRST usuable host address for the subnetwork of 204.32.24.64

11001100.00100000.00011000.01 111110 == 204.32.24.126

This is the LAST usuable host address for the subnetwork of 204.32.24.64

You can, of course, use any IP address in btn the two.

11001100.00100000.00011000.01 111111 == 204.32.24.127

This is the broadcast address for the subnetwork of 204.32.24.64  you

cannot use this as a host address.  It's the second of the special IP

addresses that is reserved.  This address denotes EVERY host on this

particular SUBNETWORK.

The above is the first set of the two usuable subnetworks.

Next SUBNETWORK is:

11001100.00100000.00011000.10 000000  == 204.32.24.128

This denotes the 'current' subnetwork.  Again, this is one of the special IP

addresses that is reserved.

11001100.00100000.00011000.10 000001 == 204.32.24.129

This is the FIRST usuable host address for the subnetwork of 204.32.24.128

11001100.00100000.00011000.10 111110 == 204.32.24.190

This is the LAST usuable host address for the subnetwork of 204.32.24.128

You can, of course, use any IP address in btn the two.

11001100.00100000.00011000.10 111111 == 204.32.24.191

This is the broadcast address for the subnetwork of 204.32.24.128.  you

cannot use this as a host address.  It's the second of the special IP

addressess that is reserved.  Again, it denotes every HOST on this

particular SUBNETWORK.

Here are some more examples (taken from TCP/IP Network Administration.)

IP             SUBNET MASK       Interpretation

A) 128.66.12.1      255.255.255.0      host 1 on subnetwork 128.66.12.0

(note: this is a class b address)

B) 130.97.16.132    255.255.255.192    host 4 on subnet 130.97.16.128

C) 192.178.16.66    255.255.255.192    host 2 on subnet 192.1778.16.64

D) 132.90.132.5     255.255.240.0      host 4.5 on subnet 132.90.128.0

E) 18.20.16.91      255.255.0.0        host 16.91 on subnet 18.20.0.0

Let's break each on down and look at it in detail.

Case A.

You can tell that this is a class B by looking at the first octet

(128.x.x.x)

Class B address have a NETWORK mask of 255.255.0.0.  This is not an

option.  It's required.

It's subnetted using 8 bits from the third octet.  Probably the most

common way of subnetting a Class B address.  This gives you 254 (1-254)

usuable subnetworks.  Again, subnetwork of 0 and 255 (all ones) are not

allowed.  Each subnetwork can have 254 hosts.  Host number 0 and 255

are not allowed.  It denotes 'this network' and 'every host'

respectively. i.e. 128.66.12.0 means subnetwork of 128.66.12.

128.66.12.255 means every host on subnetwork 128.66.12.0.

Becuase of the NETWORK maks of 255.255.0.0 and the SUBNETWORK mask of

0.0.255.0, your effective network mask is 255.255.255.0.  This means

that you ignore the first three octets since it belongs to the network

portion.  The last octet alone is used to determine the host.  Since

it is a "1" (deciaml, not binary), it means it's the FIRST host.

Again, there is no host "0"

Case B.

Again, it's a class B address, with network mask of 255.255.0.0

It's subnetted using ALL of the third octet (8 bits) and two BITS from

the fourth octet.  Subnet mask is 0.0.1111 1111.11 000000  or

0.0.255.192 in decimal.  The effective network mask

(network mask + subnetwork mask) is 255.255.255.192.

So our IP address of 130.97.16.132 in binary (last octet only) is:

130. 97. 16.10000100   This is the IP address.

255.255.255.11000000   This is the mask.

--------------------   Wherever there is a zero (remember

that 255=11111111) in the mask, allow

the number to filter down. So

***.***.***.**000100   Is the host number.

This is 4 in decimal notation.

So what is the subnetwork number?  Well, the subnetwork consists of all

three octets (130.97.16) and the first TWO bits from the fourth octet.

In this case, that's "10" in binary.  But remember that their place is

significant.  i.e. it's (1 x 2^8) + (0 x 2^7) = 128.  So your

subnetwork is: 130.97.16.128.

Case C.

This is a class C address. Network mask = 255.255.255.0

It's subnetted with the first two BITS of the fourth octet. i.e.

11000000 = 192 in decimal.

192.178. 16. 01000010   This is the IP address.

255.255.255. 11000000   This is the mask.

---------------------   Do as above (B) example.

***.***.***.**0000010   Is the host number.  This is

2 in decimal.

Subnet is 192.178.16.01xxxxxx  or more explicitely

192.178.16.(0x2^8)+(1x2^7) == 192.178.16.64.

The 'x" belong to the host.  It's only used as a place holder.

Case D.

This is a class B address.  Network mask = 255.255.0.0

It's subnetted using the first half of the third octet (4 bits).

132. 90.10000100.00000101  This is the IP address.

255.255.11110000.00000000  This is the mask.

-------------------------  Do as above (B) example

***.***.****0100.00000101  Is the host number.  This is

4.5 in decimal.  This is not

"four and a half!"  It's host

"four dot five."

The subnetwork number is:

132.90.1000****.******** or

132.90.(1x2^8)+(0x2^7)+(0x2^6)+(0x2^5)**** == 132.90.128

Notice that this is 'different' than the previous examples.  The host

has two portions to its ID (4.5).  Why?  Because the third octet is

part of the SUBNET *AND* the HOST portion.  Plus you have ALL of the

last octet as your HOST portion.  In the previous examples, ALL of the

third octet were being used as the SUBNETwork portion.  As a result,

only the LAST octet was used in the HOST portion.

--------------------------------------------------------------------

| Hansang Bae, CNE  |  Network Engineer,  Providence Medical Ctr, WA |

| hbae@primenet.com |  Voice: (206) 320-2278   Fax: (206) 320-2273   |

--------------------------------------------------------------------