HDP - Hard Disk Partition. The Partition Reader (C) 1997/8/9 Craig Hart. V1.10�

� Shareware notice

This program is released as freeware but the Copyright is retained by the

author. Do with it as you please! Distribute it to anyone you want to! It's

totally FREE, including commercial, military and government use!!

There is no warranty what so ever on this software, but all care has been taken

in it's programming to avoid mistakes. If you are unsure, use other methods to

verify the information and conclusions of HDP.

� What's new to this version?

HDP can now calculate the 'sectors in partition' and 'sectors before partition'

numbers and estimate if they are correct or not. See the rest of this doc for

more info.

� Hardware requirements

16k RAM

One or more Hard Disks with BIOS (INT 13h) support

XT or greater PC

In other words, "If it's a PC with a Hard Disk, HDP can handle it!"

HDP runs under any version of DOS or DOS-liks OS that provides a normal DOS

environment and access to BIOS int 13h, such as Win95, Win 3.x. It does NOT run

under Windows NT. Operation under OS/2 is unknown, however it IS known to

report the correct information on a HDD containing OS/2 or NT, when the PC in

question is booted from a DOS floppy.

� The Program

This is a very simple utility designed to assist you in finding out just "how

the hell" that mystery HDD was configured in it's previous life. In the past it

was often guesswork to find out what parameters an unknown HDD was setup with.

Even with your own HDD, if you loose your CMOS, do you know what parameters

your HDD is currently setup with? Would you be able to relibably find out?

HDP answers these problems by looking at the partition table of a drive and

working out what the correct geometry is to access the data the drive contains.

(Geometry is the term describing the heads, cylinders and sectors per track

numbers). HDP relies on the fact that certain critical values are stored in the

partition table during partitioning, and a little math and clever

interpretational work can give us the right numbers.

HDP can also help you find out what the CMOS type is for a PC without a

built-in CMOS setup program (Compaq, NEC, Epson, older AMI, Phoenix & Award

BIOS, etc), or a proprietary setup program you do not have. This is especially

useful when upgrading old machines and using the old HDD, or repairing a dead

PC and salvaging the data.

HDP is really for used drives, from which you wish to salvage data. HDP won't

do very much at all on a brand new "virgin" drive as the drive will never have

been partitioned! In this case, simply refer to manufacturers specs and put HDP

away for another day. HDP can not help you with unpartitioned drives.

Note: HDP is not really useful on drives not seen by the BIOS. This means that

HDP is not much help with SCSI LUN's >1, XT and ESDI HDD systems, in some

cases. SCSI is easy to setup; the CMOS type is ALWAYS "Not Installed". The

controller performs all operations onboard, with it's own onboard BIOS. XT's

don't have a CMOS, but if you insist on having an XT type HDC in a 286+, the

CMOS type is still "not installed"!!

HDP can still tell you what parameters a working drive is using, if you are

planning on switching controllers or verifying correct drive capacity on one of

those which has no parameters displayed, unlabelled jumpers type BIOS's. Note

that, generally, you cannot move a drive from one family of SCSI or XT

controller to another, different family, and still recover the data. SCSI is

generally more likely to work, and even with XT controllers it can be done. Try

to stick to the original family of controller where possible. IDE is the only

type of drive where you can generally switch controllers 'at random' and not

worry about data integrity.

There is one exception to the IDE rule, and that's IDE Controllers which have

an onboard BIOS, and you're using that BIOS. Examples of this card are the SIDE

ProJr and the Promise DC-4030VL. If it has a BIOS, and you're using that BIOS,

then swapping controllers for a non-identical BIOS is probably asking for

trouble.

ESDI's come in 2 flavours - BIOS'd and non-BIOS'd. For non-BIOS'd cards, treat

as per IDE.

With BIOS'd ESDI controllers, the controller usually doesn't allow DOS-level

access to the drive until you already have the right numbers in the CMOS, but

you can usually start with type 1, which is usually correct for most ESDI

drives. HDP will show you the selected translation mode for ESDI's if you are

considering reformatting or wondering if sector sparing format mode was used or

not (look at the sectors per track value - if it's one less that the number of

sectors per track the drive supports, the drive was formatted with sector

sparing.) It is not recommended to change models or brands of ESDI controller

when wishing to recover data - nearly every brand and model of controller has a

different and incompatible format of storing data & error information, sector

spacings etc on the disk surface.

For IDE, MFM, RLL, and other types using the normal M/B BIOS/CMOS, HDP will

always work for you. HDP works through the BIOS, so if you can access the drive

with DOS, HDP can report on it.

Many of you, by now, will be saying "But I have the numbers on my HDD's lid, in

TheRef(tm), written down, etc etc". The sorry fact is that just because you

know the manufacturer's RECOMMENDED translation, you don't necessarily know the

PHYSICAL VALUES THAT WERE ACTUALLY USED. You can reduce ANY of the three

parameters to ANY other lesser value and STILL get just about any HDD

working... for example a 540Mb HDD may have been setup as an LBA device, or as

a normal device on a non-LBA BIOS with a lower cylinder value... how do you

know? An MFM or RLL drive can have any number of cylinders from about 40 to

it's manufacturing maximum (and sometimes a few more; the classic example being

a Miniscribe 3650 which has a factory of 812 Cylinders but is usually formatted

out to 820 cylinders so that it fits the "standard" BIOS type 40 entry -

820x6x17). The person installing the drive may have guessed the values, made a

typo, gone on the wrong info etc etc. The values on the lid of the drive won't

tell you. HDP WILL.

So what, you say? The drive still works with "any" values that are pretty

close...it even boots up properly. Please read on, as this is a common

misconception leading to data loss and, possibly, damaged drives.

As a bonus, HDP also tells you about the actual values stored in the partition

table, shows the current bootable partition, and what operating system is

installed on each partition (and HDP knows plenty of OS's - over 65), and can

handle "virgin" drives with no partition table, and even read a corrupted

table - something FDISK won't do.

� Using HDP

1. Install the drive in hardware as per normal. Set the CMOS type for this HDD

to type 1, or any other type that allows boot-up without a HDD related boot

error message.

2. Boot from a DOS floppy, main HDD etc. DO NOT allow ANY disk writes to the

drives(s) under question! I STRONGLY recommend the drive in question is NOT

used as the boot device, since you may not be able to interrupt the boot

sequence before a disk-write occurrs (for example pre-dos 6 systems).

3. Run HDP. Supply a value from 0 to 7 indicating which PHYSICAL HDD you wish

to check out (remember, physical drive - on a one HDD system with 4 partitions

C: D: E: & F:, HDP's only valid parameter is 0). Enter HDP ? for a summary of

commands and copyright notice. Up to 8 physical HDD's are accessible with HDP -

enough to cover just about any combination of controllers and drives.

4. Observe HDP's output.. the last line of text will tell you what HDP has

determined are the right parameters for this drive. Reboot, re-enter the

appropriate setup (CMOS, card-BIOS, etc) and enter the values HDP recommends.

For IDE Drives: Turn on translation (Usually LBA) if HDP recommends it. If your

M/B doesn't support translation and HDP recommends it, you cannot access the

data on that HDD on that M/B. This is a BIOS limitation and HDP has helped you

find it out (sorry, you can't get around this one - you must use a translation

capable BIOS in this case).

Please note: if HDP is run on a drive that really does have 'garbage' stored in

it's partition table, HDP may make some really 'wierd' suggestions. If the

numbers HDP displays are 'way out' you can probably assume the partiton table

has been trashed. The usual dead give-away of a trashed partition table is an

invalid signature combined with insane figures for the partitions -  things

like cylinder, sector and head numbers that are outside the normal range

(0-1024, 1-63, 0-255) are dead give aways.

At this point, you may wish to 'write off' the data and start afresh. A corrupt

partition table is often (but not always) a good general sign that the rest of

the data is corrupted too. If you _still_ want to recover the data, you will

need to re-build the partition table by hand (or restore it from a backup -

many anti-virus software packages can do this when you make/use am 'emergency'

rescue floppy disk created for the system when it was in a working state) - but

discussion of taht topic is outside the scope of HDP. If you nee to know how,

email me and I'll try to write a doc on it some time.

� Possible Error Messages:

Partition Invalid!

The partition table has invalid signature bytes that are required to

indicate that the data contained therein is correct. The MBR code will

not try to boot from any partition if these bytes are incorrect.

Possible virgin drive!

Usually in conjunction with Partition Invalid. HDP senses NO valid

partitions defined on this drive. Drive is probably a virgin.

(Ie never configured).

Failed to read partition or BIOS Settings!

This is a low-level error, basically the partition table could not be

retrieved from the disk. Try another CMOS type to try and get the

drive to respond. User type 10x2x17 is OK, or type 1,2 or 17 are good

starting points. This may mean the disk is physically faulty.

� Why you need to use the right parameters!

HDP was written because it is NOT OK to just use any more-or-less randomly

selected parameters to access data on a drive. A drive may appear to work OK at

first, but it is a recipe for disaster to try and use a drive driven by the

wrong parameters. CHKDSK, FDISK, SCANDISK, NDD etc etc are sure to have a

hernia coping with a HDD that's incorrectly defined. You may even end up

corrupting the FAT, directories, data or even the partition table itself once

disk writes begin.

Only by using the same parameters that the drive was originally setup with

(including LBA and/or translation on or off) can you be assured of absolute

data integrity. This is especially important in LBA situations as the geometry

is converted into a single number by a relatively complex mathematical sequence

for actual access by the drive - so even a cylinders count one less than

correct can cause severe data loss. An inaccuracy of 1 in the head or SPT

numbers can cause a disk access many megabytes away from the 'proper' location.

HDP isn't that much use if you just want to wipe the drive and start over (in

that case, use the manufacturer's recommended settings) but for accessing data,

HDP is essential!

� Just what IS translation, anyhow?

Translation is, basically, the mathematical conversion of the CHS numbers into

another format (usually by the system BIOS); the re-formatted numbers are used

to physicly address the drive. A reverse translation occurs also for

drive-to-software responses.

LBA is a form of translation, but many BIOS's also offer other forms of

translation. Under LBA, the CHS value is mathematically transformed into a

32-bit number, and that is loaded into the drive directly. The drive is told

that the BIOS is working in LBA mode, and accepts the numbers directly (in much

the same way as the SCSI protocol). The formula for CHS to LBA translation is:

LBA = (((Cyl# * max heads) + Head#) * max SPT) + (Sector# - 1)

CHS-CHS translation is a pre-LBA scheme designed to overcome the 1024 cylinders

limitation inherent in all BIOS's. Simply halve the cylinders and double the

heads. Enter these values into the BIOS and you're done. The BIOS 'sees' that

the number of heads is >16 and that it should therfore 'undo' the halve-double

before it accesses the drive itself. This requires only a tiny ammount of extra

effort in the BIOS code and is quite fast. This method of translation was very

popular in Award BIOS's were the translation mode was labelled 'LARGE'. This

'feature' limited one to a maximum of a 1Gb drive support though.

Software that bypasses the BIOS must be aware of the use of translation, since

the CMOS/BIOS CHS parameters are no longer necessarily the drive's physical

parameters. To this end, several software protocols now exist for the

communication of such information to operating systems like Windows 95, NT,

etc.

� Notes on Novell & UNIX variants, DOS with cylinders > 1024.

The standarc PC's BIOS cannot access cylinders past 1024.

Novell, UNIX, and some other OS's replace the BIOS with their own disk

management routines and can quite happily access the hard disk past cylinder

1024 without any drama, and without the use of LBA. MS-DOS and Windows can not,

since they rely on the BIOS to do the access for them.

Because of this, often with >504Mb HDD's (or HDD's with cylinders > 1024), UNIX

and Novell will define partitions that seem unusual or incorrect. In these

cases HDP may incorrectly tell you to turn on translation.

If the cylinders value reported is greater than 1024 and the heads value is

less than 17, simply change the CMOS cylinder value to 1024 and leave the heads

value alone... this is a rare case where the cylinders above 1024 are handled

by the special drivers that are part of the Operating System. DOS type

partitions will be limited to the first 504Mb of the disk only. The drive is

most likely not an LBA capable unit. (ie older ESDI drives, usually) Obviously

you should only attempt to enable LBA on a drive that supports it, and the only

type of drive that does are modern IDE type drives > 504Mb. A 10 year old 120Mb

ESDI sure as hell isn't going to support LBA!! (LBA wasn't invented until 1995)

If, however, the cylinders value is below 1024 (and the heads value is greater

than 16) then you are probably dealing with an LBA capable EIDE Drive. In this

case turn on translation. Novell 3.11 and later now have drivers for this type

of configuration so it IS possible, indeed it is the preferred method for new,

entry level servers where SCSI isn't justified.

The MOST common fault I see with DOS partitions is an attempt to access past

the 504Mb point of a HDD with a BIOS that cannot do so (ie doesn't have

translation support); and this usually appears in the form of a cylinder number

>1024. This is often because the person setting the PC up used the 'auto

detect' function of the CMOS setup to determine the drive type and 'forgot' to

trim the cylinders value back to 1024.

NEVER EVER set any BIOS controlled drive to a Cylinder number greater than 1024

unless your BIOS specificly supports it. Depending on your BIOS, you will

either corrupt the drive, reduce the drive to a very small capacity, or damage

the drive. Some AWARD BIOS's have a setting called 'LARGE' which supports

cylinder numbers up to 2048 (without LBA), however use of this mode is quite

uncommon since it is only available on Award BIOS, and is often the source of

incompatabilities with DOS disk software tools. This mode was designed mainly

to provide BIOS support to old ESDI drives with >1024 cylinders. Do not use it

on modern HDD's.

Some BIOS's I've seen perform a binary bit-mask operation on the cylinder

number - the result is that cylinder 1025 becomes cylinder 0, cylinder 1026

becomes cylinder 1, and so on. So as soon as something tries to store data on

cylinder 1025, cylinder 0 is really overwritten, and data loss is the result.

Other BIOS's I've seen limit the cylinder number to 1025 - data loss is

avoided, since a "too big" partition cannot be created in the first place, but

it is still not technicly 'correct' to leave this situation be, since any

program relying purely on CMOS parameters will surely give trouble.

Some AMI Graphics Mode CMOS Setups also examine the partitions on the disk and

decide for themselves wether LBA should be on or not, despite also having a

'manual' LBA on/off selection in the CMOS. Watch the AMI Status box carefully

(the text box that lists CPU type, RAM type etc just before booting) to see if

it reports on the LBA status or not. Trust this status box over your own

settings in the CMOS. The only way to 'fool' this AMI BIOS is to wipe all

partitions, reboot, set your preferred LBA setting in the CMOS, re-boot again &

re-partition, then reboot again, and DOS format to arrive at a proper working

setup.

� What the 'Sectors b4 part' and 'Sectors in part' values are.

Although somewhat self explanatory, these two values form a doubly linked list

of the allocation space on the disk. I am unsure WHY these values exist

(they're just a simple calculation of the other numbers in each partition

entry), and do not appear to be actually used by any software in my experience,

but none the less, they are part of the documented standard and should be set

correctly if one is to expect proper operation.

12-12-98: Actually, that's not quite true. FDISK uses these numbers to

calculate the "percentage disk space used" figures; FDISK has some simple error

checking to avoid displaying rubbish values onscreen, however FDISK never

validates the numbers themselves; so if you get a drive that seems to be

partitioned OK, but FDISK shows wierdo percentage readings, these two numbers

are probably wrong.  Outside of FDISK, the numbers appear unused, and even

tools like the (normally excellent) Norton Diskedit, chkdsk, scandisk, etc

don't seem to validate them in any way.

'Sectors b4 part' is the total number of sectors on the HDD from the beginning

of the HDD to (one sector before) the start of the current partition.

'Sectors in part' is the total number of sectors within the current partition.

'Sectors in part' is calculated as:

((End_Cyl-Start_Cyl)*heads*spt) +

(((start_cyl_end_head-start_cyl_start_head)+1)*spt)

ie: start_cyl: 0    start_head: 1   start_spt: 1

end_cyl: 527    end_head: 5     end_spt: 55

((527-0)*6*55)+(((5-1)+1)*55) = 174185 sectors in partition

'sectors b4 part' is calculated as:

((start_cyl)*heads*spt)+((start_cyl_start_head)*spt)

ie: start_cyl: 528    start_head: 0   start_spt: 1

end_cyl: 1008     end_head: 5     end_spt: 55

((528)*6*55)+((0)*55) = 174240 sectors before partition

HDP now performs these calculations, and displays them onscreen next to the

actual read values. The values are drawn in dark green if found to be valid, or

red if found to disagree. Note that HDP _assumes_ (when working out these

values) that a partition always ends on the highest Head and sector of a given

cylinder. If some wierdo partitioning scheme doesn't do this (DOS FDISK

_always_ does) HDP will get it wrong!. HDP uses the head and sector numbers

retrieved from the partition table to do the calculations - so the current BIOS

translation has no effect on the validity of the calculations.

� About the Partition table, the numbers and the MBR.

The following is a sort of reference, a collection of thoughts, bizarre facts

and miscellaneous information that any data recovery technician worth his salt

should know by heart. You may find it useful.

Cylinder and Head numbers range from 0 to maximum. A cylinder is sometimes

called a track, espescially when referring to floppies. Sector numbers range

from 1 to maximum.

For non-translating BIOS's the maximum cylinder number is 1024, the maximum

head number is 16, and the maximum sector number is 63. The largest drive size

supported is 504Mb.

For translating BIOS's, the maximum cylinder number is still 1024, but the

maximum head number is 255 and the maximum sector number is 63. The largest

drive size supported is 8.4Gb. If the type of translation is LBA, then the

drive itself must also support LBA.

The partition table is _ALWAYS_ found on Cylinder 0, Head 0, Sector 1 which is

the first physical sector on the drive. DOS's Master Boot Record is stored in

the SAME place - the MBR and the partition table are stored together in the

same physical sector. The undocumented command FDISK /MBR can be used to

restore a corrupt MBR without damaging the partition data, if need be (this is

useful to recover from a crash where the partition table is being rebuilt by

hand). The MBR is sometimes referred to as the bootsrap loader - it is the

software designed to find the active partition, then load and execute the OS

from this partition. The MBR is OS Specific - when running a multiple OS

computer, the MBR as WELL as the active partition are usually changed. This is

because the DOS MBR has some design limitations that sometimes make it

unsuitable for booting OS's other than DOS. (booting from anything other than

the first hard disk, for example).

It is generally accepted practice to start a partition at head 0, sector 1,

cylinder xxx on a drive - never on any other head or sector. Similarly, it is

always accepted practice to end a partition on the maximum head and sector

numbers that apply so that there is no "waste of space" between partitions. In

fact, prior to DOS 4.00, the _ONLY_ way to tell FDISK how to create multiple

partitions was by specifying the cylinder number you wished to end/start a

partition on! DOS 4.00 introduced the "percentage/megabytes" system of space

allocation. The exception to the rule (sigh) is the FIRST partition which

usually starts on head 1, sector 1, Cylinder 0 - one head away from the MBR.

Cylinder 0, head 0, sectors 2-maximum are wasted. Some replacement BIOS drivers

(such as Ontrack disk Manager), disk security programs and viruses use these

'wasted' sectors to store parts of themselves that could not be fitted into the

MBR, or as extra data storage.

This is also the reason why partitions never seem to come out "exactly" the

size you ask for. The _actual_ space you get is rounded (up or down, I'm not

sure - the 0.5 rule we all learn in primary school applies, I think) to the

nearest Cylinder. You may get a little more, or a little less.  This rounding

function is also why FDISK sometimes reports that is disk is only 98% used,

despite the fact that it also reports no space to create a partition!

The BIOS _NEVER_ uses the last cylinder of a drive. It is _always_ unused. The

BIOS also calls this a test cylinder. The BIOS tells DOS there is one cylinder

less than the actual configured number of cylinders present.

DOS _NEVER_ uses the last cylinder of a drive. It is _always_ unused. It is

called the "diagnostic" or "test" cylinder. I'm unaware of anything that

actually uses this cylinder, EXCEPT for "Norton Utilities Calibrate", and

"SpinRite" programs.

So... Under MS-DOS and a PC Compatable BIOS, you ALWAYS loose 2 cylinders worth

of potential disk space!! The BIOS tells DOS there is one less than there

really is, and in turn DOS then leaves the last cylinder as well. One wonders

if a total of one cylinder was ever meant to be set aside, and this 'doubling

up' was a design bug due to lack of communication between the DOS programmers

at Microsoft and the BIOS programmers at IBM?. On a 32 head, 63 SPT drive

that's almost 2Mb worth of space lost! ouch!

OS's like Novell and Windows NT _DO_ go all the way and use all the space; in

fact, installing Win NT from a DOS FDISK'd drive then running the disk

administrator always reveals that 'extra' 0.5-1Mb or so of unpartitioned space,

which you can then make use of!

The last two bytes of the partition table/MBR must be 55h, AAh to indicate a

valid partition table. If these 2 bytes are corrupt, FDISK will show no

partitions defined, REGARDLESS of the actual values in the partition table.

The reason this system was defined was that if the partition contains "random

garbage" and no sanity check existed, drives could be physically damaged (by

seeking past their last cylinder, for example) and/or the computer could be

locked up. Remember that, even on a floppy bootup, the HDD's partition table is

read and interpreted in order to assign the drive letters for the hard disk(s)

partition(s). Garbage read from the partiton table may crash the bootstrap

loader, so that you reach a catch 22 situation: to fix it you need to boot, but

you can't boot until it's fixed.

The presence of the 55AAh signature is a simple sanity check to avoid this

problem. I have seen computers that can't be booted (even from floppy) because

the partition table contained garbage that DOS's boot programs can not cope

with, yet had a valid signature. The only solution was to set the drive up as a

non-BIOS accessible drive (say, by setting the CMOS type to 'not installed')

then using direct hardware access to wipe the HDD's partition table. Not

a pleasent experience, since almost every method of HDD recovery requires you

to first boot DOS, and will then not run without BIOS support for the drive.

� All about clusters

The following is not really relevant to HDP, but I see it mis-stated so often,

that I felt I just had to act, by stating the facts.

What is a cluster? A cluster is a group of one or more sectors. The sectors are

contiguous (that is, physically one after the other on the disk). A cluster may

cross a head or cylinder boundary. DOS allocates space to files in units of

clusters.

The cluster size is NOT related directly to the partition size. This is so

commonly mis-stated that it's not funny. ANY size drive can have ANY cluster

size you like (with one general limit).

The cluster size is determined by the utility that places the DOS format on the

disk, AFTER the partitions are defined. MS-DOS's FORMAT.COM command decides on

the cluster size internally, and is not reconfigurable to any other cluster

size. It is MS-DOS's format utility that places this restriction, NOT the

partitions. If you wrote your own DOS formatter, you could easily incorporate

any size clusters. A commercial product called "Partition Magic" does this

quite nicely. IT IS THIS FACT THAT IS MISUNDERSTOOD.

Here is how DOS's FORMAT.COM chooses cluster sizes:

Partition Size               Cluster size

0 Mb to 15.99 Mb              4k

16 Mb to 31.99 Mb              2k

32 Mb to 63.99 Mb              2k

64 Mb to 127.99 Mb             2k

128 Mb to 255.99 Mb             4k

256 Mb to 511.99 Mb             8k

512 Mb to 1023.99 Mb           16k

1024 Mb to 2047.99 Mb           32k

DOS 2 to 4.01 does not support partitions larger than 1023.99Mb.

DOS 5+ (icl. Win95) does not support partitions larger than 2047.99Mb.

Finally, there cannot be more than 65,535 clusters on a partition, since MS-DOS

only uses a 16-bit variable for holding the calculated cluster number.  This

means that with larger drives, not all cluster sizes can be selected. for

example, a 1Gb partition can only have 32k clusters.

Win95B (OEM Service Release 2) introduced 32-bit FAT, and thus support for

partitions >2047.99Mb. Technical details are unclear at this time, as I don't

have Win95B to experiment with, so I cannot document that yet. I understand

that Win95B also breaks the 65535 cluster limitation. 32-bit FAT is optional,

so either configuration may be found in the field. Other disk formats that also

break the traditional cluster size limitations are HPFS, NTFS, doubleSpace and

Drivespace, all of which have various ways of reducing cluster sizes to reduce

wasted space.

Cluster sizes are a trade-off. The smaller the cluster size, the bigger the FAT

Tables. The bigger the FAT, the slower the DOS filesystem. The bigger the

cluster sizes, however, the more wasteful the filesystem becomes. A 1 byte file

stored on a disk with 16k allocation units takes ... 16k! DOS cannot allocate

space in units smaller than a cluster so the other 15.99kb is basically

"wasted".

This is both good and bad. The "wasted" space is often referred to as slack

space. "Xtree", for example, can display this figure. The bad thing is that if

you store a lot of small files, the slack space can be very high (as much as

30%, or more). The good thing about slack space is that if the file grows a

little, no extra diskspace is taken until the file fills the cluster.  That is

why you can sometimes save a longer version of a file onto a disk with 0 bytes

free, and STILL HAVE IT FIT!!

To determine the amount of space taken by the FATs (there are 2 on a Hard

disk), do the following:

Use DOS's CHKDSK to find the "total allocation units on disk" or do the math:

"disk size in bytes divided by cluster size in bytes"

Now, for a 12-bit FAT, multiply the number by 3. for a 16-bit FAT, multiply the

number by 4. That is the number of bytes set aside for the FAT. (Use HDP to

find out if the FAT is 12-bit or 16-bit). Round this number up to the closest

multiple of 1k to find the space consumed by the FATs. (This method isn't 100%

accurate as there are a few FAT entries we have not counted, nor have we

allowed for some quirks of how DOS relates FAT locations to physical Cylinders,

but they are trivial and not relevant enough to be worth counting in these

simple examples)

Example:

(From CHKDSK: CHKDSK uses the phrase 'allocation units' instead of clusters,

but they mean the same thing)

527,654,912 bytes total disk space

22,536,192 bytes in 80 hidden files

2,514,944 bytes in 298 directories

408,485,888 bytes in 7,595 user files

94,117,888 bytes available on disk

8,192 bytes in each allocation unit

64,411 total allocation units on disk

11,489 available allocation units on disk

( The slack space on this disk is approx 40Mb or 12.6% )

# of clusters        :  527654912/8192 = 64411 clusters.

# of bytes in FAT's  :  64411 * 4 = 257644 (rounded up =) 252k

If we decided to have the smallest cluster size possible (1 sector) the FAT

tables would take much more space:

527654912/512 = 1030576 clusters.

1030576*4     = 4122304 (rounded up =) 4026k

Now, consider a 'worst case' example: a 2Gb HDD & 512b clusters:

2147483648/512 = 4194304 clusters

4194304*4      = 16777216 (rounded up =) 16384k. 16Mb of diskspace consumed in

FAT's alone!! (this is not physicly possilbe because you can't have more than

65535 clusters.. it's for theoretical FAT size demonstration only).

with 16k clusters:

2147483648/32768 = 65536 clusters

65536*4         = 262144 (rounded up =) 256k. A saving of 15.75Mb!!!

Which example do YOU think will result is a faster file system?

� The Author

Craig Hart is a qualified electronic technician, computer technician with 11

years experience in the field of PC's & PC Compatables, published technical

author, programmer and hardware expert working in Melbourne, Australia. E-MAIL

may be sent to him at chart@hyperlink.net.au Replies at the author's

discretion.  Feedback is welcomed, if not encouraged, however unsolicited SPAM

email will be prosecuted to the fullest extent of the law.

Visit my home page at http://home.hyperlink.net.au/~chart for further support,

information, and the latest version of this software.