Beep tone independant of processor speed. Works even for those troublesome

Pentiums :) SPEAK.COM produces a 440 Hz tone of 10/18ths second delay based on

clock tics rather than clock speed.  Barry Block's message in BATPOWER

containing the debug script and offsets to adjust tone and duration is

provided below.

� Area: Fido Bat Power �������������������������������������������������������

Msg#: 502                                          Date: 19 Feb 98  06:17:55

From: Barry Block                                  Read: Yes    Replied: No

To: Rex Stewart                                  Mark:

Subj: No Beep on Pentium

������������������������������������������������������������������������������

Hello Rex, for everybody else, this Debug script beeps the speaker.

RS> ...received a reply from Barry Block with a slightly different

RS> approach, and a somewhat different tone...

RS> I wish Barry would have documented his source...

RS> because varying the tone is a neat thing,

I'm only doing this because your name is Rex Stewart :-)

Here's a straight thru version. No proc's, so shorter.

===cut===

N SPEAK.COM

E 0100 FA B0 B6 E6 43 B8 8C 0A E6 42 88 E0 E6 42 E4 61

E 0110 0C 03 E6 61 B9 0A 00 FB 1E B8 40 00 8E D8 A1 6C

E 0120 00 3B 06 6C 00 74 FA E2 F5 1F E4 61 24 FC E6 61

E 0130 B4 4C CD 21

RCX

0034

W

Q

===cut===       DEBUG < TheAboveScript

Pitch    is the word at 6,7   - Trial&Error   as is, about 440Hz

Duration is the byte at 15h   - in 1/18 seconds

OffSets

0006: 8C

0007: 0A

0015: 0A     (offset 21 decimal)

Regards, Barry

-!- Terminate 5.00/Pro

! Origin: EBO-BBS A'dam +31-20-6002828 (2:280/901.42)