The following is from Vince, WB8NWQ

OK now I will try and explain how to setup OS2 so you can run the RLI

program under OS2. The information I am providing I obtained from the

OS2 support group.

Section 1 - Create a VMB Diskette:

a. Boot with a Dos 5.0 bootable diskette with the Format.COM command in

command in the Path statement in the AUTOEXEC.BAT file. Replace the

bootable diskette in Drive A with the diskette you want to use for

the VMB.  At the C> prompt type "Format A: /S ".

b. If you have pure Dos 5.0 installed on your computer, boot up from there.

Insert the blank formatted diskette you want to use for the VMB in Drive

A, and from the Dos root directory type "SYS A:".  Command.com should be

on your diskette when you have completed this process.

Note: I have Dos 5.0, and have not tried to do this with Dos 6.0 or

higher, but I would suspect it also works for Dos higher than version

5.0.Also, I am using OS2/0 and I would also suspect that version 2.1

will also work.

2.  Boot up in OS2/2 and copy the following files fron the OS2\MDOS directory

to the VMB diskette:

Ansi.Sys

FSFILTER.SYS

FSACCESS.EXE

EMM386.SYS

HIMEM.SYS

LPTDD.SYS

MOUSE.COM

3.  On the VMB diskette create the following AUTOEXEC.BAT file with a text

editor:

ECHO OFF

PROMPT $P$G

MOUSE.COM

PATH C:\;C:\VMB

FSACCESS M=A

C:

CD\VMB

NOTE: if you start DOS from an Image file. you cannot use physical Drive A

until you release the Image File.  FSACCESS M=A releases Drive A and

assigns 'M or whatever unused loical drive you select to name in its

place.

4.  On the VMB diskette create the following Config.sys file with a text

editor:

DEVICE= HIMEM.SYS

DEVICE= EMM386.SYS

DOS= HIGH, UMB

DEVICE= FSFILTER.SYS

DEVICE= ANSI.SYS

DEVICE= LPTDD.SYS

FILES= 40

BUFFERS= 30

LASTDRIVE = Z

5.  Make a directory called C:\VMB to house  the VMB image.  Copy the

Command.com from the VMB diskette to the VMB directory. Got to C:\VMB

and type: VMBDISK A: DOS5.IMG <enter>

This will create an iamfe of the diskette you created.  The image file

will be named DOS5.IMG and will be located in the C:\VMB directory.

6.  When completed, return to the Desktop.

Section II - Create a VMB DESKTOP ICON

1. Open the OS/2 System folder.

a.  Double click on the command prompt Icon.

b.  Copy the Dos full screen icon to the Desktop by holding down the CTRL

key and dragging the Icon to the Desktop.

c.  Close the "command prompts" and the OS/2 system folder.

2.  Change the Dos settings for the VMB Icon

a.  Select the Dos FullScreen icon by clicking on it once.

b.  Bring up its menu by clicking the right mouse button once.

c.  Click on the arrow on the "open" line.

d.  Select settings.

e.  Select session.

f.  Select dos settings.

g.  Change the follwoing settings:

Dos_UMB		to:  ON

DOS_HIGH		to:  ON

DOS_STARTUP DRIVE	to:  C:\VMB\DOS5.IMG

XMS_Memory_Limit	to:  Default is 2048 (Minimum Requirement)

h.  Use these settings for applications that use DOS Protected Mode

Interface (DPMI)

DPMI_Memory Limit	to:  4	(Minimum Requirement)

DPMI_DOS_API	to:  ENABLE

- OR -

I.  Use these settings for applications or device drivers that Load High.

EMS_Memory Limit	to: 0

DPMI_Memory_Limit	to: 0

3.  Save the settings.

4.  Select the General and change the Title field to "VMB" or whaever name

you wish to call your Desktop Icon.

5.  Click twice in the upper hand corner to close the Window.

6.  Double click on your new DOS VMB DESKTOP and there you are.

7.  To EXIT the VMB, type CTRL-ESC, form the 'Window List, the "VM" session,

click the right mouse button, and select 'Close'.

Once you have the VMB destop open, you load rli as if it were under

regular dos.I have a batch file that I run under the VMB session that

will load share,bpqcode,lm,etc., and then will load dv and the windows.