Documentation for EGADUMP.COM

Copyright 1989 Infinity Design Concepts Inc. All Rights Reserved

1052 Parkway Drive

Louisville, Kentucky 40217

(502) 636-1234

This program is a TSR (memory resident) utility that adds

more capabilities to the DOS print screen functions. DOS

by itself, will handle the text modes, but if the CGA graphics

modes need to be dumped to the printer, then GRAPHICS.COM

must be loaded. There is a severe limitation to GRAPHICS.COM's

capabilites and that is, that it will not work on EGA graphics

modes, that is the following modes;

320x200 - 16 color	(mode d)

640x200 - 16 color	(mode e)

640x350 monochrome	(mode f)

640x350 - 16 color	(mode 10)

EGADUMP should be loaded AFTER graphics.com, since graphics.com

will try to handle these modes if it gets control before

EGADUMP. In order to be able to dump all screen modes to the

printer, GRAPHICS.COM (or CGRAPHIC.COM) must be loaded

in addition to EGADUMP.

EGADUMP is set up to work with the IBM graphics printer, which

means that if you have a printer that has an EPSON or IBM mode,

EGADUMP will work with it. If your print screen works now in

graphics mode, then it will work with EGADUMP.

There are two printer strings used in EGADUMP;

ESC,"L",n	- set up for double density graphics

ESC,"3",n	- set to n/216" linefeeds

Command line switches:

The /I switch given when EGADUMP is loaded

will cause black to be printed as black and white

to be printed as white. The default for EGADUMP

and GRAPHICS.COM is to reverse black and white

when printing. The switch can be invoked as follows;

EGADUMP /I

The /F switch given when EGADUMP is loaded will

cause a formfeed to be sent to the printer. The

switch can be invoked as follows;

EGADUMP /F

The switches /I or /F may be used in tandom and

may be invoked as follows;

EGADUMP /I /F

Technical:

EGADUMP inserts itself into the Interrupt 5 (print screen)

chain. When the print screen key is depressed,

EGADUMP will get control (if it is loaded last) and

determine which video mode is active. If it is

one of the modes listed above, then EGADUMP will

handle sending the screen to the printer. If it is not

one of the above modes, then EGADUMP passes control

back to the routine that would have been called, had

EGADUMP not been loaded.