PIDSIM

PID Control Loop Simulator

GENERAL

PIDSIM is a program for IBM PC compatibles that simulates PID control

loops, including level, pressure, and flow.  Virtually all aspects of

a loop's dynamic behavior can be changed to see the effects of white

noise, backlash, stiction, tuning, etc on control loop performance.

PIDSIM is a great teaching tool for at home study or formal in-plant

instruction.

HARDWARE REQUIREMENTS

PIDSIM requires an IBM-compatible PC with a VGA display, mouse, and

MSDOS 3.3 or higher.  A fast 386 or 486 machine is preferable, but it

will run adequately on a 286 or XT class machine. It has been tested

under MSDOS 3.3 - 5.0 and OS/2 Warp.

REGISTRATION

Please feel free to evaluate PIDSIM and distribute copies of

PIDSIM as long as all files are unaltered and distributed

together.  If you continue to use PIDSIM you are obligated

to register this program by sending a U.S. check or money order

for $25 to:

CMAR Software

RR 1 Box 24k

Sheridan, AR 72150

Arkansas residents should include 5% sales tax.

Upon registration, you will receive the latest version of PIDSIM

without the shareware notice.  The registered version of PIDSIM

will be licensed to you for single computer system use and may

not be distributed in any manner.

Please send any comments or suggestions you may have with regard

to PIDSIM. If feasible, they may be incorporated in future

updates.

DISCLAIMER

Although PIDSIM has been tested extensively and is believed

to be free of errors if used correctly, no guarantee is

provided.  CMAR Software disclaims all warranties relating

to this software, whether express or implied, as to the

merchantability or fitness of this program for any purpose.

CMAR Software also assumes no liability for any damages,

either direct or inconsequential, which may result from use of

this program.

INSTALLATION

Compressed File - Unzip the file into any convenient location.  Run

install.exe to install the program

Distribution Disk - run install.exe to install the program

PROGRAM FILES

Install.exe - install program

Pidsim.exe  - program executable

Pidsim.doc  - this documentation file

File_id.diz - BBS file description

DEFINITION OF TERMS

PID - Proportional, Integral, Derivative

Proportional Gain - aka Controller Gain, generates a control

action proportional to the control error.

Integral - (Reset), generates a control action proportional

to the integral of the control error.

Derivative - (Rate), generates a control action proportional

to the derivative of the control error.

Setpoint -  value set by operator which loop should control to.

Process Value -  current value of the control loop input

Process Output - current value of the control loop output

Backlash - a lost response from the process when the direction

of movement of the actuator changes, due to actuator type, linkage

slop, etc.

Stiction - a result of static friction. When the actuator has

been at rest, a certain amount of force is required to overcome

the static friction, resulting in no movement for small changes in

output.

White Noise - completely random noise (in this simulation it is

only as good as the compiler's random number generator, which is

never completely random)

PID Algorithm - this simulation uses the standard, noninteracting

or ISA form of the algorithm.

Time Constant - the time required for 63.2% of a control action

to be seen after the process output is stepped, aka tau.

Deadtime - a period of time in which no change in process input

is seen in response to the process output being stepped

Open Loop - the controller is not acting on the process input, ie,

the loop is in manual mode.

Closed Loop - the controller is acting on the process input to

maintain the current setpoint value, ie, automatic mode.

Total Settling Time - time required for the process value to

reach a new steady state after the process output is stepped.

Deadband - a specified range around the setpoint within which the

controller will not change the output.

Lambda - closed loop time constant

Process Gain - the ratio of the change in the output to the change

in the input

Cut-Off Period - period at which the frequency response starts to

decay

Nominal Phase Margin - amount of additional phase lag that can be

introduced before the system becomes unstable

Nominal Gain Margin - factor by which the loop gain can increase

and the loop will remain stable

Phase Crossover Frequency - frequency at which the phase of the

closed loop system is -180 degrees

Gain Crossover Frequency - frequency at which the gain of the closed

loop system is unity

Time to Overflow - seconds required for the tank to overflow at

maximum input and zero output from the present level

Time to Empty - seconds required for the tank to empty at maximum

output and zero input from the present level

MANIPULATING THE PROGRAM

The optional command-line parameters for PIDSIM are as follows:

PIDSIM {<filename> /H /L /S}

where:

<filename> is a previously saved session file

/H displays a command-line help screen

/L locks out the setup and autotune buttons

/S displays the program serial number (registered version)

If a previously saved session file named default.dat exists in the

current directory upon startup, it will be loaded into the program,

thus allowing the ability to change the program default state.

After starting the program, the overview screen is visible, showing

the three control loops and valves.  The buttons at the bottom of the

screen allow control of the program via the mouse.  You may switch

back and forth from the overview to the graphic plots of the

individual loops.  To toggle execution of the simulation, press the

button labelled Str/Stp.  The rest of the buttons allow changing

the individual loop parameters.  The simulation must be stopped to

change these parameters.  The parameters to be changed apply to the

loop which is currently selected.

Loop Button

change the setpoint, process value, process output, and

loop status for the currently selected loop

Tuning Button

change proportional gain, integral, derivative and deadband

values for the currently selected loop

Limits Button

change high and low range values for the setpoint, process

value, and process output for the currently selected loop.

set values for hi and lo alarms for current loop

Setup Button

change the following simulation parameters:

process time constant- sets the time response for the currently

selected valve

process deadtime- sets the deadtime for the current loop

controller rate- defines the rate in seconds used for pid

control calculations (global setting for all loops)

simulation rate- actual execution rate of the simulation

(global for all loops)

zoom factor- display compression factor for the graphics plot

(global for all loops)

valve- selection of control valve for the selected loop.  The

generic valve types are:

quick-open

linear

parabolic

equal-percentage

the user-defined valve allows setting the actual

valve response by entering the desired process value

for a specific % valve opening

backlash- defines amount of backlash present in % of output

stiction- defines amount of stiction present in % of output

white noise- defines amount of white noise present in % of

output

units- the program can be set to allow input of tuning

parameters in % proportional band or engineering units

log data- log simulation data to a text file which is

compatible with the Entech Analyse (c) program.

Specify the number of points to log and the filename

of the text file.

sine wave- injects a sine wave into the input of the currently

selected loop.  The amplitude and period of the

sine wave may be adjusted.

tank volume- change the total tank volume

Read Button

reads in settings from a previously saved simulation session

Save Button

saves current program settings to a session file, do not enter

a file extension, as the extension will be forced to .dat

LIC1000 Button

make the level control loop the current loop

PIC2000 Button

make the pressure control loop the current loop

FIC3000 Button

make the flow control loop the current loop

Overview Button

displays the overview graphic

Graph Button

displays the trend plot for the current loop

AutoTune Button

Calculates suggested tuning for the current loop

Str/Stp Button

toggles the simulation on and off

Reset Button

changes all settings to the default startup state

TRADEMARKS

MSDOS is a trademark of Microsoft Corporation

OS/2 WARP is a trademark of International Business Machines

ANALYSE is a trademark of Entech Control Engineering

Entech Control Engineering Inc.

16 Four Seasons Place

Suite 100

Toronto, Ontario

(416) 626-6908

VERSION CHANGES

v2.1:

first public release

v2.2:

(1)  added ability to inject sine-wave into loop input

(2)  fixed bug in loop plot to correctly display values

when mouse clicked on plot

(3)  changed plotting algorithm to reduce flicker when plot

is wrapped at right edge

(4)  fixed bug in process deadtime when loop in manual

(5)  made pressure loop reverse-acting, which it should have

been from the start

(6)  added autotune fuction for calculating tuning values

(7)  reworked algorithm for modelling level loop, added

global parameter for tank volume

(8)  added loop parameters for high and low alarms (currently

only used for the level loop)

COMMENTS, SUGGESTIONS, BUG REPORTS

INTERNET      cmarshal@cris.com

U.S. MAIL     CMAR Software

RR 1 Box 24k

Sheridan, AR 72150