��������������������������������������ͻ

� X-10 Events             Version 1.09 �

� Copyright 1988       F.C. Wilczynski �

��������������������������������������ͼ

Much of this file is duplicated in a HELP file within the program. To

access the HELP, simply type HELP at the Xe> prompt, or enter

XE HELP

at the DOS command prompt.

X-10 Events allows you to access many of the features of the  X-10

Computer Interface.  This program allows you to access the X-10

Computer Interface from the command line, or from within the program.

In addition, commands can be piped thru the program from a list of

commands contained in an ASCII text file.

COM PORT

The default COM port of this program is COM1, but can be overridden to

address COM 1 thru COM 4 by adding a /x as the final parameter on the

command line at the DOS prompt; where x is a number 1, 2, 3 or 4.  You

can permanently change the COM port by entering:

XE PORT x

Where x is the COM port value, 1 thru 4, you wish to use.  The program

must be in the current directory of the current drive, and there must

be sufficient disk space remaining to allow the program to make a copy

of itself.

This program allow the following:

Clear any selected Timer Events

Change the Interface House Code

Read the Interface Time and House Code

Review Timer Events

Set the Interface Time

Send Direct Command thru the Interface

Set Interface Timer Event

The syntax of the program is not case sensitive, and commands can be

entered in upper or lower case.

The program can operate from a command line, or within the program.

To operate from the command line, enter the program name and the

command you wish to execute:

XE RV

would produce a list of the current X-10 Interface Timer Events, and

return back to the DOS prompt.

If you wish to execute several commands, you may execute them from

within the program, eliminating the need to enter the program name

with each command.  The operation will also be faster since the

program need not be loaded for each command.  Entering:

XE

will cause a "Xe>" prompt to be displayed.  Enter your commands just

as you would on the DOS command line, without the program name:

RV

would produce the save list indicated above.  To exit from the program

press the ESC key.  If the programs default COM port is not set to the

port to which the X-10 Interface is attached, the port override (/x)

would have to be entered for each command entered from the DOS command

line.  Running from within the program, the override is only entered

when the program is initially started.

Clear Timer Event

Syntax:  cl  L

Where 'L' is a valid event value in the range of 1 thru 128.  A

range of events can be specified by separating two values, the first

less then the second, by a dash.  A list of events can be specified

by separating the values by commas.

Examples:

cl 10        Clears timer event 10.

cl 10-20     Clears timer events 10 thru 20.

cl 10,11,12  Clears timer events 10 thru 12.

Change House Code

Syntax:  hc H

Where 'H' is a valid house code in the range of 'A' thru 'P'.  All

timer events are maintained.

Read Time

Syntax:  rt

Displays the current Day, Time, and House Code settings of the X-10

Computer Interface.

Review Events

Syntax: rv [p]

Displays a list of the current timer events currently in the X-10

Computer Interface:

------------------ Event Number

|    -------------- Mode of Operation

|   |     --------- House Code

|   |    |   ------ Units

|   |    |  |       Time ---------------------------------------------

|   |    |  |       Days ------------------------------------         |

|   |    |  |       Command -----------------------          |        |

|   |    |  |                                      |         |        |

5 Secure A1,2,3,4,5,6,7,8,9,10 .................. DIM   8. Weekends  7:30 pm

6 Normal A15 .................................... SW/ON .. Everyday  6:00 pm

7 Normal A14  ....................................ON ..... Everyday  7:00 pm

8 Normal A16 .................................... OFF .... Everyday  7:30 pm

Using the optional 'P' command will cause the listing to be sent to the

printer.

Set Time

Syntax  st T

Sets the time of the X-10 Computer Interface to the specified day

and time, or to the current day and time established by the

computer.

Examples:

st mon,7:25 pm    Sets the day to Monday and the time to 7:25 pm

st mon,19:25      Sets the day and time as in example one, using

a 24 time format

st C              Sets the day and time the current day and time

of the computer.

Direct Command

Syntax  HU C

Where 'H' is a valid house code in the range of 'A' thru 'P', 'U' is

a valid unit value, in the range of 1 thru 16, and 'C' is a valid

module command. Unit values may be specified as a range by

separating two values by a dash, or as a list, with values separated

by commas.  Valid commands are:

ON  ....... Turns Specified Units ON

OFF ....... Turns Specified Units OFF

DIMxx ..... DIM Lights to a value of 1 thru 16 (16=bright)

LIGHTOFF .. Turns Specified Units off, but light are

Dimmed off.

SWITCH .... Turns All Wall Switch Lights OFF.

SW/ON ..... Turns All Switch Lights OFF & Specified Units ON

ALLON ..... Turns All LIGHTS set to HOUSE CODE ON

ALLOFF .... TURNS ALL HOUSE CODE UNITS OFF

Examples:

A1 ON         Turns Unit 1 of House Code 'A' ON.

A1-2 OFF      Turns Units 1 and 2 of House Code 'A' OFF.

A1,2,5 DIM8   Dims light Units 1,2 and 5 of House Code 'A' to

DIM LEVEL 8.

Set Timer Event

Syntax  HU D T C [S] [L]

Where 'H' is a valid house code in the range of 'A' thru 'P', 'U' is

a valid unit value, in the range of 1 thru 16.  'D' represents the

days of the week, and 'T' the time of the day when the command will be

effected. 'C' is a valid module command as indicated in the Direct

Command above.  The optional 'S' will cause the timer event to be

entered in Security Mode.  The optional 'L' allows you to designate

the timer event position within the interface.

HOUSE AND UNIT

The house code and unit values are entered just as they are for the

Direct Command described above.

DAY(S) OF WEEK

The indication of the days of the week, when the timer event will be

executed, can be described as follows:

TODAY  ........... An event will execute TODAY only.

TODAY+ ........... An event will execute TODAY and TOMORROW only.

TOMORROW ......... An event will execute TOMORROW only.

WEEKENDS ......... The event will execute on Saturday and Sunday.

WEEKDAYS ......... The event will execute only on weekdays.

EVERYDAY ......... The event will execute everyday of the week

TD ............... Same as TODAY

TD+ .............. Same as TODAY+

TM ............... Same as TOMORROW

WE ............... Same as WEEKENDS

WD ............... Same as WEEKDAYS

ED ............... Same as EVERYDAY

Other sequences of days can be designated in one of the following

manners:

Enter a list of numbers to indicate the days the event is to

occur, with 1 representing SUNDAY, and 7 representing SATURDAY.

For example if you want an event to occur on TUESDAY, THURSDAY,

and SATURDAY, enter a the 3 digits list 357.

Enter a quoted string, 7 characters in length, filling in a

character for each day the event to be effective.  The string

"--t-t-s" would have the same effect as the 357 in the above

example.  You can use the - or a space, but the string must be

quoted.

TIME

The time can be entered in 12 or 24 hour format. if the 12 hour

format is used the optional AM, PM is used.  The time can also be

set relative to either the computer clock, or the X-10 Computer

Interface clock.  Examples of valid times:

10:40     = 10:40 am

22:40     = 10:40 pm

10:25 am  = 10:25 am

9:30 pm  =  9:30 pm

C         = Uses current time of computer clock.

X         = Uses current time of X-10 Computer Interface clock.

C+1:15    = Current computer clock time, plus one hour and 15

minutes.

X-:10     = Current X-10 clock time, minus 10 minutes.

COMMANDS

The commands for a timer event are identical to those described in

the Direct command.

SECURITY

The use of the optional 'S' marks the timer event as SECURITY Mode,

causing the events actual time of occurrence to vary within an hour of

the specified time.

TIMER EVENT LIST ENTRY

The X-10 Computer Interface has room for 128 timer events.  When a

timer event is transmitted to the X-10 interface, the actual

position in the event list must be specified.  If the listing is

not specified, the program will download the current list of timer

events in order to locate an unused position for the new timer

event.  If you wish to change an exiting timer event it is faster

to specify the list entry value, because the program does not have

to download the existing events, and you will not have to remove

the previous entry, because it will be overwritten.

Timer event examples:

A1,3 ED 23:30 OFF

Turns units 1 and 3, on house code 'A', OFF at 11:30 pm everyday

of the week.

A6 TODAY C+:20 ON

Turns unit A6 on 20 minutes from the current time.  Attached to a

signal device, this can be used as a timer.

A9-12 TODAY+ 6:30 PM DIM10 L6

Turns on units 9 thru 12 on house code 'A', and dims them to dim

level 10 at 6:30 pm.  The timer event in place in the 6th position

in the time list, overwriting any previous event existing in that

position.  This timer event will occur today and tomorrow, and

then the event will be cleared from the list by the X-10 Computer

Interface.

Piping Files

This program allows an ASCII text file of commands to be piped

thru as if the commands were typed at the keyboard.  This allows you to

maintain a timer event list in a file which can be directly loaded into

X-10 Interface.  Any changes can be made to the file with a text

editor, and then reloaded.  To pipe a file enter:

XE < filename

The file "TEST" is included to provide an example.  You may wish to

take a good look at this file.  A couple of things you may wish to

note:  1) The last line in the file is "ESC" which cause the program

to terminate operation after sending all commands to the Interface.

If the ESC is not entered, the program will hang up after executing the

last command in the file, and will require you to reset your computer.

2) The first command sets the X-10 Computer Interface to Noon, and all

the timer events are set relative to the X-10 clock. The final

commands set the X-10 Interface clock to the same time as the computer

clock.  This technique could be of value in setting dawn and dusk

times by changing only the initial time setting of the X-10 clock in

the first command.