ODIPKT(8)                                               ODIPKT(8)

NAME

odipkt  -  translate  Open  Data-Link  Interface to Packet

Driver interface

SYNOPSIS

odipkt [ logical_board [ vector ] ]

odipktpn [ logical_board [ vector ] ]

DESCRIPTION

Odipkt.com provides a Packet Driver API over Novell's Open

Data-Link  Interface, allowing a single network adapter to

service IPX, tcp/ip, and other protocol stacks  which  use

packet  drivers.   Odipkt.com  understands Ethernet, Token

Ring, and ARCnet frame types.   Ethernet  and  Token  Ring

frames are passed unchanged to packet driver applications,

but ARCnet  frames  are  internally  mapped  to  Ethernet.

Thus,  packet  driver  applications  do  not need specific

knowledge of ARCnet in order to operate in  such  environ-

ments.

The logical_board argument indicates which logical adapter

odipkt should bind to.  Under ODI, there  is  one  logical

adapter  per  frame  type.  There may be multiple physical

adapters, and multiple frame types per  physical  adapter.

The logical adapter number starts at zero and increases by

one for each frame type.  For example, if  a  system  con-

tained  two  physical  adapters with two frame types each,

there would be four logical adapters  numbered  0-3.   The

frame  types  are specified in the NET.CFG file which must

be in the current directory (or on the APPEND  path)  when

the ODI drivers are loaded.

Typically,  the frame type used by default by Netware on a

given adapter will not be  appropriate  for  tcp/ip.   The

correct  frame  type  for  tcp/ip  on Token Ring is TOKEN-

RING_SNAP and the correct frame type for tcp/ip on  Ether-

net  is  almost  always ETHERNET_II.  Assuming an existing

installation with one adapter running the default  Netware

frame  type, it will usually be necessary to define a sec-

ond frame type (as listed above) and to specify the  logi-

cal_board  argument  as  1.   An exception to this rule is

ARCnet which uses only one frame type.  Below is a NET.CFG

entry for a 3c501 using the default address and interrupt.

Other Ethernet adapters would use similar configurations:

Link driver 3c501

envelope type ETHERNET_802.3

envelope type ETHERNET_II

Given the above entry, the logical_board argument of oipkt

would be 1.  Token Ring uses different frame types:

Link driver token

envelope type TOKEN-RING

March 3, 1992                         1

ODIPKT(8)                                               ODIPKT(8)

envelope type TOKEN-RING_SNAP

Again, the logical_board should be specified as 1 for this

example.  ARCnet installations require no frame-type defi-

nitions and should use a logical_board of 0.

Because  odipkt.com uses buffers provided by the Link Sup-

port Layer, the Link support section of NET.CFG must spec-

ify  at  least  one  buffer  of  a size appropriate to the

medium in use.  Some adapters  require  more  buffers  for

optimal performance, e.g., the 3c503 can use at least five

during reception of back-to-back packets.   The  following

lines configure five buffers large enough for Ethernet:

Link support

Buffers 5 1600

The correct buffer size for Token Ring will vary depending

on the installation.  Specifying too small a  buffer  size

will result in unpredictable behavior.

Vector  is the packet driver interrupt at which to provide

service.  The default is 105 and the parameter is decimal.

Odipkt.com should be loaded after lsl.com and the adapter-

specific driver(s).  It must, of course, be loaded  before

the  packet driver applications which use it.  The Netware

shell and ipxodi.com can be loaded  before  or  after  the

odipkt.com program.

Odipktpn.com supports an older version of the ODI specifi-

cation as used in Farallon's PhoneNET Talk product.  Oper-

ation  of  opipktpn.com is identical to that of opipkt.com

but configuration of NET.CFG is  somewhat  different:   no

``envelope''  statements are required or allowed.  Because

multiple  frame  types  are  not  supported,  odipktpn.com

treats all Ethernet cards as ETHERNET_II.

AUTHOR

Dan Lanciani

185 Atlantic Road

Gloucester, MA 01930

March 3, 1992                         2