============================================================================

ACORN ATOM EMULATOR v1.0 (c) Wouter Ras, Delft, November 1997

============================================================================

CONTENTS

--------

1. Introduction

2. Configuration

3. Keyboard

4. Emulator desktop

4.1 Tape

4.2 Options

4.3 Monitor

5. Atom Memory

5.1 ROM

5.2 RAM

5.3 I/O-devices

6. The AtomTape utility

7. Compatibility

8. Future expansions

9. Problem solving

10. References

11. Contacts

1. INTRODUCTION

---------------

Welcome to the Acorn Atom software emulator for PC's. This program

enables you to run most of the original Acorn Atom software on your PC.

Minimum requirements:

-80286 CPU (80486 DX-40 required for real time emulation)

-VGA display

-300 kB RAM

-MS-DOS 3.0

The full emulation pack contains the following files:

-ATOM.EXE       - Acorn Atom software emulator (286+)

-ATOMTAPE.EXE   - Utility to read Atom tapes into PC-memory

-ATOM.DOC       - Documentation

The emulator has been tested on several computers and turned out to work on

every one of them. The program is designed to run under an MS-DOS environ-

ment. It also works with Windows 3.1 and Windows 95, however, the full

screen mode is required. Timing, synchronisation and sound are better if run

under MS-DOS than under Windows.

2. CONFIGURATION

----------------

The emulator allows you to configure the program to your own satisfaction.

The preferences can be saved in the ATOM.INI file. If there's no .INI-file

at startup, the emulator uses default values (See also chapter 4.2).

Apart from the in chapter 4.2 described variables, the following variables

can be specified in the .INI-file:

-SpeedIndex : If you can't get the emulator to run at the right speed, you

might want to specify a number here which sets emulation at a

certain speed. The higher the number, the slower the emulation.

For 486/Pentium machines this value should be about the speed

of your CPU in MHz.

-AltExtROM  : The emulator is equipped with the AXR1 chip (see chapter 5.1)

in the memory range A000h-AFFFh. If you would like to use

another (EP)ROM at this address, you can specify the filename

of that particular ROM-image. The file's length must be 4118

bytes. Its format should be the same as .ATM-files (see ch.6).

3. KEYBOARD

-----------

Due to the fact that the Atom's keyboard layout differs form the PC's

keyboard layout, a few keys have been changed. The most important are:

-F12         : Break

-Tab         : Copy

-Alt         : Rept

-CapsLock    : Lock

-`           : ^

Furthermore the Atom-cursorkeys have been replaced by the PC-cursorkeys.

Therefore the SHIFT keys are no longer required to move the cursor down or

left. (Note: If NumLock is on, only the arrows on the numeric keypad will

give the correct results!)

In a lot of games, however, it would be better if some keys had been placed

elsewhere or would have been swapped. This can be done by toggling between

the standard keyboard (as described above) and an alternate keyboard. When

the alternate keyboard has been selected, the following keys should be used:

-Tab         : Left/right arrow

-CapsLock    : Up/down arrow

-Left Shift  : Ctrl

-Ctrl/Alt    : Shift

-Right Shift : Rept

-Cursor Lf   : Lock

-Cursor Rt   : Copy

Swapping between the standard and the alternate keyboard layout is done

during emulation by pressing the <F4> key. On BREAK, the standard keyboard

is selected automatically.

The following keys have a special function:

-F4          : Toggle standard/alternate keyboard

-F5          : Toggle original/maximum CPU speed

-F9          : Enter emulator desktop

-Ctrl+F12    : Exit the emulator

4. EMULATOR DESKTOP

-------------------

Press F9 during emulation to enter the emulator desktop. On the top line

three divisions of the desktop are shown. They can be selected by pressing

<Alt> and the highlighted letter simultaneously.

-Tape    : Alt+T

-Options : Alt+O

-Monitor : Alt+M

The desktop interface is very much like the C64S desktop interface by Miha

Peternel. So if you are familiar with that interface, you should have no

problems at all using the desktop.

4.1 TAPE

~~~~~~~~

The Tape screen enables you to select the tape images (.TAP files) from

which you can load Atom files. The screen has two separate areas; the one

on the left hand side shows the directory with all the tape images, the

area on the right hand side shows the contents of the selected tape image.

A tape image consists of a number of Atom files (.ATM files; see chapter 6

for a description) all appended to each other. One tape image can hold up

to a maximum of 500 Atom files.

If the cursor is in the left hand area, where the tape images are shown, the

following actions can be taken:

-Press the <Cursor Up> or <Page Up> key to move the cursor up.

-Press the <Cursor Down> or <Page Down> key to move the cursor down.

-Press the <N> key to create a new tape image. You must enter a file name.

If there's no file with that name already, a new file will be created and

its length will be set to 0.

-Press <O>, <Enter> or the <Cursor Right> key to move the cursor to the

right hand side of the screen, in which the Atom files are shown.

If the cursor is shown in the right hand area, the selected tape image can

be used and/or manipulated.The contents of the tape image are shown line by

line. Each line represents one Atom file, from which the name is displayed

followed by three hexadecimal numbers: The first one is the program's start

address, the second number is the program's execution address and the last

number is the program's size in bytes.

The following actions can be taken:

-Press the <Cursor Up> or <Page Up> key to move the cursor up.

-Press the <Cursor Down> or <Page Down> key to move the cursor down.

-Press <S> or <Enter> to set the tape file pointer to the cursor position.

-Press <L> to load the file the cursor is pointing to and return to the

emulator. Now there are three possibilities:

� If the program's start address is 2900h and its execution address is

C2B2h, a BASIC "LOAD" command will be issued. This means that the file

is loaded into the Atom's text area at 2900h. Then the text area is

scanned through to set the value of TOP and the free space pointer.

Remember, if the file is not a valid BASIC program, the prompt may not

appear.

� If the program's start address is not 2900h, but the execution address

is C2B2h, the COS command "*LOAD" will be issued. The value of TOP will

not be set.

� In any other case the COS command "*RUN" will be issued. This will load

the file at its start address and then execution is transferred to the

execution address specified in the file. The value of TOP is not set.

Remember that all commands that are preceded by an asterisk (*) are COS

commands and not BASIC commands, which in short means that the value op

TOP will not be set after the file has been loaded. Statements like DIM

will generate "Error 216", unless the END or OLD statement are entered

prior to RUNning the program.

If you would like to open a file using another command than the desktop

generates, or if you would like to load it at another address, it must

be typed in manually from the Atom's BASIC prompt. If no file name is

specified (or "" with the BASIC "LOAD" command), the file at the current

cursor position will be loaded. The emulator also supports the "*" wild-

card at the end of the file name.

If you try to load a file that is not in the current tape image or if a

DOS load error occurs, "ERROR 6" (the infamous checksum error) will be

generated.

-Press <R> to rename the current Atom file.

-Press <I> to import an Atom file. In the right hand area all the importable

files in the current directory will be shown. You can select one by

pressing <Enter> and it will then be inserted in the tape image at the

current cursor position.

-Press <E> to export the current file. An MS-DOS name will generated by the

emulator. The file cannot be exported if there already is a file with the

same name.

-Press <D> to delete the current file. You will be prompted for confir-

mation.

-Press <F> to freeze the current state of the emulator.

-Press <T> or the <Cursor Left> key to move the cursor back to left hand

side of the screen, in which the tape images are shown.

4.2 OPTIONS

~~~~~~~~~~~

The Options screen allows you to change the emulator settings and to save

them to the .INI-file. The changeable settings are:

-CPU speed    : The emulator can run at two speeds, namely

� Original : The emulator will try to be as close as possible

to the original 6502 operating speed of 1 MHz.

� Maximum  : The emulator will run at maximum speed.

You can also press <F5> during emulation to toggle between

these two CPU speed modes. On BREAK the CPU speed is reset

to original.

-ASM layout   : The assembly listings in the built-in monitor can be

displayed in two different ways, either using the Atom

notation (@ for immediate, # for hexadecimal numbers) or the

regular notation (# for immediate, $ for hex. numbers).

-Colors       : You can set all seven colors that are used during emulation.

� BackGr is the backgroundcolor.

� ForeGr is the foregroundcolor.

� GreySb is the color of the "grey symbols" (C0h-FFh).

� Color0-Color3 are the colors in HIRES color modes.

Each color is a six digit (3 bytes) hexadecimal number; the

first byte stands for the amount of RED, the second for the

amount of GREEN and the third for the amount of BLUE. The

range for each of these bytes is 00h to 3Fh.

-Sound output : You can select whether or not you wish to have sound effects

outputted to the PC-speaker or through the SoundBlaster.

-Save options : This will save the preferences to the .INI-file. The previous

.INI-file will be overwritten.

4.3 MONITOR

~~~~~~~~~~~

The built-in monitor lets you watch the programs when running. In the

monitor screen the following things are shown:

-6502 registers (top right)

-Assembly code (top left)

-Memory dump (bottom)

Move around with cursor keys, move faster with Page Up, Page Down keys.

The actions available are:

-Trace    : Executes one instruction and moves the assembly code display.

Screen updates are not turned off, so sometimes you might see

a pixel or two appearing between the monitor-information.

-Step     : Steps over the instruction, skipping the assembly code display

of subroutines (JSR). In case of a subroutine, it is executed

in debug mode. You can interrupt debug mode by pressing <F1>.

-Goto     : Prompts for a new address and jumps to it.

-Here     : Sets a breakpoint at the position of code display and runs

emulation in debug mode until CPU reaches the instruction at

the breakpoint or the <F1> key is pressed.

-Mode     : Switches focus between code and dump.

-Where    : Moves the assembly code display to the position of the Program

Counter (PC), where the emulation was interrupted.

When the emulator is running in debug mode, the Break key (F12) doesn't

work. Press F1 to quit the debug mode.

The dump-values of various I/O-ports sometimes do not reflect their actual

value.

5. ATOM MEMORY

--------------

5.1 ROM

~~~~~~~

The emulated Acorn Atom has 16 kB of Read Only Memory:

-Assembler and Casette Operating System at F000h (4 kB)

-Extension ROM (Floating Point) at D000h (4 kB)

-Basic interpreter at C000h (4 kB)

-AXR1 at A000h (4 kB)

All the basic Atom function are supported, as well as floating point

operations. The casette load & save routines have been replaced by fast

ROM-trapped routines. However, the Atom ROM has not been changed.

The AXR1 is an EPROM developed by Stichting Electronica Workshop (ECD/SEW),

Delft in 1981 to 1983 and provides the Atom with nineteen extra commands.

Because it is not a standard Acorn ROM, the extra commands are briefly

described below:

Command  Abbr. Description

-------  ----- -----------------------------------------------------------

DATA     DA.   A sequential one line file marker

Syntax: [<ARGUMENT0> ... <ARGUMENTn>]

RESTORE  RES.  Resets the file pointer to a specified DATA line

Syntax: [<LINENUMBER>|<EXPRESSION>|<LABEL>]

READ     REA.  Get records from a DATA line

Syntax: <VARIABLE0>[,<VARIABLE1> ... <VARIABLEn>]

FIND     -     Locates specified strings in a BASIC program

Syntax: <LITTERAL STRING>|<STRING VARIABLE>

RENUM    REN.  Renumbers a BASIC program and adjusts GOTO's etc.

Syntax: [<START LINE>,<INCREMENT>]

FCOS     FC.   Fast 1200 bps COS with visible tape transfer indication

SCOS     SC.   Same as FCOS, but with 300 bps

XDUMP    X.    Gives ASCII/hexadecimal memory dumps

Syntax: <START ADDRESS>[,<END ADDRESS>]

HDUMP    H.    Gives hexadecimal memory dumps

Syntax: <START ADDRESS>[,<END ADDRESS>]

DISAS    DI.   A fast 6502 machine code disassembler

Syntax: <START ADDRESS>[,<END ADDRESS>]

COPY     COP.  Copies memory blocks to a specified destination

Syntax: <START ADDRESS>,<END ADDRESS>,<DESTINATION ADDRESS>

RELOC    REL.  Relocates 6502 machine code to a specified destination

Syntax: <START ADDRESS>,<END ADDRESS>,<DESTINATION ADDRESS>

STEP     STE.  A versatile 6502 machine code debugger

Syntax: <START ADDRESS>[,<DISPLAY ADDRESS>]

PLAY     PL.   Provides a versatile method of sound output

Syntax: <NOTE0>[,<NOTE1> ... <NOTEn>]

KEY      K.    Performs a single keyboard scan

Syntax: <VARIABLE>

ON ERR   O.    Redefines error handling

Syntax: <BASIC STATEMENT>

SHAPE    SHA.  Draws anything you want on the screen in any mode

GRMOD    GR.   Allows HIRES graphics to be mixed with normal characters

TXMOD    T.    Disables GRMOD and returns control to the 6847

FCOS and SCOS have no effect in the emulator, because the tape load and

save routines are ROM-trapped. Most of the AXR1 commands cannot be traced

with the STEP command because STEP and the traced AXR1 commands may both

use the same zero page locations. From within the built-in monitor

(see chapter 4.3), however, everything can be stepped through.

All the AXR1 commands are shown in the AXR1 demo-tape, which can be loaded

and run in the emulator.

5.2 RAM

~~~~~~~

The emulated Acorn Atom has 29 kB of Random Access Memory:

-Zero Block RAM at 0000h (1 kB)

-Text RAM and floating point variable space at 2800h (22 kB)

-Graphics RAM at 8000h (6 kB)

5.3 I/O-DEVICES

~~~~~~~~~~~~~~~

There are two I/O-devices implemented in the emulator. However, both of them

are not fully implented, because not all of their functions are required in

the emulator. A description is listed below:

-8255 Programmable Peripheral Interface Adapter (PPIA)

� Port A at B000h: Output bits:     Function:

0 - 3       Keyboard row

4 - 7       Graphics mode

� Port B at B001h: Input bits:      Function:

0 - 5       Keyboard column

6         CTRL key (low when pressed)

7         SHIFT keys (low when pressed)

� Port C at B002h: Output bits:     Function:

0         Tape output (not implemented)

1         Enable 2.4 kHz to casette output (n.i.)

2         Loudspeaker

3         Intensity

Input bits:      Function:

4         2.4 kHz input (not implemented)

5         Casette input (not implemented)

6         REPT key (low when pressed)

7         60 Hz sync signal (low during flyback)

Note:

� Port C, bit 3 actually selects between the two possible sets of four

colors. However, I have chosen to use only one colorset, which can be

altered freely as described in chapter 4.2, and to use Port C, bit 3

as an intensity selector for the "ForeGr" and "Color0" colors, because

that's how I've always seen it on my real Atom with its green monitor!

If this bit is set (1), the amounts of red, green and blue of the

"ForeGr" and "Color0" colors are multiplied by 1.5.

-6522 Versatile Interface Adapter (VIA)

Register:                 Address:   Name:

� Data Register B              B800h       DB (partially implemented)

� Data Register A              B801h       DA (not implemented)

� Data Direction Register B    B802h     DDRB (partially implemented)

� Data Direction Register A    B803h     DDRA (not implemented)

� Timer 1 low counter, latch   B804h     T1CL

� Timer 1 high counter         B805h     T1CH

� Timer 1 low latch            B806h     T1LL

� Timer 1 high latch           B807h     T1LH

� Timer 2 low counter, latch   B808h     T2CL

� Timer 2 high counter         B809h     T2CH

� Shift Register               B80Ah       SR (not implemented)

� Auxiliary Control Register   B80Bh      ACR (partially implemented)

� Peripheral Control Register  B80Ch      PCR (not implemented)

� Interrupt Flag Register      B80Dh      IFR (partially implemented)

� Interrupt Enable Register    B80Eh      IER (partially implemented)

� Data Register A              B80Fh       DA (reads/writes to B801h)

Notes:

� DB and DDRB partially implemented means that only the polarity signal

on PB7, generated by Timer 1, is implemented.

� Timer 2 pulse mode not implemented (DA not implemented). Timer 2 only

runs in one shot mode and stands still in pulse mode.

� ACR only bits 7 - 5 implemented.

� IFR only bits 7 - 5 implemented.

� IER only bits 7 - 5 implemented.

In short: The emulator fully supports both independent 16-bit timers and

interrupts generated by these timers.

6. THE ATOMTAPE UTILITY

-----------------------

The ATOMTAPE.EXE utility enables you to transfer files from an original

Acorn Atom or from tape to the PC. However, a soundcard is necessary,

because the ATOMTAPE.EXE utility analyzes VOC-files.

You must connect your Atom or tape recorder to the "Line In" connector

on your soundcard, using line 2 as ground and line 1 as output signal.

(Line 3 is the input signal to the Atom from the

���������ͻ        recorder). See adjacent diagram.

�ͼ    �    �ͻ

�  �   2   �  �      Start a VOC/WAV maker on your PC and record from the

�             �      "Line In" connector.

� �1       3� �      The VOC file must be 8-bits, mono and recorded at

�             �      22050 Hz. The VOC-file may consist of only one

�ͻ   �Ŀ   �ͼ      datablock (type 1). This block may or may not be

���; ���ͼ        preceded by one blocktype 8.

Back view of DIN plug   Press "Play" on the tape-recorder or save a file on

i.e. Cable Side      your original Atom at either 300 or 1200 bps (The

latter according to the AXR1 1200 bps protocol).

The program can only evaluate named files; which are files, containing a

control block / header and a data block of 257 bytes (the last being a

checksum byte), the next control block, the next data block, etc.

About the tape speeds:

~~~~~~~~~~~~~~~~~~~~~~

Every byte on tape is surrounded by a startbit (0) and a stopbit (1).

In between there are the 8 bits of the databyte. In 300 bps mode every

cleared bit (0) consists of four 1.2 kHz waves and every set bit (1)

consists of eigth 2.4 kHz waves. In 1200 bps mode every cleared bit (0)

consists of one 1.2 kHz wave and every set bit (1) consists of two 2.4 kHz

waves. The stopbit (1) always has one extra 2.4 kHz wave.

After the VOC file has been recorded, start the ATOMTAPE.EXE utility.

Enter the VOC filename and whether it has been recorded at 300 bps or

1200 bps. The utility then checks whether the VOC file is of the right

format (8-bits, mono, 22050 Hz) and starts analyzing the data. If all goes

well the subsequent control blocks (headers) of the file are shown and

the data blocks are loaded.

If no errors occur, the file will be saved to the same name as the VOC

file, but with the .ATM extension.

The format for the .ATM files is as follows:

Offset Size     Description

------ -------- -----------------------------------------------------------

0000h  16 BYTEs Atom filename (if less than 16 BYTEs, rest is 00h bytes)

0010h  WORD     Start address for load

0012h  WORD     Execution address

0014h  WORD     Size of data in BYTEs

0016h  Size     Data

7. COMPATIBILITY

----------------

CPU

~~~

-6502 instruction set (flags not 100% compatible in decimal mode)

-6502 illegal opcodes

VIDEO

~~~~~

-all graphic modes

SOUND

~~~~~

-supports sound through PC speaker and SoundBlaster or compatible

I/O PORTS

~~~~~~~~~

-8255 PPIA keyboard/graphics ports

-6522 VIA timers/timer interrupts

COS

~~~

-ROM trapped LOAD and SAVE routines

-tape image files with directory

-ATOMTAPE.EXE utility reads original tapes from .VOC-files

8. FUTURE EXPANSIONS

--------------------

I have planned to add the following features to the emulator in the near

future:

-Printer communication in VIA,

-Extended monitor,

-Any ideas from other people ?!?

9. PROBLEM SOLVING

------------------

-Every time the emulator is executed, it will determine the speed of your

CPU. If this is not done correctly, you must set the value of "SpeedIndex"

in the .INI-file manually. It should be about the value of the CPU speed

of your PC (in MHz) for 486/Pentium machines.

Examples: On an AMD 486 DX-40 MHz, it should be approx. 40,

On an Intel 486 DX2-66 MHz, it should be approx. 66,

On a Pentium 120 MHz, it should be approx. 120.

On a Pentium 233 MHz, it should be approx. 233.

Note: The higher your CPU, speed the larger the absolute deviation. I've run

the emulator on a P233, and it was about 3% too fast. If you find that

anoying, set the value of SpeedIndex manually in the .INI-file.

You might want to test the emulation speed by running the following program

in the emulator:

10F.I=1TO40

20PL.C1

30N.

40E.

This program plays the C1 note 40 times. It should last about 29.5 seconds.

If it lasts shorter, set SpeedIndex to a higher number. If it lasts longer,

set SpeedIndex to a lower number.

Note: The values mentioned above, apply to the emulator run under an MS-DOS

environment. If run under Windows, the emulator runs a little bit slower.

ATOM MUSIC from the AXR1 demo-tape lasts 413 seconds (6 mins, 53 secs) on a

real Atom.

-If you would like to use another extension ROM than the AXR1 at memory

address A000h, you can specify its filename in the .INI-file as follows:

AltExtROM=filename.ext

When you startup the emulator, the relevant part of the file will be

loaded at memory address A000h, but only if its length is exactly 4118

bytes.

You can retrieve the data of your alternative extension ROM from your

real Atom, by connecting the Atom to your PC, as described in chapter 6

and saving from the Atom into a .VOC-file:

*SAVE"A000"A000 B000

and then saving that as an .ATM-file by using the ATOMTAPE.EXE utility.

You may give the file any name & extension you like, as long as the

file's format is the same as the format of an .ATM-file.

-If you find that sometimes a few keys don't work (Shift, Alt, Ctrl etc.),

remember that you can toggle between the standard and the alternate

keyboard during emulation by pressing the <F4> key. You may have switched

to the alternate keyboard especially for a game, and forgotten to switch

back to the standard keyboard. On BREAK (F12) the standard keyboard is

selected automatically.

10. REFERENCES

-------------

The following sources have been used during the construction of the

emulator:

-Johnson-Davies, David: "Atomic theory and practice", second ed. 1980,

Acorn Computers Ltd.

-"Atom Technical Manual", 2nd issue, october 1980, Acorn Computers Ltd.

-Greenley, Larry et al.: "Commodore 128 Programmer's Reference Guide",

february 1986, Bantam Books.

-Lincklaen Arriens, H.J. and Horsmeier, J.A.: "Atom Extension ROM (AXR1)

Instructions Manual", second edition 1983, ECD/SEW Stichting Elektronica

Workshop.

-Vardy, Adam: "Extra instructions of the 65xx series CPU", text document

of 27 september 1996, downloaded from "http://www.funet.fi/".

-Kontros, Frank: "R6522 Versatile Interface Adapter", text document

downloaded from "http://www.funet.fi/".

-Brown, Ralf: "Interrupt List Release 54", "file PORTS.LST", june 1997.

11. CONTACTS

------------

If you would like to contact me with questions, remarks, comments etc.,

feel free to do so. In fact, I would very much appreciate some feedback!

If you have more Atom programs that did not come with the emulator, please

send them to me and I will put them on my page with the emulator, so that

all Atom fans (including myself) can enjoy them. I'd love to see what more

Atom stuff is still out there!

I prefer e-mail (In case of any attachments, please use MIME or BinHex

encoding) to either one or both of these addresses:

ras@dutccis.ct.tudelft.nl

theeroos@globalxs.nl

You can also write to me the old-fashioned way:

Wouter Ras

Nieuwe Schoolstraat 48

2613 SJ  Delft

The Netherlands