Print controller

As  we  all  know,  printer  control codes are rather confusing and rather

cumbersome  to  sure;  some word processors (eg. View) go some way to

help you by using a  printer driver, but it is usually not a very

user-friendly method ! (eg. 2 highlights  for  bold  or suchlike). What

is required is a simple  to use yet powerfull program; this should  let

you  control  yourprinter by  using plain English commands, & should be

adjustable by you touse any printer  codes  or  command names.

Interested ? (Hopefully!) - Then read on !

The program lets you change fonts and styles by getting you to send to

the printer the font or style required,  eg.  by placing the command

[BOLD ON] before a heading in Wordwise, and (of course)  [BOLD  OFF]

after  it.  To enter the command, all you have to do is quite literally

type "[BOLD ON]" into  your  document;  when the document is printed,

the command [BOLD ON] will appear on the  screen,  but instead of coming

out on the printer, the control codes (to turn on the  bold  style)  are

sent  instead. This is far easier to show with an example :

In the Document: normal type [BOLD ON]bold type[BOLD OFF] normal type

On the screen  : normal type [BOLD ON]bold type[BOLD OFF] normal type

On the printer : normal type [EMPHAS ON]bold type[EMPHAS OFF] normal type

The Program

One advantage of the approach that  I have used in  writing the program

is that : The program will work in ANY ENVIRONMENT THAT USES THE

PRINTER !!! This may not seem immediately apparent, but this means that

you could send the message "[BOLD ON]" to the printer  from  BASIC, & the

bold style will be selected ... This is possible by the program

re-directing the INSV (INSert into buffer) vector to itself. It keeps

reading in data, until  it  meets  a  character that  is  bound  for the

printer; if the character is an "[", then it will check all incoming

characters  to  see  if  they  form  a  known  command. Otherwise,  the

characters are just sent to printer as if the routine was not even there.

If  it  meets  an  "["  &  it  isn't  a command, then the characters  are

sent  straight out to the printer (unadulterated). If, on the other hand,

the  characters  form  a  know  commands, then the routine sends  out the

appropriate control characters to the printer,  instead  of the command.

The commands  are  all  listed  at  the  end of the program; the "Start

Of Command" & "End Of Command" characters, the start address for the

machine code, maximum length of coammnds, etc. are all defined at the

start of the program. The program is fairly well documented, so  if  you

need to change anything (eg. "[" "]" to "{" "}" in case of  any  clashes

with some other software,  or  preference  !)  just  look  at the source

code & all should become clear (I hope !).

Usual disclaimer : I make no responsibility for any malfunction,  damage,

nuclear explosion,etc. that this program may (directly or indirectly)

cause (I'm a poor stude, so no claims for money will be accepted).

This  code  is put into the Public Domain in good faith; if any errors

occur, then please contact  me,  or  if  you  have any ideas

(or solutions to errors !). This program may be copied in  whole  free

of  charge,  as  long  as I am properly credited (or bribed).

Let me know (via email) what you think ! Long live the beeb - let's see

some more PD for it ! (Ok, so I  own an A440 as well, but I might as well

still use it). Has anyone got any routines for connecting up a beeb to

an  archi  ?  I'm interested in a routine that lets the archi use the

beeb's peripherals; an Opus  Challenger  has a nice 5.25" disc drive, but

is not worth ripping to pieces to connect to an archi; any ideas for

using the drive via the archi? (eg. for  an  IBM disc reader ...). How

about using the printer I've got connected the beeb as well as the one

I've got linked to the arc - a choieof printers, depending on quality

/ speed. (Why only 1 parallel, 1 serial & 4 expansion slots Acorn ???

PC's  have  (eg)  2 serials, 1 parallel & 8expansion slots; I'm

listening ... )

Written by Ian "the Grim" Grimstead;

JANET email: ijg@uk.ac.cf.cm (ijg@uk.ac.cardiff.computing-maths)