IDEAcomm 5251/AH v1.21

IDEAH & Windows:

----------------

Version 1.21 of IDEAH.EXE supports IRQs. Earlier versions of this software

did not. If IDEAH is to execute correctly with Microsoft Windows, it must

be executed with the IRQ command line switch documented below. If the IRQ

switch is not used, IDEAH does not use IRQs. On PS2 style PCs, the IRQ

can be selected when booted from the system reference diskette supplied

with that PC. The @6060.ADF file from the IDEAcomm setup diskette must

first be copied to the system reference diskette, as described below.

Command Line Switches:

----------------------

This release of the IDEA Adapter Handler software includes support of IRQ's

to make it compatable with Microsoft Windows. To run with IRQ's, the

appropriate switch settings must be set on the board and IDEAH then

executed with the following command line switch.

IDEAH IRQ # (where # = 2, 3, 5, or 7)

IDEAH may now be removed from memory using the command line switch

U or UNINSTALL.

IDEAH U     (if IDEAH is installed, it will be removed)

Micro Code files & 4 Lu Cards:

------------------------------

Version 1.11 contained the files IDEAH6.MC and TWINAX4.UCM in the

sub directory 4LU. These files do no exist in version 1.21. TWINAX7.UCM

is now used for both the 4 Lu and 7 Lu cards. The file IDEAH6.MC

is now named IDEAH.MC and is still found in the sub directory 4LU. This

is the 6Mhz version of the micro code.

Enhancements to the SETUP program:

----------------------------------

The SETUP program has been enhanced to prompt for a 7 Lu or 4 Lu card

type and 6Mhz or 8Mhz micro code. If 6Mhz micro code is selected, IDEAH.MC

is copied from the setup diskette sub directory 4LU. If 8Mhz is chosen,

IDEAH.MC is taken from the root directory of the setup diskette. Choosing

the 4 Lu or 7 Lu installation copies the same TWINAX7.UCM file from the

setup diskette to the destination directory. The file is modified to

inform IDEAcomm 5251 Plus Emulation of which card type was installed.

IDEAH itself ignores this information.

PS2 Configuration File @6060.ADF:

---------------------------------

If a micro channel PS2 card is being installed, in addition to running SETUP

to install IDEAcomm software, the PC must be booted from the System Reference

Diskette supplied by the computer manufacturer and the desired memory, IO

port, and IRQ settings selected for the IDEAcomm card. The setup diskette

contains two copies of the file @6060.ADF. One of these @6060.ADF files must

be copied from the setup diskette onto the system reference diskette. The

configuration program on the system reference diskette will then use this

file to configure the IDEAcomm 5251/Plus card.

If a 7 Lu card is being installed, the copy of @6060.ADF that is in the

root directory of the setup diskette should be copied to the system

reference diskette. If a 4 Lu card is being installed, copy @6060.ADF

from the sub directory 4LU on the setup diskette.

The version of @6060.ADF in the root directory contains the correct

PS2 POS register bit settings to configure the IDEAcomm card for 32K

of memory while the version 4LU\@6060.ADF will configure it for 16K

of memory.