#

# CLREGS User's Manual

#		 				Version 1.00a1

05-05-1992

Chi-Jen Huang

Command line option

-------------------

CLREGS ,VGA Register Editor Engine, Version 1.00�1

Copyright (C) Cirrus Logic, 1992. All right reserved.

Usage: CLREGS option

where option can be

-U	: uninstall previously-loaded copy.

or  [-L] [-V] [-A] [-E] [-K=[CS[A-Z]|AS[A-Z]] [COM[1|2]] [BAUD]

here

-L	: terminate and stay resident (TSR).

-V 	: run as a standard VGA.

-A 	: activate 132-column mode.

-E 	: use EMS memory.

-K	: specify user-defined hot key.

COM?	: output will be directed to serial port.

BAUD	: can be 300, 1200, 2400, 9600, 19200 or 38400.

or  -H or -?	: display this help message.

[-V] : To run as a standard VGA CLREGS, standard menus appeared are

as follow:

Vga  - contains all the standard VGA registers.

Stat - contains chip/BIOS/CRTC status.

Rmdc - RAMDAC menu

Info - contains Video BIOS data area and related vectors.

Peek - a mini debugger (all available commands listed later).

eXit - exit CLREGS to DOS.

[-K] : To specify user-defined hot key to override default <plus> or

<shift-shift> key.

CS[A-Z] stands for Control-Shift A to Control-Shift Z.

AS[A-Z] stands for Alt-Shift A to Alt-Shift Z.

A mini debugger: 'Peek' menu

----------------------------

Use '?' to display valid commands:

? - display this messgae

a - toggle 132-column mode

c - compare (use m first): user should issue 'm' to memorize a

copy of register data and later user 'c' to compare. CLREGS

will display delta list either on the screen or a file if

specified.

d <address> - dump memory.

For example: d c000:0

e[type] <address> <list>- enter data to memory location.

For example:

eb 40:49 04 - enter a byte data 04 to location 40:49

ew 40:50 1234 5678 - enter 2 words to location 40:50 and

40:52.

ed 0:40 f000:f065 - change video interrupt vector to

point to F000:f065.

f <range> <list> - fill memory location with data.

For example:

f a000:0 100 55 aa

or

f a000:0 l4000 ff

h - display macro command as follow:

de - wait DE

vsync - wait VSYNC

# comment line

i <io_port>

o <io_port> <value>

o <io_port> <and_mask> <or_mask>

o <io_port> <index> <and_mask> <or_mask>

i <port> - input from port.

k <cs[a-z]|as[a-z]> - hot key (same as command line option [-K]).

m - memorize current register data.

o <port> <data> - output to port (both byte and word supported).

For example:

o 3ce 80

or

0 3ce 8080

q - quit and back to menu mode.

r <reg>=<value> - set CPU register and ready to issue video

interrupt. This command is useful to test Cirrus Logic Video

external function. For example

The follow command are ready to enable SimulSCAN:

r ax=1202

r bx=92

You should issue 'v' to execute an interrupt 10H NOW.

s <file> - save video register to a file.

v - issue an video interrupt.

x <macro file> - execute a macro file. This command allows user

execute a series of action specified in a macro file. See

'CLREGS macro language syntax' section for details.

CLREGS macro language syntax

----------------------------

Supported commands:

# comment line

- any line with leading character '#' is treated as a

comment line. No action will be performed.

eg.

# this is comment line

i <io_port>

- input from io_port.

eg.

i 3da

o <io_port> <value>

- output value to io_port

- value can be 8 bit or 16 bit

- for 16 bit value, it contains data and index and has to be

EXACTLY 4 digits

eg.

8 bit	- o 3c2 12

16 bit	- o 3ce 80a1

o <io_port> <index> <and_mask> <or_mask>

- input current data from index, ANDed with and_mask, ORed with

or_mask and then write it back to index/data port.

eg.

o 3c4 c2 fc 01

o <io_port> <and_mask> <or_mask>

- input current from io_port, ANDed with and_mask, ORed with

or_mask and then write it back to io_port.

eg.

o 3cc 4f 80

vsync

- wait for VSYNC

eg.	wait VSYNC before programming 3c4/c2

vsync

o 3c4 03c2

de

- wait for Display Enable

eg.	wait DE before programming 3c4/c2

de

o 3c4 03c2

A completed example:

File name: <myfile.tst>

#

# set overscan to be red color

#

i 3da

o 3c0 11

o 3c0 4

o 3c0 20

#

# set cursor type to be full block

#

o 3d4 010a

o 3d4 0f0b

#

# turn on GD610/620 RevC expanded graphics: erc2[1]

# - do it during vertical retrace

#

vsync

o 3c4 c2 fd 02

#end

How CLREGS works

----------------

CLREGS has build-in standard VGA register definition and handles

chip-dependent register help message by reading external help

binary file during run-time. The chip identification routine is

part of the help file and will be executed by CLREGS to identify

what kind of VGA controller that we are currently running at. This

mechanism provides handy support to any new controller by giving

identification code and register defintion in the hlep file.

Depending on the number of register of the controller, there will

be some additional menu appears before 'eXit' menu. They are usually

named as 'Ext1' and 'exT2'.

There is the case that no identification code is available to be able

to check the existence of a certain controller. Take AVGA2 for

example, there is no register that user can read back an chip-id to

identify it. CLREGS will read an initialization file, called

CLREGS.INI, to check if user intends to run CLREGS as a particular

unable-to-identify chip. For example:

CLREGS.INI:

GD6420=N

GD5410=N

GD5426=Y

GD5424=N

GD5422=N

GD5420=N

GD5402=N

GD6205=N

will tell CLREGS to run as AVGA2. In some situation, CLREGS will

identify currently running chip as A, but user might force it to run

as B for some reason. Of course, user is always allowed to use [-V]

in command line option to run as standard VGA CLREGS. This is useful

while examining competitor's controller.

Miscellaneous enhancements

--------------------------

. At 132-column mode, the help message will be displayed at same time

as register menu.

. Type capital letter of the menu name to bring up that menu directly.

. While editing a register, F2 key can be used to increment register

data by 1, whereas F3 key to decrement register data by 1. This

feature is useful in observing some of the panel effect.

. The size of resident portion of program is reduced.