������������������������������������������������������������������������Ŀ

�                                                                        �

�   CLPANEL  <--->  A Flat Panel Utility Program                         �

�                                                                        �

�   This program is designed as an aid to the task of LCD Flat Panel     �

�   optimization. Optimization, defined as minute adjustments made to    �

�   better image quality and or performance.                             �

�                                                                        �

�   This program only affects Extended Registers that have to do with    �

�   the LCD Flat Panel operation. For other extended register functions  �

�   CLREGS would be used.                                                �

�                                                                        �

�                                                                        �

�   Last Date Modified:  09-17-93                                        �

�   Copyright Cirrus Logic Inc.     All Rights Reserved                  �

��������������������������������������������������������������������������

There are two version of CLPANEL. The file names are: CLPANEL2 and

CLPANEL4. CLPANEL2 will operate with any member of the CL-GD62xx family.

CLPANEL4 will operate only with the CL-GD6440. The operation of the two

programs are identical with the exception that there are two pages of

optimization registers for the GD6440. The GD6410/12/20 products are not

supported by CLPANEL.

The equipment requirements to make use of CLPANEL are as follows:

CLPANEL -------------------- the program

A COMM port ---------------- COMM1, COMM2,3 or 4 or whatever

A NULL modem cable --------- this cable can be only three wires,

at each end: wire pin 2 to 3

and one ground pin (6)

Also a standard LAPLINK serial cable

can be used, since it is a NULL

modem cable.

A DUMB Terminal or

another computer running

communications software

set to ANSI mode ----------- ANSI compatible terminal calls have been

used throughout the program to make it

easier to find somthing to connect to.

A WYSE terminal was used to develop the

program, and another computer running

PROCOMM + was tested as well.

The two CLPANEL files are archived using the "ZIP" form and PKZIP 1.04G.

PKZIP can be obtained from most bulletin boards including the Cirrus

Logic bulletin board.

In each ZIP file is the executable program, CLPANEL2 or CLPANEL4, and

a series of Microsoft Windows compatible BMP image files. The BMP files

are identical in both ZIP files. The BMP image file format was chosen

because most users are interested in optimizing panels for Windows.

NOTE:

To generate your own Windows BMP image, do the following:

While running Windows, and adjusting the Windows desktop to

where you want to capture it, press the PRINT SCRN key. (Not

SHIFT PRINT SCRN like under DOS). Everything you see on the

screen will be captured onto the Windows CLIPBOARD. Open Windows

PAINT, (or any other image program that outputs the BMP format),

and under the EDIT menu, select PASTE. The captured desktop

image is now in the paint program and can be saved as a BMP

file through the FILE SAVE selection.

The syntax for executing CLPANEL is:

CLPANEL2 comx

or

CLPANEL4 comx

The "comx" is the only argument to CLPANEL and should be the COMM port

that is connected to the terminal.

The first screen shown is a menu, operated with the up and down arrow

keys and ENTER, that allows you to select these options:

-----------------------------------------------------------------------

CLPANEL2 Version x.xx <-> An LCD Flat Panel Utility Program

Move to menu selection with cursor keys

Press ENTER to select

Flat Panel Applications - Cirrus Logic Inc. 1993

���������������������������������Ŀ

� Quit                            �

� Load a 4- or 8-bit BMP Image    �

� Load a Mode 3h TEXT test Screen �

� Optimize LCD Panel              �

� Switch to CRT                   �

� Switch to LCD                   �

� Switch to SimulScan             �

� Help                            �

�����������������������������������

Quit -------------------------------------- Return to DOS

Load a 4- or 8-bit BMP image -------------- a sub-menu follows that

is explained below

Load a Mode 3h TEXT test Screen ----------- another sub-menu explained

below

Optimize LCD Panel ------------------------ This selection will load

the register editor to

the terminal if it is not

desired to load an image

first

Switch to CRT ----------------------------- assuming LCD mode, this

option switches the VGA

controller output to CRT

Switch to LCD ----------------------------- assuming CRT mode, this

option switches the VGA

controller output to LCD

Switch to SimulScan ----------------------- this option selects both

CRT and LCD as the video

output device

Help -------------------------------------- this is one page of help

listing key assignments

and syntax - a terse

version of this document

Load BMP image Menu ........

����������������������������������Ŀ

� Vertical Color Bars - Mode 12h   �

� Horizontal Color Bars - Mode 12h �

� Dots - Mode 12h                  �

� CrossHatch - Mode 12h            �

� Text Pattern - Mode 12h          �

� Vertical Lines - Mode 12h        �

� Horizontal Lines - Mode 12h      �

� 1 Pixel Text - Mode 12h          �

� 1 Pixel Text Reversed _ mode 12h �

� RED Screen - Mode 12h            �

� GREEN Screen - Mode 12h          �

� BLUE Screen - Mode 12h           �

� Solitare - Mode 12h              �

� Solitare - 640x480x256           �

� Windows Desktop -  320x200x256   �

� Windows Desktop -  Mode 12h      �

� Windows Desktop -  640x480x256   �

� Windows Desktop - 1024x768x256   �

� Enter Filename.BMP               �

������������������������������������

The above test screens were chosen because of the different type of

issues that arise on some panels when these images are viewed. You,

the user, may have a favorite image of your own. In this case choose

the last menu option, "Enter Filename.BMP". This will bring up a full

screen menu which is a directory listing of the current directory, and

allow you to type in a filename.

There are no restrictions to the size and color depth of the image

chosen, except for the limitations of the VGA controller used. For

instance: a GD6225 equiped with 256k of video memory could not load

a 640x480x256 image. (this image would require 307k of video memory)

If an image is chosen that is beyond the controllers capability,

CLPANEL will just refuse to load it.

Two keys are active while viewing an image, they are:

ESCAPE ------------------ returns to the main menu

SPACE  ------------------ load the register editor thru

the selected COMM port. If you

forgot to add the COMM port

argument when CLPANEL was executed,

the program will tell you that.

Load a TEXT Screen ......

�����������������������������������Ŀ

� Cross Hatch - Mode 3              �

� Cross Hatch w/numbers - Mode 3    �

� Plain Double Line Border - Mode 3 �

� Numeric Charaters - Mode 3        �

� Enter Filename.TXT                �

�������������������������������������

The TEXT test menu works like the BMP menu. These are all VGA text mode

3 screen captures. They are different that the graphics images in that

they are better for trying to optimize flicker and crosstalk. When a

selection is made from this menu, some text appears that advises you

to press a key 0-f. Any of these keys will finish loading the test

screen. Keys 0-f coorespond to the 16 colors available in text mode 3.

Each time you press 0-f the color text in the test screen will change

to color 0-16. Since 0 is BLACK, press 0 twice and the screen video

will reverse. When viewing these screens, as in the BMP images, ESCAPE

returns you to the main menu and SPACE load the register editor thru

the comm port.

THE REGISTER EDITOR:

The difference between the register editor for the GD62xx family and the

GD6440 is the quantity of registers that are programmable. The GD62xx

screen is one page and separated with lines by function. The GD6440 has

two screens of registers that can be toggled with the PGUP and PGDOWN

keys.

The register editor differs from CLREGS in that all parameters are

entered as actual values instead of register level values.

Example:

To change Vertical Total with CLREGS you would have to have the data

book to know that VTOT is a 10-bit value and is stored 8-bits in one

register and the other 2-bits as bits 0 and 5 in another register.

With CLPANEL you just select the key that cooresponds to Vertical

Total as shown on the screen, and enter a new value. (i.e. value is

800, select 'c', the bottom screen prompt changes to "Enter Value",

and you type in 900, or whatever, followed by RETURN.

All parameters are treated this way. When selecting a new clock value,

you have to type in the entire 6-digit value, like 25.000, before

pressing RETURN. Some selections are binary in nature, in that the

function is either ON or OFF, therefore they are listed that way.

Notice that when a key selection is pressed, additional information

about the selection choices is made available on the last line of the

display. One exception to that rule is when selecting panel types with

the GD6440. There are so many of them that they are listed on the

right hand side of the screen. When that key selection is chosen, type

in the number from the list on the right.

One very imporatant feature of CLPANEL is the PANIC key. This key is the

* (asteriks) key.

Scenario:

While modifying register values you make a selection and the panel

appears to be melting, or something equaly bad. By pressing the

'*' key all registers are loaded with default values as they were

when CLPANEL was first executed. Unfortunately CLPANEL cannot

restore just the last register modified because the "melting"

scenario could have been caused by the change of more than one

parameter. If you feel that it was the last parameter modified,

you can elect to quickly type back in the original value. If this

doesn't help the panel, press the '*' key.

Three other keys are available to the user besides all the parameter

keys.

ESCAPE ------------------ brings you back to the main menu

END --------------------- Writes a text file called "REG_DUMP.TXT"

to the current directory. In this file

is a "delta" list of all parameters

modified in a session, the before and

after values, both high-level lables

like VTOT and register names like CR06

HOME -------------------- Writes a text file by the same name

to the current directory. With this

key all parameters and register values

are listed

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Be advised that this is a "living" program. By this term it is meant to

convey the fact that there could be modifications made at any time, that

would constitute a revision change, by Cirrus Logic. The Cirrus Logic

bulletin board will always have the latest version.