This is version 1.06 of the RVGNDIS driver, dated 25-May-96.     It is for

CPL    100VG/AnyLAN Ethernet cards using Windows/NT version 3.5 and up, or

using Windows 95.

Beginning with version 1.05, the following "defaults" will be used:

a)  Standard  PCI cards  will use the

adapter's "I-O" operational mode exclusively.

b)  "Bus-master" PCI cards  will use "bus-master" operational mode

as their default.  On PCI systems allowing slot configuration,

the user MUST insert a "bus-master" card in a PCI slot that is

configured as a "bus-master" slot, or the system will CRASH!!!

To run such cards  using "I-O" mode,  the user must access the

Windows/95 registry  and edit the adapter's "IOMODE" parameter

from "1" to "0".   This is UNRECOMMENDED except for diagnostic

work (adapter throughput is MUCH faster in "bus-master" mode).

c)  All 100VG cards will default to using "auto-select" speed.  To

force a card to use 10MB-only or 100VG-only mode, the user can

access the Windows/95 registry  and edit the adapter's "SPEED"

parameter  from "3" (auto-select)  to "2" (100VG-only)  or "1"

(10MB-only).   This is UNRECOMMENDED except for diagnostics.

Note that "shared-memory" mode has been deleted from the V1.05 driver.

Driver Installation on Windows 95:

1)  Install the desired CPL    adapter card in your computer.

2)  Boot Windows 95.

3)  If you are using a PCI adapter, the system will display the

"New Hardware Found" dialog.   For any other adapter, enter

the Network configuration dialog and select "Add Adapter".

4)  Select "Driver from disk provided by hardware manufacturer" and

click on "OK".

5)  Enter the path "A:\WIN95" and click on "OK".

6)  A dialog box showing the various CPL    100VG adapters will be

displayed.   Select your desired adapter.

7)  Click on "OK" to accept the selected adapter.

8)  You will be asked to insert the WIN95 CD  and provide a path.

At this time, the system will be updated.

9)  A final  "Systems Settings Change"  dialog will be displayed.

Restart the computer to establish a network connection.

Revision History:

V1.00

Original version.

V1.01

Converted to a "miniport" driver, for compatibility with new versions

of Windows/NT and for better performance.

V1.02

Added support for standard PCI cards.

V1.03

Corrected PCI card-scan, to avoid "conflicts" with any ISA cards that

may be present in the same system.

V1.04

Modified driver parameters and PCI-card logic, for compatibility with

Windows 95.

V1.05

Added "bus-master" PCI card support; deleted "shared-memory" support.

V1.06

Corrected driver to pass Windows/NT certification.