This is version 1.06 of the RVGNDIS driver, dated 25-May-96.     It is for

CPL    100VG/AnyLAN Ethernet cards using Windows/NT version 3.5 and up, or

using Windows 95.

Revision History:

V1.00

Original version.

V1.01

Converted to a "miniport" driver, for compatibility with new versions

of Windows/NT and for better performance.

V1.02

Added support for PCI cards.

V1.03

Corrected PCI card-scan, to avoid "conflicts" with any ISA cards that

may be present in the same system.

V1.04

Modified driver parameters and PCI-card logic, for compatibility with

Windows 95.

V1.05

Added PCI "bus-master" card support; deleted "shared-memory" support.

V1.06

Corrected driver to pass Windows/NT certification.

SPECIAL NOTES:

1)  Windows/NT V3.1, even using the special "NDIS.SYS" file (supplied with

later versions of NT), has "bugs" that prevent a driver from finding a

PCI card, unless the driver does the following special functions:

a) The driver must declare a PCI card to NT V3.1 as an "ISA card".

b) The driver must do its own scan of the I-O bus for PCI cards.

Even with the above, motherboards using PCI BIOS versions 2.1 or later

MAY not "enable" a PCI card's I-O addresses; thus a scan for PCI cards

might still find nothing.

Users of Windows/NT V3.1 wishing to run 100VG/AnyLAN cards (and who do

NOT have motherboards using PCI BIOS V2.1 and up) can use version 1.03

of the RVGNDIS driver, which may be ordered from CPL    Field Service.

Beginning in version 1.04, the RVGNDIS driver no longer does the above

special functions, for compatibility with Windows 95, and thus may NOT

be taken back into Windows/NT V3.1.

2)  Beginning with version 1.05, the following "defaults" will be used:

a)  IStandard PCI cards will use the adapter's "I-O"

operational mode exclusively.

b)  PCI "bus-master" cards  will use "bus-master" operational mode

by default.    On PCI systems allowing slot configuration, the

user MUST insert a PCI "bus-master" card into a PCI slot which

has been configured as a "bus-master" slot, or the system will

CRASH!!!   To force a PCI "bus-master" card to use "I-O" mode,

the user must access the Windows/NT registry file and edit the

"IOMODE" parameter from "1" to "0".    This is NOT RECOMMENDED

except for diagnostic work (adapter performance is MUCH faster

in "bus-master" mode).

c)  All 100VG cards will default to using "auto-select" speed.  To

force a card to use 10MB-only or 100VG-only mode, the user can

access the Windows/NT registry  and edit the adapter's "SPEED"

parameter  from "3" (auto-select)  to "2" (100VG-only)  or "1"

(10MB-only).   This is UNRECOMMENDED except for diagnostics.

Note that "shared-memory" mode has been deleted from the V1.05 driver.

Installation:

1) Login to Windows/NT using an adminstrator-capable user I.D.

2) Select the "Main" screen.

3) Select the "Control Panel" icon of the main screen.

4) Select the "Network" icon of the control panel.

5) Within the network-setup window, select "Add Adapter".

6) When installation asks for an adapter-type, select "Other" and insert

this distribution diskette into drive A:.

7) When installation asks for the diskette path, type in "a:\winnt".

8) Select "CPL    100VG/AnyLAN NDIS Driver".

9) A configuration panel will prompt for the base I-O address, IRQ level,

and network-address of the desired 100VG card.      Make the necessary

choices and then select "Continue".

10) The system will ask if you wish to reboot.  Select "Yes".

11) Reboot the Windows/NT system.