Microsoft LAN Manager V2.x Installation --

Follow the installation procedures in the LAN Manager Installation

Guide.   The "Importing Additional Network Device Drivers" section

describes the process of loading a driver.    The "Network Drivers

Dialog Box" section describes how to specify which driver to use.

Beginning in version 1.14, the RVGNDIS drivers will run all 100VG-

AnyLAN adapters in "I-O" mode only  ("shared-memory" mode has been

deleted, since it was confusing and did not increase performance).

Microsoft LAN Manager V1.0 Installation --

Install LAN Manager  as described in the LAN Manager  installation

guide, if you have not already done so.     Then, do the following

steps to install the 100VG-AnyLAN adapter drivers:

1.  Copy the appropriate driver, RVGNDIS.DOS or RVGNDIS.OS2,  from

the installation diskette  to the desired  hard-disk drive and

directory.     If installing under OS/2, also copy the message

file RVGNDIS.MSG to the hard-disk.

2.  Add the following line to the CONFIG.SYS file:

DEVICE=[drive:directory]RVGNDIS.DOS

or

DEVICE=[drive:directory]RVGNDIS.OS2

This line should be entered  immediately after the line which

installs the PROTMAN.DOS or PROTMAN.OS2 device driver.

3.  Add RVGNDIS_NIF to the bindings list  of the protocol in the

PROTOCOL.INI file.   An example is as follows:

[NETBEUI_XIF]

DriverName = NETBEUI$

Bindings = RVGNDIS_NIF

4.  Add the following lines at the end of the PROTOCOL.INI file:

[RVGNDIS_NIF]

DriverName = R100VG$

IOADDRESS = 0xNNNN

.  .  .

.  .  .

(Other parameters as required; see below)

.  .  .

.  .  .

NOTE:  The IOADDRESS parameter is REQUIRED on most CPUs; if unspecified,

a default ISA address of 0x300 will be used!

RVGNDIS Driver Parameters --

The PROTOCOL.INI file defines the configuration of protocol  and

NDIS drivers.     The following table summarizes all the RVGNDIS

parameters.    For additional information about the PROTOCOL.INI

file  and network device drivers,  see the Microsoft LAN Manager

Network Device Driver Guide.

�������������������������������������������������������������������Ŀ

�                �                       �            �             �

�  Parameter     �         Range         �  Default   �   Default   �

�                �                       �    DOS     �     OS/2    �

�                �                       �            �             �

�������������������������������������������������������������������Ĵ

�                �                       �            �             �

�  DRIVERNAME    �  R100VG$ or R100VG$n  �  Required  �   Required  �

�                �                       �            �             �

�  IOADDRESS     �  0x100 - 0xFFE0       �  Required  �   Required  �

�                �                       �            �             �

�  INTERRUPT     �  3 - 15               �   EEPROM   �    EEPROM   �

�                �                       �   or PCI   �    or PCI   �

�                �                       �            �             �

�  BUFFERSIZE    �  1 or 2               �     1      �   Ignored   �

�                �                       �            �             �

�  NETADDRESS    �  "000000000000" thru  �   EEPROM   �    EEPROM   �

�                �  "FEFFFFFFFFFF"       �            �             �

�                �                       �            �             �

���������������������������������������������������������������������

DRIVERNAME

Identifies the network device driver.  The DRIVERNAME is R100VG$

for the first driver, R100VG$2 for the second, etc.    Note that

a separate driver is required in NDIS version 2 for each adapter

card.   This parameter is required.

IOADDRESS

Identifies the I-O address of the adapter.    For EISA adapters,

the I-O address is 0x?C38, where ? is the EISA slot in which the

adapter is located (0x1C38 for slot 1, 0x2C38 for slot 2, etc.).

For ISA adapters, the I-O address is programmed into the adapter

by the VGCFG.EXE utility;  and for PCI adapters, the I-O address

must be determined using the FINDPCI.COM utility  to examine the

system's PCI BIOS.    If not specified, a default ISA address of

0x300 will be used.

INTERRUPT

This entry overrides  the EEPROM-programmed interrupt level  for

ISA and EISA adapters.     It must be a decimal number between 3

and 15.   Note that interrupt level 8 is illegal, and some other

levels may be required for other peripherals.     This parameter

is always ignored by PCI adapters,  which must use the interrupt

level assigned to them by the PCI BIOS.    This parameter should

be used by ISA and EISA adapters only when an override is needed

(e.g., for diagnostic testing).

BUFFERSIZE

Specifies the DOS driver's input buffer size.     This parameter

is ignored by the OS/2 driver (which always allocates 63K for an

input buffer).    A value of 1 allocates a 34K input buffer, and

a value of 2 allocates a 60K buffer  (for "peer" workstations or

other high-usage DOS systems).

NETADDRESS

This entry permits the user to override the adapter node address

programmed in the 100VG EEPROM.     The entry must be a 12-digit

string of hexadecimal characters and must be enclosed in quotes.

Also, the low-order bit of byte 0 must be zero.