32 Bit Programmers Toolkit:							4/23/96

The 32 Bit Programmers Toolkit contains dynamic link libraries, import libraries, include files, a sample Visual Basic Application with source code, and a sample C Language Application with source code so that you can create your own applications to use the WinTV from Hauppauge Computer Works. Below is list of the files that are on the disk along with some useful notes.

readme.doc	this word document

dll

pcvid32.dll	32 bit version of dynamic link library pcvideo.dll

wtvacc32.dll  	32 bit version of dynamic link library wintvacc.dll

accugnte.dll	32 bit evaluation version of image processing dynamic link library

include

wintv.h	Include file for primary WinTV functions

wintvacc.h	Include file for WTVACC functions

lib

pcvid32.lib	Library file to be used in the link statement when linking applications that will call 			routines in pcvid32.dll

wtvacc32.lib	Library file to be used in the link statement when linking applications that will call

routines in wtvacc.dll

vbexampl

example.vbp	Visual Basic Version 4, 32 bit project file

example.exe	Compiled Visual Basic 32 bit sample application

wintv.frm	Visual Basic Sample Application

wtvdec32.bas	Declaration file for WinTV function calls

wtvcon.bas	Declaration file for WinTV variables

global.bas	Global declarations for Visual Basic sample 32 bit application

winexamp

winw32tv.mak   Visual C++ 2.1 project file

winw32tv.exe  	Compilied 32 bit 32 bit sample application

winwtv.c	C source code for the Windows 32 bit sample application

winwtv.h	Include file for 32 bit sample application

winwtv.def	Definition file for 32 bit sample application

winwtv.rc	Resource file for 32 bit sample application

wintvio

wintvio.sys	Release candidate kernel driver  to perform loads and saves (version 2)

regini.exe	Utility from the NT DDK that will update the registry using a script file.

wintvio.ini	Script file containing the keys and their respective values for the registry.

Notes:

1. The WinTV API for the 32 bit enviroment is exactly the same as the 16 bit enviroment. Therefore, it will be very easy to port your existing 16 bit applications to 32 bits, there is no need to learn a new API and you can use the existing Software Development Guide.

2. The WinTV 32 bit dll's can be used on both Windows NT and Windows 95 operating systems.

3. Load, Save, Print, and Clipboard Copy/Paste have been implemented using an evaluation copy of accugnte.dll.

4. When using pcvid32.dll and wtvacc32.dll under Windows NT, the virtual device driver winvdd.dll is no longer required but you will still need the kernel driver wintvio.sys. The kernel driver is not required for Windows 95.

5. The pcvid32.dll and wtvacc32.dll use the existing pcvideo.ini file.

6. To determine the version of the kernel driver, in File Manager, highlight wintvio.sys in the \winnt35\system32\drivers\ directory, select File from the File Manager menu, and then select properties from the drop down menu.

Installation:

1. Copy pcvid32.dll, wtvacc32.dll, and to the \winnt35\system32 directory for Windows NT or to the \windows\system\ directory for Windows 95. For Windows NT only, copy wintvio.sys to the \winnt35\system32\drivers\ directory. Copy pcvideo.ini and wintvstr.ini to the \winnt35\ or \windows\ directory from the application disk that was supplied with the board. If you installed the the Hauppauge WinTV application, these files will already be on your hard drive. The new dll’s will have to be used.

2. For Windows NT, to install the kernel driver that will do loads and saves, copy the file wintvio.sys to the \winnt35\system32\drivers directory. If you have already installed a previous version of the kernel driver using the Hauppauge NT driver setup disk, the registry will already be configured. If you have not previously installed the kernel driver, run the regini.exe program with wintvio.ini as an argument.

(i.e. regini wintvio.ini)

Determining If There Is A Memory Conflict:

If the WinTV cannot save or load images, the most probable cause is a memory conflict. Below is a list of steps to follow to resolve this problem.

Windows NT

1. Run the program winmsd.exe which is located in the Administrative Tools group, icon labeled Windows NT Diagnostics. Select the DMA/Memory button. Scroll through the list box to see what address in memory are being used.

2. Change the address in the pcvideo.ini file to an unused address. For example, under the section [AVSettings] , change the value for the address key to Address=D000h. For compatiblity with prior operating systems, the segment is still used. The WinTV software will convert the segment into the proper physical address. The address can also be changed by using the Configure Hardware dialog box by using the function WTVacc_ ConfigureHardware().

3. When there are no memory conflicts and the WinTV software is running, wintvio will be listed in the NT Diagnostics along with the physical memory and lenght that is being used.

Windows 95

1. Move the mouse cursor over My Computer and click the right mouse button. From the pop up menu, select properties. Select the Device Manager Tab. Highlight Computer and click the properties button or double click Computer.

2. Click the radio button for memory. In the View Resources Tab, verify that the memory range you want to use is not already in use. Click the Reserve Resources Tab. Click the memory radio button. Click the Add button. Put in the start and end values. example, start 0D0000 end 0D7FFF. Click OK.

3. Change the address in the pcvideo.ini file to an unused address. For example, under the section [AVSettings] , change the value for the address key to Address=D000h. For compatiblity with prior operating systems, the segment is still used. The WinTV software will convert the segment into the proper physical address. The address can also be changed by using the Configure Hardware dialog box by using the function WTVacc_ ConfigureHardware().