DATE: JUN,1,1993

LCS-8684/8784 MS-DOS Device Driver API (MSC Library) Documentation

------------------------------------------------------------------

I. Functions Description :

1.int initialization (int port_no, int baud_rate, int parity, int stop_bit,

int character_length)

Input:

int port_no = 5..36

int baud_rate = 0 : 110  bps	8  : 19.2k  bps

1 : 150  bps	9  : 38.4k  bps

2 : 300  bps	10 : 56k    bps

3 : 600  bps	11 : 57.6k  bps

4 : 1200 bps	12 : 64k    bps

5 : 2400 bps	13 : 76.8k  bps

6 : 4800 bps	14 : 115.2k bps

7 : 9600 bps

int parity = 0 : no parity

1 : odd parity

2 : even parity

int stop_bit = 1 : one stop bit

2 : two stop bits

int character_length = 7 : one character 7 bits

8 : one character 8 bits

Output : 0  = Normal (OK.)

-x = the x'th input parameter error

-1 = port_no invalid

2  = Tx/Rx time out

3  = data overrun error

4  = parity error

5  = framing error

6  = break interrupt

Function : Initialize each port, include baud rate, parity bit, stop bits,

character length.

2.int output_character (int port_no, char c)

Input:

int port_no = 5..36

char c = output character

Output : 0  = Normal (OK.)

-1 = port_no invalid

2  = Tx/Rx time out

Function : Output one character.

3.int input_character (int port_no, char *c)

Input:

int port_no = 5..36

Output : 0  = Normal (OK.)

char *c = input character pointer

-1 = port_no invalid

2  = Tx/Rx time out

3  = data overrun error

4  = parity error

5  = framing error

6  = break interrupt

Function : Input one character.

4.int get_port_status (int port_no, int *status)

Input:

int port_no = 5..36

Output : 0  = Normal (OK.)

int *status = pointer of return status value

bits definition of status value :

(1 : yes , 0 : no)

b0 - b3 : reversed b8	: Rx data in queue

b4 : CTS	    b9	: Rx data overrun error

b5 : DSR	    b10 : Rx data parity error

b6 : RI	    b11 : Rx data framing error

b7 : CD	    b12 : break interrupt

b13 : Tx holding register empty (Tx ready)

b14 : Tx shift register empty (Tx empty)

b15 : time out error bit

-1 = port_no invalid

Function : Get port sataus, include tx/rx sataus.

5.int Rx_buf_check (int port_no, int *rx_length)

Input:

int port_no = 5..36

Output : 0  = Normal (OK.)

int *rx_length = pointer of return value of rx character

length (If length equal 0, it means no rx data.)

-1 = port_no invalid

Function : Check rx character length.

6.int string_input (int port_no, char far *input_buffer, int *buffer_size)

Input:

int port_no = 5..36

int *input_buffer = pointer of rx buffer

int *buffer_size = pointer of rx buffer size

Output : 0  = Normal (OK.)

int *buffer_size = pointer of return value of physical rx

character length

(If length equal 0, it means rx buffer empty.)

-1 = port_no invalid

Function : Input string from rx buffer.

7.int Tx_buf_check (int port_no, int *tx_length)

Input:

int port_no = 5..36

Output : 0  = Normal (OK.)

int *tx_length = pointer of return value of tx buffer length

(If length equal 0, it means no tx data.)

-1 = port_no invalid

Function : Check tx buffer available length.

8.int string_output (int port_no, char far *output_buffer, int *buffer_size)

Input:

int port_no = 5..36

int *output_buffer = pointer of tx buffer

int *buffer_size = pointer of tx buffer size

Output : 0  = Normal (OK.)

int *buffer_size = pointer of return value of physical tx

character length

(If length equal 0, it means tx buffer full.)

-1 = port_no invalid

Function : Output string to tx buffer.

9.int get_port_initialization (int port_no, int *baud_rate, int *parity,

int *stop_bit, int *character_length)

Input:

int port_no = 5..36

Output : 0  = Normal (OK.)

int *baud_rate = 0 : 110  bps	 8  : 19.2k  bps

1 : 150  bps	 9  : 38.4k  bps

2 : 300  bps	 10 : 56k    bps

3 : 600  bps	 11 : 57.6k  bps

4 : 1200 bps	 12 : 64k    bps

5 : 2400 bps	 13 : 76.8k  bps

6 : 4800 bps	 14 : 115.2k bps

7 : 9600 bps

int *parity = 0 : no parity

1 : odd parity

2 : even parity

int *stop_bit = 1 : one stop bit

2 : two stop bits

int *character_length = 7 : one character 7 bits

8 : one character 8 bits

-1 = port_no invalid

Function : Get each port initialization, include baud rate, parity bit,

stop bit, character length.

10.int select_buf_flush (int port_no, int flush_operation)

Input:

int port_no = 5..36

int flush_operation = 1 : rx buffer flush

2 : tx buffer flush

3 : tx/rx buffer flush

Output : 0  = Normal (OK.)

-x = the x'th input parameter error

-1 = port_no invalid

Function : Select tx/rx buffer flush.

11.int Non_destructive_input (int port_no, char *c)

Input:

int port_no = 5..36

Output : 0  = Normal (OK.)

char *c = pointer of character to be dequeued

-1 = port_no invalid

7  = Rx buffer empty

Function : Get first character from rx buffer,no change the buffer pointer.

12.int xon_xoff_flow_control (int port_no, int operation)

Input:

int port_no = 5..36

int operation = 0 : enable xon/xoff flow control

1 : disable xon/xoff flow control

Output : 0  = Normal (OK.)

-x = the x'th input parameter error

-1 = port_no invalid

Function : Enable/Disable xon/xoff flow control.

The default xon/xoff character code are 11h and 13h.

II.  Sample Program :

1. Program as flow:

/* ----------- porgram start ----------------- */

/**************************************************************************

1.int initialization (int port_no, int baud_rate, int parity, int stop_bit,

int character_length)

2.int output_character (int port_no, char c)

3.int input_character (int port_no, char *c)

4.int get_port_status (int port_no, int *status)

5.int Rx_buf_check (int port_no, int *rx_length)

6.int string_input (int port_no, char *input_buffer, int *buffer_size)

7.int Tx_buf_check (int port_no, int *tx_length)

8.int string_output (int port_no, char *output_buffer, int *buffer_size)

9.int get_port_initialization (int port_no, int *baud_rate, int *parity,

int *stop_bit, int *character_length)

10.int select_buf_flush (int port_no, int flush_operation)

11.int Non_destructive_input (int port_no, char *c)

12.int xon_xoff_flow_control (int port_no, int operation)

****************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include <malloc.h>

#define BUF_SIZE 20

main()

{

int port_no,baud_rate,parity,stop_bit,character_length;

int status;

int rx_length;

char far *input_buffer;

int buffer_size;

int tx_length;

char far *output_buffer = "1234567890abcdef";

int flush_operation;

char c = 'H';

int operation;

/* initialize the communication port COM7: 9600 bps,none parity,one stop bit,

8 bits character size	   */

port_no = 7;

baud_rate = 7;

parity = 0;

stop_bit = 1;

character_length = 8;

initialization (port_no,baud_rate,parity,stop_bit,character_length);

/* put one character to communication port */

port_no = 7;

output_character (port_no,c);

/* get one character form communication port */

port_no = 7;

if ((input_character (port_no,&c)) == 0)

printf("\n The input character : %s ",c);

/* get communication port status */

port_no = 7;

get_port_status (port_no,&status);

printf("\n The port status value : %x ",status);

/* check receive buffer data length */

port_no = 7;

Rx_buf_check (port_no,&rx_length);

printf("\n The receive data length : %d ",rx_length);

/* get block data from communication port */

port_no = 7;

buffer_size = BUF_SIZE;

input_buffer = _fmalloc(BUF_SIZE); /* allocate memory */

string_input (port_no,input_buffer,&buffer_size);

if (buffer_size != 0)

printf("\n The input block data : %20Fs ",input_buffer);

_ffree(input_buffer);	       /* free memory */

/* check transmit buffer free space */

port_no = 7;

Tx_buf_check (port_no,&tx_length);

/* put block data to communication port */

port_no = 7;

buffer_size = 16;

string_output (port_no,output_buffer,&buffer_size);

/* get communication port initialization data, include baud rate, parity,stop

bit(s), character size.	*/

port_no = 7;

get_port_initialization (port_no,&baud_rate,&parity,&stop_bit,

&character_length);

printf("\n The port initial value: baud rate = %d bps, parity = %d, stop

bit(s) = %d, character size = %d ",baud_rate,parity,stop_bit,

character_length);

/* flush communication port transmit/receive buffer data */

port_no = 7;

flush_operation = 3;

select_buf_flush (port_no,flush_operation);

/* get one character */

port_no = 7;

if ((Non_destructive_input (port_no,&c)) == 0) ;

printf("\n The input character : %s ",c);

/* enable XON/XOFF flow control */

port_no = 7;

operation = 0;

xon_xoff_flow_control (port_no,operation);

}

/* -------- end of program --------------- */

2. Compile and link:

0) flie list:

API.OBJ	: API Object code for Microsoft C 6.00

API5.OBJ	: API Object code for Microsoft C 5.00

DOSAPI.DOC : this file.

1) Compiler method: (for Microsoft C 6.00)

cl /c sample.c

link sample+api;

2) Compiler method: (for Microsoft C 5.00)

cl /c sample.c

link sample+api5;

--------- End of ducumentation -----------