MODEM.INF

---------

This is a replacement MODEM.INF file for use with the Remote Access (RAS) part

of Windows for Work Groups and the MWave Modem in the ThinkPad 755CD/CE/CSE.

This updated file tells RAS what the proper send and return codes are for the

DSP MWave Modem.  This file does not address the memory problems encountered

with earlier Mwave code releases.  If your are experiencing "ERROR 640:  A NET

BIOS ERROR HAS OCCURED" messages when attempting to use the Mwave Modem with

RAS, you should obtain the latest level MWave drivers (release 1.10 or later).

INSTALLATION

------------

Find your existing MODEM.INF file, by default located in the system sub-

directory of the Windows directory. Make a copy of the existing file on

diskette or rename the file so that if anything goes wrong during the

installation, you can recover. Copy the new MODEM.INF file into the

directory where the existing MODEM.INF was found.

To run RAS using the new MODEM.INF the following steps must be taken:

* Windows for Work Groups must be started

* The Mwave Modem must be started

* The new (updated) MODEM.INF file must be in the \WINDOWS\SYSTEM directory

* RAS must be started

* Mwave Modem must be selected from the available list in the RAS port

configuration menu

* RAS must be restarted