How to Install NetWare Server 4.x Driver

==============================================

For a NetWare 4.x server configuration, the ETHER-32V/P Server 4.x driver

is included on the ETHER-32V/P driver diskette.The driver is located under

the 4.x subdirectory in the \NETWARE\SRV directory of the ETHER-32V/P disk.

The directory structure provided on the ETHER-32V/P driver diskette has been

setup for automatic OEM installation. The OEM Installation section in

this manual provides a brief explanation regarding OEM driver installation.

Follow the guidelines in the Novell Netware documentation for more details

to install the driver from the ETHER-32V/P driver diskettes.

Note: Some of the custom installation procedures provided here are either

an abbreviated or a duplication of the OEM installation guidelines.

Use the LOAD and BIND commands to load the NetWare 4.x open datalink

interface (ODI) server driver as described below. For more information

on these commands, refer to the NetWare 4.x LAN driver installation

documentation.  Located in the Driver Disk is the version 4x server

driver labeled PCNTNW.LAN.

To install the NetWare Server 4.x, follow the instructions given below:

1. Install NetWare 4.x on your system.

2. Copy the ODI server driver, PCNTNW.LAN, from the ETHER-32V/P Driver

Diskette to the hard disk directory where the NetWare 4.x

system files are located.The path on the ETHER-32V/P Diskette should

be as follows:

A:\NETWARE\SRV\4.X\MSM.NLM

A:\NETWARE\SRV\4.X\PCNTNW.LDI

A:\NETWARE\SRV\ETHERTSM.NLM

A:\NETWARE\SRV\PCNTNW.LAN

3. Boot NetWare by executing SERVER.EXE. Refer to NetWare documentation

on loading disk drivers and mounting volumes.

4. To load the driver manually, simply follow the instructions given

below.

For ETHER-32V  and ETHER-32P adapter boards, type the

following command at the file server console:

LOAD PCNTNW   <enter>

( You will need to specify DOS path if the driver is located in

a DOS partition).

Do not use Port, INT, and DMA keywords. It is recommended that

the user use IOP, IRQ, and DRQ keywords instead.However, for the

ETHER-32V, and ETHER-32P adapter cards,no keywords need

to be specified for interrupt

Make sure that MSM and ETHERTSM files are present in the same

directory as PCNTNW. The PCNTNW driver will autoload these files.

These files should be copied from the ETHER-32V/P disk from the

4.x subdirectory under the Netware directory

Then, bind the driver to the IPX protocol stack by typing:

BIND IPX TO PCNTNW NET=xxxx  <enter>

(Bind to ipx or any other protocol stack).

If two or more adapters to install:

For ETHER32-V card:

LOAD  PCNTNW IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO PCNTNW  NET=xxxx

(Bind to ipx or any other protocol stack).

LOAD  PCNTNW  IOP=xxx IRQ=xxx FRAME=Ethernet_xx

BIND IPX TO PCNTNW  NET=xxxx

(Bind to ipx or any other protocol stack).

.

.

.

For ETHER32-P card:

LOAD  PCNTNW BOARD=0 BUSTYPE=PCI1 FRAME=Ethernet_xx

BIND IPX TO PCNTNW NET=xxxx

(Bind to ipx or any other protocol stack).

LOAD  PCNTNW BOARD=1 BUSTYPE=PCI1 FRAME=Ethernet_xx

BIND IPX TO PCNTNW NET=xxxx

(Bind to ipx or any other protocol stack).

.

.

.

5.   To load the driver using AUTOEXEC, you must edit AUTOEXEC.NCF.

First, type "LOAD INSTALL" at the server prompt ":"  to access

the install utility. Open the MAINTENANCE/SELECTIVE INSTALL to

select the "NCF File Options" from the menu to access the

"EDIT AUTOEXEC". Add the following command lines in AUTOEXEC.NCF

file.

For example on a ETHER-32V card:

.One adapter and more frame types

LOAD  PCNTNW IRQ=3 IOP=320 FRAME=Ethernet_802.2 name=xxx

BIND IPX TO xxx NET=xxxx

LOAD  PCNTNW IRQ=3 IOP=320 FRAME=Ethernet_802.3 name=xxx

BIND IPX TO xxx NET=xxxx

.

.

.

For example on a ETHER-32P card

.One adapter and more frame types

LOAD  PCNTNW BOARD=0 BUSTYPE=PCI1 FRAME=Ethernet_802.2 name=xxx

BIND IPX TO xxx NET=xxxx

LOAD  PCNTNW BOARD=0 BUSTYPE=PCI1 FRAME=Ethernet_802.3 name=xxx

BIND IPX TO xxx NET=xxxx

.

.

.

Note:

1. You will need to specify DOS path if the  driver is located

in a DOS partition.

2. You can modify BUSTYPE=PCI2 for PCI mech#2 machines.