Date: 2/8/1995

pcimech.exe display machines PCI type mech#1 or mech#2.

Date: 6/6/1994

readme.doc - This readme file

cdcoff.com - A DOS program to switch off the write buffers of the Intel CDC

chip (see problem description for more explanation)

These driver files are modified from version 1.2 to version 1.3(beta). The difference

between the two versions is the implementation of a new keyword "CDCOFF". This

keyword can be placed in the net.cfg file for the DOSODI driver or on the load

command line for the server driver. This keyword will disable the write buffers

on the Intel CDC chip (see the section below for mode details). This change is

only implemented in the Novell drivers because this issue is only seen during

the certification testing of Novell drivers (Exchange test). Novell has agreed

with AMD to use this keyword and then run the drivers on the Intel CDC based

machines. For all other environments, we have not seen any issues with regards

to this problem. However, just to be safe we are providing the DOS program

cdcoff.com which when executed will disable the write buffers on the Intel

CDC chip (same effect as the "CDCOFF" keyword for the Novell drivers). If you

are interested in obtaining Novell certification for the PCnet-PCI device then

please use Novell drivers version 1.3 along with the "CDCOFF" keyword. The

other PCnet family members (PCnet-ISA, PCnet-ISA+, PCnet-32) are not affected

with this change. The version 1.3(beta) has not been fully tested at the

AMD labs. We beleive the changes are very minor but because of limited time

for this release, the complete testing is not complete yet.

---------------------------------------------------------------------------

Subject:        PCnet-PCI based adapter cards failing Novell

Exchange Test in Intel Based Motherboards

PROBLEM:

PCI based Ethernet cards, including AMD PCnet-PCI and DEC 21040 based

cards, are failing the Novell 4.01 Standard Exchange test with the server

and client drivers.  The failure is reported as excessive dropped packets.

The Exchange Test is required to pass Novell Certification.

After extensive testing, it has been determined that the problem with the

Exchange Test is related to the Intel PCI chipsets currently shipping,

both the Saturn (486) and Mercury (P5).  The problem relates to the write

buffers integrated into the Cache DRAM Controller (CDC).  These buffers are

intended to buffer write cycles from the processor's local bus to the PCI

bus.  These are designed to provide a small performance increase in write

intensive operations.

Unfortunately, when these write buffers are enabled on the Saturn (486)

chipset, the chipset can insert disconnect cycles while the PCI based

ethernet card has control of the bus.  This forces the Ethernet card to

do retries many times, effectively not granting access to the bus for

extended periods of time.  This results in underflows and overflows, which

cause the dropped packets reported in the Exchange Test.

When the write buffers are enabled on a Mercury (P5) chipset, the chipset

simply does not grant the bus to the Ethernet for extended periods of

time, often over 200 uS.  As a result, like the Saturn chipset, the

Mercury chipset causes underflows and overflows.

Both of these chipsets cause the Novell Exchange Test to fails with both

the AMD PCnet-PCI and the DEC 21040, when the processor to PCI bus write

buffers are enabled.  AMD has done testing with the OPTi and Contaq chipsets

and pass the exchange test with the AMD PCnet-PCI. The PCnet-PCI also

passes the exchange test on Compaq Deskpro XL PCI machine.

SOLUTION:

The long term solution to the extensive latencies is to work with the PCI

chipset vendors and convince them that these latencies are not acceptable,

and help them figure out how to solve this problem.

The short term solution has two options:

A patch program, which is run before the ethernet drivers are loaded in

the Novell environment, both client and server, before running the Novell

Exchange Test.  This program reprograms the bit which controls the write

buffers and disables the processor bus to PCI bus write buffers.

This program is currently available on the AMD Corporate Systems Engineering

bulletin board, and may be downloaded for testing.

Otherwise, a modification is being made to the Novell Ethernet drivers,

both client and server, which allows a command line switch to disable the

write buffers.  These Novell drivers will be released as version 1.3 by

June 10, 1994.  They will be available on the Corporate Systems Engineering

bulletin board, and may be downloaded to perform Novell certification testing.

Note:  There is no difference in driver functionality between the version 1.2

and 1.3 drivers.  Customers should ONLY upgrade to version 1.3 if they

plan to submit for Novell Certification.