MMTEST.COM    Tablet Test Utility

Copyright Summagraphics Corporation 1990, 1991.  All rights reserved.

MMTEST allows you to verify that your Summagraphics tablet is fully operational.

MMTEST will display tablet output (X,Y coordinates) on your screen.

The data format is:

XXXXXX,YYYYYY,F,P

X = X coordinate data. Y = Y coordinate data. F = button data. P = proximity.

If you find that your tablet is working with MMTEST, but not working

with your application, run the application's setup procedure and be

sure all the options have been correctly configured. If you continue to have

problems contact the software manufacturer who produced the application.

Be sure to reset the tablet by turning it off and then on, or by using MMRST

before running other software.

Usage: MMTEST [ options ]

Options are:

/1 = (default) selects serial port 1 (PC, XT, AT, PS/2)

/2 = selects serial port 2 (PC, XT, AT, PS/2)

/3 = selects serial port 3 (PS/2 only)

/4 = selects serial port 4 (PS/2 only)

/5 = selects serial port 5 (PS/2 only)

/6 = selects serial port 6 (PS/2 only)

/7 = selects serial port 7 (PS/2 only)

/8 = selects serial port 8 (PS/2 only)

/BAxxxx = selects serial port with base address of xxxx (in HEX)

/Ix = selects serial port with IRQ line number x (allowable IRQ 2 through 7)

/U = selects UIOF configuration for program use when tablet is configured for

UIOF format or has a 16-button cursor attached.

/N = selects no parity configuration

/? or /H = displays help screen

Ctrl-X = Exit program and return to DOS.

Notes:

- When using the BA option (Base Address) all four digits must be

specified (example:  for Com 1 - /BA03F8)

- This program transmits in SummaSketch default serial communication

configuration which is 9600 baud, 8 data bits, odd parity, and 1 stop bit.

- Use the /BA switch only when the serial port is located at a non-standard

address within the computer.

- A summary of command line switches is always available by starting the

program with the/? or /H switch.

- When selecting non-standard serial ports, the /BA and /I options must both

be specified.

Serial Port Address Table

Command Line	Serial	   Serial Port	 System

Switch		Port	   Address

______________________________________________________________________________

/1		1	   03F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/2		2	   02F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/3		3	   3220 Hex	 Any PS/2

/4		4	   3228 Hex	 Any PS/2

/5		5	   4220 Hex	 Any PS/2

/6		6	   4228 Hex	 Any PS/2

/7		7	   5220 Hex	 Any PS/2

/8		8	   5228 Hex	 Any PS/2

______________________________________________________________________________

************************ 87-1011-001 *****************************