Tablet Driver for IBM OS/2 V2.1, OS/2 V2.11 and OS/2 V3.0 Warp

Version 1.02

Date: May 1, 1995.

The driver "Tablet.sys" is the tablet driver for Tablet-1812, Tablet 1212,

Tablet 66, Tablet 345 and SK-32.

The driver will work as Tablet Mode ( Absolute Address ).

Tablet-1812 and Tablet 1212 will map Upper-left 6"X6" to screen.

Tablet 66 will map whole area 6"X6" to Screen.

Tablet 345 will map 3"X4.5" to Screen.

SK-32 will map 3"X2" to Screen.

Install Tablet.sys Driver to OS/2.

1. Copy the file "Tablet.SYS" to C:\OS2\BOOT. ( C: is the Driver where OS/2

located.)

2. Use Tedit or any text editer modify c:\config.sys.

3. Find the line

DEVICE=C:\OS2\BOOT\MOUSE.SYS SERIAL=COM1

4. Delete the line if there is another mouse device as

DEVICE=C:\OS2\BOOT\PCLOGIC.SYS SERIAL=COM1

5. Change the Mouse driver as

DEVICE=C:\OS2\BOOT\TABLET.SYS SERIAL=COM1

DEVICE=C:\OS2\BOOT\MOUSE.SYS TYPE=TABLET$

( COM1 can be COM2, COM3, or COM4 )

6. Shutdown the system and reboot the system.

Use Tablet Driver.

The Tip Button is same as Left Button of Mouse.

The side Button is same as Right Button of Mouse.

Low down the double click speed or use right button to double click,

if you find the double click is not easy to use.