INSTALLATION INSTRUCTIONS

OS2 VER. 2.1 DRIVER

APEX DATA PCMCIA MODEMS

OCT. 13,1993

Software Required:        IBM OS2 ver. 2.1

IBM Socket Services ( provided with your Thinkpad Computer)

Apex OS2 Driver for the PCMCIA modem ( APEXDFM1.OS2)

Step 1: Do a directory of \OS2\COM.SYS

Check the creation date making sure it is 6/17/93 or later. If the date is earlier 	phone IBM OS2 Support at 1-800-992-4777 and request the latest COM.SYS file. 	Upon receipt of the new file, replace the excising COM.SYS.

Step 2: Copy the Apex OS2 Driver ( APEXDFM1.OS2 ) to the \OS2 directory.

Step 3: From the OS2 Command Prompt folder select OS2 Window

Go to the OS2 root directory ( C:\ or D:\ ect. )

Bring up your CONFIG.SYS file in the OS2 editor.

Example: [D:\] E CONFIG.SYS

When the file appears verify that PCMCIA.SYS and COM.SYS are present and are being 	loaded. If they are not present add them , the PCMCIA.SYS must be loaded first.

Example: DEVICE=D:\OS2\PCMCIA.SYS

Add the following as the last line of your CONFIG.SYS.

DEVICE=D:\OS2\APEXDFM1.OS2 S1C2,S2C2

|     |    |    |_com2

|     |    |_slot2

|     |_com2

|_slot1

( Please note, drive letter D:\ is just an example, your drive letter may be different.)

! ! ! IMPORTANT ! ! !

The Apex OS2 Driver MUST be installed after the COM.SYS driver and before the Socket Services Drivers.

Step 4: Install your IBM Socket Services from the ref. diskette provided with your IBM Thinkpad. Check your Thinkpad documentation for instructions. The install program which comes with the socket services must be used for installation of the socket services drivers.

Step 5: Save and reboot the system.

Sample CONFIG.SYS file showing the order of the previously mentioned device drivers:

( Driver provided by OS2 ) - DEVICE=D:\OS2\PCMCIA.SYS

*

*

( Driver provided by OS2 ) - DEVICE=D:\OS2\COM.SYS

*

*

( Apex OS2 driver ) ---------  DEVICE=D:\OS2\APEXDFM1.OS2 S1C2,S2C2

( 3 examples of IBM ---------  DEVICE=D:\OS2\IBM2SS02.SYS

socket services drivers ) --  DEVICE=D:\OS2\ICRMU02.SYS

* * *	          ---  DEVICE=D:\OS2\$ICPMOS2.SYS

<<end of file>>

*** Please note the IBM socket services drivers may or may not be the same as the example above, depending on the ver. or model of your IBM Thinkpad.

Application Notes

When installing a Fax or Comm software package please make sure that any device drivers which were loaded on installation are located below the Socket Services drivers. This also applies to any Fax or Comm package installed before the Apex OS2 driver.

If there are any problems please contact Apex Data Technical Support 510-803-2031