VIDPARK: Video Blanker/Hard Disk Head Parker Utility

Version 1.06, 03-Jun-1991

SMG Software

(C) Copyright 1987,1991 by

Steven Georgiades

All Rights Reserved

This program is a memory resident utility that combines two common

functions: a video  screen blanker and  a hard disk  head parker.   The

screen blanker  will  cause the  video  display  to turn  off  after  a

specified number of minutes of inactivity of both the keyboard and  the

video.  Once the screen has blanked, it may be restored by pressing ANY

key on the  keyboard (including SHIFT,  CTRL, ALT, SYSREQ,  etc).   The

head parker will cause the hard  disk heads to be positioned just  past

the last cylinder  of the disk  (which is usually  a dedicated  landing

zone) after a  specified number of  minutes of inactivity  of the  hard

disk.  Note that  in a system  with two hard  disks, this program  will

park each independently.  In other words, activity on drive C: will not

prevent drive D: from parking, and  visa versa.  This program will  NOT

park more than two hard disks, nor will  it park a second hard disk  if

the second hard disk uses a  different controller card than the  first.

Either function  (Video Blanking  or Head  Parking) my  be disabled  by

entering a timeout period of zero (0).

This program WILL work as a Video Blanker, even if you do not have

a hard disk.  The program will be able to detect the absence of a  hard

disk controller, and will behave accordingly.

Command Format:

VIDPARK [?] | [[/0|/1|/B]/P[n]][/V[n]]

? = Query current timeout periods

/0 = Select Hard Disk 0

/1 = Select Hard Disk 1

/B = Select Hard Disks 0 and 1

/P = Set Selected Hard Disk Parking Period

/V = Set Video Blanking Period

n = Timeout period, in minutes

(0 to 60, 0 = disable, default = 5)

Examples:

VIDPARK ?           (Will respond with current timeout settings)

VIDPARK /V0/P10     (Will disable  video  blanking  and  set  hard

disks to park after 10 minutes of inactivity)

VIDPARK /1P7        (Will set hard disk 1 to park after 7  minutes

of inactivity)

Revison History:

Version 1.05        Initial Release

Version 1.06        Added /0, /1 and /B options

Video Blanker/Hard Disk Head Parker Utility, Version 1.06                 Page 2

This software is distributed as ShareWare.  "ShareWare" means that

you are granted free license to use  and copy this software as much  as

you like, within the following restrictions:

1).  It is  distributed  in  its  original,  unmodified  form,

including program and documentation.

2).  No fee is charged for use, copying or distribution  (With

the exception of nominal "per disk" copying charges,  not

to exceed $6.00 per disk).

3).  The program  may  not be  included  with other  goods  or

services supplied for a fee, unless written permission to

do so is obtained in advance from the author.

No registration fees are  required for this program,  but if you  would

like to have the program customized in some minor way, a nominal fee of

$25 is required.  The fee for major customization is negotiable, but in

most cases  will  not  exceed  $50.    Source  code  for  this  program

(Microsoft Macro Assembler, Version 5.1) can be purchased for $50.

This program  is provided  AS  IS without  any warranty,  expressed  or

implied, including  but  not limited  to  fitfulness for  a  particular

purpose.

If you have any questions or comments about this or any other  SMG

program, or if you would like to register any SMG Software, write to me

at the following address:

Steven M. Georgiades, SMG Software

P.O. Box 19054

Anaheim, CA  92817-9054

A diskette containing all of the latest SMG Software can be obtained by

sending a check or money  order for $6.00 to  the above address.   Make

all checks payable to Steven M. Georgiades.  Thank you.