LOTUS(R)/INTEL(R)/MICROSOFT(R)

EXPANDED MEMORY SPECIFICATION [1]

Version 4.0

300275-005

October, 1987

Copyright (C) 1987

Lotus Development Corporation

55 Cambridge Parkway

Cambridge, MA  02142

Intel Corporation

5200 NE Elam Young Parkway

Hillsboro, OR  97124

Microsoft Corporation

16011 NE 35th Way

Box 97017

Redmond, WA  98073

[1] Transcribed into machine-readable form by Dick Flanagan,

Ben Lomond, California.  This transcription is released into the

public domain without warranty or assumption of liability.

This specification was jointly developed by Lotus Develop-

ment Corporation, Intel Corporation, and Microsoft Corpora-

tion.  Although it has been released into the public domain

and is not confidential or proprietary, the specification is

still the copyright and property of Lotus Development

Corporation, Intel Corporation, and Microsoft Corporation.

DISCLAIMER OF WARRANTY

LOTUS DEVELOPMENT CORPORATION, INTEL CORPORATION, AND MICRO-

SOFT CORPORATION EXCLUDE ANY AND ALL IMPLIED WARRANTIES,

INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE.  NEITHER LOTUS NOR INTEL NOR MICROSOFT

MAKE ANY WARRANTY OF REPRESENTATION, EITHER EXPRESS OR

IMPLIED, WITH RESPECT TO THIS SPECIFICATION, ITS QUALITY,

PERFORMANCE, MERCHANTABILITY, OR FITNESS FOR A PARTICULAR

PURPOSE.  NEITHER LOTUS NOR INTEL NOR MICROSOFT SHALL HAVE

ANY LIABILITY FOR SPECIAL, INCIDENTAL, OR CONSEQUENTIAL

DAMAGES ARISING OUT OF OR RESULTING FROM THE USE OR MODIF-

ICATION OF THIS SPECIFICATION.

This specification uses the following trademarks:

Intel is a trademark of Intel Corporation

Lotus is a trademark of Lotus Development Corporation

Microsoft is a trademark of Microsoft Corporation

ii

CONTENTS

Chapter 1

INTRODUCTION

What is Expanded Memory? . . . . . . . . . . . . . . . . .    1

How Expanded Memory Works  . . . . . . . . . . . . . . . .    1

Chapter 2

WRITING PROGRAMS THAT USE EXPANDED MEMORY

What Every Program Must Do . . . . . . . . . . . . . . . .    4

Advanced Programming . . . . . . . . . . . . . . . . . . .    5

Saving The State of Mapping Hardware . . . . . . . . . .    6

Retrieving Handle and Page Counts  . . . . . . . . . . .    6

Mapping and Unmapping Multiple Pages . . . . . . . . . .    6

Reallocating Pages . . . . . . . . . . . . . . . . . . .    6

Using Handles and Assigning Names to Handles . . . . . .    6

Using Handle Attributes  . . . . . . . . . . . . . . . .    7

Altering Page Maps and Jumping/Calling . . . . . . . . .    7

Moving or Exchanging Memory Regions  . . . . . . . . . .    7

Getting the Amount of Mappable Memory  . . . . . . . . .    8

Operating System Functions . . . . . . . . . . . . . . .    8

Programming Guidelines . . . . . . . . . . . . . . . . . .   12

Examples . . . . . . . . . . . . . . . . . . . . . . . . .   14

Example 1  . . . . . . . . . . . . . . . . . . . . . . .   14

Example 2  . . . . . . . . . . . . . . . . . . . . . . .   19

Example 3  . . . . . . . . . . . . . . . . . . . . . . .   30

Example 4  . . . . . . . . . . . . . . . . . . . . . . .   32

Chapter 3

EMM FUNCTIONS

Function 1. Get Status . . . . . . . . . . . . . . . . . .   37

Function 2. Get Page Frame Address . . . . . . . . . . . .   38

Function 3. Get Unallocated Page Count . . . . . . . . . .   40

Function 4. Allocate Pages . . . . . . . . . . . . . . . .   42

Function 5. Map/Unmap Handle Pages . . . . . . . . . . . .   46

Function 6. Deallocate Pages . . . . . . . . . . . . . . .   49

Function 7. Get Version  . . . . . . . . . . . . . . . . .   51

Function 8. Save Page Map  . . . . . . . . . . . . . . . .   53

Function 9. Restore Page Map . . . . . . . . . . . . . . .   55

Function 10. Reserved  . . . . . . . . . . . . . . . . . .   57

Function 11. Reserved  . . . . . . . . . . . . . . . . . .   58

Function 12. Get Handle Count  . . . . . . . . . . . . . .   59

Function 13. Get Handle Pages  . . . . . . . . . . . . . .   61

Function 14. Get All Handle Pages  . . . . . . . . . . . .   63

Function 15. Get/Set Page Map  . . . . . . . . . . . . . .   65

Get Page Map subfunction . . . . . . . . . . . . . . . .   65

Set Page Map subfunction . . . . . . . . . . . . . . . .   67

Get & Set Page Map subfunction . . . . . . . . . . . . .   69

Get Size of Page Map Save Array subfunction  . . . . . .   71

iii

Function 16. Get/Set Partial Page Map  . . . . . . . . . .   73

Get Partial Page Map subfunction . . . . . . . . . . . .   73

Set Partial Page Map subfunction . . . . . . . . . . . .   76

Get Size of Partial Page Map Save Array subfunction  . .   78

Function 17. Map/Unmap Multiple Handle Pages . . . . . . .   80

Mapping Multiple Pages . . . . . . . . . . . . . . . . .   80

Unmapping Multiple Pages . . . . . . . . . . . . . . . .   80

Mapping and Unmapping Multiple Pages Simultaneously  . .   80

Alternate Mapping and Unmapping Methods  . . . . . . . .   81

Logical Page/Physical Page Method  . . . . . . . . . . .   82

Logical Page/Segment Address Method  . . . . . . . . . .   85

Function 18. Reallocate Pages  . . . . . . . . . . . . . .   88

Function 19. Get/Set Handle Attribute  . . . . . . . . . .   91

Design Considerations  . . . . . . . . . . . . . . . . .   91

Get Handle Attribute subfunction . . . . . . . . . . . .   92

Set Handle Attribute subfunction . . . . . . . . . . . .   94

Get Attribute Capability subfunction . . . . . . . . . .   96

Function 20. Get/Set Handle Name . . . . . . . . . . . . .   98

Get Handle Name subfunction  . . . . . . . . . . . . . .   98

Set Handle Name subfunction  . . . . . . . . . . . . . .  100

Function 21. Get Handle Directory  . . . . . . . . . . . .  102

Get Handle Directory subfunction . . . . . . . . . . . .  102

Search For Named Handle subfunction  . . . . . . . . . .  105

Get Total Handles subfunction  . . . . . . . . . . . . .  107

Function 22. Alter Page Map & Jump . . . . . . . . . . . .  109

Function 23. Alter Page Map & Call . . . . . . . . . . . .  113

Alter Page Map & Call subfunction  . . . . . . . . . . .  113

Get Page Map Stack Space Size subfunction  . . . . . . .  118

Function 24. Move/Exchange Memory Region . . . . . . . . .  120

Move Memory Region subfunction . . . . . . . . . . . . .  120

Exchange Memory Region subfunction . . . . . . . . . . .  126

Function 25. Get Mappable Physical Address Array . . . . .  132

Get Mappable Physical Address Array subfunction  . . . .  132

Get Mappable Physical Address Array Entries subfunction   136

Function 26. Get Expanded Memory Hardware Information  . .  138

Get Hardware Configuration Array subfunction . . . . . .  138

Get Unallocated Raw Page Count subfunction . . . . . . .  142

Function 27. Allocate Standard/Raw Pages . . . . . . . . .  144

Allocate Standard Pages subfunction  . . . . . . . . . .  144

Allocate Raw Pages subfunction . . . . . . . . . . . . .  147

Function 28. Alternate Map Register Set  . . . . . . . . .  151

Design Considerations  . . . . . . . . . . . . . . . . .  151

System DMA Capabilities & Expanded Memory Support of DMA  151

Expanded Memory Support of DMA Register Sets . . . . . .  151

Get Alternate Map Register Set subfunction . . . . . . .  153

Set Alternate Map Register Set subfunction . . . . . . .  157

Get Alternate Map Save Array Size subfunction  . . . . .  161

Allocate Alternate Map Register Set subfunction  . . . .  163

Deallocate Alternate Map Register Set subfunction  . . .  166

Allocate DMA Register Set subfunction  . . . . . . . . .  168

Enable DMA on Alternate Map Register Set subfunction . .  170

Disable DMA on Alternate Map Register Set subfunction  .  173

iv

Deallocate DMA Register Set subfunction  . . . . . . . .  175

Function 29. Prepare Expanded Memory Hardware For Warm Boot 177

Function 30. Enable/Disable OS/E Function Set Functions  .  179

Enable OS/E Function Set subfunction . . . . . . . . . .  179

Disable OS/E Function Set subfunction  . . . . . . . . .  182

Return Access Key subfunction  . . . . . . . . . . . . .  185

Appendix A

FUNCTION AND STATUS CODE CROSS REFERENCE TABLES

Function and Status Code Cross Reference . . . . . . . . .  187

Status and Function Code Cross Reference . . . . . . . . .  192

Appendix B

TESTING FOR THE PRESENCE OF THE EXPANDED MEMORY MANAGER

Which method should your program use?  . . . . . . . . . .  199

The "open handle" technique  . . . . . . . . . . . . . . .  199

The "get interrupt vector" technique . . . . . . . . . . .  204

Appendix C

EXPANDED MEMORY MANAGER IMPLEMENTATION GUIDELINES

The amount of expanded memory supported  . . . . . . . . .  206

The number of handles supported  . . . . . . . . . . . . .  206

Handle Numbering . . . . . . . . . . . . . . . . . . . . .  206

New handle type:  Handles versus Raw Handles . . . . . . .  206

The system Raw Handle (Raw Handle = 0000h) . . . . . . . .  207

Terminate and Stay Resident (TSR) Program Cooperation  . .  208

Accelerator Cards  . . . . . . . . . . . . . . . . . . . .  208

Appendix D

OPERATING SYSTEM/ENVIRONMENT USE OF FUNCTION 28

Examples . . . . . . . . . . . . . . . . . . . . . . . . .  209

Example 1  . . . . . . . . . . . . . . . . . . . . . . .  209

Example 2  . . . . . . . . . . . . . . . . . . . . . . .  210

Example 3  . . . . . . . . . . . . . . . . . . . . . . .  211

GLOSSARY

INDEX

v

Chapter 1

INTRODUCTION

Because even the maximum amount (640K bytes) of conventional

memory isn't always enough for large application programs,

Lotus Development Corporation, Intel Corporation, and Micro-

soft Corporation created the Lotus/Intel/Microsoft (LIM)

Expanded Memory Specification.

The LIM Expanded Memory Specification defines the software

interface between the Expanded Memory Manager (EMM) -- a

device driver that controls and manages expanded memory --

and application programs that use expanded memory.

What is Expanded Memory?

Expanded memory is memory beyond DOS's 640K-byte limit.  The

LIM specification supports up to 32M bytes of expanded

memory.  Because the 8086, 8088, and 80286 (in real mode)

microprocessors can physically address only 1M bytes of

memory, they access expanded memory through a window in

their physical address range.  The next section explains how

this is done.

How Expanded Memory Works

Expanded memory is divided into segments called logical

pages.  These pages are typically 16K bytes of memory.  Your

computer accesses logical pages through a physical block of

memory called a page frame.  The page frame contains

multiple physical pages, pages that the microprocessor can

address directly.  Physical pages are also typically 16K

bytes of memory.

This page frame serves as a window into expanded memory.

Just as your computer screen is a window into a large

spreadsheet, so the page frame is a window into expanded

memory.

A logical page of expanded memory can be mapped into (made

to appear in) any one of the physical pages in the page

frame.  Thus, a read or write to the physical page actually

becomes a read or write to the associated logical page.  One

logical page can be mapped into the page frame for each

physical page.

Figure 1-1 shows the relationship among the page frame,

physical pages, and logical pages.

Introduction                                                    1

32M +--------------+

/|              |

|              |

/     |              |

|              |

/          |              |

|              |

/               |              |

|   Expanded   |

/                    |    Memory    |

1024K +--------------+                           |              |

| / / / / / /  | /                         |              |

960K +--------------+                           |              |

|  Page Frame  |                           |              |

|              |                           |              |

| 12 16K-Byte  |                           |              |

|   Physical   |                           |              |

|    Pages     |                           |              |

768K +--------------+                           | Divided into |

| / / / / / /  | \                         |   logical    |

640K +--------------+                           |    pages     |

|              |   \                       |              |

|              |                           |              |

|              |     \                     |              |

|              |                           |              |

| 24 16K-Byte  |       \                   |              |

|   Physical   |                           |              |

|    Pages*    |         \                 |              |

|              |                           |              |

|              |           \               |              |

|              |                           |              |

|              |             \             |              |

256K +--------------+                           |              |

|              |               \           |              |

| / / / / / /  |                           |              |

|              |                 \         |              |

| / / / / / /  |                           |              |

|              |                   \       |              |

| / / / / / /  |                           |              |

|              |                     \     |              |

0 +--------------+                           |              |

\   |              |

|              |

*Intended for operating                        \ |              |

system/environment use only                   0 +--------------+

Figure 1-1.  Expanded Memory

Introduction                                                    2

The page frame is located above 640K bytes.  Normally, only

video adapters, network cards, and similar devices exist

between 640K and 1024K.

This specification also defines methods for operating

systems and environments to access expanded memory through

physical pages below 640K bytes.  These methods are intended

for operating system/environment developers only.

Introduction                                                    3

Chapter 2

WRITING PROGRAMS THAT USE EXPANDED MEMORY

This chapter describes what every program must do to use

expanded memory and describes more advanced techniques of

using expanded memory.

This chapter also lists programming guidelines you should

follow when writing programs that use expanded memory and

provides the listings of some example programs.

What Every Program Must Do

This section describes the steps every program must take to

use expanded memory.

In order to use expanded memory, applications must perform

these steps in the following order:

1.  Determine if EMM is installed.

2.  Determine if enough expanded memory pages exist for your

application.  (Function 3)

3.  Allocate expanded memory pages.  (Function 4, 18, or 27)

4.  Get the page frame base address.  (Function 2)

5.  Map in expanded memory pages.  (Function 5 or 17)

6.  Read/write/execute data in expanded memory, just as if

it were conventional memory.

7.  Return expanded memory pages to expand memory pool

before exiting.  (Function 6 or 18)

Table 2-1 overviews the functions while Chapter 3 describes

each of these functions in detail.  Example programs at the

end of this chapter illustrate using expanded memory.

Writing Programs That Use Expanded Memory                       4

Table 2-1.  The Basic Functions

----------------------------------------------------------------

Function                        Description

----------------------------------------------------------------

1         The Get Status Function returns a status code

indicating whether the memory manager hardware is

working correctly.

2         The Get Page Frame Address function returns the

address where the 64K-byte page frame is located.

3         The Get Unallocated Page Count function returns the

number of unallocated pages (pages available to your

program) and the total number of pages in expanded

memory.

4         The Allocate Pages function allocates the number of

pages requested and assigns a unique EMM handle to

these pages.

5         The Map/Unmap Handle Page function maps a logical

page to a specific physical page anywhere in the

mappable regions of system memory.

6         The Deallocate Pages deallocates the logical pages

currently allocated to an EMM handle.

7         The Get Version function returns the version number

of the memory manager software.

----------------------------------------------------------------

Advanced Programming

In addition to the basic functions, the Lotus/Intel/Micro-

soft Expanded Memory Specification provides several advanced

functions which enhance the capabilities of software that

uses expanded memory.

The following sections describe the advanced programming

capabilities and list the advanced EMM functions.

Note............................................................

Before using the advanced functions, programs should first

call Function 7 (Get Version) to determine whether the

installed version of EMM supports these functions.

Writing Programs That Use Expanded Memory                       5

Saving The State of Mapping Hardware

Some software (such as interrupt service routines, device

drivers, and resident software) must save the current state

of the mapping hardware, switch mapping contexts, manipulate

sections of expanded memory, and restore the original

context of the memory mapping hardware.  Use Functions 8 and

9 or 15 and 16 to save the state of the hardware.

Retrieving Handle and Page Counts

Some utility programs need to keep track of how expanded

memory is being used; use Functions 12 through 14 to do

this.

Mapping and Unmapping Multiple Pages

Mapping multiple pages reduces the overhead an application

must perform during mapping.  Function 17 lets a program map

(or unmap) multiple pages at one time.

In addition, you can map pages using segment addresses

instead of physical pages.  For example, if the page frame

base address is set to D000, you can map to either physical

page 0 or segment D000.  Function 25 (Get Mappable Physical

Address Array) returns a cross reference between all

expanded memory physical pages and their corresponding

segment values.

Reallocating Pages

Reallocating pages (Function 18) lets applications dynami-

cally allocate expanded memory pages without acquiring

another handle or obtain a handle without allocating pages.

Reallocating pages is an efficient means for applications to

obtain and release expanded memory pages.

Using Handles and Assigning Names to Handles

This specification lets you associate a name with a handle,

so a family of applications can share information in

expanded memory.  For example, a software package consisting

of a word processor, spreadsheet, and print spooler can

share the same data among the different applications.  The

print spooler could use a handle name to reference data that

either the spreadsheet or word processor put in expanded

memory and could check for data in a particular handle

name's expanded memory pages.

Writing Programs That Use Expanded Memory                       6

Use Function 20 (Set Handle Name subfunction) to assign a

handle name to an EMM handle or Function 21 (Search for

Named Handle subfunction) to obtain the EMM handle as-

sociated with the handle name.  In addition, you can use

Function 14 (Get Handle Pages) to determine the number of

expanded memory pages allocated to an EMM handle.

Using Handle Attributes

In addition to naming a handle, you can use Function 19 to

associate an attribute (volatile or non-volatile) with a

handle name.  A non-volatile attribute enables expanded

memory pages to preserve their data through a warmboot.

With a volatile attribute, the data is not preserved.  The

default attribute for handles is volatile.

Because using this function depends on the capabilities of

the expanded memory hardware installed in the system, you

should use the Get Attribute Capability subfunction before

attempting to assign an attribute to a handle's pages.

Altering Page Maps and Jumping/Calling

You can use Functions 22 (Alter Page Map & Jump) and 23

(Alter Page Map & Call) to map a new set of values into the

map registers and transfer program control to a specified

address within expanded memory.  These functions can be used

to load and execute code in expanded memory.  An application

using this feature can significantly reduce the amount of

conventional memory it requires.  Programs can load needed

modules into expanded memory at run time and use Functions

22 and 23 to transfer control to these modules.

Using expanded memory to store code can improve program

execution in many ways.  For example, sometimes programs

need to be divided into small overlays because of conven-

tional memory size limitations.  Overlays targeted for

expanded memory can be very large because LIM EMS 4.0

supports up to 32M bytes of expanded memory.  This method of

loading overlays improves overall system performance by

conserving conventional memory and eliminating conventional

memory allocation errors.

Moving or Exchanging Memory Regions

Using Function 24 (Move/Exchange Memory Region), you can

easily move and exchange data between conventional and

expanded memory.  Function 24 can manipulate up to one

megabyte of data with one function call.  Although applica-

Writing Programs That Use Expanded Memory                       7

tions can perform this operation without this function,

having the expanded memory manager do it reduces the amount

of overhead for the application.

In addition, this function checks for overlapping regions

and performs all the necessary mapping, preserving the

mapping context from before the exchange/move call.

Getting the Amount of Mappable Memory

Function 25 enables applications to determine the total

amount of mappable memory the hardware/system currently

supports.  Not all expanded memory boards supply the same

number of physical pages (map registers).

The Get Mappable Physical Address Array Entries subfunction

returns the total number of physical pages the expanded

memory hardware/system is capable of supporting.  The Get

Mappable Physical Array subfunction returns a cross refer-

ence between physical page numbers and the actual segment

address for each of the physical pages.

Operating System Functions

In addition to the functions for application programs, this

specification defines functions for operating systems/en-

vironments.  These functions can be disabled at any time by

the operating system/environment, so programs should not

depend on their presence.  Applications that avoid this

warning and use these functions run a great risk of being

incompatible with other programs, including the operating

system.

Writing Programs That Use Expanded Memory                       8

Table 2-2.  The Advanced Functions

----------------------------------------------------------------

Function                        Description

----------------------------------------------------------------

8         The Save Page Map saves the contents of the page

mapping registers from all expanded memory boards in

an internal save area.

9         The Restore Page Map function restores (from an

internal save area) the page mapping register

contents on the expanded memory boards for a

particular EMM handle.

10        Reserved.

11        Reserved.

12        The Get Handle Count function returns the number of

open EMM handles in the system.

13        The Get Handle Pages function returns the number of

pages allocated to a specific EMM handle.

14        The Get All Handle Pages function returns an array

of the active EMM handles and the number of pages

allocated to each one.

15        The Get/Set Page Map subfunction saves or restores

the mapping context for all mappable memory regions

(conventional and expanded) in a destination array

which the application supplies.

16        The Get/Set Partial Page Map subfunction provides a

mechanism for saving a partial mapping context for

specific mappable memory regions in a system.

17        The Map/Unmap Multiple Handle Pages function can, in

a single invocation, map (or unmap) logical pages

into as many physical pages as the system supports.

18        The Reallocate Pages function can increase or

decrease the amount of expanded memory allocated to

a handle.

19        The Get/Set Handle Attribute function allows an

application program to determine and set the

attribute associated with a handle.

Writing Programs That Use Expanded Memory                       9

Table 2-2.  The Advanced Functions (continued)

----------------------------------------------------------------

Function                        Description

----------------------------------------------------------------

20        The Get/Set Handle Name function gets the eight

character name currently assigned to a handle and

can assign an eight character name to a handle.

21        The Get Handle Directory function returns informa-

tion about active handles and the names assigned to

each.

22        The Alter Page Map & Jump function alters the memory

mapping context and transfers control to the

specified address.

23        The Alter Page Map & Call function alters the speci-

fied mapping context and transfers control to the

specified address.  A return can then restore the

context and return control to the caller.

24        The Move/Exchange Memory Region function copies or

exchanges a region of memory from conventional to

conventional memory, conventional to expanded

memory, expanded to conventional memory, or expanded

to expanded memory.

25        The Get Mappable Physical Address Array function

returns an array containing the segment address and

physical page number for each mappable physical page

in a system.

26        The Get Expanded Memory Hardware Information

function returns an array containing the hardware

capabilities of the expanded memory system.

27        The Allocate Standard/Raw Pages function allocates

the number of standard or non-standard size pages

that the operating system requests and assigns a

unique EMM handle to these pages.

28        The Alternate Map Register Set function enables an

application to simulate alternate sets of hardware

mapping registers.

29        The Prepare Expanded Memory Hardware for Warm Boot

function prepares the expanded memory hardware for

an "impending" warm boot.

Writing Programs That Use Expanded Memory                      10

Table 2-2.  The Advanced Functions (continued)

----------------------------------------------------------------

Function                        Description

----------------------------------------------------------------

30        The Enable/Disable OS/E function enables operating

systems developers to enable and disable functions

designed for operating system use.

----------------------------------------------------------------

Writing Programs That Use Expanded Memory                      11

Programming Guidelines

The following section contains guidelines for programmers

writing applications that use EMM.

o   Do not put a program's stack in expanded memory.

o   Do not replace interrupt 67h.  This is the interrupt

vector the EMM uses.  Replacing interrupt 67h could

result in disabling the Expanded Memory Manager.

o   Do not map into conventional memory address space your

application doesn't own.  Applications that use the EMM

to swap into conventional memory space, must first

allocate this space from the operating system.  If the

operating system is not aware that a region of memory it

manages is in use, it will think it is available.  This

could have disastrous results.  EMM should not be used

to "allocate" conventional memory.  DOS is the proper

manager of conventional memory space.  EMM should only

be used to swap data in conventional memory space

previously allocated from DOS.

o   Applications that plan on using data aliasing in

expanded memory must check for the presence of expanded

memory hardware.  Data aliasing occurs when mapping one

logical page into two or more mappable segments.  This

makes one 16K-byte expanded memory page appear to be in

more than one 16K-byte memory address space.  Data

aliasing is legal and sometimes useful for applications.

Software-only expanded memory emulators cannot perform

data aliasing.  A simple way to distinguish software

emulators from actual expanded memory hardware is to

attempt data aliasing and check the results.  For

example, map one logical page into four physical pages.

Write to physical page 0.  Read physical pages 1-3 to

see if the data is there as well.  If the data appears

in all four physical pages, then expanded memory

hardware is installed in the system, and data aliasing

is supported.

o   Applications should always return expanded memory pages

to the expanded memory manager upon termination.  These

pages will be made available for other applications.  If

unneeded pages are not returned to the expanded memory

manager, the system could "run out" of expanded memory

pages or expanded memory handles.

o   Terminate and stay resident programs (TSR's) should

ALWAYS save the state of the map registers before

changing them.  Since TSR's may interrupt other programs

Writing Programs That Use Expanded Memory                      12

which may be using expanded memory, they must not change

the state of the page mapping registers without first

saving them.  Before exiting, TSR's must restore the

state of the map registers.

The following sections describe the three ways to save

and restore the state of the map registers.

1.  Save Page Map and Restore Page Map (Functions 8 and

9).  This is the simplest of the three methods.  The

EMM saves the map register contents in its own data

structures -- the application does not need to

provide extra storage locations for the mapping

context.  The last mapping context to be saved,

under a particular handle, will be restored when a

call to Restore Page Map is issued with the same

handle.  This method is limited to one mapping

context for each handle and saves the context for

only LIM standard 64K-byte page frames.

2.  Get/Set Page Map (Function 15).  This method

requires the application to allocate space for the

storage array.  The EMM saves the mapping context in

an array whose address is passed to the EMM.  When

restoring the mapping context with this method, an

application passes the address of an array which

contains a previously stored mapping context.

This method is preferable if an application needs to

do more than one save before a restore.  It provides

a mechanism for switching between more than one

mapping context.

3.  Get/Set Partial Page Map (Function 16).  This method

provides a way for saving a partial mapping context.

It should be used when the application does not need

to save the context of all mappable memory.  This

function also requires that the storage array be

part of the application's data.

o   All functions using pointers to data structures must

have those data structures in memory which will not be

mapped out.  Functions 22 and 23 (Alter Map & Call and

Alter Map & Jump) are the only exceptions.

Writing Programs That Use Expanded Memory                      13

Examples

This section lists four example programs that demonstrate

the use of expanded memory.

Example 1

This program was written using the Microsoft C compiler

Version 3.0.  EMM function calls are made with the int86

function found in the dos.h library.  To create an ex-

ecutable program use the following compile command line:

msc /Gs /Oat /Ml program,,program;

#include <dos.h>

#include <stdio.h>

#define EMM_INT                 0x67  /* EMM interrupt number */

#define GET_PAGE_FRAME          0x41  /* EMM get page frame */

/* function number */

#define GET_UNALLOC_PAGE_COUNT  0x42  /* EMM get unallocated */

/* page count */

/* function number */

#define ALLOCATE_PAGES          0x43  /* EMM allocate pages */

/* function number */

#define MAP_PAGES               0x44  /* EMM map pages */

/* function number */

#define DEALLOCATE_PAGES        0x45  /* EMM deallocate pages */

/* function number */

#define DEVICE_NAME_LENGTH      8     /* length of a device */

/* name string */

#define TRUE                    1

#define FALSE                   0

union REGS input_regs, output_regs;

struct SREGS segment_regs;

int pf_addr;

/*------------------------------------------------------------*/

/* Routine to convert a segment:offset pair to a far ptr.     */

/*------------------------------------------------------------*/

char *build_ptr (segment, offset)

unsigned int segment;

unsigned int offset;

{

char *ptr;

ptr = (char *)(((unsigned long)segment << 16) + offset);

return (ptr);

}

Writing Programs That Use Expanded Memory                      14

/*------------------------------------------------------------*/

/* Function which determines whether EMM device driver        */

/* is installed.                                              */

/*------------------------------------------------------------*/

char emm_installed()

{

char *EMM_device_name = "EMMXXXX0";

char *int_67_device_name_ptr;

/*--------------------------------------------------------*/

/* AH = DOS get interrupt vector function.                */

/*--------------------------------------------------------*/

input_regs.h.ah = 0x35;

/*--------------------------------------------------------*/

/* AL = EMM interrupt vector number.                      */

/*--------------------------------------------------------*/

input_regs.h.al = EMM_INT;

intdosx (&input_regs, &output_regs, &segment_regs);

/*--------------------------------------------------------*/

/* Upon return ES:0Ah points to location where            */

/* device name should be.                                 */

/*--------------------------------------------------------*/

int_67_device_name_ptr = build_ptr (segment_regs.es, 0x0A);

/*--------------------------------------------------------*/

/* Compare memory with EMM device name.                   */

/*--------------------------------------------------------*/

if (memcmp (EMM_device_name, int_67_device_name_ptr,

DEVICE_NAME_LENGTH) == 0)

return (TRUE);

else

return (FALSE);

}

/*------------------------------------------------------------*/

/* Function which determines if there are enough unallocated  */

/* expanded memory pages for the application.                 */

/*------------------------------------------------------------*/

char enough_unallocated_pages (pages_needed)

int pages_needed;

{

input_regs.h.ah = GET_UNALLOCATED_PAGE_COUNT;

int86 (EMM_INT, &input_regs, &output_regs);

if (output_regs.h.ah != 0 || pages_needed > output_regs.x.bx)

return (FALSE);

else

return (TRUE);

}

Writing Programs That Use Expanded Memory                      15

/*------------------------------------------------------------*/

/* Function which allocates expanded memory pages and passes  */

/* back to the main EMM handle.                               */

/*------------------------------------------------------------*/

char allocate_expanded_memory_pages (pages_needed,emm_handle_ptr)

int pages_needed;

unsigned int *emm_handle_ptr;

{

input_regs.h.ah = ALLOCATE_PAGES;

input_regs.x.bx = pages_needed;

int86 (EMM_INT, &input_regs, &output_regs);

if (output_regs.h.ah == 0) {

*emm_handle_ptr = output_regs.x.dx;

return (TRUE);

}

else

return (FALSE);

}

/*------------------------------------------------------------*/

/* Routine to map a logical page to a physical page.          */

/*------------------------------------------------------------*/

char map_expanded_memory_pages (emm_handle, physical_page,

logical_page)

unsigned int emm_handle;

int physical_page;

int logical_page;

{

input_regs.h.ah = MAP_PAGES;

input_regs.h.al = physical_page;

input_regs.x.bx = logical_page;

input_regs.x.dx = emm_handle;

int86 (EMM_INT, &input_regs, &output_regs);

if (output_regs.h.ah == 0)

return (TRUE);

else

return (FALSE);

}

Writing Programs That Use Expanded Memory                      16

/*------------------------------------------------------------*/

/* Routine which gets the page frame base address from EMM.   */

/*------------------------------------------------------------*/

char get_page_frame_address (pf_ptr)

char **pf_ptr;

{

input_regs.h.ah = GET_PAGE_FRAME;

int86 (EMM_INT, &input_regs, &output_regs);

if (output_regs.h.ah != 0)       /* check EMM status */

return (FALSE);

else

*pf_ptr = build_ptr (output_regs.x.bx, 0);

return (TRUE);

}

/*------------------------------------------------------------*/

/* Routine to release all expanded memory pages allocated     */

/* by an EMM handle.                                          */

/*------------------------------------------------------------*/

char deallocate_expanded_memory_pages (emm_handle)

unsigned int emm_handle;

{

input_regs.h.ah = DEALLOCATE_PAGES;

input_regs.x.dx = emm_handle;

int86 (EMM_INT, &input_regs, &output_regs);

if (output_regs.h.ah == 0)

return (TRUE);

else

return (FALSE);

}

main()

{

unsigned int emm_handle;

char *pf_addr;

int pages_needed;

int physical_page;

int logical_page;

int index;

/*--------------------------------------------------------*/

/* Determine if EMM is installed.                         */

/*--------------------------------------------------------*/

if (!emm_installed())

exit(1);

Writing Programs That Use Expanded Memory                      17

/*--------------------------------------------------------*/

/* Determine if enough expanded memory pages exist for    */

/* application.                                           */

/*--------------------------------------------------------*/

pages_needed = 1;

if (!enough_unallocated_pages (pages_needed))

exit(1);

/*--------------------------------------------------------*/

/* Allocate expanded memory pages.                        */

/*--------------------------------------------------------*/

if (!allocate_expanded_memory_pages (pages_needed,

&emm_handle))

exit(1);

/*--------------------------------------------------------*/

/* Map in the required pages.                             */

/*--------------------------------------------------------*/

physical_page = 0;

logical_page = 0;

if (!map_expanded_memory_pages (emm_handle, physical_page,

logical_page))

exit(1);

/*--------------------------------------------------------*/

/* Get expanded memory page frame address.                */

/*--------------------------------------------------------*/

if (!get_page_frame_address (&pf_addr))

exit(1);

/*--------------------------------------------------------*/

/* Write to expanded memory.                              */

/*--------------------------------------------------------*/

for (index = 0; index < 0x3fff; index++)

pf_addr[index] = index;

/*--------------------------------------------------------*/

/* Return expanded memory pages before exiting.           */

/*--------------------------------------------------------*/

if (!deallocate_expanded_memory_pages (emm_handle))

exit(1);

}

Writing Programs That Use Expanded Memory                      18

Example 2

This program shows you how to use the basic functions of the LIM

Expanded Memory Specification with Turbo Pascal.  The program

does the following:

1.  Makes sure the LIM Expanded Memory Manager (EMM) has

been installed.

2.  Displays the version number of the EMM.

3.  Determines if there are enough pages of memory for the

program.  It then displays the total number of EMM pages

present in the system and the number available for use.

4.  Requests the desired number of pages from the EMM.

5.  Maps a logical page into one of the physical pages.

6.  Displays the base address of our EMM memory page frame.

Performs a simple read/write test on the EMM memory.

7.  Returns the EMM memory given to us back to the EMM.

8.  Exits.

All the calls are structured to return the result or error code

of the Expanded Memory function performed as an integer.  If the

error code is not zero, an error has occurred, a simple error

procedure is called, and the program terminates.

Type

ST3  = string[3];

ST80 = string[80];

ST5  = string[5];

Registers = record

case integer of

1: (AX,BX,CX,DX,BP,SI,DI,DS,ES,FLAGS: Integer);

2: (AL,AH,BL,BH,CL,CH,DL,DH         : Byte);

end;

Const

EMM_INT                    = $67;

DOS_Int                    = $21;

GET_PAGE_FRAME             = $41;

GET_UNALLOCATED_PAGE_COUNT = $42;

ALLOCATE_PAGES             = $43;

MAP_PAGES                  = $44;

DEALLOCATE_PAGES           = $45;

GET_VERSION                = $46;

STATUS_OK                  = 0;

Writing Programs That Use Expanded Memory                      19

{------------------------------------------------------------}

{ Assume the application needs one EMM page.                 }

{------------------------------------------------------------}

APPLICATION_PAGE_COUNT = 1;

Var

Regs: Registers;

Emm_handle,

Page_Frame_Base_Address,

Pages_Needed,

Physical_Page,

Logical_Page,

Offset,

Error_Code,

Pages_EMM_Available,

Total_EMM_Pages,

Available_EMM_Pages: Integer;

Version_Number,

Pages_Number_String: ST3;

Verify: Boolean;

{------------------------------------------------------------}

{ The function Hex_String converts an integer into a four    }

{ character hexadecimal number (string) with leading zeros.  }

{------------------------------------------------------------}

Function Hex_String (Number: Integer): ST5;

Function Hex_Char (Number: Integer): Char;

Begin

If Number < 10 then

Hex_Char := Char (Number + 48)

else

Hex_Char := Char (Number + 55);

end; { Function Hex_char }

Var

S: ST5;

Begin

S := '';

S := Hex_Char ((Number shr 1) div 2048);

Number := (((Number shr 1) mod 2048) shl 1) + (Number and 1);

S := S + Hex_Char (Number div 256);

Number := Number mod 256;

S := S + Hex_Char (Number div 16);

Number := Number mod 16;

S := S + Hex_Char (Number);

Hex_String := S + 'h';

end; { Function Hex_String }

Writing Programs That Use Expanded Memory                      20

{------------------------------------------------------------}

{ The function Emm_Installed checks to see if the            }

{ EMM is loaded in memory.  It does this by looking          }

{ for the string 'EMMXXXX0', which should be located         }

{ at 10 bytes from the beginning of the code segment the     }

{ EMM interrupt, 67h, points to.                             }

{------------------------------------------------------------}

Function Emm_Installed: Boolean;

Var

Emm_Device_Name   : string[8];

Int_67_Device_Name: string[8];

Position          : integer;

Regs              : registers;

Begin

Int_67_Device_Name := '';

Emm_Device_Name    := 'EMMXXXX0';

with Regs do

Begin

{----------------------------------------------------}

{ Get the code segment interrupt 67h points to       }

{ the EMM interrupt by using DOS function 35h.       }

{ (get interrupt vector)                             }

{----------------------------------------------------}

AH := $35;

AL := EMM_INT;

Intr (DOS_Int, Regs);

{----------------------------------------------------}

{ The ES pseudo-register contains the segment        }

{ address pointed to by interrupt 67h.  Create an    }

{ eight character string from the eight successive   }

{ bytes at address ES:$000A (10 bytes from ES)       }

{----------------------------------------------------}

For Position := 0 to 7 do

Int_67_Device_Name :=

Int_67_Device_Name + Chr (mem[ES:Position + $0A]);

Emm_Installed := True;

{----------------------------------------------------}

{ If the string is the EMM manager signature,        }

{ 'EMMXXXX0', then EMM is installed and ready for    }

{ use.  If not, then EMM is not present.             }

{----------------------------------------------------}

If Int_67_Device_Name <> Emm_Device_Name

then Emm_Installed := False;

end; { with Regs do }

end; { Function Emm_Installed }

Writing Programs That Use Expanded Memory                      21

{------------------------------------------------------------}

{ This function returns the total number of EMM pages        }

{ present in the system, and the number of EMM pages that    }

{ are available.                                             }

{------------------------------------------------------------}

Function EMM_Pages_Available

(Var Total_EMM_Pages, Pages_Available: Integer): Integer;

Var

Regs: Registers;

Begin

with Regs do

Begin

{----------------------------------------------------}

{ Get the number of currently unallocated pages and  }

{ the total number of pages in the system from EMM.  }

{ Load pseudo-registers prior to invoking EMM.       }

{    AH = get unallocated page count function        }

{----------------------------------------------------}

AH := GET_UNALLOCATED_PAGE_COUNT;

Intr (EMM_INT, Regs);

{----------------------------------------------------}

{ Unload the pseudo-registers after invoking EMM.    }

{    BX = currently unallocated pages                }

{    DX = total pages in the system                  }

{    AH = status                                     }

{----------------------------------------------------}

Pages_Available := BX;

Total_EMM_Pages := DX;

EMM_Pages_Available := AH;

end;

end; { Function EMM_Pages_Available }

{------------------------------------------------------------}

{ This function requests the specified number of pages       }

{ from the EMM.                                              }

{------------------------------------------------------------}

Function Allocate_Expanded_Memory_Pages

(Pages_Needed: Integer; Var Handle: Integer): Integer;

Var

Regs: Registers;

Writing Programs That Use Expanded Memory                      22

Begin

with Regs do

Begin

{----------------------------------------------------}

{ Allocate the specified number of pages from EMM.   }

{ Load pseudo-registers prior to invoking EMM.       }

{    AH = allocate pages function.                   }

{    BX = number of pages to allocate.               }

{----------------------------------------------------}

AH := ALLOCATE_PAGES;

BX := Pages_Needed;

Intr (EMM_INT, Regs);

{----------------------------------------------------}

{ Unload the pseudo-registers after invoking EMM.    }

{    DX = EMM handle                                 }

{    AH = status                                     }

{----------------------------------------------------}

Handle := DX;

Allocate_Expanded_Memory_Pages := AH;

end;

end; { Function Allocate_Expanded_Memory_Pages }

{------------------------------------------------------------}

{ This function maps a logical page allocated by the         }

{ Allocate_Expanded_Memory_Pages function into one of the    }

{ four physical pages.                                       }

{------------------------------------------------------------}

Function Map_Expanded_Memory_Pages

(Handle, Logical_Page, Physical_Page: Integer): Integer;

Var

Regs: Registers;

Begin

with Regs do

Begin

{----------------------------------------------------}

{ Map a logical page at a physical page.             }

{ Load pseudo-registers prior to invoking EMM.       }

{    AH = map page function                          }

{    DX = handle                                     }

{    BX = logical page number                        }

{    AL = physical page number                       }

{----------------------------------------------------}

AH := MAP_PAGES;

DX := Handle;

BX := Logical_Page;

AL := Physical_Page;

Intr (EMM_INT, Regs);

Writing Programs That Use Expanded Memory                      23

{----------------------------------------------------}

{ Unload the pseudo-registers after invoking EMM.    }

{    AH = status                                     }

{----------------------------------------------------}

Map_Expanded_Memory_Pages := AH;

end; { with Regs do }

end; { Function Map_Expanded_Memory_Pages }

{------------------------------------------------------------}

{ This function gets the physical address of the EMM page    }

{ frame we are using.  The address returned is the segment   }

{ of the page frame.                                         }

{------------------------------------------------------------}

Function Get_Page_Frame_Base_Address

(Var Page_Frame_Address: Integer): Integer;

Var

Regs: Registers;

Begin

with Regs do

Begin

{----------------------------------------------------}

{ Get the page frame segment address from EMM.       }

{ Load pseudo-registers prior to invoking EMM.       }

{    AH = get page frame segment function            }

{----------------------------------------------------}

AH := GET_PAGE_FRAME;

Intr (EMM_INT, Regs);

{----------------------------------------------------}

{ Unload the pseudo-registers after invoking EMM.    }

{    BX = page frame segment address                 }

{    AH = status                                     }

{----------------------------------------------------}

Page_Frame_Address := BX;

Get_Page_Frame_Base_Address := AH;

end; { with Regs do }

end; { Function Get_Page_Frame_Base_Address }

{------------------------------------------------------------}

{ This function releases the EMM memory pages allocated to   }

{ us, back to the EMM memory pool.                           }

{------------------------------------------------------------}

Function Deallocate_Expanded_Memory_Pages

(Handle: Integer): Integer;

Var

Regs: Registers;

Writing Programs That Use Expanded Memory                      24

Begin

with Regs do

Begin

{----------------------------------------------------}

{ Deallocate the pages allocated to an EMM handle.   }

{ Load pseudo-registers prior to invoking EMM.       }

{    AH = deallocate pages function                  }

{    DX = EMM handle                                 }

{----------------------------------------------------}

AH := DEALLOCATE_PAGES;

DX := Handle;

Intr (EMM_INT, Regs);

{----------------------------------------------------}

{ Unload the pseudo-registers after invoking EMM.    }

{    AH = status                                     }

{----------------------------------------------------}

Deallocate_Expanded_Memory_Pages := AH;

end; { with Regs do }

end; { Function Deallocate_Expanded_Memory_Pages }

{------------------------------------------------------------}

{ This function returns the version number of the EMM as     }

{ a three-character string.                                  }

{------------------------------------------------------------}

Function Get_Version_Number (Var Version_String: ST3): Integer;

Var

Regs: Registers;

Integer_Part, Fractional_Part: Char;

Begin

with Regs do

Begin

{----------------------------------------------------}

{ Get the version of EMM.                            }

{ Load pseudo-registers prior to invoking EMM.       }

{    AH = get EMM version function                   }

{----------------------------------------------------}

AH := GET_VERSION;

Intr (EMM_INT, Regs);

Writing Programs That Use Expanded Memory                      25

{----------------------------------------------------}

{ If the version number returned was OK, then        }

{ convert it to a three-character string.            }

{----------------------------------------------------}

If AH=STATUS_OK then

Begin

{------------------------------------------------}

{ The upper four bits of AH are the integer      }

{ portion of the version number, the lower four  }

{ bits are the fractional portion.  Convert the  }

{ integer value to ASCII by adding 48.           }

{------------------------------------------------}

Integer_Part    := Char (AL shr 4  + 48);

Fractional_Part := Char (AL and $F + 48);

Version_String  := Integer_Part + '.' +

Fractional_Part;

end; { If AH=STATUS_OK }

{----------------------------------------------------}

{ Unload the pseudo-registers after invoking EMM.    }

{    AH = status                                     }

{----------------------------------------------------}

Get_Version_Number := AH;

end; { with Regs do }

end; { Function Get_Version_Number }

{------------------------------------------------------------}

{ This procedure prints an error message passed by the       }

{ caller, prints the error code passed by the caller in hex, }

{ and then terminates the program with an error level of 1.  }

{------------------------------------------------------------}

Procedure Error (Error_Message: ST80; Error_Number: Integer);

Begin

Writeln (Error_Message);

Writeln ('  Error_Number = ', Hex_String (Error_Number));

Writeln ('EMM test program aborting.');

Halt (1);

end; { Procedure Error }

{--------------------------------------------------------------}

{ This program is an example of the basic EMM functions that   }

{ you need in order to use EMM memory with Turbo Pascal.       }

{--------------------------------------------------------------}

Begin

ClrScr;

Window (5,2,77,22);

Writing Programs That Use Expanded Memory                      26

{------------------------------------------------------------}

{ Determine if the Expanded Memory Manager is installed.  If }

{ not, then terminate 'main' with an ErrorLevel code of 1.   }

{------------------------------------------------------------}

If not (Emm_Installed) then

Begin

Writeln ('The LIM EMM is not installed.');

Halt (1);

end

else

Begin

{ Get the version number and display it }

Error_Code := Get_Version_Number (Version_Number);

If Error_Code <> STATUS_OK then

Error ('Error getting EMM version number.', Error_Code)

else

Writeln ('LIM Expanded Memory Manager, version ',

Version_Number, ' is ready for use.');

end;

Writeln;

{------------------------------------------------------------}

{ Determine if there are enough expanded memory pages for    }

{ this application.                                          }

{------------------------------------------------------------}

Pages_Needed := APPLICATION_PAGE_COUNT;

Error_Code   := EMM_Pages_Available (Total_EMM_Pages,

Available_EMM_Pages);

If Error_Code <> STATUS_OK then

Error ('Error determining number of EMM pages available.',

Error_Code);

Writeln ('There are a total of ', Total_EMM_Pages,

' expanded memory pages present in this system.');

Writeln ('  ', Available_EMM_Pages,

' of those pages are available for use.');

Writeln;

{------------------------------------------------------------}

{ If there is an insufficient number of pages for the        }

{ application, then report the error and terminate the EMM   }

{ example program.                                           }

{------------------------------------------------------------}

If Pages_Needed > Available_EMM_Pages then

Begin

Str (Pages_Needed, Pages_Number_String);

Error ('We need ' + Pages_Number_String +

' EMM pages.  There are not that many available.',

Error_Code);

end; { Pages_Needed > Available_EMM_Pages }

Writing Programs That Use Expanded Memory                      27

{------------------------------------------------------------}

{ Allocate expanded memory pages for our use.                }

{------------------------------------------------------------}

Error_Code :=

Allocate_Expanded_Memory_Pages (Pages_Needed, Emm_Handle);

Str (Pages_Needed, Pages_Number_String);

If Error_Code <> STATUS_OK then

Error ('EMM test program failed trying to allocate '

+ Pages_Number_String

+ ' pages for usage.', Error_Code);

Writeln (APPLICATION_PAGE_COUNT,

' EMM page(s) allocated for the EMM test program.');

Writeln;

{------------------------------------------------------------}

{ Map in the required logical pages to the physical pages    }

{ given to us, in this case just one page.                   }

{------------------------------------------------------------}

Logical_Page  := 0;

Physical_Page := 0;

Error_Code := Map_Expanded_Memory_Pages (Emm_Handle,

Logical_Page,

Physical_Page);

If Error_Code <> STATUS_OK then

Error ('EMM test program failed trying to map '

+ 'logical pages into physical pages.',

Error_Code);

Writeln ('Logical Page ',

Logical_Page,

' successfully mapped into Physical Page ',

Physical_Page);

Writeln;

{------------------------------------------------------------}

{ Get the expanded memory page frame address.                }

{------------------------------------------------------------}

Error_Code := Get_Page_Frame_Base_Address

(Page_Frame_Base_Address);

If Error_Code <> STATUS_OK then

Error ('EMM test program unable to get the base Page'

+ ' Frame Address.',

Error_Code);

Writeln ('The base address of the EMM page frame is = '

+ Hex_String (Page_Frame_Base_Address));

Writeln;

Writing Programs That Use Expanded Memory                      28

{------------------------------------------------------------}

{ Write a test pattern to expanded memory.                   }

{------------------------------------------------------------}

For Offset := 0 to 16382 do

Begin

Mem[Page_Frame_Base_Address:Offset] := Offset mod 256;

end;

{------------------------------------------------------------}

{ Make sure that what is in EMM memory is what was just      }

{ written.                                                   }

{------------------------------------------------------------}

Writeln ('Testing EMM memory.');

Offset := 1;

Verify := True;

while (Offset <= 16382) and (Verify = True) do

Begin

If Mem[Page_Frame_Base_Address:Offset] <> Offset mod 256

then Verify := False;

Offset := Succ (Offset);

end; { while (Offset <= 16382) and (Verify = True) }

{------------------------------------------------------------}

{ If what is read does not match what was written,           }

{ an error occurred.                                         }

{------------------------------------------------------------}

If not Verify then

Error ('What was written to EMM memory was not found during'

+ ' memory verification test.',

0);

Writeln ('EMM memory test successful.');

Writeln;

{------------------------------------------------------------}

{ Return the expanded memory pages given to us back to the   }

{ EMM memory pool before terminating our test program.       }

{------------------------------------------------------------}

Error_Code := Deallocate_Expanded_Memory_Pages (Emm_Handle);

If Error_Code <> STATUS_OK then

Error ('EMM test program was unable to deallocate '

+ 'the EMM pages in use.',

Error_Code);

Writeln (APPLICATION_PAGE_COUNT,

' pages(s) deallocated.');

Writeln;

Writeln ('EMM test program completed.');

end.

Writing Programs That Use Expanded Memory                      29

Example 3

This program is written in Microsoft's macro assembler.

CODE SEGMENT

ASSUME CS:CODE, DS:CODE

MOV   AX,CS

MOV   DX,AX

.

.

.

check_emm_installed:

MOV   AH,35h                  ; AH = DOS get interrupt vector

; function

MOV   AL,67h                  ; AL = EMM interrupt vector number

INT   21h

MOV   DI,0Ah                  ; ES:DI points to where device

; name should be

LEA   SI,EMM_device_name      ; DS:SI points to ASCII string

; containing EMM device name

MOV   CX,device_name_length   ; set up loop counter for string op

CLD                           ; set up direction flag for forward

REPE  CMPSB                   ; Compare the strings

JNE   exit                    ; IF strings not equal THEN exit

; ELSE

check_enough_unallocated_pages:

MOV   AH,41h                  ;    AH = EMM get unallocated page

;    count function code

INT   67h

OR    AH,AH                   ; Check EMM status

JNZ   emm_error_handler       ; IF error THEN goto error handler

; ELSE

allocate_expanded_memory_pages:

MOV   AH,43h                  ;    AH = EMM allocate pages

;    function code

MOV   BX,2                    ;    BX = number of pages needed

INT   67h

OR    AH,AH                   ; Check EMM status

JNZ   emm_error_handler       ; IF error THEN goto error handler

; ELSE

MOV   emm_handle,DX           ;    save EMM handle

map_expanded_memory_pages:

MOV   AH,44h                  ; AH = EMM map pages function

MOV   DX,emm_handle           ; DX = application's handle

Writing Programs That Use Expanded Memory                      30

map_0_to_0:

MOV   BX,0                    ; BX = logical page 0

MOV   AL,0                    ; AL = physical page 0

INT   67h

OR    AH,AH                   ; Check EMM status

JNZ   emm_error_handler       ; If error THEN goto error handler

; ELSE

get_page_frame_address:

MOV   AH,41h                  ; AH = EMM get page frame base

; address function

INT   67h

OR    AH,AH                   ; Check EMM status

JNZ   emm_error_handler       ; IF error THEN goto error handler

MOV   pf_addr,BX              ; ELSE save pf_addr

write_to_expanded_memory:     ; Write zeros to memory mapped at

; physical page 0

MOV   AX,pf_addr

MOV   ES,AX                   ; ES points to physical page 0

MOV   DI,0                    ; DI indexes into physical page 0

MOV   AL,0                    ; Initialize AL for string STOSB

MOV   CX,4000h                ; Initialize loop counter to length

; of expanded memory page size

CLD                           ; set up direction flag for forward

REP   STOSB

deallocate_pages:

MOV   AH,45h                  ; AH = EMM deallocate pages

; function

MOV   DX,emm_handle

INT   67h                     ; return handle's pages to EMM

OR    AH,AH                   ; Check EMM status

JNZ   emm_error_handler       ; IF error THEN goto error handler

exit:

MOV   AH,4Ch                  ; AH = DOS exit function

INT   21h                     ; return to DOS

EMM_device_name DB 'EMMXXXX0' ; ASCII EMM device name string

device_name_length EQU 8

CODE ENDS

END

Writing Programs That Use Expanded Memory                      31

Example 4

This program is an example of how to exchange a 256K-byte

block of data from conventional memory to expanded memory.

CODE SEGMENT

ASSUME CS:CODE, DS:CODE

.

.

.

xchg_packet_set_up:

;DS:SI = xchg_packet

MOV   AX,SEG xchg_packet

MOV   DS,AX

MOV   SI,OFFSET xchg_packet

;Moving 256K of data from conventional memory to expanded memory

MOV   WORD PTR [SI].region_length[0],0

MOV   WORD PTR [SI].region_length[2],4

MOV   [SI].src_mem_type,0

MOV   [SI].dest_mem_type,1

;starting at segment: 4000h, offset: 0

MOV   [SI].src_init_seg_page,4000h

MOV   [SI].src_init_offset,0

;Move data into expanded memory logical page 0, offset 0.

MOV   [SI].dest_init_seg_page,0

MOV   [SI].dest_init_offset,0

;Initialize for future compatibility

MOV   [SI].src_handle,0

;Need handle for expanded memory destination.

MOV   DX,emm_handle

MOV   [SI].dest_handle,DX

;AX = EMM Exchange Memory function

MOV   AX,5701h

INT   67h

OR    AH,AH

JNZ   emm_error_handler

Writing Programs That Use Expanded Memory                      32

xchg_struct                    STRUC

region_length               DD ?

src_mem_type                DB ?

src_handle                  DW ?

src_init_offset             DW ?

src_init_seg_page           DW ?

dest_mem_type               DB ?

dest_handle                 DW ?

dest_init_offset            DW ?

dest_init_seg_page          DW ?

xchg_struct                    ENDS

xchg_packet                    xchg_struct

CODE  ENDS

END

Writing Programs That Use Expanded Memory                      33

Chapter 3

EMM FUNCTIONS

This chapter provides you with a standardized set of

expanded memory functions.  Because they are standardized,

you avoid potential compatibility problems with other

expanded memory programs that also adhere to the memory

manager specification.  Programs that deal directly with the

hardware or that don't adhere to this specification will be

incompatible.

Table 3-1 presents a sequential list of the EMM functions.

The remainder of this chapter provides detailed descriptions

of each function.

Table 3-1.  List of EMM Functions

----------------------------------------------------------------

Number             Function Name                Hex Value   Page

----------------------------------------------------------------

1    Get Status                                40h         37

2    Get Page Frame Address                    41h         38

3    Get Unallocated Page Count                42h         40

4    Allocate Pages                            43h         42

5    Map/Unmap Handle Page                     44h         46

6    Deallocate Pages                          45h         49

7    Get Version                               46h         51

8    Save Page Map                             47h         53

9    Restore Page Map                          48h         55

10    Reserved                                  49h         57

11    Reserved                                  4Ah         58

12    Get Handle Count                          4Bh         59

13    Get Handle Pages                          4Ch         61

14    Get All Handle Pages                      4Dh         63

EMM Functions                                                  34

Table 3-1.  List of EMM Functions (continued)

----------------------------------------------------------------

Number             Function Name                Hex Value   Page

----------------------------------------------------------------

15    Get Page Map                              4E00h       65

Set Page Map                              4E01h       67

Get & Set Page Map                        4E02h       69

Get Size of Page Map Save Array           4E03h       71

16    Get Partial Page Map                      4F00h       73

Set Partial Page Map                      4F01h       76

Get Size of Partial Page Map

Save Array                             4F02h       78

17    Map/Unmap Multiple Handle Pages

(Physical page number mode)               5000h       82

Map/Unmap Multiple Handle Pages

(Segment address mode)                    5001h       85

18    Reallocate Pages                          51h         88

19    Get Handle Attribute                      5200h       92

Set Handle Attribute                      5201h       94

Get Handle Attribute Capability           5202h       96

20    Get Handle Name                           5300h       98

Set Handle Name                           5301h      100

21    Get Handle Directory                      5400h      102

Search for Named Handle                   5401h      105

Get Total Handles                         5402h      107

22    Alter Page Map & Jump

(Physical page number mode)               5500h      109

Alter Page Map & Jump

(Segment address mode)                    5501h      109

23    Alter Page Map & Call

(Physical page number mode)               5600h      113

Alter Page Map & Call

(Segment address mode)                    5601h      113

Get Page Map Stack Space Size             5602h      118

24    Move Memory Region                        5700h      120

Exchange Memory Region                    5701h      126

25    Get Mappable Physical Address Array       5800h      132

Get Mappable Physical Address Array

Entries                                5801h      136

EMM Functions                                                  35

Table 3-1.  List of EMM Functions (continued)

----------------------------------------------------------------

Number             Function Name                Hex Value   Page

----------------------------------------------------------------

26    Get Hardware Configuration Array          5900h      138

Get Unallocated Raw Page Count            5901h      142

27    Allocate Standard Pages                   5A00h      144

Allocate Raw Pages                        5A01h      147

28    Get Alternate Map Register Set            5B00h      153

Set Alternate Map Register Set            5B01h      157

Get Alternate Map Save Array Size         5B02h      161

Allocate Alternate Map Register Set       5B03h      163

Deallocate Alternate Map Register Set     5B04h      166

Allocate DMA Register Set                 5B05h      168

Enable DMA on Alternate Map

Register Set                           5B06h      170

Disable DMA on Alternate Map

Register Set                           5B07h      173

Deallocate DMA Register Set               5B08h      175

29    Prepare Expanded Memory Hardware

for Warmboot                           5Ch        177

30    Enable OS/E Function Set                  5D00h      179

Disable OS/E Function Set                 5D01h      182

Return OS/E Access Key                    5D02h      185

----------------------------------------------------------------

EMM Functions                                                  36

Function 1. Get Status

PURPOSE

The Get Status function returns a status code indicating

whether the memory manager is present and the hardware is

working correctly.

CALLING PARAMETERS

AH = 40h

Contains the Get Status Function.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager is present in the system, and the hardware

is working correctly.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EXAMPLE

MOV   AH,40h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  37

Function 2. Get Page Frame Address

PURPOSE

The Get Page Frame Address function returns the segment

address where the page frame is located.

CALLING PARAMETERS

AH = 41h

Contains the Get Page Frame Address function.

RESULTS

These results are valid only if the status returned is zero.

BX = page frame segment address

Contains the segment address of the page frame.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the page frame address in the

BX register.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EMM Functions                                                  38

Function 2. Get Page Frame Address

EXAMPLE

page_frame_segment             DW ?

MOV   AH,41h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   page_frame_segment,BX    ; save page frame address

EMM Functions                                                  39

Function 3. Get Unallocated Page Count

PURPOSE

The Get Unallocated Page Count function returns the number

of unallocated pages and the total number of expanded memory

pages.

CALLING PARAMETERS

AH = 42h

Contains the Get Unallocated Page Count function.

RESULTS

These results are valid only if the status returned is zero.

BX = unallocated pages

The number of expanded memory pages that are currently

available for use (unallocated).

DX = total pages

The total number of expanded memory pages.

REGISTERS MODIFIED

AX, BX, DX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the number of unallocated pages

and the number of total pages in expanded memory.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EMM Functions                                                  40

Function 3. Get Unallocated Page Count

EXAMPLE

un_alloc_pages                 DW ?

total_pages                    DW ?

MOV   AH,42h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   un_alloc_pages,BX        ; save unallocated page count

MOV   total_pages,DX           ; save total page count

EMM Functions                                                  41

Function 4. Allocate Pages

The Allocate Pages function allocates the number of pages

requested and assigns a unique EMM handle to these pages.

The EMM handle owns these pages until the application

deallocates them.

Handles which are assigned using this function will have

16K-byte pages, the size of a standard expanded memory page.

If the expanded memory board hardware isn't able to supply

16K-byte pages, it will emulate them by combining multiple

non-standard size pages to form a single 16K-byte page.  All

application programs and functions that use the handles this

function returns will deal with 16K-byte pages.

The numeric value of the handles the EMM returns are in the

range of 1 to 254 decimal (0001h to 00FEh).  The OS handle

(handle value 0) is never returned by the Allocate Pages

function.  Also, the uppermost byte of the handle will be

zero and cannot be used by the application.  A memory

manager should be able to supply up to 255 handles, includ-

ing the OS handle.  An application can use Function 21 to

find out how many handles an EMM supports.

Allocating zero pages to a handle is not valid.  If an

application needs to allocate 0 pages to a handle it should

use Function 27 (Allocate Standard Pages subfunction)

provided for this purpose.

Note............................................................

This note affects expanded memory manager implementors and

operating system developers only.  Applications should not

use the following characteristics of the memory manager.  An

application violating this rule will be incompatible with

future versions of Microsoft's operating systems and

environments.

To be compatible with this specification, an expanded memory

manager will provide a special handle which is available to

the operating system only.  This handle will have a value of

0000h and will have a set of pages allocated to it when the

expanded memory manager driver installs.  The pages that the

memory manager will automatically allocate to handle 0000h

are those that backfill conventional memory.  Typically,

this backfill occurs between addresses 40000h (256K) and

9FFFFh (640K).  However, the range can extend below and

above this limit if the hardware and memory manager have the

capability.

EMM Functions                                                  42

Function 4. Allocate Pages

An operating system won't have to invoke Function 4 to

obtain this handle because it can assume the handle already

exists and is available for use immediately after the

expanded memory device driver installs.  When an operating

system wants to use this handle, it uses the special handle

value of 0000h.  The operating system will be able to invoke

any EMM function using this special handle value.  To

allocate pages to this handle, the operating system need

only invoke Function 18 (Reallocate Pages).

There are two special cases for this handle:

1.  Function 4 (Allocate Pages).  This function must never

return zero as a handle value.  Applications must always

invoke Function 4 to allocate pages and obtain a handle

which identifies the pages which belong to it.  Since

Function 4 never returns a handle value of zero, an

application will never gain access to this special

handle.

2.  Function 6 (Deallocate Pages).  If the operating system

uses it to deallocate the pages which are allocated to

this special handle, the pages the handle owns will be

returned to the manager for use.  But the handle will

not be available for reassignment.  The manager should

treat a deallocate pages function request for this

handle the same as a reallocate pages function request,

where the number of pages to reallocate to this handle

is zero.

CALLING PARAMETERS

AH = 43h

Contains the Allocate Pages function.

BX = num_of_pages_to_alloc

Contains the number of pages you want your program to

allocate.

EMM Functions                                                  43

Function 4. Allocate Pages

RESULTS

These results are valid only if the status returned is zero.

DX = handle

Contains a unique EMM handle.  Your program must use

this EMM handle (as a parameter) in any function that

requires it.  You can use up to 255 handles.  The

uppermost byte of the handle will be zero and cannot be

used by the application.

REGISTERS MODIFIED

AX, DX

STATUS

AH = 0   SUCCESSFUL.

The manager has allocated the requested pages to the

assigned EMM handle.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 85h   RECOVERABLE.

All EMM handles are being used.

AH = 87h   RECOVERABLE.

There aren't enough expanded memory pages present in the

system to satisfy your program's request.

AH = 88h   RECOVERABLE.

There aren't enough unallocated pages to satisfy your

program's request.

AH = 89h   RECOVERABLE.

Your program attempted to allocate zero pages.

EMM Functions                                                  44

Function 4. Allocate Pages

EXAMPLE

num_of_pages_to_alloc          DW ?

emm_handle                     DW ?

MOV   BX,num_of_pages_to_alloc ; load number of pages

MOV   AH,43h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   emm_handle,DX            ; save EMM handle

EMM Functions                                                  45

Function 5. Map/Unmap Handle Pages

PURPOSE

The Map/Unmap Handle Page function maps a logical page at a

specific physical page anywhere in the mappable regions of

system memory.  The lowest valued physical page numbers are

associated with regions of memory outside the conventional

memory range.  Use Function 25 (Get Mappable Physical

Address Array) to determine which physical pages within a

system are mappable and determine the segment addresses

which correspond to a specific physical page number.

Function 25 provides a cross reference between physical page

numbers and segment addresses.

This function can also unmap physical pages, making them

inaccessible for reading or writing.  You unmap a physical

page by setting its associated logical page to FFFFh.

You might unmap an entire set of mapped pages, for example,

before loading and executing a program.  Doing so ensures

the loaded program, if it accesses expanded memory, won't

access the pages your program has mapped.  However, you must

save the mapped context before you unmap the physical pages.

This enables you to restore it later so you can access the

memory you mapped there.  To save the mapping context, use

Function 8, 15, or 16.  To restore the mapping context, use

Function 9, 15, or 16.

The handle determines what type of pages are being mapped.

Logical pages allocated by Function 4 and Function 27

(Allocate Standard Pages subfunction) are referred to as

pages and are 16K bytes long.  Logical pages allocated by

Function 27 (Allocate Raw Pages subfunction) are referred to

as raw pages and might not be the same size as logical

pages.

CALLING PARAMETERS

AH = 44h

Contains the Map Handle Page function.

AL = physical_page_number

Contains the number of the physical page into which the

logical page number is to be mapped.  Physical pages are

numbered zero-relative.

EMM Functions                                                  46

Function 5. Map/Unmap Handle Pages

BX = logical_page_number

Contains the number of the logical page to be mapped at

the physical page within the page frame.  Logical pages

are numbered zero-relative.  The logical page must be in

the range zero through (number of pages allocated to the

EMM handle - 1).  However, if BX contains logical page

number FFFFh, the physical page specified in AL will be

unmapped (be made inaccessible for reading or writing).

DX = emm_handle

Contains the EMM handle your program received from

Function 4 (Allocate Pages).

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has mapped the page.  The page is ready to

be accessed.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The memory manager couldn't find the EMM handle your

program specified.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager isn't

defined.

AH = 8Ah   RECOVERABLE.

The logical page is out of the range of logical pages

which are allocated to the EMM handle.  This status is

also returned if a program attempts to map a logical

page when no logical pages are allocated to the handle.

EMM Functions                                                  47

Function 5. Map/Unmap Handle Pages

AH = 8Bh   RECOVERABLE.

The physical page number is out of the range of allow-

able physical pages.  The program can recover by

attempting to map into memory at a physical page which

is within the range of allowable physical pages.

EXAMPLE

emm_handle                     DW ?

logical_page_number            DW ?

physical_page_number           DB ?

MOV   DX,emm_handle            ; load EMM handle

MOV   BX,logical_page_number   ; load logical page number

MOV   AL,physical_page_number  ; load physical page number

MOV   AH,44h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  48

Function 6. Deallocate Pages

PURPOSE

Deallocate Pages deallocates the logical pages currently

allocated to an EMM handle.  Only after the application

deallocates these pages can other applications use them.

When a handle is deallocated, its name is set to all ASCII

nulls (binary zeros).

Note............................................................

A program must perform this function before it exits to DOS.

If it doesn't, no other programs can use these pages or the

EMM handle.  This means that a program using expanded memory

should trap critical errors and control-break if there is a

chance that the program will have allocated pages when

either of these events occur.

CALLING PARAMETERS

AH = 45h

Contains the Deallocate Pages function.

DX = handle

Contains the EMM handle returned by Function 4 (Allocate

Pages).

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has deallocated the pages previously allo-

cated to the EMM handle.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager couldn't find the specified EMM handle.

EMM Functions                                                  49

Function 6. Deallocate Pages

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 86h   RECOVERABLE.

The memory manager detected a save or restore page

mapping context error (Function 8 or 9).  There is a

page mapping register state in the save area for the

specified EMM handle.  Save Page Map (Function 8) placed

it there and a subsequent Restore Page Map (Function 9)

has not removed it.

If you have saved the mapping context, you must restore

it before you deallocate the EMM handle's pages.

EXAMPLE

emm_handle                     DW ?

MOV   DX,emm_handle            ; load EMM handle

MOV   AH,45h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  50

Function 7. Get Version

PURPOSE

The Get Version function returns the version number of the

memory manager software.

CALLING PARAMETERS

AH = 46h

Contains the Get Version function.

RESULTS

These results are valid only if the status returned is zero.

AL = version number

Contains the memory manager's version number in binary

coded decimal (BCD) format.  The upper four bits contain

the integer digit of the version number.  The lower four

bits contain the fractional digit of version number.

For example, version 4.0 is represented like this:

0100 0000

/   \

4  .  0

When checking for a version number, an application

should check for a version number or greater.  Vendors

may use the fractional digit to indicate enhancements or

corrections to their memory managers.  Therefore, to

allow for future versions of memory managers, an

application shouldn't depend on an exact version number.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager is present in the system and the hardware is

working correctly.

EMM Functions                                                  51

Function 7. Get Version

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EXAMPLE

emm_version                    DB ?

MOV   AH,46h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   emm_version,AL           ; save version number

EMM Functions                                                  52

Function 8. Save Page Map

PURPOSE

Save Page Map saves the contents of the page mapping

registers on all expanded memory boards in an internal save

area.  The function is typically used to save the memory

mapping context of the EMM handle that was active when a

software or hardware interrupt occurred.  (See Function 9,

Restore Page Map, for the restore operation.)

If you're writing a resident program, an interrupt service

routine, or a device driver that uses expanded memory, you

must save the state of the mapping hardware.  You must save

this state because application software using expanded

memory may be running when your program is invoked by a

hardware interrupt, a software interrupt, or DOS.

The Save Page Map function requires the EMM handle that was

assigned to your resident program, interrupt service

routine, or device driver at the time it was initialized.

This is not the EMM handle that the application software was

using when your software interrupted it.

The Save Page Map function saves the state of the map

registers for only the 64K-byte page frame defined in

versions 3.x of this specification.  Since all applications

written to LIM versions 3.x require saving the map register

state of only this 64K-byte page frame, saving the entire

mapping state for a large number of mappable pages would be

inefficient use of memory.  Applications that use a mappable

memory region outside the LIM 3.x page frame should use

Function 15 or 16 to save and restore the state of the map

registers.

CALLING PARAMETERS

AH = 47h

Contains the Save Page Map function.

DX = handle

Contains the EMM handle assigned to the interrupt

service routine that's servicing the software or

hardware interrupt.  The interrupt service routine needs

to save the state of the page mapping hardware before

mapping any pages.

EMM Functions                                                  53

Function 8. Save Page Map

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has saved the state of the page mapping

hardware.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The memory manager couldn't find the EMM handle your

program specified.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Ch   NON-RECOVERABLE.

There is no room in the save area to store the state of

the page mapping registers.  The state of the map

registers has not been saved.

AH = 8Dh   CONDITIONALLY-RECOVERABLE.

The save area already contains the page mapping register

state for the EMM handle your program specified.

EXAMPLE

emm_handle                     DW ?

MOV   DX,emm_handle            ; load EMM handle

MOV   AH,47h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  54

Function 9. Restore Page Map

PURPOSE

The Restore Page Map function restores the page mapping

register contents on the expanded memory boards for a

particular EMM handle.  This function lets your program

restore the contents of the mapping registers its EMM handle

saved.  (See Function 8, Save Page Map for the save opera-

tion.)

If you're writing a resident program, an interrupt service

routine, or a device driver that uses expanded memory, you

must restore the mapping hardware to the state it was in

before your program took over.  You must save this state

because application software using expanded memory might

have been running when your program was invoked.

The Restore Page Map function requires the EMM handle that

was assigned to your resident program, interrupt service

routine, or device driver at the time it was initialized.

This is not the EMM handle that the application software was

using when your software interrupted it.

The Restore Page Map function restores the state of the map

registers for only the 64K-byte page frame defined in

versions 3.x of this specification.  Since all applications

written to LIM versions 3.x require restoring the map

register state of only this 64K-byte page frame, restoring

the entire mapping state for a large number of mappable

pages would be inefficient use of memory.  Applications that

use a mappable memory region outside the LIM 3.x page frame

should use Function 15 or 16 to save and restore the state

of the map registers.

CALLING PARAMETERS

AH = 48h

Contains the Restore Page Map function.

DX = emm_handle

Contains the EMM handle assigned to the interrupt

service routine that's servicing the software or

hardware interrupt.  The interrupt service routine needs

to restore the state of the page mapping hardware.

EMM Functions                                                  55

Function 9. Restore Page Map

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has restored the state of the page mapping

registers.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The memory manager couldn't find the EMM handle your

program specified.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Eh   CONDITIONALLY-RECOVERABLE.

There is no page mapping register state in the save area

for the specified EMM handle.  Your program didn't save

the contents of the page mapping hardware, so Restore

Page can't restore it.

EXAMPLE

emm_handle                     DW ?

MOV   DX,emm_handle            ; load EMM handle

MOV   AH,48h                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  56

Function 10. Reserved

In earlier versions of the Lotus/Intel/Microsoft Expanded

Memory Specification, Function 10 returned the page mapping

register I/O array.  This function is now reserved and new

programs should not use it.

Existing programs that use this function may still work

correctly if the hardware is capable of supporting them.

However, programs that use Functions 16 through 30 in

Version 4.0 of this specification must not use Functions 10

and 11.  These functions won't work correctly if your

program attempts to mix the use of the new functions

(Functions 16 through 30) and Functions 10 and 11.  Func-

tions 10 and 11 are specific to the hardware on Intel

expanded memory boards and will not work correctly on all

vendors' expanded memory boards.

EMM Functions                                                  57

Function 11. Reserved

In earlier versions of the Lotus/Intel/Microsoft Expanded

Memory Specification, Function 11 returned a page transla-

tion array.  This function is now reserved and new programs

should not use it.

Existing programs that use this function may still work

correctly if the hardware is capable of supporting them.

However, programs that use Functions 16 through 30 in

Version 4.0 of this specification must not use Functions 10

and 11.  These functions won't work correctly if your

program attempts to mix the use of the new functions

(Functions 16 through 30) and Functions 10 and 11.  Func-

tions 10 and 11 are specific to the hardware on Intel

expanded memory boards and will not work correctly on all

vendors' expanded memory boards.

EMM Functions                                                  58

Function 12. Get Handle Count

PURPOSE

The Get Handle Count function returns the number of open EMM

handles (including the operating system handle 0) in the

system.

CALLING PARAMETERS

AH = 4Bh

Contains the Get Handle Count function.

RESULTS

These results are valid only if the status returned is zero.

BX = total_open_emm_handles

Contains the number of open EMM handles [including the

operating system handle (0)].  This number will not

exceed 255.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the number of active EMM

handles.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EMM Functions                                                  59

Function 12. Get Handle Count

EXAMPLE

total_open_emm_handles              DW ?

MOV   AH,4Bh                        ; load function code

INT   67h                           ; call the memory manger

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

MOV   total_open_emm_handles,BX     ; save total active handle

; count

EMM Functions                                                  60

Function 13. Get Handle Pages

PURPOSE

The Get Handle Pages function returns the number of pages

allocated to a specific EMM handle.

CALLING PARAMETERS

AH = 4Ch

Contains the Get Handle Pages function.

DX = emm_handle

Contains the EMM handle.

RESULTS

These results are valid only if the status returned is zero.

BX = num_pages_alloc_to_emm_handle

Contains the number of logical pages allocated to the

specified EMM handle.  This number never exceeds 2048

because the memory manager allows a maximum of 2048

pages (32M bytes) of expanded memory.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the number of pages allocated

to the EMM handle.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The memory manager couldn't find the EMM handle your

program specified.

EMM Functions                                                  61

Function 13. Get Handle Pages

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EXAMPLE

emm_handle                          DW ?

pages_alloc_to_handle               DW ?

MOV   DX,emm_handle                 ; load EMM handle

MOV   AH,4Ch                        ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

MOV   pages_alloc_to_handle,BX      ; save number of pages

; allocated to specified

; handle

EMM Functions                                                  62

Function 14. Get All Handle Pages

PURPOSE

The Get All Handle Pages function returns an array of the

open EMM handles and the number of pages allocated to each

one.

CALLING PARAMETERS

AH = 4Dh

Contains the Get All Handle Pages function.

handle_page_struct              STRUC

emm_handle                  DW ?

pages_alloc_to_handle       DW ?

handle_page_struct              ENDS

ES:DI = pointer to handle_page

Contains a pointer to an array of structures where a

copy of all open EMM handles and the number of pages

allocated to each will be stored.  Each structure has

these two members:

.emm_handle

The first member is a word which contains the value

of the open EMM handle.  The values of the handles

this function returns will be in the range of 0 to

255 decimal (0000h to 00FFh).  The uppermost byte of

the handle is always zero.

.pages_alloc_to_handle

The second member is a word which contains the

number of pages allocated to the open EMM handle.

RESULTS

These results are valid only if the status returned is zero.

BX = total_open_emm_handles

Contains the number of open EMM handles (including the

operating system handle [0]).  The number cannot be zero

because the operating system handle is always active and

cannot be deallocated.  This number will not exceed 255.

EMM Functions                                                  63

Function 14. Get All Handle Pages

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the array.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EXAMPLE

handle_page                    handle_page_struct 255 DUP (?)

total_open_handles             DW ?

MOV   AX,SEG handle_page

MOV   ES,AX

LEA   DI,handle_page           ; ES:DI points to handle_page

MOV   AH,4Dh                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check the EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   total_open_handles,BX    ; save total open handle count

EMM Functions                                                  64

Function 15. Get/Set Page Map

Get Page Map subfunction

PURPOSE

The Get Page Map subfunction saves the mapping context for

all mappable memory regions (conventional and expanded) by

copying the contents of the mapping registers from each

expanded memory board to a destination array.  The applica-

tion must pass a pointer to the destination array.  This

subfunction doesn't require an EMM handle.

Use this function instead of Functions 8 and 9 if you need

to save or restore the mapping context but don't want (or

have) to use a handle.

CALLING PARAMETERS

AX = 4E00h

Contains the Get Page Map subfunction.

ES:DI = dest_page_map

Contains a pointer to the destination array address in

segment:offset format.  Use the Get Size of Page Map

Save Array subfunction to determine the size of the

desired array.

RESULTS

These results are valid only if the status returned is zero.

dest_page_map

The array contains the state of all the mapping regis-

ters on all boards in the system.  It also contains any

additional information necessary to restore the boards

to their original state when the program invokes a Set

subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the array.

EMM Functions                                                  65

Function 15. Get/Set Page Map

Get Page Map subfunction

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

dest_page_map                  DB ? DUP (?)

MOV   AX,SEG dest_page_map

MOV   ES,AX

LEA   DI,dest_page_map         ; ES:DI points to dest_page_map

MOV   AX,4E00h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  66

Function 15. Get/Set Page Map

Set Page Map subfunction

PURPOSE

The Set Page Map subfunction restores the mapping context

for all mappable memory regions (conventional and expanded)

by copying the contents of a source array into the mapping

registers on each expanded memory board in the system.  The

application must pass a pointer to the source array.  This

subfunction doesn't require an EMM handle.

Use this function instead of Functions 8 and 9 if you need

to save or restore the mapping context but don't want (or

have) to use a handle.

CALLING PARAMETERS

AX = 4E01h

Contains the Set Page Map subfunction.

DS:SI = source_page_map

Contains a pointer to the source array address in

segment:offset format.  The application must point to an

array which contains the mapping register state.  Use

the Get Size of Page Map Save Array subfunction to

determine the size of the desired array.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has passed the array.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

EMM Functions                                                  67

Function 15. Get/Set Page Map

Set Page Map subfunction

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A3h   NON-RECOVERABLE.

The contents of the source array have been corrupted, or

the pointer passed to the subfunction is invalid.

EXAMPLE

source_page_map                DB ? DUP (?)

MOV   AX,SEG source_page_map

MOV   DS,AX

LEA   SI,source_page_map       ; DS:SI points to source_page_map

MOV   AX,4E01h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  68

Function 15. Get/Set Page Map

Get & Set Page Map subfunction

PURPOSE

The Get & Set Page Map subfunction simultaneously saves a

current mapping context and restores a previous mapping

context for all mappable memory regions (both conventional

and expanded).  It first copies the contents of the mapping

registers from each expanded memory board in the system into

a destination array.  (The application must pass a pointer

to the destination array.)  Then, the subfunction copies the

contents of a source array into the mapping registers on

each of the expanded memory boards.  (The application must

pass a pointer to the source array.)

Use this function instead of Functions 8 and 9 if you need

to save or restore the mapping context but don't want (or

have) to use a handle.

CALLING PARAMETERS

AX = 4E02h

Contains the Get & Set Page Map subfunction.

ES:DI = dest_page_map

Contains a pointer to the destination array address in

segment:offset format.  The current contents of the map

registers will be saved in this array.

DS:SI = source_page_map

Contains a pointer to the source array address in

segment:offset format.  The contents of this array will

be copied into the map registers.  The application must

point to an array which contains the mapping register

state.  This address is required only for the Set or Get

and Set subfunctions.

RESULTS

These results are valid only if the status returned is zero.

dest_page_map

The array contains the mapping state.  It also contains

any additional information necessary to restore the

original state when the program invokes a Set subfunc-

tion.

EMM Functions                                                  69

Function 15. Get/Set Page Map

Get & Set Page Map subfunction

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned and passed both arrays.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A3h   NON-RECOVERABLE.

The contents of the source array have been corrupted, or

the pointer passed to the subfunction is invalid.

EXAMPLE

dest_page_map                  DB ? DUP (?)

source_page_map                DB ? DUP (?)

MOV   AX,SEG dest_page_map

MOV   ES,AX

MOV   AX,SEG source_page_map

MOV   DS,AX

LEA   DI,dest_page_map         ; ES:DI points to dest_page_map

LEA   SI,source_page_map       ; DS:SI points to source_page_map

MOV   AX,4E02h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  70

Function 15. Get/Set Page Map

Get Size of Page Map Save Array subfunction

PURPOSE

The Get Size of Page Map Save Array subfunction returns the

storage requirements for the array passed by the other three

subfunctions.  This subfunction doesn't require an EMM

handle.

CALLING PARAMETERS

AX = 4E03h

Contains the Get Size of Page Map Save Array subfunc-

tion.  The size of this array depends on how the

expanded memory system is configured and how the

expanded memory manager is implemented.  Therefore, the

size must be determined after the memory manager is

loaded.

RESULTS

These results are valid only if the status returned is zero.

AL = size_of_array

Contains the number of bytes that will be transferred to

the memory area an application supplies whenever a

program requests the Get, Set, or Get and Set subfunc-

tions.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the array size.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

EMM Functions                                                  71

Function 15. Get/Set Page Map

Get Size of Page Map Save Array subfunction

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

size_of_array                  DB ?

MOV   AX,4E03h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   size_of_array,AL         ; save array size

EMM Functions                                                  72

Function 16. Get/Set Partial Page Map

Get Partial Page Map subfunction

PURPOSE

The Get Partial Page Map subfunction saves a partial mapping

context for specific mappable memory regions in a system.

Because this function saves only a subset of the entire

mapping context, it uses much less memory for the save area

and may be potentially faster than Function 15.  The

subfunction does this by copying the contents of selected

mapping registers from each expanded memory board to a

destination array.

The application must pass a pair of pointers.  The first

points to a structure which specifies which mappable

segments to save; the second points to the destination

array.

Use this function instead of Functions 8 and 9 if you need

to save or restore the mapping context but don't want (or

have) to use a handle.

CALLING PARAMETERS

AX = 4F00h

Contains the Get Partial Page Map subfunction.

partial_page_map_struct         STRUC

mappable_segment_count      DW ?

mappable_segment            DW (?) DUP (?)

partial_page_map_struct         ENDS

DS:SI = partial_page_map

Contains a pointer to a structure which specifies only

those mappable memory regions which are to have their

mapping context saved.  The structure members are

described below.

.mappable_segment_count

The first member is a word which specifies the

number of members in the word array which immediate-

ly follows it.  This number should not exceed the

number of mappable segments in the system.

EMM Functions                                                  73

Function 16. Get/Set Partial Page Map

Get Partial Page Map subfunction

.mappable_segment

The second member is a word array which contains the

segment addresses of the mappable memory regions

whose mapping contexts are to be saved.  The segment

address must be a mappable segment.  Use Function 25

to determine which segments are mappable.

ES:DI = dest_array

Contains a pointer to the destination array address in

segment:offset format.  To determine the size of the

required array, see the Get Size of Partial Page Map

Save Array subfunction.

RESULTS

These results are valid only if the status returned is zero.

dest_array

The array contains the partial mapping context and any

additional information necessary to restore this context

to its original state when the program invokes a Set

subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has saved the partial map context.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EMM Functions                                                  74

Function 16. Get/Set Partial Page Map

Get Partial Page Map subfunction

AH = 8Bh   NON-RECOVERABLE.

One of the specified segments is not a mappable segment.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A3h   NON-RECOVERABLE.

The contents of the partial page map structure have been

corrupted, the pointer passed to the subfunction is

invalid, or the mappable_segment_count exceeds the

number of mappable segments in the system.

EXAMPLE

partial_page_map               partial_page_map_struct <>

dest_array                     DB ? DUP (?)

MOV   AX,SEG partial_page_map

MOV   DS,AX

LEA   SI,partial_page_map      ; DS:SI points to partial_page_map

MOV   AX,SEG dest_array

MOV   ES,AX

LEA   DI,dest_array            ; ES:DI points to dest_array

MOV   AX,4F00h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  75

Function 16. Get/Set Partial Page Map

Set Partial Page Map subfunction

PURPOSE

The Set Partial Page Map subfunction provides a mechanism

for restoring the mapping context for a partial mapping

context for specific mappable memory regions in a system.

Because this function restores only a subset of the entire

mapping context and not the entire systems mapping context,

it uses much less memory for the save area and is potential-

ly faster than Function 15.  The subfunction does this by

copying the contents of the source array to selected mapping

registers on each expanded memory board.  The application

passes a pointer to the source array.

Use this function instead of Functions 8 and 9 if you need

to save or restore the mapping context but don't want (or

have) to use a handle.

CALLING PARAMETERS

AX = 4F01h

Contains the Set Partial Page Map subfunction

source_array     DB ? DUP (?)

DS:SI = source_array

Contains a pointer to the source array in segment:offset

format.  The application must point to an array which

contains the partial mapping register state.  To deter-

mine the size of the required array, see the Get Size of

Partial Page Map Save Array subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has restored the partial mapping context.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

EMM Functions                                                  76

Function 16. Get/Set Partial Page Map

Set Partial Page Map subfunction

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A3h   NON-RECOVERABLE.

The contents of the source array have been corrupted, or

the pointer passed to the subfunction is invalid.

EXAMPLE

MOV   AX,SEG source_array

MOV   DS,AX

LEA   SI,source_array          ; DS:SI points to source_array

MOV   AX,4F01h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  77

Function 16. Get/Set Partial Page Map

Get Size of Partial Page Map Save Array subfunction

PURPOSE

The Get Size of Partial Page Map Save Array subfunction

returns the storage requirements for the array passed by the

other two subfunctions.  This subfunction doesn't require an

EMM handle.

CALLING PARAMETERS

AX = 4F02h

Contains the Get Size of Partial Page Map Save Array

subfunction.  The size of this array depends on the

expanded memory system configuration and the implementa-

tion of the expanded memory manager.  Therefore, it will

vary between hardware configurations and implementations

and must be determined after a specific memory manager

is loaded.

BX = number of pages in the partial array

Contains the number of pages in the partial map to be

saved by the Get/Set Partial Page Map subfunctions.

This number should be the same as the mappable_seg-

ment_count in the Get Partial Page Map subfunction.

RESULTS

These results are valid only if the status returned is zero.

AL = size_of_partial_save_array

Contains the number of bytes that will be transferred to

the memory areas supplied by an application whenever a

program requests the Get or Set subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the array size.

EMM Functions                                                  78

Function 16. Get/Set Partial Page Map

Get Size of Partial Page Map Save Array subfunction

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Bh   NON-RECOVERABLE.

The number of pages in the partial array is outside the

range of physical pages in the system.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

number_of_pages_to_map              DW ?

size_of_partial_save_array          DB ?

MOV   BX,number_of_pages_to_map

MOV   AX,4F02h                      ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

MOV   size_of_partial_save_array,AL ; save array size

EMM Functions                                                  79

Function 17. Map/Unmap Multiple Handle Pages

PURPOSE

This function can, in a single invocation, map (or unmap)

logical pages into as many physical pages as the system

supports.  Consequently, it has less execution overhead than

mapping pages one at a time.  For applications which do a

lot of page mapping, this is the preferred mapping method.

Mapping Multiple Pages

The handle passed to this function determines what type of

logical pages are being mapped.  Logical pages that Function

4 and Function 27 (Allocate Standard Pages subfunction)

allocate are referred to as pages and are 16K bytes.

Logical pages that Function 27 (Allocate Raw Pages subfunc-

tion) allocates are referred to as raw pages and might not

be the same size as the pages Function 4 and Function 27

(Allocate Standard Pages subfunction) allocate.

Unmapping Multiple Pages

This function can make specific physical pages unavailable

for reading or writing.  A logical page which is unmapped

from a specific physical page cannot be read or written from

that physical page.  The logical page which is unavailable

(unmapped) can be made available again by mapping it, or a

new logical page, at the physical page that was unmapped.

Unmapping a physical page is accomplished by setting the

logical page it is associated with to FFFFh.

You might unmap an entire set of mapped pages, for example,

before loading and executing a program.  This ensures that

the loaded program won't be able to access the pages your

program has mapped.  However, you must save the mapping

context before you unmap the physical pages.  This enables

you to restore it later so that you may access the memory

you had mapped there.  You can save the mapping context with

Functions 8, 15, or 16.  You can restore the mapping context

with Functions 9, 15, or 16.

Mapping and Unmapping Multiple Pages Simultaneously

Both mapping and unmapping pages can be done in the same

invocation.

EMM Functions                                                  80

Function 17. Map/Unmap Multiple Handle Pages

Mapping or unmapping no pages is not considered an error.

If a request to map or unmap zero pages is made, nothing is

done and no error is returned.

Alternate Mapping and Unmapping Methods

You can map or unmap pages using two methods.  Both methods

produce identical results.

1.  The first method specifies both a logical page and a

physical page at which the logical page is to be mapped.

This method is an extension of Function 5 (Map Handle

Page).

2.  The second method specifies both a logical page and a

corresponding segment address at which the logical page

is to be mapped.  While this is functionally the same as

the first method, it may be easier to use the actual

segment address of a physical page than to use a number

which only represents its location.  The memory manager

verifies whether the specified segment address falls on

the boundary of a mappable physical page.  The manager

then translates the segment address passed to it into

the necessary internal representation to map the pages.

EMM Functions                                                  81

Function 17. Map/Unmap Multiple Handle Pages

Logical Page/Physical Page Method

CALLING PARAMETERS

AX = 5000h

Contains the Map/Unmap Multiple Handle Pages subfunction

using the logical page/physical page method.

log_to_phys_map_struct          STRUC

log_page_number             DW ?

phys_page_number            DW ?

log_to_phys_map_struct          ENDS

DX = handle

Contains the EMM handle.

CX = log_to_phys_map_len

Contains the number of entries in the array.  For

example, if the array contained four pages to map or

unmap, then CX would contain 4.  The number in CX should

not exceed the number of mappable pages in the system.

DS:SI = pointer to log_to_phys_map array

Contains a pointer to an array of structures that

contains the information necessary to map the desired

pages.  The array is made up of the following two

elements:

.log_page_number

The first member is a word which contains the number

of the logical page which is to be mapped.  Logical

pages are numbered zero-relative, so the number for

a logical page can only range from zero to (maximum

number of logical pages allocated to the handle -

1).

If the logical page number is set to FFFFh, the

physical page associated with it is unmapped rather

than mapped.  Unmapping a physical page makes it

inaccessible for reading or writing.

.phys_page_number

The second member is a word which contains the

number of the physical page at which the logical

page is to be mapped.  Physical pages are numbered

zero-relative, so the number for a physical page can

only range from zero to (maximum number of physical

pages supported in the system - 1).

EMM Functions                                                  82

Function 17. Map/Unmap Multiple Handle Pages

Logical Page/Physical Page Method

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The logical pages have been mapped, or unmapped, at the

specified physical pages.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager couldn't find the specified EMM handle.  The

manager doesn't currently have any information pertain-

ing to the specified EMM handle.  The program has

probably corrupted its EMM handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Ah   RECOVERABLE.

One or more of the mapped logical pages is out of the

range of logical pages allocated to the EMM handle.  The

program can recover by attempting to map a logical page

which is within the bounds for the specified EMM handle.

When this error occurs, the only pages mapped were the

ones valid up to the point that the error occurred.

AH = 8Bh   RECOVERABLE.

One or more of the physical pages is out of the range of

mappable physical pages, or the log_to_phys_map_len

exceeds the number of mappable pages in the system.  The

program can recover from this condition by attempting to

map into memory at the physical page which is in the

range of the physical page numbers supported by the

system.  When this error occurs, the only pages mapped

were the ones valid up to the point that the error

occurred.

EMM Functions                                                  83

Function 17. Map/Unmap Multiple Handle Pages

Logical Page/Physical Page Method

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

log_to_phys_map            log_to_phys_map_struct ? DUP (?)

emm_handle                 DW ?

MOV   AX,SEG log_to_phys_map

MOV   DS,AX

LEA   SI,log_to_phys_map            ; DS:SI points to

; log_to_phys_map

MOV   CX,LENGTH log_to_phys_map     ; set length field

MOV   DX,emm_handle

MOV   AX,5000h                      ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

EMM Functions                                                  84

Function 17. Map/Unmap Multiple Handle Pages

Logical Page/Segment Address Method

CALLING PARAMETERS

AX = 5001h

Contains the Map/Unmap Multiple Handle Pages subfunction

using the logical page/segment address method.

log_to_seg_map_struct           STRUC

log_page_number             DW ?

mappable_segment_address    DW ?

log_to_seg_map_struct           ENDS

DX = handle

Contains the EMM handle.

CX = log_to_segment_map_len

Contains the number of entries in the array.  For

example, if the array contained four pages to map or

unmap, then CX would contain four.

DS:SI = pointer to log_to_segment_map array

Contains a pointer to an array of structures that

contains the information necessary to map the desired

pages.  The array is made up of the following elements:

.log_page_number

The first member is a word which contains the number

of the logical pages to be mapped.  Logical pages

are numbered zero-relative, so the number for a

logical page can range from zero to (maximum number

of logical pages allocated to the handle - 1).

If the logical page number is set to FFFFh, the

physical page associated with it is unmapped rather

than mapped.  Unmapping a physical page makes it

inaccessible for reading or writing.

.mappable_segment_address

The second member is a word which contains the

segment address at which the logical page is to be

mapped.  This segment address must correspond

exactly to a mappable segment address.  The mappable

segment addresses are available with Function 25

(Get Mappable Physical Address Array).

REGISTERS MODIFIED

AX

EMM Functions                                                  85

Function 17. Map/Unmap Multiple Handle Pages

Logical Page/Segment Address Method

STATUS

AH = 0   SUCCESSFUL.

The logical pages have been mapped (or unmapped) at the

specified physical pages.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager could not find the specified EMM handle.

The manager doesn't currently have any information

pertaining to the specified EMM handle.  The program has

probably corrupted its EMM handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Ah   RECOVERABLE.

One or more of the logical pages to be mapped is out of

the range of logical pages allocated to the EMM handle.

The program can recover from this condition by mapping a

logical page which is within the bounds for the speci-

fied EMM handle.  When this error occurs, the only pages

mapped or unmapped were the ones valid up to the point

that the error occurred.

AH = 8Bh   RECOVERABLE.

One or more of the mappable segment addresses specified

is not mappable, the segment address doesn't fall

exactly on a mappable address boundary, or the log_to_-

segment_map_len exceeds the number of mappable segments

in the system.  The program can recover from this

condition by mapping into memory on an exact mappable

segment address.  When this error occurs, the only pages

mapped were the ones valid up to the point that the

error occurred.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EMM Functions                                                  86

Function 17. Map/Unmap Multiple Handle Pages

Logical Page/Segment Address Method

EXAMPLE

log_to_seg_map             log_to_seg_map_struct 4 DUP (?)

emm_handle                 DW ?

MOV   AX,SEG log_to_seg_map

MOV   DS,AX

LEA   SI,log_to_seg_map             ; DS:SI points to

; log_to_seg_map

MOV   CX,LENGTH log_to_seg_map

MOV   DX,emm_handle

MOV   AX,5001h                      ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

EMM Functions                                                  87

Function 18. Reallocate Pages

PURPOSE

This function allows an application program to increase or

decrease (reallocate) the number of logical pages allocated

to an EMM handle.  There are four reallocation cases of

interest:

1.  A reallocation count of zero.  The handle assigned to

the application remains assigned and is still available

for use by the application.  The memory manager won't

reassign the handle to any other application.  However,

the handle will have any currently allocated pages

returned to the memory manager.  The application must

invoke the Deallocate Pages function (Function 6) before

returning to DOS, or the handle will remain assigned and

no other application will be able to use it.

2.  A reallocation count equal to the current allocation

count.  This is not treated as an error, and a success-

ful status is returned.

3.  A reallocation count greater than the current allocation

count.  The memory manager will attempt to add new pages

to those pages already allocated to the specified EMM

handle.  The number of new pages added is the difference

between the reallocation count and the current alloca-

tion count.  The sequence of logical pages allocated to

the EMM handle remains continuous after this operation.

The newly allocated pages have logical page numbers

which begin where the previously allocated pages ended,

and continue in ascending sequence.

4.  A reallocation count less than the current allocation

count.  The memory manager will attempt to subtract some

of the currently allocated pages and return them to the

memory manager.  The number of old pages subtracted is

the difference between the current allocation count and

the re-allocation count.  The pages are subtracted from

the end of the sequence of pages currently allocated to

the specified EMM handle.  The sequence of logical pages

allocated to the EMM handle remains continuous after

this operation.

EMM Functions                                                  88

Function 18. Reallocate Pages

The handle determines what type of logical pages are being

reallocated.  Logical pages which were originally allocated with

Function 4 or Function 27 (Allocate Standard Pages subfunction)

are called pages and are 16K bytes long.  Logical pages which

were allocated with Function 27 (Allocate Raw Pages subfunction)

are called raw pages and might not be the same size as pages

allocated with Function 4.

CALLING PARAMETERS

AH = 51h

Contains the Reallocate Handle Pages function.

DX = handle

Contains the EMM handle.

BX = reallocation_count

Contains the total number of pages this handle should

have allocated to it after this function is invoked.

RESULTS

BX = number of pages allocated to handle after reallocation

Contains the number of pages now allocated to the EMM

handle after the pages have been added or subtracted.

If the status returned is not zero, the value in BX is

equal to the number of pages allocated to the handle

prior to the invocation of this function.  This informa-

tion can be used to verify that the request generated

the expected results.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The pages specified have been added to or subtracted

from the handle specified.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

EMM Functions                                                  89

Function 18. Reallocate Pages

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager could not find the specified EMM handle.

The manager doesn't have any information pertaining to

the specified EMM handle.  The program may have cor-

rupted its EMM handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 87h   RECOVERABLE.

The number of pages that are available in the system is

insufficient for the new allocation request.  The

program can recover from this condition by specifying

fewer pages be allocated to the EMM handle.

AH = 88h   RECOVERABLE.

The number of unallocated pages is insufficient for the

new allocation request.  The program can recover from

this condition by either requesting again when addition-

al pages are available or specifying fewer pages.

EXAMPLE

emm_handle                          DW ?

realloc_count                       DW ?

current_alloc_page_count            DW ?

MOV   DX,emm_handle                 ; specify EMM handle

MOV   BX,realloc_count              ; specify count

MOV   AH,51h                        ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

MOV   current_alloc_page_count,BX

EMM Functions                                                  90

Function 19. Get/Set Handle Attribute

Design Considerations

This function is an option which will probably not be

available in a typical expanded memory manager, system, or

memory board.  Most personal computer systems disable memory

refresh signals for a considerable period during a warm

boot.  This can corrupt some of the data in memory boards,

even though there is no problem with the design of the

memory board, its operation, or the memory chips.  This

memory refresh deficiency is present in the software design

of the ROM BIOS in most personal computer systems.

The majority of memory board designs, chip types, or

personal computer systems won't be able to support the non-

volatility feature.  The reason that this ROM BIOS deficien-

cy is not evident in the conventional or extended memory

area is that the ROM BIOS always initializes this area

during a warm boot.  Memory data integrity is not a problem

with the conventional or extended memory region, because it

isn't physically possible to have data retained there across

a warm boot event -- the ROM BIOS sets it to zero.

Consequently, expanded memory board manufacturers should not

supply this function unless their board can guarantee the

integrity of data stored in the board's memory during a warm

boot.  Generally, this means the memory board has an

independent memory refresh controller which does not depend

on the system board's memory refresh.

If the expanded memory manager, system, or memory board

cannot support this feature, it should return the not

supported status described in the function.

EMM Functions                                                  91

Function 19. Get/Set Handle Attribute

Get Handle Attribute subfunction

PURPOSE

This subfunction returns the attribute associated with a

handle.  The attributes are volatile or non-volatile.

Handles with non-volatile attributes enable the memory

manager to save the contents of a handle's pages between

warm boots.  However, this function may be disabled with a

user option or may not be supported by the memory board or

system hardware.

If the handle's attribute has been set to non-volatile, the

handle, its name (if it is assigned one), and the contents

of the pages allocated to the handle are all maintained

after a warm boot.

CALLING PARAMETERS

AX = 5200h

Contains the Get Handle Attribute subfunction.

DX = handle

Contains the EMM handle.

RESULTS

These results are valid only if the status returned is zero.

AL = handle attribute

Contains the EMM handle's attribute.  The only at-

tributes a handle may have are volatile or non-volatile.

A value of zero indicates the handle is volatile.  A

value of one indicates that the handle is non-volatile.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The handle's attribute has been obtained.

EMM Functions                                                  92

Function 19. Get/Set Handle Attribute

Get Handle Attribute subfunction

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager couldn't find the specified EMM handle.  The

manager doesn't have any information pertaining to the

specified EMM handle.  The program may have corrupted

its EMM handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 91h   NON-RECOVERABLE.

This feature is not supported.

EXAMPLE

emm_handle                     DW ?

handle_attrib                  DB ?

MOV   DX,emm_handle            ; specify EMM handle

MOV   AX,5200h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   handle_attrib,AL         ; save handle attribute

EMM Functions                                                  93

Function 19. Get/Set Handle Attribute

Set Handle Attribute subfunction

PURPOSE

This subfunction can be used to modify the attribute which a

handle has associated with it.  The attributes which a

handle may have are volatile or non-volatile.  The non-

volatile attribute enables the EMM to save the contents of a

handle's pages between warm boots.  However, this function

may be disabled with a user option or may not be supported

by the memory board or system hardware.

If the handle's attribute has been set to non-volatile, the

handle, its name (if it is assigned one), and the contents

of the pages allocated to the handle are all maintained

after a warm boot.

CALLING PARAMETERS

AX = 5201h

Contains the Set Handle Attribute function.

DX = handle

Contains the EMM handle.

BL = new handle attribute

Contains the handle's new attribute.  A value of zero

indicates that the handle should be made volatile.  A

value of one indicates that the handle should be made

non-volatile.

A volatile handle attribute instructs the memory manager

to deallocate both the handle and the pages allocated to

it after a warm boot.  If all handles have the volatile

attribute (the default attribute) at warm boot, the

handle directory will be empty and all of expanded

memory will be initialized to zero immediately after a

warm boot.

REGISTERS MODIFIED

AX

EMM Functions                                                  94

Function 19. Get/Set Handle Attribute

Set Handle Attribute subfunction

STATUS

AH = 0   SUCCESSFUL.

The handle's attribute has been modified.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager could not find the specified EMM handle.

The manager doesn't have any information pertaining to

the specified EMM handle.  The program may have cor-

rupted its EMM handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 90h   NON-RECOVERABLE.

The attribute type is undefined.

AH = 91h   NON-RECOVERABLE.

This feature is not supported.

EXAMPLE

emm_handle                     DW ?

new_handle_attrib              DB ?

MOV   DX,emm_handle            ; specify EMM handle

MOV   BL,new_handle_attrib     ; specify the set attribute

MOV   AX,5201h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  95

Function 19. Get/Set Handle Attribute

Get Attribute Capability subfunction

PURPOSE

This subfunction can be used to determine whether the memory

manager can support the non-volatile attribute.

CALLING PARAMETERS

AX = 5202h

Contains the Get Attribute Capability subfunction.

RESULTS

These results are valid only if the status returned is zero.

AL = attribute capability

Contains the attribute capability.  A value of zero

indicates that the memory manager and hardware supports

only volatile handles.  A value of one indicates that

the memory manager/hardware supports both volatile and

non-volatile handles.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The attribute capability has been returned.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EMM Functions                                                  96

Function 19. Get/Set Handle Attribute

Get Attribute Capability subfunction

EXAMPLE

attrib_capability              DB ?

MOV   AX,5202h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   attrib_capability,AL     ; save attribute capability

EMM Functions                                                  97

Function 20. Get/Set Handle Name

Get Handle Name subfunction

PURPOSE

This subfunction gets the eight character name currently

assigned to a handle.  There is no restriction on the

characters which may be used in the handle name (that is,

anything from 00h through FFh).

The handle name is initialized to ASCII nulls (binary zeros)

three times:  when the memory manager is installed, when a

handle is allocated, and when a handle is deallocated.  A

handle with a name which is all ASCII nulls, by definition,

has no name.  When a handle is assigned a name, at least one

character in the name must be a non-null character in order

to distinguish it from a handle without a name.

CALLING PARAMETERS

AX = 5300h

Contains the Get Handle Name function.

DX = handle number

Contains the EMM handle.

ES:DI = pointer to handle_name array

Contains a pointer to an eight-byte array into which the

name currently assigned to the handle will be copied.

RESULTS

These results are valid only if the status returned is zero.

handle_name array

Contains the name associated with the specified handle.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The handle name has been returned.

EMM Functions                                                  98

Function 20. Get/Set Handle Name

Get Handle Name subfunction

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager couldn't find the specified EMM handle.  The

manager doesn't have any information on the specified

EMM handle.  The program may have corrupted its EMM

handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

handle_name                    DB 8 DUP (?)

emm_handle                     DW ?

MOV   AX,SEG handle_name

MOV   ES,AX

LEA   DI,handle_name           ; ES:DI points to handle_name

MOV   DX,emm_handle            ; specify EMM handle

MOV   AX,5300h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                  99

Function 20. Get/Set Handle Name

Set Handle Name subfunction

PURPOSE

This subfunction assigns an eight character name to a

handle.  There is no restriction on the characters which may

be used in the handle name.  The full range of values may be

assigned to each character in a name (that is, 00h through

FFh).

At installation, all handles have their name initialized to

ASCII nulls (binary zeros).  A handle with a name consisting

of all ASCII nulls has no name.  When a handle is assigned a

name, at least one character in the name must be a non-null

character in order to distinguish it from a handle without a

name.  No two handles may have the same name.

A handle can be renamed at any time by setting the handle's

name to a new value.  A handle can have its name removed by

setting the handle's name to all ASCII nulls.  When a handle

is deallocated, its name is removed (set to ASCII nulls).

CALLING PARAMETERS

AX = 5301h

Contains the Set Handle Name function.

DX = handle number

Contains the EMM handle.

DS:SI = pointer to handle_name

Contains a pointer to a byte array which contains the

name that is to be assigned to the handle.  The handle

name must be padded with nulls if the name is less than

eight characters long.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The handle name has been assigned.

EMM Functions                                                 100

Function 20. Get/Set Handle Name

Set Handle Name subfunction

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager couldn't find the specified EMM handle.  The

manager doesn't currently have any information pertain-

ing to the specified EMM handle.  The program may have

corrupted its EMM handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A1h   RECOVERABLE.

A handle with this name already exists.  The specified

handle was not assigned a name.

EXAMPLE

handle_name                    DB 'AARDVARK'

emm_handle                     DW ?

MOV   AX,SEG handle_name

MOV   DS,AX

LEA   SI,handle_name           ; DS:SI points to handle_name

MOV   DX,emm_handle            ; specify EMM handle

MOV   AX,5301h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 101

Function 21. Get Handle Directory

Get Handle Directory subfunction

PURPOSE

This function returns an array which contains all active

handles and the names associated with each.  Handles which

have not been assigned names have a default name of all

ASCII nulls (binary zeros).  When a handle is first allo-

cated, or when all the pages belonging to a handle are

deallocated (that is, an open handle is closed), its default

name is set to ASCII nulls.  It takes a subsequent assign-

ment of a name for a handle to have a name after it has been

opened.  The full range of values may be assigned to each

character in a name (that is, 00h through FFh).

The number of bytes required by the array is:

10 bytes * total number of handles

The maximum size of this array is:

(10 bytes/entry) * 255 entries = 2550 bytes.

CALLING PARAMETERS

AX = 5400h

Contains the Get Handle Directory function.

handle_dir_struct               STRUC

handle_value                DW ?

handle_name                 DB 8 DUP (?)

handle_dir_struct               ENDS

ES:DI = pointer to handle_dir

Contains a pointer to an area of memory into which the

memory manager will copy the handle directory.  The

handle directory is an array of structures.  There are

as many entries in the array as there are open EMM

handles.  The structure consists of the following

elements:

.handle_value

The first member is a word which contains the value

of the open EMM handle.

EMM Functions                                                 102

Function 21. Get Handle Directory

Get Handle Directory subfunction

.handle_name

The second member is an 8 byte array which contains

the ASCII name associated with the EMM handle.  If

there is no name currently associated with the

handle, it has a value of all zeros (ASCII nulls).

RESULTS

These results are valid only if the status returned is zero.

handle_dir

Contains the handle values and handle names associated

with each handle value.

AL = number of entries in the handle_dir array

Contains the number of entries in the handle directory

array.  This is also the same as the number of open

handles.  For example, if only one handle is active, AL

will contain a one.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The handle directory has been returned.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EMM Functions                                                 103

Function 21. Get Handle Directory

Get Handle Directory subfunction

EXAMPLE

handle_dir                    handle_dir_struct 255 DUP (?)

num_entries_in_handle_dir     DB ?

MOV   AX,SEG handle_dir

MOV   ES,AX

LEA   DI,handle_dir                 ; ES:DI points to handle_dir

MOV   AX,5400h                      ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

MOV   num_entries_in_handle_dir,AL  ; save number of entries

EMM Functions                                                 104

Function 21. Get Handle Directory

Search For Named Handle subfunction

PURPOSE

This subfunction searches the handle name directory for a

handle with a particular name.  If the named handle is

found, this subfunction returns the handle number associated

with the name.  At the time of installation, all handles

have their names initialized to ASCII nulls.  A handle with

a name which is all ASCII nulls has, by definition, no name.

When a handle is assigned a name, at least one character in

the name must be a non-null character in order to distin-

guish it from a handle without a name.

CALLING PARAMETERS

AX = 5401h

Contains the Search for Named Handle subfunction.

DS:SI = handle_name

Contains a pointer to an 8-byte string that contains the

name of the handle being searched for.

RESULTS

These results are valid only if the status returned is zero.

DX = value of named handle

The value of the handle which matches the handle name

specified.

REGISTERS MODIFIED

AX, DX

STATUS

AH = 0   SUCCESSFUL.

The handle value for the named handle has been found.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

EMM Functions                                                 105

Function 21. Get Handle Directory

Search For Named Handle subfunction

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A0h   NON-RECOVERABLE.

No corresponding handle could be found for the handle

name specified.

AH = A1h   NON-RECOVERABLE.

A handle found had no name (all ASCII nulls).

EXAMPLE

named_handle                   DB 'AARDVARK'

named_handle_value             DW ?

MOV   AX,SEG named_handle

MOV   DS,AX

LEA   SI,named_handle          ; DS:SI points to named_handle

MOV   AX,5401h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   named_handle_value,DX    ; save value of named handle

EMM Functions                                                 106

Function 21. Get Handle Directory

Get Total Handles subfunction

PURPOSE

This subfunction returns the total number of handles that

the memory manager supports, including the operating system

handle (handle value 0).

CALLING PARAMETERS

AX = 5402h

Contains the Get Total Handles subfunction.

RESULTS

These results are valid only if the status returned is zero.

BX = total_handles

The value returned represents the maximum number of

handles which a program may request the memory manager

to allocate memory to.  The value returned includes the

operating system handle (handle value 0).

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The total number of handles supported has been returned.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EMM Functions                                                 107

Function 21. Get Handle Directory

Get Total Handles subfunction

EXAMPLE

total_handles                  DW ?

MOV   AX,5402h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   total_handles,BX         ; save total handle count

EMM Functions                                                 108

Function 22. Alter Page Map & Jump

PURPOSE

This function alters the memory mapping context and trans-

fers control to the specified address.  It is analogous to

the FAR JUMP in the 8086 family architecture.  The memory

mapping context which existed before the invocation of this

function is lost.

Mapping no pages and jumping is not considered an error.  If

a request to map zero pages and jump is made, control is

transferred to the target address, and this function

performs a far jump.

CALLING PARAMETERS

AH = 55h

Contains the Alter Page Map & Jump function.

log_phys_map_struct             STRUC

log_page_number             DW ?

phys_page_number_seg        DW ?

log_phys_map_struct             ENDS

map_and_jump_struct             STRUC

target_address              DD ?

log_phys_map_len            DB ?

log_phys_map_ptr            DD ?

map_and_jump_struct             ENDS

AL = physical page number/segment selector

Contains a code which indicates whether the value

contained in the

.log_phys_map.phys_page_number_seg

members are physical page numbers or are the segment

address representation of the physical page numbers.  A

zero in AL indicates that the values are physical page

numbers.  A one in AL indicates that the values in these

members are the segment address representations of the

physical page numbers.

DX = handle number

Contains the EMM handle.

EMM Functions                                                 109

Function 22. Alter Page Map & Jump

DS:SI = pointer to map_and_jump structure

Contains a pointer to a structure that contains the

information necessary to map the desired physical pages

and jump to the target address.  The structure consists

of the following elements:

.target_address

The first member is a far pointer which contains the

target address to which control is to be trans-

ferred.  The address is represented in segment:of-

fset format.  The offset portion of the address is

stored in the low portion of the double word.

.log_phys_map_len

The second member is a byte which contains the

number of entries in the array of structures which

immediately follows it.  The array is as long as the

application developer needs in order to map the

desired logical pages into physical pages.  The

number of entries cannot exceed the number of

mappable pages in the system.

.log_phys_map_ptr

The third member is a pointer to an array of struc-

tures which contain the logical page numbers and

physical pages or segment address at which they are

to be mapped.  Each entry in the array of structures

contains the following two elements:

.log_page_number

The first member of this structure is a word which

contains the number of the logical page to be

mapped.

.phys_page_number_seg

The second member of this structure is a word which

contains either the physical page number or the

segment address representation of the physical page

number at which the previous logical page number is

to be mapped.  The value passed in AL determines the

type of representation.

REGISTERS MODIFIED

AX

EMM Functions                                                 110

Function 22. Alter Page Map & Jump

Note............................................................

Values in registers which don't contain required parameters

maintain the values across the jump.  The values in regis-

ters (with the exception of AX) and the flag state at the

beginning of the function are still in the registers and

flags when the target address is reached.

STATUS

AH = 0   SUCCESSFUL.

Control has been transferred to the target address.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager could not find the specified EMM handle.

The manager does not currently have any information

pertaining to the specified EMM handle.  The program may

have corrupted its EMM handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Ah   RECOVERABLE.

One or more of the logical pages to map into a cor-

responding physical page is out of the range of logical

pages which are allocated to the EMM handle.  The

program can recover from this condition by mapping a

logical page which is within the bounds for the EMM

handle.

AH = 8Bh   RECOVERABLE.

One or more of the physical pages is out of the range of

allowable physical pages, or the log_phys_map_len

exceeds the number of mappable pages in the system.

Physical page numbers are numbered zero-relative.  The

program can recover from this condition by mapping into

memory at a physical page which is in the range of

supported physical pages.

EMM Functions                                                 111

Function 22. Alter Page Map & Jump

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

log_phys_map              log_phys_map_struct (?) DUP (?)

map_and_jump              map_and_jump_struct (?)

emm_handle                          DW ?

phys_page_or_seg_mode               DB ?

MOV   AX,SEG map_and_jump

MOV   DS,AX

LEA   SI,map_and_jump               ; DS:SI points to

; map_and_jump

MOV   DX,emm_handle

MOV   AH,55h                        ; load function code

MOV   AL,phys_page_or_seg_mode      ; specify physical page

; or segment mode

INT   67h                           ; call memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

EMM Functions                                                 112

Function 23. Alter Page Map & Call

Alter Page Map & Call subfunction

PURPOSE

This subfunction saves the current memory mapping context,

alters the specified memory mapping context, and transfers

control to the specified address.  It is analogous to the

FAR CALL in the 8086 family architecture.  Just as a return

from a FAR CALL restores the original value in the code

segment register, this subfunction restores the state of the

specified mapping context after the return.

There is no explicit expanded memory subfunction which

emulates a return from a FAR CALL.  However, this facility

is implicitly available through the standard return from a

FAR CALL.  The following paragraphs describe how this works:

After this function is invoked, unless an error is detected,

the memory manager will transfer control to the address

specified.  If an error occurs, the memory manager returns

immediately with the error code in the AH register.

Otherwise, the memory manager pushes on the stack informa-

tion which enables it to restore the mapping context after

the return.

When the called procedure wants to return to the calling

procedure, it simply issues a standard FAR RETURN.  The

memory manager traps this return, restores the specified

mapping context, and returns to the calling procedure.  The

memory manager also returns a status from a successful

return just as it does for all functions.

Developers using this subfunction must make allowances for

the additional stack space this subfunction will use.

CALLING PARAMETERS

AH = 56h

Contains the Alter Page Map & Call function.

log_phys_map_struct             STRUC

log_page_number             DW ?

phys_page_number_seg        DW ?

log_phys_map_struct             ENDS

EMM Functions                                                 113

Function 23. Alter Page Map & Call

Alter Page Map & Call subfunction

map_and_call_struct             STRUC

target_address              DD ?

new_page_map_len            DB ?

new_page_map_ptr            DD ?

old_page_map_len            DB ?

old_page_map_ptr            DD ?

reserved                    DW 4 DUP (?)

map_and_call_struct             ENDS

AL = physical page number/segment selector

Contains a code which indicates whether the value

contained in the

.new_page_map.phys_page_number_seg

.old_page_map.phys_page_number_seg

members are physical page numbers or are the segment

address representation of the physical page numbers.  A

value of zero in AL indicates that the values in these

members are physical page numbers.  A value of one in AL

indicates that the values in these members are the

segment address representations of the physical page

numbers.

DX = handle number

Contains the EMM handle.

DS:SI = pointer to map_and_call structure

Contains a pointer to a structure which contains the

information necessary to map the desired physical pages

and call the target address.  The structure members are

described here:

.target_address

The first member is a far pointer which contains the

target address to which control is to be trans-

ferred.  The address is represented in segment:of-

fset format.  The offset portion of the address is

stored in the low portion of the pointer.  The

application must supply this value.

.new_page_map_len

The second member is a byte which contains the

number of entries in the new mapping context to

which new_page_map_ptr points.  This number cannot

exceed the number of mappable pages in the system.

EMM Functions                                                 114

Function 23. Alter Page Map & Call

Alter Page Map & Call subfunction

.new_page_map_ptr

The third member is a far pointer that points to an

array of structures which contains a list of the

logical page numbers and the physical page num-

bers/segments at which they are to be mapped im-

mediately after the call.  The contents of the new

array of structures are described at the end of the

map_and_call structure.

.old_page_map_len

The fourth member is a byte which contains the

number of entries in the old mapping context to

which old_page_map_ptr points.  This number cannot

exceed the number of mappable pages in the system.

.old_page_map_ptr

The fifth member is a far pointer that points to an

array of structures which contains a list of the

logical page numbers and the physical page num-

bers/segments at which they are to be mapped im-

mediately after the return.  The contents of the old

array of structures are described at the end of the

map_and_call structure.

.reserved

The sixth member is reserved for use by the memory

manager.

Each entry in the old and new array of structures contains

two elements:

.log_page_number

The first member of this structure is a word which

contains a logical page number which is to be mapped

at the succeeding physical page number/segment

immediately after the CALL (in the case of the new

array of structures) or after the RETURN (in the

case of the old array of structures).

.phys_page_number_seg

The second member of this structure is a word which

contains either the physical page number or the

segment address representation of the physical page

number/segment at which the preceding logical page

is to be mapped immediately after the CALL (in the

case of the new array of structures) or after the

RETURN (in the case of the old array of structures).

EMM Functions                                                 115

Function 23. Alter Page Map & Call

Alter Page Map & Call subfunction

REGISTERS MODIFIED

AX

Note............................................................

Values in registers which don't contain required parameters

maintain the values across the call.  The values in regis-

ters (with the exception of AX) and the flag state at the

beginning of the function are still in the registers and

flags when the target address is reached.

STATUS

AH = 0   SUCCESSFUL.

Control has been transferred to the target address.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager couldn't find the specified EMM handle.  The

manager doesn't have any information pertaining to the

specified EMM handle.  The program may have corrupted

its EMM handle.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Ah   RECOVERABLE.

One or more of the logical pages to map into a cor-

responding physical page is out of the range of logical

pages which are allocated to the EMM handle.  The

program can recover from this condition by mapping a

logical page which is within the bounds for the EMM

handle.

EMM Functions                                                 116

Function 23. Alter Page Map & Call

Alter Page Map & Call subfunction

AH = 8Bh   RECOVERABLE.

One or more of the physical pages is out of the range of

allowable physical pages, or you've specified more

physical pages than exist in the system.  Physical page

numbers are numbered zero-relative.  The program can

recover from this condition by mapping a physical page

which is in the range from zero to three.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

new_page_map              log_phys_map_struct (?) DUP (?)

old_page_map              log_phys_map_struct (?) DUP (?)

map_and_call              map_and_call_struct (?)

emm_handle                DW ?

phys_page_or_seg_mode     DB ?

MOV   AX,SEG map_and_call

MOV   DS,AX

LEA   SI,map_and_call               ; DS:SI points to

; map_and_call

MOV   DX,emm_handle                 ; specify EMM handle

MOV   AH,56h                        ; load function code

MOV   AL,phys_page_or_seg_mode      ; specify physical page

; or segment mode

INT   67h                           ; control is actually

; transferred to the called

; procedure at this point

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

EMM Functions                                                 117

Function 23. Alter Page Map & Call

Get Page Map Stack Space Size subfunction

PURPOSE

Since the Alter Page Map & Call function pushes additional

information onto the stack, this subfunction returns the

number of bytes of stack space the function requires.

CALLING PARAMETERS

AX = 5602h

Contains the Get Page Map Stack Space Size subfunction.

RESULTS

These results are valid only if the status returned is zero.

BX = stack space required

Contains the number of bytes which the Alter Page Map &

Call function will require.  In other words, BX contains

the number (including the return address) which has to

be added to the stack pointer to remove all elements

from the stack.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The size of the array has been returned.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

EMM Functions                                                 118

Function 23. Alter Page Map & Call

Get Page Map Stack Space Size subfunction

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

stack_space_reqd               DW ?

MOV   AX,5602h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   stack_space_reqd,BX      ; save required stack size count

EMM Functions                                                 119

Function 24. Move/Exchange Memory Region

Move Memory Region subfunction

PURPOSE

This subfunction copies a region of memory in the following

memory source/destination combinations.

o   conventional memory to conventional memory

o   conventional memory to expanded memory

o   expanded memory to conventional memory

o   expanded memory to expanded memory

You do not have to save and restore the expanded memory

mapping context to perform these move operations.  The

current mapping context is maintained throughout this

operation.

The length of the region is limited by the amount of

expanded memory allocated to the handles specified.

However, in most practical applications, the region length

will be considerably smaller.  A region length of zero is

not an error, and no move will be performed.

A region length which exceeds 16K bytes is not an error.  In

this case the function assumes that a group of logical pages

is the target for the move.  The logical page specified

represents the first logical page in which the move will

take place.  If the region length exceeds 16K bytes, or if

the region is less than 16K bytes but spans logical pages,

there must be sufficient logical pages remaining after the

first logical page for the entire region to fit.

If your application needs to save a region of conventional

memory in expanded memory, you can move it without having to

perform a save or restore of the current mapping context.

The memory manager maintains the context.  A move of up to

1M bytes may be performed, although practical lengths are

substantially less than this value.

If the source and destination handles are identical, the

source and destination regions are tested for overlap before

the move.  If they overlap, the move direction is chosen so

that the destination region receives an intact copy of the

source region.  A status will be returned indicating that

this overlap has occurred.

EMM Functions                                                 120

Function 24. Move/Exchange Memory Region

Move Memory Region subfunction

CALLING PARAMETERS

AX = 5700h

Contains the Move Memory Region function.

move_source_dest_struct         STRUC

region_length               DD ?

source_memory_type          DB ?

source_handle               DW ?

source_initial_offset       DW ?

source_initial_seg_page     DW ?

dest_memory_type            DB ?

dest_handle                 DW ?

dest_initial_offset         DW ?

dest_initial_seg_page       DW ?

move_source_dest_struct         ENDS

DS:SI = pointer to move_source_dest structure

Contains a pointer to a data structure which contains

the source and destination information for the move.

The structure members are described here:

.region_length

The first member is a double word which specifies

the length of the memory region (in bytes) to be

moved.

.source_memory_type

The second member is a byte which specifies the type

of memory where the source region resides.  A value

of zero indicates that the source region resides in

conventional memory (excluding the page frame seg-

ment).  A value of one indicates that the source

region resides in expanded memory.

.source_handle

If the source region resides in expanded memory, the

third member is a word which specifies the handle

number associated with the source memory region.  If

the source region resides in conventional memory,

this variable has no meaning and should be set to

zero for future compatibility.

.source_initial_offset

The fourth member is a word which specifies the

offset within the source region from which to begin

the move.

EMM Functions                                                 121

Function 24. Move/Exchange Memory Region

Move Memory Region subfunction

If the source region resides in expanded memory, the

source_initial_offset is relative to the beginning

of the 16K logical page.  Because the offset is

relative to the beginning of a 16K expanded memory

page, it may only take on values between 0000h and

3FFFh.

If the source region resides in conventional memory,

the source_initial_offset is a word which specifies

the offset, relative to the beginning of the source

segment, from which to begin the move.  Because the

offset is relative to the beginning of a 64K-byte

conventional memory segment, it may take on values

between 0000h and FFFFh.

.source_initial_seg_page

The fifth member is a word which specifies the

initial segment or logical page number within the

source region from which to begin the move.

If the source region resides in expanded memory, the

value specifies the logical page within the source

region from which to begin the move.

If the source region resides in conventional memory,

the source_initial_seg_page specifies the initial

segment address within conventional memory from

which to begin the move.

.dest_memory_type

The sixth member is a byte which specifies the type

of memory where the destination region resides.  A

value of zero indicates conventional memory; a value

of one indicates expanded memory.

.dest_handle

If the destination region resides in expanded

memory, the seventh member is a word which specifies

the handle number associated with the destination

memory region.  If the destination region resides in

conventional memory, this variable has no meaning

and should be set to zero for future compatibility.

.dest_initial_offset

The eighth member is a word which specifies the

offset within the destination region from which to

begin the move.

EMM Functions                                                 122

Function 24. Move/Exchange Memory Region

Move Memory Region subfunction

If the destination region resides in expanded

memory, the dest_initial_offset is relative to the

beginning of the 16K-byte logical page.  Because the

offset is relative to the beginning of a 16K-byte

expanded memory page, it may only take on values

between 0000h and 3FFFh.

If the destination region resides in conventional

memory, the dest_initial_offset is a word which

specifies the offset, relative to the beginning of

the destination segment, to begin the move.  Because

the offset is relative to the beginning of a 64K

conventional memory segment, it may take on values

between 0000h and FFFFh.

.dest_initial_seg_page

The ninth member is a word which specifies the

initial segment or logical page number within the

destination region from which to begin the move.

If the destination region resides in expanded memory

then the value specifies the logical page within the

destination region from which to begin the move.

If the destination region resides in conventional

memory, the dest_initial_seg_page specifies the

initial segment address within conventional memory

from which to begin the move.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The memory regions have been moved.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

EMM Functions                                                 123

Function 24. Move/Exchange Memory Region

Move Memory Region subfunction

AH = 83h   NON-RECOVERABLE.

The manager couldn't find either the source or destina-

tion EMM handles.  The memory manager doesn't have any

information on the handles specified.  The program may

have corrupted its EMM handles.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Ah   NON-RECOVERABLE.

One or more of the logical pages is out of the range of

logical pages allocated to the source/destination

handle.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 92h   SUCCESSFUL.

The source and destination expanded memory regions have

the same handle and overlap.  This is valid for a move.

The move has been completed and the destination region

has a full copy of the source region.  However, at least

a portion of the source region has been overwritten by

the move.  Note that the source and destination expanded

memory regions with different handles will never physi-

cally overlap because the different handles specify

totally different regions of expanded memory.

AH = 93h   CONDITIONALLY-RECOVERABLE.

The length of the source or destination expanded memory

region specified exceeds the length of the expanded

memory region allocated either the source or destination

handle.  Insufficient pages are allocated to this handle

to move a region of the size specified.  The program can

recover from this condition by allocating additional

pages to the destination or source handle and attempting

to execute the function again.  However, if the applica-

tion program allocated as much expanded memory as it

thought it needed, this may be a program error and is

not recoverable.

AH = 94h   NON-RECOVERABLE.

The conventional memory region and expanded memory

region overlap.  This is invalid, the conventional

memory region cannot overlap the expanded memory region.

EMM Functions                                                 124

Function 24. Move/Exchange Memory Region

Move Memory Region subfunction

AH = 95h   NON-RECOVERABLE.

The offset within the logical page exceeds the length of

the logical page.  The initial source or destination

offsets within an expanded memory region must be between

0000h and 3FFFh (16383 or (length of a logical page

- 1)).

AH = 96h   NON-RECOVERABLE.

Region length exceeds 1M bytes.

AH = 98h   NON-RECOVERABLE.

The memory source and destination types are undefined.

AH = A2h   NON-RECOVERABLE.

An attempt was made to wrap around the 1M-byte address

space of conventional memory during the move.  The

combination of source/destination starting address and

length of the region to be moved exceeds 1M bytes.  No

data was moved.

EXAMPLE

move_source_dest               move_source_dest_struct (?)

MOV   AX,SEG move_source_dest

MOV   DS,AX

LEA   SI,move_source_dest      ; DS:SI points to move_source_dest

MOV   AX,5700h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 125

Function 24. Move/Exchange Memory Region

Exchange Memory Region subfunction

PURPOSE

This subfunction exchanges (using a string move) a region of

memory in any of the following memory source/destination

combinations.

o   conventional memory to conventional memory

o   conventional memory to expanded memory

o   expanded memory to conventional memory

o   expanded memory to expanded memory

The term expanded memory region refers only to the area of

memory above 640K bytes (9FFFFh).  If a system provides

mappable conventional memory, this function treats the

mappable conventional memory regions as ordinary convention-

al memory.  The contents of the source region and the

destination region are exchanged.

The exchange operation can be performed without having to

save and restore the expanded memory mapping context.  The

current mapping context is maintained throughout this

operation.  The length of the region is limited to the

amount of expanded memory allocated to the specified EMM

handles.  A length of zero is not an error; however, no

exchange will be performed.  A region length which exceeds

16K bytes is not an error.  In this case the function

assumes that a group of logical pages is the target for the

exchange.  The logical page specified represents the first

logical page in which the exchange will take place.  If the

region length exceeds 16K bytes, or if the region is less

than 16K bytes but spans logical pages, there must be

sufficient logical pages remaining after the first logical

page for the entire region to fit.

If your application needs to exchange a region of conven-

tional memory with expanded memory, you can simply exchange

it with the region of interest without having to perform a

save or restore of the current mapping context.  An exchange

of up to 1M bytes may be performed, although practical

lengths are obviously below that value.  Checking is done

before starting the exchange to prevent the possibility of

overlap during the exchange operation.  Overlapping source

and destination regions for an exchange are invalid, and the

exchange will not take place.

EMM Functions                                                 126

Function 24. Move/Exchange Memory Region

Exchange Memory Region subfunction

CALLING PARAMETERS

AX = 5701h

Contains the Exchange Memory Region function.

xchg_source_dest_struct         STRUC

region_length               DD ?

source_memory_type          DB ?

source_handle               DW ?

source_initial_offset       DW ?

source_initial_seg_page     DW ?

dest_memory_type            DB ?

dest_handle                 DW ?

dest_initial_offset         DW ?

dest_initial_seg_page       DW ?

xchg_source_dest_struct         ENDS

DS:SI = pointer to xchg_source_dest structure

Contains a pointer to the data structure which contains

the source and destination information for the exchange.

The structure members are described here:

.region_length

The first member is a double word which specifies

the length of the memory region to be exchanged.

.source_memory_type

The second member is a byte which specifies the type

of memory where the source region resides.  A value

of zero indicates that the source region resides in

conventional memory.  A value of one indicates that

the source region resides in expanded memory.

.source_handle

If the source region resides in expanded memory, the

third member is a word which specifies the handle

number associated with the source memory region.  If

the source region resides in conventional memory,

this variable has no meaning and should be set to

zero for future compatibility.

.source_initial_offset

The fourth member is a word which specifies the

offset within the source region from which to begin

the exchange.

EMM Functions                                                 127

Function 24. Move/Exchange Memory Region

Exchange Memory Region subfunction

If the source region resides in expanded memory, the

source_initial_offset is relative to the beginning

of the 16K logical page.  Because the offset is

relative to the beginning of a 16K expanded memory

page, it may only take on values between 0000h and

3FFFh.

If the source region resides in conventional memory,

the source_initial_offset is a word which specifies

the offset, relative to the beginning of the source

segment, from which to begin the exchange at.

Because the offset is relative to the beginning of a

64K-byte conventional memory segment, it may take on

values between 0000h and FFFFh.

.source_initial_seg_page

The fifth member is a word which specifies the

initial segment or logical page number within the

source region from which to begin the exchange.

If the source region resides in expanded memory then

the value specifies the logical page within the

source region from which to begin the exchange.

If the source region resides in conventional memory,

the source_initial_seg_page specifies the initial

segment address within conventional memory from

which to begin the exchange.

.dest_memory_type

The sixth member is a byte which specifies the type

of memory where the destination region resides.  A

value of zero indicates that the destination region

resides in conventional memory (excluding the page

frame segment).  A value of one indicates that the

destination region resides in expanded memory.

.dest_handle

If the destination region resides in expanded

memory, the seventh member is a word which specifies

the handle number associated with the destination

memory region.  If the destination region resides in

conventional memory, this variable has no meaning

and should be set to zero for future compatibility.

EMM Functions                                                 128

Function 24. Move/Exchange Memory Region

Exchange Memory Region subfunction

.dest_initial_offset

The eighth member is a word which specifies the

offset within the destination region from which to

begin the exchange.

If the destination region resides in expanded

memory, the dest_initial_offset is relative to the

beginning of the 16K-byte logical page.  Because the

offset is relative to the beginning of a 16K-byte

expanded memory page, it may only take on values

between 0000h and 3FFFh.

If the destination region resides in conventional

memory, the dest_initial_offset is a word which

specifies the offset, relative to the beginning of

the destination segment, to begin the exchange at.

Because the offset is relative to the beginning of a

64K conventional memory segment, it may take on

values between 0000h and FFFFh.

.dest_initial_seg_page

The ninth member is a word which specifies the

initial segment or logical page number within the

destination region from which to begin the exchange.

If the destination region resides in expanded memory

then the value specifies the logical page within the

destination region from which to begin the exchange.

If the destination region resides in conventional

memory, the dest_initial_seg_page specifies the

initial segment address within conventional memory

from which to begin the exchange.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The memory regions have been exchanged.

EMM Functions                                                 129

Function 24. Move/Exchange Memory Region

Exchange Memory Region subfunction

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE.

The manager could not find either the source or destina-

tion EMM handles.  The memory manager does not currently

have any information pertaining to the handles speci-

fied.  The program may have corrupted its EMM handles.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Ah   NON-RECOVERABLE.

One or more of the logical pages is out of the range of

logical pages allocated to the source/destination

handle.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 93h   CONDITIONALLY-RECOVERABLE.

The length of the source or destination expanded memory

region specified, exceeds the length of the expanded

memory region allocated to the source or destination

specified EMM handle.  There are insufficient pages

allocated to this handle to exchange a region of the

size specified.  The program can recover from this

condition by attempting to allocate additional pages to

the destination or source handle and attempting to

execute the function again.  However, if the application

program was allocated as much expanded memory as it

thought it needed, this may be a program error and is

therefore not recoverable.

AH = 94h   NON-RECOVERABLE.

The conventional memory region and expanded memory

region overlap.  This is invalid, the conventional

memory region cannot overlap the expanded memory region.

EMM Functions                                                 130

Function 24. Move/Exchange Memory Region

Exchange Memory Region subfunction

AH = 95h   NON-RECOVERABLE.

The offset within the logical page exceeds the length of

the logical page.  The initial source or destination

offsets within an expanded memory region must be between

0000h and 3FFFh (16383 or (length of a logical page

- 1)).

AH = 96h   NON-RECOVERABLE.

Region length exceeds 1M-byte limit.

AH = 97h   NON-RECOVERABLE.

The source and destination expanded memory regions have

the same handle and overlap.  This is invalid, the

source and destination expanded memory regions cannot

have the same handle and overlap when they are being

exchanged.  Note that the source and destination

expanded memory regions which have different handles

will never physically overlap because the different

handles specify totally different regions of expanded

memory.

AH = 98h   NON-RECOVERABLE.

The memory source and destination types are undefined.

AH = A2h   NON-RECOVERABLE.

An attempt was made to wrap around the 1M-byte address

space of conventional memory during the exchange.  The

combination of source/destination starting address and

length of the region to be exchanged exceeds 1M bytes.

No data was exchanged.

EXAMPLE

xchg_source_dest               xchg_source_dest_struct (?)

MOV   AX,SEG xchg_source_dest

MOV   DS,AX

LEA   SI,xchg_source_dest      ; DS:SI points to xchg_source_dest

MOV   AX,5701h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 131

Function 25. Get Mappable Physical Address Array

Get Mappable Physical Address Array subfunction

PURPOSE

This subfunction returns an array containing the segment

address and physical page number for each mappable physical

page in a system.  The contents of this array provide a

cross reference between physical page numbers and the actual

segment addresses for each mappable page in the system.  The

array is sorted in ascending segment order.  This does not

mean that the physical page numbers associated with the

segment addresses are also in ascending order.

CALLING PARAMETERS

AX = 5800h

Contains the Get Mappable Physical Address Array

subfunction

mappable_phys_page_struct       STRUC

phys_page_segment           DW ?

phys_page_number            DW ?

mappable_phys_page_struct       ENDS

ES:DI = mappable_phys_page

Contains a pointer to an application-supplied memory

area where the memory manager will copy the physical

address array.  Each entry in the array is a structure

containing two members:

.phys_page_segment

The first member is a word which contains the

segment address of the mappable physical page

associated with the physical page number following

it.  The array entries are sorted in ascending

segment address order.

.phys_page_number

The second member is a word which contains the

physical page number which corresponds to the

previous segment address.  The physical page numbers

are not necessarily in ascending order.

EMM Functions                                                 132

Function 25. Get Mappable Physical Address Array

Get Mappable Physical Address Array subfunction

Example 1

An expanded memory board has its page frame starting

at address C0000h and has no mappable conventional

memory.  For this configuration, physical page 0

corresponds to segment address C000h, physical page

1 corresponds to segment address C400h, etc.  The

array would contain the following data (in this

order):

C000h, 00h

C400h, 01h

C800h, 02h

CC00h, 03h

Example 2

An expanded memory board has a large page frame

starting at address C0000h and has mappable conven-

tional memory from 90000h through 9FFFFh.  For this

configuration, physical page 0 corresponds to

segment address C000h, physical page 1 corresponds

to segment address C400h, etc.  The array would

contain the following data in the order specified.

Note that the expanded memory region always has the

lowest numerically valued physical page numbers.

9000h, 0Ch

9400h, 0Dh

9800h, 0Eh

9C00h, 0Fh

C000h, 00h

C400h, 01h

C800h, 02h

CC00h, 03h

D000h, 04h

D400h, 05h

D800h, 06h

DC00h, 07h

E000h, 08h

E400h, 09h

E800h, 0Ah

EC00h, 0Bh

EMM Functions                                                 133

Function 25. Get Mappable Physical Address Array

Get Mappable Physical Address Array subfunction

RESULTS

These results are valid only if the status returned is zero.

CX = number of entries in the mappable_phys_page

Multiply this number by (SIZE mappable_phys_page_struct)

to determine the number of bytes the physical page

address array requires.

REGISTERS MODIFIED

AX, CX

STATUS

AH = 0   SUCCESSFUL.

The hardware configuration array has been returned.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EMM Functions                                                 134

Function 25. Get Mappable Physical Address Array

Get Mappable Physical Address Array subfunction

EXAMPLE

mappable_phys_page             mappable_phys_page_struct (?)

mappable_page_entry_count      DW ?

MOV   AX,SEG mappable_phys_page

MOV   ES,AX

LEA   DI,mappable_phys_page         ; ES:DI points to

; mappable_phys_page

MOV   AX,5800h                      ; load function code

INT   67h                           ; call the memory

; manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler

; on error

MOV   mappable_page_entry_count,CX  ; save mappable

; page entry count

EMM Functions                                                 135

Function 25. Get Mappable Physical Address Array

Get Mappable Physical Address Array Entries subfunction

PURPOSE

This subfunction gets the number of entries which will be

required for the array the first subfunction returns.

CALLING PARAMETERS

AX = 5801h

Contains the Get Physical Page Address Array Entries

subfunction.  This subfunction returns a word which

represents the number of entries in the array returned

by the previous subfunction.  This number also repre-

sents the number of mappable physical pages in a system.

RESULTS

These results are valid only if the status returned is zero.

CX = number of entries in the mappable_phys_page

Multiply this number by (SIZE mappable_phys_page_struct)

to determine the number of bytes the physical page

address array will require.

REGISTERS MODIFIED

AX, CX

STATUS

AH = 0   SUCCESSFUL.

The number of mappable physical pages has been returned.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

EMM Functions                                                 136

Function 25. Get Mappable Physical Address Array

Get Mappable Physical Address Array Entries subfunction

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

mappable_page_entry_count           DW ?

MOV   AX,5801h                      ; load function code

INT   67h                           ; call memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler

; on error

MOV   mappable_page_entry_count,CX  ; save mappable

; page entry count

EMM Functions                                                 137

Function 26. Get Expanded Memory Hardware Information

Get Hardware Configuration Array subfunction

Note............................................................

This function is for use by operating systems only.  This

function can be disabled at any time by the operating

system.  Refer to Function 30 for a description of how an

operating system does this.

PURPOSE

This subfunction returns an array containing expanded memory

hardware configuration information for use by an operating

system/environment.

CALLING PARAMETERS

AX = 5900h

Contains the Get Hardware Configuration Array subfunc-

tion.

hardware_info_struct            STRUC

raw_page_size               DW ?

alternate_register_sets     DW ?

context_save_area_size      DW ?

DMA_register_sets           DW ?

DMA_channel_operation       DW ?

hardware_info_struct            ENDS

ES:DI = hardware_info

Contains a pointer to a memory area that the operating

system supplies where the memory manager will copy

expanded memory hardware information.  The structure

contains these five members:

.raw_page_size

The first member is a word which contains the size

of a raw mappable physical page in paragraphs (16

bytes).  LIM standard pages are always 16K bytes.

However, other implementations of expanded memory

boards do not necessarily comply with this standard

and can emulate a 16K-byte page by mapping in

multiple smaller pages.  This member specifies the

size of a mappable physical page viewed from the

hardware implementation level.

EMM Functions                                                 138

Function 26. Get Expanded Memory Hardware Information

Get Hardware Configuration Array subfunction

.alternate_register_sets

The second member is a word which specifies the

number of alternate mapping register sets.  The

additional mapping register sets are termed alter-

nate mapping register sets in this document.

All expanded memory boards have at least one set of

hardware registers to perform the logical to

physical page mapping.  Some expanded memory boards

have more than one set of these mapping registers.

This member specifies how many of these alternate

mapping register sets exist (beyond the one set that

all expanded memory boards have) on the expanded

memory boards in the system.  If an expanded memory

card has only one set of mapping registers (that is,

no alternate mapping register sets) this member has

a value of zero.

.context_save_area_size

The third member is a word which contains the

storage requirements for the array required to save

a mapping context.  The value returned in this

member is exactly the same as that returned by

Function 15 (Get Size of Page Map Save Array

subfunction).

.DMA_register_sets

The fourth member is a word which contains the

number of register sets that can be assigned to DMA

channels.  These DMA register sets, although similar

in use to alternate register sets, are for DMA

mapping and not task mapping.

If the expanded memory hardware does not support DMA

register sets, care must be taken when DMA is taking

place.

In a multitasking operating system, when one task is

waiting for DMA to complete, it is useful to be able

to switch to another task.  However, if the DMA is

taking place in memory that the second task will

need to remap, remapping would be disastrous.

EMM Functions                                                 139

Function 26. Get Expanded Memory Hardware Information

Get Hardware Configuration Array subfunction

If the expanded memory hardware can detect when DMA

is occurring, the OS/E should allow task switches

and remapping during DMA.  If no special support for

DMA is available, no remapping should be done when

DMA is in progress.

.DMA_channel_operation

The fifth member is a word which specifies a special

case for the DMA register sets.  A value of zero

specifies that the DMA register sets behave as

described in Function 28.  A value of one specifies

that the expanded memory hardware has only one DMA

register set.  In addition, if any channel is mapped

through this register set, then all channels are

mapped through it.  For LIM standard boards, this

value is zero.

RESULTS

These results are valid only if the status returned is zero.

hardware_info

Contains the expanded memory hardware-specific informa-

tion described above.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The hardware configuration array has been returned.

AH = 80h   NON-RECOVERABLE.

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE.

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the manager is not defined.

EMM Functions                                                 140

Function 26. Get Expanded Memory Hardware Information

Get Hardware Configuration Array subfunction

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE.

Access to this function has been denied by the operating

system.  The function cannot be used at this time.

EXAMPLE

hardware_info                  hardware_info_struct (?)

MOV   AX,SEG hardware_info

MOV   ES,AX

LEA   DI,hardware_info         ; ES:DI points to hardware_info

MOV   AX,5900h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 141

Function 26. Get Expanded Memory Hardware Information

Get Unallocated Raw Page Count subfunction

PURPOSE

The Get Unallocated Raw Page Count subfunction returns the

number of unallocated non-standard length mappable pages as

well as the total number of non-standard length mappable

pages in expanded memory to the operating system.

One variety of expanded memory board has a page size which

is a sub-multiple of 16K bytes.  An expanded memory page

which is a sub-multiple of 16K is termed a raw page.  An

operating system may deal with mappable physical page sizes

which are sub-multiples of 16K bytes.

If the expanded memory board supplies pages in exact

multiples of 16K bytes, the number of pages this function

returns is identical to the number Function 3 (Get Unallo-

cated Page Count) returns.  In this case, there is no

difference between a page and a raw page.

CALLING PARAMETERS

AX = 5901h

Contains the Get Unallocated Raw Page Count subfunction.

RESULTS

These results are valid only if the status returned is zero.

BX = unallocated raw pages

The number of raw pages that are currently available for

use.

DX = total raw pages

The total number of raw pages in expanded memory.

REGISTERS MODIFIED

AX, BX, DX

EMM Functions                                                 142

Function 26. Get Expanded Memory Hardware Information

Get Unallocated Raw Page Count subfunction

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the number of unallocated raw

pages and the number of total raw pages in expanded

memory.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

unalloc_raw_pages              DW ?

total_raw_pages                DW ?

MOV   AX,5901h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   unalloc_raw_pages,BX     ; save unallocated raw page count

MOV   total_raw_pages,DX       ; save total raw page count

EMM Functions                                                 143

Function 27. Allocate Standard/Raw Pages

Allocate Standard Pages subfunction

PURPOSE

The Allocate Standard Pages subfunction allocates the number

of standard size (16K bytes) pages that the operating system

requests and assigns a unique EMM handle to these pages.

The EMM handle owns these pages until the operating system

deallocates them.  This subfunction allows you to allocate

zero pages to a handle, unlike Function 4 (Allocate Pages).

Note............................................................

This note affects expanded memory manager implementors and

operating system developers only.  Applications should not

use the following characteristic of the memory manager.  An

application violating this rule will be incompatible with

future versions of Microsoft's operating systems and

environments.

To be compatible with this specification, an expanded memory

manager will provide a special handle which is available to

the operating system only.  This handle will have a value of

0000h and will have a set of pages allocated to it when the

expanded memory manager driver installs.  The pages that the

memory manager will automatically allocate to handle 0000h

are those that backfill conventional memory.  Typically,

this backfill occurs between addresses 40000h (256K) and

9FFFFh (640K).  However, the range can extend below and

above this limit if the hardware and memory manager have the

capability.

An operating system won't have to invoke Function 27 to

obtain this handle because it can assume the handle already

exists and is available for use immediately after the

expanded memory device driver installs.  When an operating

system wants to use this handle, it uses the special handle

value of 0000h.  The operating system will be able to invoke

any EMM function using this special handle value.  To

allocate pages to this handle, the operating system need

only invoke Function 18 (Reallocate Pages).

There are two special cases for this handle:

1.  Function 27 (Allocate Standard Pages subfunction).  This

function must never return zero as a handle value.

Applications must always invoke Function 27 to allocate

pages and obtain a handle which identifies the pages

which belong to it.  Since Function 27 never returns a

EMM Functions                                                 144

Function 27. Allocate Standard/Raw Pages

Allocate Standard Pages subfunction

handle value of zero, an application will never gain

access to this special handle.

2.  Function 6 (Deallocate Pages).  If the operating system

uses it to deallocate the pages which are allocated to

this handle, the pages the handle owns will be returned

to the manager for use.  But the handle will not be

available for reassignment.  The manager should treat a

deallocate pages function request for this handle the

same as a reallocate pages function request, where the

number of pages to reallocate to this handle is zero.

CALLING PARAMETERS

AX = 5A00h

Contains the Allocate Standard Pages subfunction.

BX = num_of_standard_pages_to_alloc

Contains the number of standard pages the operating

system wants to allocate.

RESULTS

These results are valid only if the status returned is zero.

DX = handle

Contains a unique EMM handle.  The operating system must

use this EMM handle as a parameter in any function that

requires it.  Up to 255 handles may be obtained.  (Both

Function 27 and Function 4 must share the same 255

handles.)

For all functions using this handle, the length of the

physical and logical pages allocated to it are standard

length (that is, 16K bytes).

REGISTERS MODIFIED

AX, DX

EMM Functions                                                 145

Function 27. Allocate Standard/Raw Pages

Allocate Standard Pages subfunction

STATUS

AH = 0   SUCCESSFUL.

The manager has allocated the pages to an assigned EMM

standard handle.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 85h   RECOVERABLE.

All EMM handles are being used.

AH = 87h   RECOVERABLE.

There aren't enough expanded memory pages present in the

system to satisfy the operating system's request.

AH = 88h   RECOVERABLE.

There aren't enough unallocated pages to satisfy the

operating system's request.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

num_of_standard_pages_to_alloc      DW ?

emm_handle                          DW ?

MOV   BX,num_of_standard_pages_to_alloc

MOV   AX,5A00h                      ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler on

; error

MOV   emm_handle,DX                 ; save handle

EMM Functions                                                 146

Function 27. Allocate Standard/Raw Pages

Allocate Raw Pages subfunction

PURPOSE

The Allocate Raw Pages function allocates the number of non-

standard size pages that the operating system requests and

assigns a unique EMM handle to these pages.  The EMM handle

owns these pages until the operating system deallocates

them.  This function allows you to allocate zero pages to a

handle, unlike Function 4 (Allocate Pages).

A hardware vendor may design an expanded memory board that

has a page size which is a sub-multiple of 16K bytes.  A

physical page which is a sub-multiple of 16K is termed a raw

page.  The operating system may deal with page sizes which

are sub-multiples of 16K bytes.  The memory manager must

treat any function using a handle with raw pages allocated

to it by Function 27 (Allocate Raw Pages subfunction)

differently than it does a handle that has normal 16K-byte

pages allocated to it.

Handles which are assigned using Function 4 (Allocate Pages)

or Function 27 (Allocate Standard Pages subfunction) must

have pages which are 16K bytes -- this is the length of a

standard expanded memory page.  If the expanded memory board

hardware is not able to supply 16K-byte pages, the memory

manager must emulate pages which are 16K bytes combining

multiple non-standard size pages to form a single 16K-byte

page.

Handles which are assigned using Function 27 (Allocate Raw

Pages subfunction) are called raw handles.  All logical

pages allocated to a raw handle may have a non-standard

length (that is, not 16K bytes).  However, once the operat-

ing system has allocated a number of raw pages to a handle,

it is the responsibility of the memory manager to recognize

that raw handle as one that has non-standard size pages

allocated to it.  The memory manager must identify these

handles and treat all functions which use handles which have

non-standard page lengths differently.  The logical page

length becomes the length of the non-standard size page for

any raw handle that Function 27 assigns.

Note............................................................

This note affects expanded memory manager implementors and

operating system developers only.  Applications should not

use the following characteristic of the memory manager.  An

application violating this rule will be incompatible with

EMM Functions                                                 147

Function 27. Allocate Standard/Raw Pages

Allocate Raw Pages subfunction

future versions of Microsoft's operating systems and

environments.

To be compatible with this specification, an expanded memory

manager will provide a special handle which is available to

the operating system only.  This handle will have a value of

0000h and will have a set of pages allocated to it when the

expanded memory manager driver installs.  The pages that the

memory manager will automatically allocate to handle 0000h

are those that backfill conventional memory.  Typically,

this backfill occurs between addresses 40000h (256K) and

9FFFFh (640K).  However, the range can extend below and

above this limit if the hardware and memory manager have the

capability.

An operating system won't have to invoke Function 27 to

obtain this handle because it can assume the handle already

exists and is available for use immediately after the

expanded memory device driver installs.  When an operating

system wants to use this handle, it uses the special handle

value of 0000h.  The operating system will be able to invoke

any EMM function using this special handle value.  To

allocate pages to this handle, the operating system need

only invoke Function 18 (Reallocate Pages).

There are two special cases for this handle:

1.  Function 27 (Allocate Raw Pages subfunction).  This

function must never return zero as a handle value.

Applications must always invoke Function 27 to allocate

pages and obtain a handle which identifies the pages

which belong to it.  Since Function 27 never returns a

handle value of zero, an application will never gain

access to this special handle.

2.  Function 6 (Deallocate Pages).  If the operating system

uses it to deallocate the pages which are allocated to

this handle, the pages the handle owns will be returned

to the manager for use.  But the handle will not be

available for reassignment.  The manager should treat a

deallocate pages function request for this handle the

same as a reallocate pages function request, where the

number of pages to reallocate to this handle is zero.

EMM Functions                                                 148

Function 27. Allocate Standard/Raw Pages

Allocate Raw Pages subfunction

CALLING PARAMETERS

AX = 5A01h

Contains the Allocate Raw Pages subfunction.

BX = num_of_raw_pages_to_alloc

Contains the number of raw pages the operating system

wishes to allocate.

RESULTS

These results are valid only if the status returned is zero.

DX = raw handle

Contains a unique EMM raw handle.  The operating system

must use this EMM raw handle as a parameter in any

function that requires it.  Up to 255 handles may be

obtained.  (Both Function 4 and Function 27 must share

the same 255 handles).

For all functions using this raw handle, the length of

the physical and logical pages allocated to it may be

non-standard (that is, not 16K bytes).

REGISTERS MODIFIED

AX, DX

STATUS

AH = 0   SUCCESSFUL.

The manager has allocated the raw pages to an assigned

EMM raw handle.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

EMM Functions                                                 149

Function 27. Allocate Standard/Raw Pages

Allocate Raw Pages subfunction

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 85h   RECOVERABLE.

All EMM handles are being used.

AH = 87h   RECOVERABLE.

There aren't enough expanded memory raw pages present in

the system to satisfy the operating system's request.

AH = 88h   RECOVERABLE.

There aren't enough unallocated raw pages to satisfy the

operating system's request.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EXAMPLE

num_of_raw_pages_to_alloc           DW ?

emm_raw_handle                      DW ?

MOV   BX,num_of_raw_pages_to_alloc

MOV   AX,5A01h                      ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler

; on error

MOV   emm_raw_handle,DX             ; save raw handle

EMM Functions                                                 150

Function 28. Alternate Map Register Set

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

Design Considerations

The hardware support for the entire set of subfunctions

described is generally not present on every expanded memory

board from every vendor of expanded memory board products.

For some of the subfunctions, software emulation is provid-

ed.  For other subfunctions, a certain protocol in their use

must be observed.  The subfunctions for which this is most

crucial are those which address system DMA capabilities.

System DMA Capabilities & Expanded Memory Support of DMA

In a multitasking operating system, when one task is waiting

for DMA to complete, it is useful to be able to switch to

another task.  This specification describes a capability

which may be designed into expanded memory boards to provide

DMA into memory regions which may be mapped out while the

DMA is occurring.  For expanded memory boards that do not

provide this, it is crucial to understand that while DMA is

in progress into a region of mappable memory, the memory

mapping context cannot be changed.  That is, all DMA action

must be complete before any remapping of pages can be done.

Expanded Memory Support of DMA Register Sets

Expanded memory boards which have DMA register sets could

support DMA into a region of mappable memory while the

memory mapping context is being switched.  It is important

to realize that these DMA register sets are separate from

the alternate map register sets.  An example of how an OS/E

might use DMA register sets follows:

Example 1

1.  Allocate a DMA register set.

2.  Get current register set.

3.  Set the DMA register set.

EMM Functions                                                 151

Function 28. Alternate Map Register Set

4.  Map in the memory desired.

5.  Get the DMA register set.

6.  Set the original register set.

7.  Assign the desired DMA channel to the DMA register set.

The preceding set of calls makes all DMA accesses for the

desired DMA channel get mapped through the current DMA

register set regardless of the current register set.  In

other words, the DMA register set overrides the current

mapping register set for DMA operations on the DMA channel

specified.  A DMA channel that is not assigned to a DMA

register set has all its DMA operations mapped through the

current mapping register set.

EMM Functions                                                 152

Function 28. Alternate Map Register Set

Get Alternate Map Register Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

The subfunction does one of two things depending on the map

register set which is active at the time this function is

invoked:

1.  If the preceding Set Alternate Map Register Set call was

done with the alternate map register set equal to zero

(BL = 0), these points apply:

a.  The context save area pointer saved within EMM by

the Set Alternate Map Register subfunction is

returned by this call.  This pointer is always

returned for boards which do not supply alternate

mapping register sets.

b.  If the context save area pointer returned is not

equal to zero, this subfunction copies the contents

of the mapping registers on each expanded memory

board in the system into the save area specified by

the pointer.  The format of this save area is the

same as that returned by Function 15 (Get Page Map

subfunction).  This is intended to simulate getting

an alternate map register set.  Note that the memory

manager does not allocate the space for the context:

the operating system must do so.

c.  If the context save area pointer returned is equal

to zero, this subfunction does not copy the contents

of the mapping registers on each expanded memory

board in the system into the save area specified by

the pointer.

d.  The context save area pointer must have been

initialized by a previous Set Alternate Map Register

Set call.  Note that the value of the context save

area pointer saved within EMM is zero immediately

after installation.

EMM Functions                                                 153

Function 28. Alternate Map Register Set

Get Alternate Map Register Set subfunction

e.  The context save area must be initialized by a

previous Get Page Map call (Function 15).

2.  If the preceding Set Alternate Map Register Set call was

done with the alternate map register set greater than

zero (BL > 0), then the number of the alternate map

register set which is in use at the time that this

function is invoked is returned.  The context save area

pointer is not returned in this case.

CALLING PARAMETERS

AX = 5B00h

Contains the Get Alternate Map Register Set subfunction.

RESULTS

These results are valid only if the status returned is zero.

If BL <> 0, current active alternate map register set number

Contains the alternate map register set which was active

at the time that this function was invoked.

ES:DI   Unaffected.

If BL = 0

Indicates that a pointer to an area which contains the

state of all the map registers on all boards in the

system, and any additional information necessary to

restore the boards to their original state, has been

returned.

ES:DI = pointer to a map register context save area

Contains a pointer to an operating system supplied

context save area.  The pointer is in standard seg-

ment:offset format.  This pointer is always returned if

the expanded memory hardware does not supply alternate

mapping register sets.

EMM Functions                                                 154

Function 28. Alternate Map Register Set

Get Alternate Map Register Set subfunction

The operating system first passes this pointer to the

memory manager whenever it invokes a Set Alternate Map

Register Set subfunction (the description follows).  If

the OS/E invokes this function before invoking a Set

Alternate Map Register Set subfunction, this function

returns a pointer value of zero.  The OS/E must have

allocated the space for the save area.  However, the OS

must request that the memory manager initialize the

contents of this save area before it contains any useful

information.

The OS/E must initialize the save area it has allocated

by invoking Function 15 (Get Page Map subfunction).

After the OS/E has done this, the save area will contain

the state of all the map registers on all boards in the

system.  The save area will also contain any additional

information necessary to restore the boards to their

original state when the operating system invokes a Set

Alternate Map Register Set subfunction.

REGISTERS MODIFIED

AX, BX, ES:DI

STATUS

AH = 0   SUCCESSFUL.

The manager got the alternate map register set.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

EMM Functions                                                 155

Function 28. Alternate Map Register Set

Get Alternate Map Register Set subfunction

AH = A4h   NON-RECOVERABLE.

The operating system denied access to this function.

The function cannot be used at this time.

EXAMPLE

alt_map_reg_set                        DB ?

context_save_area_ptr_seg              DW ?

context_save_area_ptr_offset           DW ?

MOV   AX,5B00h                         ; load function code

INT   67h                              ; call the memory manager

OR    AH,AH                            ; check EMM status

JNZ   emm_err_handler                  ; jump to error handler

; on error

MOV   alt_map_reg_set,BL

TEST  BL,BL

JNZ   no_ptr_returned

MOV   context_save_area_ptr_seg,ES     ; save pointer values

MOV   context_save_area_ptr_offset,DI

no_ptr_returned:

EMM Functions                                                 156

Function 28. Alternate Map Register Set

Set Alternate Map Register Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

The subfunction does one of two things, depending on the map

register set specified:

1.  If the alternate map register set specified is zero, map

register set zero is activated.  If the map register

context restore area pointer is not equal to zero, the

contents of the restore area pointed to by ES:DI are

copied into register set zero on each expanded memory

board in the system.  If the pointer is equal to zero,

the contents are not copied.

Regardless of its value, the map register context

restore area pointer is saved within the memory manager.

It will be used during the Get Alternate Map Register

Set subfunction.

The operating system must supply the pointer to the

area.  This subfunction is intended to simulate setting

an alternate map register set.  Note that the operating

system must allocate the space for the context.  The

memory manager saves the context save area pointer

internally.

2.  If the alternate map register set specified is not zero,

the alternate map register set specified is activated.

The restore area, which the operating system is pointing

to, is not used.

CALLING PARAMETERS

AX = 5B01h

Contains the Set Alternate Map Register Set subfunction.

EMM Functions                                                 157

Function 28. Alternate Map Register Set

Set Alternate Map Register Set subfunction

BL = new alternate map register set number

Contains the number of the alternate map register set

which is to be activated.

If BL <> 0

A pointer to a map register context restore area is

not required and the contents of ES:DI is unaffected

and ignored.  The alternate map register set

specified in BL is activated if the board supports

it.

If BL = 0

A pointer to an area which contains the state of all

the map registers on all boards in the system, and

any additional information necessary to restore the

boards to their original state, has been passed in

ES:DI.

ES:DI = pointer to a map register context restore area

Contains a pointer to an OS/E supplied map register

context restore area.  The pointer is in standard

segment:offset format.  This pointer must always be

passed if the expanded memory hardware does not supply

alternate mapping register sets.

The memory manager must save this pointer whenever the

OS/E invokes this function.  The OS/E must have allo-

cated the space for the restore area.  Additionally, the

contents of this restore area must have been initialized

by the memory manager before it will contain any useful

information.  The OS/E initializes the restore area it

has allocated by invoking Function 15 (Get Page Map

subfunction).  After the OS/E has done this, the restore

area will contain the state of the map registers on all

boards in the system, and any additional information

necessary to restore the boards to their original state

when the operating system invokes a Set Alternate Map

Register Set subfunction.

REGISTERS MODIFIED

AX

EMM Functions                                                 158

Function 28. Alternate Map Register Set

Set Alternate Map Register Set subfunction

STATUS

AH = 0   SUCCESSFUL.

The manager set the alternate map register set.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 9Ah   NON-RECOVERABLE.

Alternate map register sets are supported, but the

alternate map register set specified is not supported.

AH = 9Ch   NON-RECOVERABLE.

Alternate map register sets are not supported, and the

alternate map register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE.

Alternate map register sets are supported, but the

alternate map register set specified is either not

defined or not allocated.

AH = A3h   NON-RECOVERABLE.

The contents of the source array have been corrupted, or

the pointer passed to the subfunction is invalid.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.

EMM Functions                                                 159

Function 28. Alternate Map Register Set

Set Alternate Map Register Set subfunction

EXAMPLE

alt_map_reg_set                        DB ?

context_restore_area_ptr_seg           DW ?

context_restore_area_ptr_offset        DW ?

MOV   AX,5B01h                         ; load function code

MOV   BL,alt_map_reg_set

TEST  BL,BL

JZ    no_ptr_passed

MOV   ES,context_restore_area_ptr_seg

MOV   DI,context_restore_area_ptr_offset

no_ptr_passed:

INT   67h                              ; call the memory manger

OR    AH,AH                            ; check EMM status

JNZ   emm_err_handler                  ; jump to error handler

; on error

EMM Functions                                                 160

Function 28. Alternate Map Register Set

Get Alternate Map Save Array Size subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

This subfunction returns the storage requirements for the

map register context save area referenced by the other

subfunctions.

CALLING PARAMETERS

AX = 5B02h

Contains the Get Alternate Map Save Array Size subfunc-

tion.

RESULTS

These results are valid only if the status returned is zero.

DX = size_of_array

Contains the number of bytes that will be transferred to

the memory area supplied by an operating system whenever

an operating system requests the Get, Set, or Get and

Set subfunction.

REGISTERS MODIFIED

AX, DX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the array size.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

EMM Functions                                                 161

Function 28. Alternate Map Register Set

Get Alternate Map Save Array Size subfunction

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

size_of_array                  DW ?

MOV   AX,5B02h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   size_of_array,DX         ; save size of array

EMM Functions                                                 162

Function 28. Alternate Map Register Set

Allocate Alternate Map Register Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

The Allocate Alternate Map Register Set subfunction gets the

number of an alternate map register set for an operating

system if an alternate map register set is currently

available for use.  If the hardware does not support

alternate map register sets, an alternate map register set

number of zero will be returned.

The alternate map register set allocated may be referred to

by this number when using the Get or Set Alternate Map

Register Set subfunctions.  The operating system can use

these subfunctions to switch map contexts very rapidly on

expanded memory boards with alternate map register sets.

This subfunction copies the currently active alternate map

register set's contents into the newly allocated alternate

map register set's mapping registers.  This is done so that

when the OS/E performs a Set Alternate Map Register Set

subfunction the memory mapped before the allocation of the

new alternate map will be available for reading and writing.

This function does not actually change the alternate map

register set in use, but in addition to allocating a new

alternate map register set, it prepares the new alternate

map register set for a subsequent Set Alternate Map Register

Set subfunction.

CALLING PARAMETERS

AX = 5B03h

Contains the Allocate Alternate Map Register Set

subfunction.

EMM Functions                                                 163

Function 28. Alternate Map Register Set

Allocate Alternate Map Register Set subfunction

RESULTS

These results are valid only if the status returned is zero.

BL = alternate map register set number

Contains the number of an alternate map register set.

If there are no alternate map register sets supported by

the hardware, a zero will be returned.  In this case,

the Get Alternate Map function (Function 28) should be

invoked in order to obtain a pointer to a map register

context save area.  The OS/E must supply this area.  The

save area is necessary because the hardware doesn't

support alternate map register sets.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The manager has returned the alternate map register set

number.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 9Bh   NON-RECOVERABLE.

Alternate map register sets are supported.  However, all

alternate map register sets are currently allocated.

EMM Functions                                                 164

Function 28. Alternate Map Register Set

Allocate Alternate Map Register Set subfunction

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

alt_map_reg_num                DB ?

MOV   AX,5B03h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   alt_map_reg_num,BL       ; save number of

; alternate map register set

EMM Functions                                                 165

Function 28. Alternate Map Register Set

Deallocate Alternate Map Register Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

The Deallocate Alternate Map Register Set subfunction

returns the alternate map register set to the memory manager

for future use.  The memory manager may reallocate the

alternate map register set when needed.

This subfunction also makes the mapping context of the

alternate map register specified unavailable for reading or

writing (unmapping).  This protects the pages previously

mapped in an alternate map register set by making them

inaccessible.  Note that the current alternate map register

set cannot be deallocated.  This makes all memory which was

currently mapped into conventional and expanded memory

inaccessible.

CALLING PARAMETERS

AX = 5B04h

Contains the Deallocate Alternate Map Register Set

subfunction.

BL = alternate register set number

Contains the number of the alternate map register set to

deallocate.  Map register set zero cannot be allocated

or deallocated.  However, if alternate map register set

zero is specified and this subfunction is invoked, no

error will be returned.  The function invocation is

ignored in this case.

REGISTERS MODIFIED

AX

EMM Functions                                                 166

Function 28. Alternate Map Register Set

Deallocate Alternate Map Register Set subfunction

STATUS

AH = 0   SUCCESSFUL.

The manager has deallocated the alternate map register

set.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 9Ch   NON-RECOVERABLE.

Alternate map register sets are not supported and the

alternate map register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE.

Alternate map register sets are supported, but the

alternate map register set specified is either not

defined or not allocated.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

alternate_map_reg_set               DB ?

MOV   BL,alternate_map_reg_set      ; specify alternate map

; register set

MOV   AX,5B04h                      ; load function code

INT   67h                           ; call the memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler

; on error

EMM Functions                                                 167

Function 28. Alternate Map Register Set

Allocate DMA Register Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

The Allocate DMA Register Set subfunction gets the number of

a DMA register set for an OS/E, if a DMA register set is

currently available for use.  If the hardware does not

support DMA register sets, a DMA register set number of zero

will be returned.

In a multitasking operating system, when one task is waiting

for DMA to complete, it is useful to be able to switch to

another task.  However, if the DMA is being mapped through

the current register set, the switching cannot occur.  That

is, all DMA action must be complete before any remapping of

pages can be done.

The operating system would initiate a DMA operation on a

specific DMA channel using a specific alternate map register

set.  This alternate map register set would not be used

again, by the operating system or an application, until

after the DMA operation is complete.  The operating system

guarantees this by not changing the contents of the alter-

nate map register set, or allowing an application to change

the contents of the alternate map register set, for the

duration of the DMA operation.

CALLING PARAMETERS

AX = 5B05h

Contains the Allocate DMA Register Set subfunction.

RESULTS

These results are valid only if the status returned is zero.

BL = DMA register set number

Contains the number of a DMA register set.  If there are

no DMA register sets supported by the hardware, a zero

will be returned.

EMM Functions                                                 168

Function 28. Alternate Map Register Set

Allocate DMA Register Set subfunction

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0   SUCCESSFUL.

The manager has allocated the DMA register set.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 9Bh   NON-RECOVERABLE.

DMA register sets are supported.  However, all DMA

register sets are currently allocated.

AH = A4h   NON-RECOVERABLE.

Access to this function has been denied by the operating

system.  The function cannot be used at this time.

EXAMPLE

DMA_reg_set_number                  DB ?

MOV   AX,5B05h                      ; load function code

INT   67h                           ; call memory manager

OR    AH,AH                         ; check EMM status

JNZ   emm_err_handler               ; jump to error handler

; on error

MOV   DMA_reg_set_number,BL         ; save number of DMA

; register set

EMM Functions                                                 169

Function 28. Alternate Map Register Set

Enable DMA on Alternate Map Register Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

This subfunction allows DMA accesses on a specific DMA

channel to be associated with a specific alternate map

register set.  In a multitasking operating system, when a

task is waiting for the completion of DMA, it is useful to

be able to switch to another task until the DMA operation

completes.

Any DMA on the specified channel will go through the speci-

fied DMA register set regardless of the current register

set.  If a DMA channel is not assigned to a DMA register

set, DMA for that channel will be mapped through the current

register set.

CALLING PARAMETERS

AX = 5B06h

Contains the Enable DMA on Alternate Map Register Set

subfunction.

BL = DMA register set number

Contains the number of the alternate map register set to

be used for DMA operations on the DMA channel specified

by DL.  If the alternate map register set specified is

zero, no special action will be taken on DMA accesses

for the DMA channel specified.

DL = DMA channel number

Contains the DMA channel which is to be associated with

the DMA map register set specified in BL.

REGISTERS MODIFIED

AX

EMM Functions                                                 170

Function 28. Alternate Map Register Set

Enable DMA on Alternate Map Register Set subfunction

STATUS

AH = 0   SUCCESSFUL.

The manager has enabled DMA on the DMA register set and

the DMA channel specified.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 9Ah   NON-RECOVERABLE.

Alternate DMA register sets are supported, but the

alternate DMA register set specified is not supported.

AH = 9Ch   NON-RECOVERABLE.

Alternate DMA register sets are not supported, and the

DMA register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE.

DMA register sets are supported, but the DMA register

set specified is either not defined or not allocated.

AH = 9Eh   NON-RECOVERABLE.

Dedicated DMA channels are not supported.

AH = 9Fh   NON-RECOVERABLE.

Dedicated DMA channels are supported, but the DMA

channel specified is not supported.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.

EMM Functions                                                 171

Function 28. Alternate Map Register Set

Enable DMA on Alternate Map Register Set subfunction

EXAMPLE

alt_map_reg_set                DB ?

DMA_channel_num                DB ?

MOV   BL,alt_map_reg_set

MOV   DL,DMA_channel_num

MOV   AX,5B06h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 172

Function 28. Alternate Map Register Set

Disable DMA on Alternate Map Register Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

This subfunction disables DMA accesses for all DMA channels

which were associated with a specific alternate map register

set.

CALLING PARAMETERS

AX = 5B07h

Contains the Disable DMA on Alternate Map Register Set

subfunction.

BL = alternate register set number

Contains the number of the DMA register set for which

all operations are to be disabled.  If the alternate map

register set specified is zero, no action will be taken.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has disabled DMA operations on the alternate

DMA register set.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

EMM Functions                                                 173

Function 28. Alternate Map Register Set

Disable DMA on Alternate Map Register Set subfunction

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 9Ah   NON-RECOVERABLE.

Alternate DMA register sets are supported, but the

alternate DMA register set specified is not supported.

AH = 9Ch   NON-RECOVERABLE.

Alternate DMA register sets are not supported, and the

DMA register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE.

DMA register sets are supported, but the DMA register

set specified is either not defined or not allocated.

AH = 9Eh   NON-RECOVERABLE.

Dedicated DMA channels are not supported.

AH = 9Fh   NON-RECOVERABLE.

Dedicated DMA channels are supported, but the DMA

channel specified is not supported.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

DMA_reg_set                    DB ?

MOV   BL,DMA_reg_set

MOV   AX,5B07h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 174

Function 28. Alternate Map Register Set

Deallocate DMA Register Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

Refer to Function 30 for a description of how an operating

system can enable or disable this function.

PURPOSE

The Deallocate DMA Register Set subfunction deallocates the

specified DMA register set.

CALLING PARAMETERS

AX = 5B08h

Contains the Deallocate DMA Register Set subfunction.

BL = DMA register set number

Contains the number of the DMA register set which should

not be used for DMA operations any longer.  The DMA

register set would have been previously allocated and

enabled for DMA operations on a specific DMA channel.

If the DMA register set specified is zero, no action

will be taken.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has deallocated the DMA register set.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

EMM Functions                                                 175

Function 28. Alternate Map Register Set

Deallocate DMA on Alternate Map Register Set subfunction

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = 9Ch   NON-RECOVERABLE.

DMA register sets are not supported, and the DMA

register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE.

DMA register sets are supported, but the DMA register

set specified is either not defined or not allocated.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

DMA_reg_set_num                DB ?

MOV   BL,DMA_reg_set_num

MOV   AX,5B08h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 176

Function 29. Prepare Expanded Memory Hardware For Warm Boot

PURPOSE

This function prepares the expanded memory hardware for an

impending warm boot.  This function assumes that the next

operation that the operating system performs is a warm boot

of the system.  In general, this function will effect the

current mapping context, the alternate register set in use,

and any other expanded memory hardware dependencies which

need to be initialized at boot time.  If an application

decides to map memory below 640K, the application must trap

all possible conditions leading to a warm boot and invoke

this function before performing the warm boot itself.

CALLING PARAMETERS

AH = 5Ch

Contains the Prepare Expanded Memory Hardware for Warm

Boot function.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The manager has prepared the expanded memory hardware

for a warm boot.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

EMM Functions                                                 177

Function 29. Prepare Expanded Memory Hardware for Warm Boot

EXAMPLE

MOV   AH,5Ch                   ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 178

Function 30. Enable/Disable OS/E Function Set Functions

Enable OS/E Function Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

PURPOSE

This subfunction provides an OS/E with the ability to enable

all programs or device drivers to use the OS/E specific

functions.  The capability is provided only for an OS/E

which manages regions of mappable conventional memory and

cannot permit programs to use any of the functions which

affect mappable conventional memory regions, but must be

able to use these functions itself.  When an OS/E disables

these functions and a program attempts to use them, the

memory manager returns a status to the program indicating

that the OS/E has denied the program access to the function.

In other words, the functions will not work when disabled.

However, all programs may use them when enabled.

The OS/E (Operating System/Environment) functions this

subfunction enables are:

Function 26.  Get Expanded Memory Hardware Information.

Function 28.  Alternate Map Register Sets.

Function 30.  Enable/Disable Operating System Functions.

It appears contradictory that the OS/E can re-enable these

functions when the function which re-enables them is itself

disabled.  An overview of the process follows.

The memory manager enables all the OS/E specific functions,

including this one, when it is loaded.  The OS/E gets

exclusive access to these functions by invoking either of

the Enable/Disable OS/E Function Set subfunctions before any

other software does.

On the first invocation of either of these subfunctions, the

memory manager returns an access_key which the OS/E must use

in all future invocations of either of these subfunctions.

The memory manager does not require the access_key on the

first invocation of the Enable/Disable OS/E Function Set

subfunctions.

EMM Functions                                                 179

Function 30. Enable/Disable OS/E Function Set Functions

Enable OS/E Function Set subfunction

On all subsequent invocations, the access_key is required

for either the Enable or Disable OS/E Function Set subfunc-

tions.  Since the access_key is returned only on the first

invocation of the Enable/Disable OS/E Function Set subfunc-

tions, and presumably the OS/E is the first software to

invoke this function, only the OS/E obtains a copy of this

key.  The memory manager must return an access key with a

random value, a fixed value key defeats the purpose of

providing this level of security for an OS/E.

CALLING PARAMETERS

AX = 5D00h

Contains the Enable OS/E Function Set subfunction.

BX,CX = access_key

Required on all function invocations after the first.

The access_key value returned by the first function

invocation is required.

RESULTS

These results are valid only if the status returned is zero.

BX,CX = access_key

Returned only on the first function invocation, the

memory manager returns a random valued key which will be

required thereafter for the execution of this function.

On all invocations after the first, this key is not

returned.  Neither BX nor CX is affected after the first

time this function is invoked.

REGISTERS MODIFIED

AX, BX, CX

STATUS

AH = 0   SUCCESSFUL.

The operating system function set has been enabled.

EMM Functions                                                 180

Function 30. Enable/Disable OS/E Function Set Functions

Enable OS/E Function Set subfunction

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.  The value of

the key which was passed to this function does not

entitle the program to execute this function.

EXAMPLE

First invocation

access_key                     DW 2 DUP (?)

MOV   AX,5D00h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   access_key[0],BX

MOV   access_key[2],CX

All invocations after the first

access_key                     DW 2 DUP (?)

MOV   BX,access_key[0]

MOV   CX,access_key[2]

MOV   AX,5D00h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 181

Function 30. Enable/Disable OS/E Function Set Functions

Disable OS/E Function Set subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

PURPOSE

This subfunction provides an OS/E with the ability to

disable all programs or device drivers from using the OS/E

specific functions.  The capability is provided only for an

OS/E which manages regions of mappable conventional memory

and cannot permit programs to use any of the functions which

would affect mappable conventional memory regions.  When an

OS/E disables these functions and a program attempts to use

them, the memory manager returns a status to the program

indicating that the OS/E has denied the program access to

the function.  In other words, the functions will not work

when disabled.

The OS/E (Operating System) functions which are disabled by

this subfunction are:

Function 26.  Get Expanded Memory Hardware Information.

Function 28.  Alternate Map Register Sets.

Function 30.  Enable/Disable Operating System Functions.

CALLING PARAMETERS

AX = 5D01h

Contains the Disable OS/E Function Set subfunction.

BX,CX = access_key

Required on all function invocations after the first.

The access_key value returned by the first function

invocation is required.

EMM Functions                                                 182

Function 30. Enable/Disable OS/E Function Set Functions

Disable OS/E Function Set subfunction

RESULTS

These results are valid only if the status returned is zero.

BX,CX = access_key

Returned only on the first function invocation, the

memory manager returns a random valued key which will be

required thereafter for the execution of this function.

On all invocations after the first, this key is not

returned.  Neither BX nor CX is affected after the first

time this function is invoked.

REGISTERS MODIFIED

AX, BX, CX

STATUS

AH = 0   SUCCESSFUL.

The operating system function set has been disabled.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.  The value of

the key which was passed to this function does not

entitle the program to execute this function.

EMM Functions                                                 183

Function 30. Enable/Disable OS/E Function Set Functions

Disable OS/E Function Set subfunction

EXAMPLE

First Function invocation

access_key                     DW 2 DUP (?)

MOV   AX,5D01h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

MOV   access_key[0],BX

MOV   access_key[2],CX

All invocations after the first

access_key                     DW 2 DUP (?)

MOV   BX,access_key[0]

MOV   CX,access_key[2]

MOV   AX,5D01h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 184

Function 30. Enable/Disable OS/E Function Set Functions

Return Access Key subfunction

Note............................................................

This function is for use by operating systems only.  The

operating system can disable this function at any time.

PURPOSE

This subfunction provides an OS/E with the ability to return

the access key to the memory manager.  Returning the access

key to the memory manager places the memory manager in the

state it is in at installation time (regarding the use of

the OS/E function set and the access key).  That is, access

to the OS/E function set is enabled.  Upon execution of the

next enable/disable OS/E function set subfunction, the

access key will once again be returned.

CALLING PARAMETERS

AX = 5D02h

Contains the Return Access Key subfunction.

BX,CX = access_key

Required on all function invocations.  The access_key

value returned by the first function invocation of the

enable or disable subfunctions is required.

REGISTERS MODIFIED

AX

STATUS

AH = 0   SUCCESSFUL.

The access key has been returned to the memory manager.

AH = 80h   NON-RECOVERABLE.

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE.

The manager detected a malfunction in the expanded

memory hardware.

EMM Functions                                                 185

Function 30. Enable/Disable OS/E Function Set Functions

Return Access Key subfunction

AH = 84h   NON-RECOVERABLE.

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE.

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE.

The operating system has denied access to this function.

The function cannot be used at this time.  The value of

the key which was passed to this function does not

entitle the program to execute this function.

EXAMPLE

access_key                     DW 2 DUP (?)

MOV   BX,access_key[0]

MOV   CX,access_key[2]

MOV   AX,5D02h                 ; load function code

INT   67h                      ; call the memory manager

OR    AH,AH                    ; check EMM status

JNZ   emm_err_handler          ; jump to error handler on error

EMM Functions                                                 186

Appendix A

FUNCTION AND STATUS CODE CROSS REFERENCE TABLES

This appendix contains two cross reference tables:  one

lists the function codes and the status codes they return;

the other lists the status codes and the functions that

return them.

Table A-1.  Function and Status Code Cross Reference

----------------------------------------------------------------

Function        Status                 Description

----------------------------------------------------------------

40h    00h, 80h, 81h, 84h   Get Memory Manager Status

41h    00h, 80h, 81h, 84h   Get Page Frame Segment Address

42h    00h, 80h, 81h, 84h   Get Unallocated Page Count

43h    00h, 80h, 81h, 84h   Allocate Pages

85h, 87h, 88h, 89h

44h    00h, 80h, 81h, 83h   Map/Unmap Handle Page

84h, 8Ah, 8Bh

45h    00h, 80h, 81h, 83h   Deallocate Pages

84h, 86h

46h    00h, 80h, 81h, 84h   Get EMM Version

47h    00h, 80h, 81h, 83h   Save Page Map

84h, 8Ch, 8Dh

48h    00h, 80h, 81h, 83h   Restore Page Map

84h, 8Eh

49h                         Reserved

4Ah                         Reserved

4Bh    00h, 80h, 81h, 84h   Get EMM Handle Count

4Ch    00h, 80h, 81h, 83h   Get EMM Handle Pages

84h

4Dh    00h, 80h, 81h, 84h   Get All EMM Handle Pages

Cross Reference Tables                                        187

Table A-1.  Function and Status Code Cross Reference (continued)

----------------------------------------------------------------

Function        Status                 Description

----------------------------------------------------------------

4E00h    00h, 80h, 81h, 84h   Get Page Map

8Fh

4E01h    00h, 80h, 81h, 84h   Set Page Map

8Fh, A3h

4E02h    00h, 80h, 81h, 84h   Get & Set Page Map

8Fh, A3h

4E03h    00h, 80h, 81h, 84h   Get Size of Page Map Save Array

8Fh

4F00h    00h, 80h, 81h, 84h   Get Partial Page Map

8Bh, 8Fh, A3h

4F01h    00h, 80h, 81h, 84h   Set Partial Page Map

8Fh, A3h

4F02h    00h, 80h, 81h, 84h   Get Size of Partial Page Map Array

8Bh, 8Fh

5000h    00h, 80h, 81h, 83h   Map/Unmap Multiple Handle Pages

84h, 8Ah, 8Bh, 8Fh   (physical page number mode)

5001h    00h, 80h, 81h, 83h   Map/Unmap Multiple Handle Pages

84h, 8Ah, 8Bh, 8Fh   (segment address mode)

51h    00h, 80h, 81h, 83h   Reallocate Pages

84h, 87h, 88h

5200h    00h, 80h, 81h, 83h   Get Handle Attribute

84h, 8Fh, 91h

5201h    00h, 80h, 81h, 83h   Set Handle Attribute

84h, 8Fh, 90h, 91h

5202h    00h, 80h, 81h, 84h   Get Handle Attribute Capability

8Fh

5300h    00h, 80h, 81h, 83h   Get Handle Name

84h, 8Fh

5301h    00h, 80h, 81h, 83h   Set Handle Name

84h, 8Fh, A1h

Cross Reference Tables                                        188

Table A-1.  Function and Status Code Cross Reference (continued)

----------------------------------------------------------------

Function        Status                 Description

----------------------------------------------------------------

5400h    00h, 80h, 81h, 84h   Get Handle Directory

8Fh

5401h    00h, 80h, 81h, 84h   Search for Named Handle

8Fh, A0h, A1h

5402h    00h, 80h, 81h, 84h   Get Total Handles

8Fh

5500h    00h, 80h, 81h, 83h   Alter Page Map & Jump (Physical

84h, 8Ah, 8Bh, 8Fh   page mode)

5501h    00h, 80h, 81h, 83h   Alter Page Map & Jump (Segment

84h, 8Ah, 8Bh, 8Fh   address mode)

5600h    00h, 80h, 81h, 83h   Alter Page Map & Call (Physical

84h, 8Ah, 8Bh, 8Fh   page mode)

5601h    00h, 80h, 81h, 83h   Alter Page Map & Call (Segment

84h, 8Ah, 8Bh, 8Fh   address mode)

5602h    00h, 80h, 81h, 84h   Get Alter Page Map & Call Stack

8Fh                  Space Size

5700h    00h, 80h, 81h, 83h   Move Memory Region

84h, 8Ah, 8Fh, 92h

93h, 94h, 95h, 96h

98h, A2h

5701h    00h, 80h, 81h, 83h   Exchange Memory Region

84h, 8Ah, 8Fh, 93h

94h, 95h, 96h, 97h

98h, A2h

5800h    00h, 80h, 81h, 84h   Get Mappable Physical Address

8Fh                  Array

5801h    00h, 80h, 81h, 84h   Get Mappable Physical Address

8Fh                  Array Entries

5900h    00h, 80h, 81h, 84h   Get Expanded Memory Hardware

8Fh, A4h             Information

5901h    00h, 80h, 81h, 84h   Get Unallocated Raw Page Count

8Fh

Cross Reference Tables                                        189

Table A-1.  Function and Status Code Cross Reference (continued)

----------------------------------------------------------------

Function        Status                 Description

----------------------------------------------------------------

5A00h    00h, 80h, 81h, 84h   Allocate Standard Pages

85h, 87h, 88h, 8Fh

5A01h    00h, 80h, 81h, 84h   Allocate Raw Pages

85h, 87h, 88h, 8Fh

5B00h    00h, 80h, 81h, 84h   Get Alternate Map Register Set

8Fh, A4h

5B01h    00h, 80h, 81h, 84h   Set Alternate Map Register Set

8Fh, 9Ah, 9Ch, 9Dh

A3h, A4h

5B02h    00h, 80h, 81h, 84h   Get Alternate Map Save Array Size

8Fh, A4h

5B03h    00h, 80h, 81h, 84h   Allocate Alternate Map Register

8Fh, 9Bh, A4h        Set

5B04h    00h, 80h, 81h, 84h   Deallocate Alternate Map Register

8Fh, 9Ch, 9Dh, A4h   Set

5B05h    00h, 80h, 81h, 84h   Allocate DMA Register Set

8Fh, 9Bh, A4h

5B06h    00h, 80h, 81h, 84h   Enable DMA on Alternate Map

8Fh, 9Ah, 9Ch, 9Dh   Register Set

9Eh, 9Fh, A4h

5B07h    00h, 80h, 81h, 84h   Disable DMA on Alternate Map

8Fh, 9Ah, 9Ch, 9Dh   Register Set

9Eh, 9Fh, A4h

5B08h    00h, 80h, 81h, 84h   Deallocate DMA Register Set

8Fh, 9Ch, 9Dh, A4h

5Ch    00h, 80h, 81h, 84h   Prepare Expanded Memory Hardware

for Warmboot

5D00h    00h, 80h, 81h, 84h   Enable Operating System Function

8Fh, A4h             Set

5D01h    00h, 80h, 81h, 84h   Disable Operating System Function

8Fh, A4h             Set

Cross Reference Tables                                        190

Table A-1.  Function and Status Code Cross Reference (continued)

----------------------------------------------------------------

Function        Status                 Description

----------------------------------------------------------------

5D02h    00h, 80h, 81h, 84h   Return Operating System Access Key

8Fh, A4h

----------------------------------------------------------------

Cross Reference Tables                                        191

Table A-2.  Status and Function Code Cross Reference

----------------------------------------------------------------

Status      Function                   Description

----------------------------------------------------------------

00h   All                  The function completed normally.

80h   All                  The memory manager has detected a

malfunction in the expanded memory

software.  A condition has been

detected which would not have

occurred if the memory manager had

been operating correctly.

81h   All                  The memory manager has detected a

malfunction in the expanded memory

hardware.  A condition has been

detected which would not occur if the

memory hardware were working correct-

ly.  Diagnostics should be run on the

expanded memory system to determine

the source of the problem.

82h   None                 This error code is not returned in

version 3.2 of the memory manager or

above.  In earlier versions of the

memory manager this code meant a

"busy" status.  This status indicated

that the memory manager was already

processing an expanded memory request

when the current request was made and

is unable to process another request.

In versions 3.2 of the memory manager

and above, the memory manager is

never "busy" and can always honor

requests.

83h   44h, 45h, 47h, 48h   The memory manager can not find the

4Ch, 5000h, 5001h    handle specified.  The program has

51h, 5200h, 5201h    probably corrupted its specified

5300h, 5301h         handle.  The memory manager does not

5500h, 5501h         have any information pertaining to

5600h, 5601h         the  specified handle.  The program

5700h, 5701h         has probably corrupted its handle.

84h   All                  The function code passed to the

manager is not currently defined.

Function codes in the range 40h

through 5Dh are currently defined.

Cross Reference Tables                                        192

Table A-2.  Status and Function Code Cross Reference (continued)

----------------------------------------------------------------

Status      Function                   Description

----------------------------------------------------------------

85h   43h, 5A00h, 5A01h    No handles are currently available.

All assignable handles are currently

in use.  The program may re-request

the assignment of a handle in the

hope that another program has

released a handle.  The maximum

number of handles that may be

supported is 255.

86h   45h                  A mapping context restoration error

has been detected.  This error occurs

when a program attempts to return a

handle and there is still a "mapping

context" on the context stack for the

indicated handle.  A program can

recover from this error by restoring

the mapping context before returning

the handle.

87h   43h, 51h, 5A00h,     The number of total pages that are

5A01h                available in the system is insuffi-

cient to honor the request.  The

program can recover from this

condition by requesting fewer pages.

88h   43h, 51h, 5A00h,     The number of unallocated pages

5A01h                currently available is insufficient

to honor the allocation request.  The

program can recover from this

condition by re-posting the request

or by requesting fewer pages.

89h   43h                  A Function 4 (Allocate Pages) request

has been made specifying zero pages.

Zero pages cannot be assigned to a

handle with Function 4 (Allocate

Pages).  If it is necessary to assign

zero pages to a handle, Function 27

(Allocate Standard Pages and Allocate

Raw Pages subfunctions) may be used.

Cross Reference Tables                                        193

Table A-2.  Status and Function Code Cross Reference (continued)

----------------------------------------------------------------

Status      Function                   Description

----------------------------------------------------------------

8Ah   44h, 5000h, 5001h    The logical page to map into memory

5500h, 5501h         is out of the range of logical pages

5600h, 5601h         which are allocated to the handle.

5700h, 5701h         The program can recover from this

condition by attempting to map a

logical page which is within the

bounds for the handle.

8Bh   44h, 4F00h, 4F02h    One or more of the physical pages is

5000h, 5001h         out of the range of allowable

5600h, 5601h         physical pages.  Physical page

5500h, 5501          numbers are numbered zero-relative.

The program can recover from this

condition by mapping at a physical

page which is in the range from zero

to three.

8Ch   47h                  The mapping register context save

area is full.  The program can

recover from this condition by

attempting to save the mapping

registers again.

8Dh   47h                  The mapping register context stack

already has a context associated with

the handle.  The program has at-

tempted to save the mapping register

context when there was already a

context for the handle on the stack.

The program can recover from this

condition by not attempting to save

the context again (this assumes the

mapping register context on the stack

for the handle is correct).

Cross Reference Tables                                        194

Table A-2.  Status and Function Code Cross Reference (continued)

----------------------------------------------------------------

Status      Function                   Description

----------------------------------------------------------------

8Eh   48h                  The mapping register context stack

does not have a context associated

with the handle.  The program has

attempted to restore the mapping

register context when there was no

context for the handle on the stack.

The program can recover from this

condition by not attempting to

restore the context again (this

assumes the current mapping register

context is correct).

8Fh   All functions        The subfunction parameter passed to

requiring            the function is not defined.

subfunction codes

90h   5201h                The attribute type is undefined.

91h   5200h, 5201h         The system configuration does not

support non-volatility.

92h   5700h                The source and destination expanded

memory regions have the same handle

and overlap.  This is valid for a

move.  The move has been completed

and the destination region has a full

copy of the source region.  However,

at least a portion of the source

region has been overwritten by the

move.  Note that the source and

destination expanded memory regions

with different handles will never

physically overlap because the

different handles specify totally

different regions of expanded memory.

Cross Reference Tables                                        195

Table A-2.  Status and Function Code Cross Reference (continued)

----------------------------------------------------------------

Status      Function                   Description

----------------------------------------------------------------

93h   5700h, 5701h         The length of the specified source or

destination expanded memory region

exceeds the length of the expanded

memory region allocated to the

specified source or destination

handle.  There are insufficient pages

allocated to this handle to move/ex-

change a region of the size speci-

fied.  The program can recover from

this condition by attempting to

allocate additional pages to the

destination or source handle or by

reducing the specified length.

However, if the application program

has allocated as much expanded memory

as it thought it needed, this may be

a program error and is therefore not

recoverable.

94h   5700h, 5701h         The conventional memory region and

expanded memory region overlap.  This

is invalid, the conventional memory

region cannot overlap the expanded

memory region.

95h   5700h, 5701h         The offset within the logical page

exceeds the length of the logical

page.  The initial source or destina-

tion offsets within an expanded

memory region must be between 0 and

the (length of a logical page - 1) or

16383 (3FFFh).

96h   5700h, 5701h         Region length exceeds 1M-byte limit.

Cross Reference Tables                                        196

Table A-2.  Status and Function Code Cross Reference (continued)

----------------------------------------------------------------

Status      Function                   Description

----------------------------------------------------------------

97h   5701h                The source and destination expanded

memory regions have the SAME handle

AND overlap.  This is invalid; the

source and destination expanded

memory regions cannot have the same

handle and overlap when they are

being exchanged.  Note that the

source and destination expanded

memory regions with different handles

will never physically overlap because

the different handles specify totally

different regions of expanded memory.

98h   5700h, 5701h         The memory source and destination

types are undefined/not supported.

9Ah   5B01h, 5B06h         Alternate map register sets are

5B07h                supported, but the alternate map

register set specified is not

supported.

9Bh   5B03h, 5B05h         Alternate map/DMA register sets are

supported.  However, all alternate

map/DMA register sets are currently

allocated.

9Ch   5B01h, 5B04h         Alternate map/DMA register sets are

5B06h, 5B07h         not supported, and the alternate

5B08h                map/DMA register set specified is not

zero.

9Dh   5B01h, 5B04h         Alternate map/DMA register sets are

5B06h, 5B07h         supported, but the alternate map

5B08h                register set specified is not

defined, not allocated, or is the

currently allocated map register set.

9Eh   5B06h, 5B07h         Dedicated DMA channels are not

supported.

9Fh   5B06h, 5B07h         Dedicated DMA channels are supported.

But the DMA channel specified is not

supported.

Cross Reference Tables                                        197

Table A-2.  Status and Function Code Cross Reference (continued)

----------------------------------------------------------------

Status      Function                   Description

----------------------------------------------------------------

A0h   5401h                No corresponding handle value could

be found for the handle name speci-

fied.

A1h   5301h, 5401h         A handle with this name already

exists.  The specified handle was not

assigned a name.

A2h   5700h, 5701h         An attempt was made to "wrap around"

the 1M-byte address space during the

move/exchange.  The source starting

address together with the length of

the region to be moved/exchanged

exceeds 1M bytes.  No data was

moved/exchanged.

A3h   4E01h, 4E02h         The contents of the data structure

4F00h, 4F01h         passed to the function have either

5B01h                been corrupted or are meaningless.

A4h   5900h, 5B00h         The operating system has denied

5B01h, 5B02h         access to this function.  The

5B03h, 5B04h         function cannot be used at this time.

5B05h, 5B06h

5B07h, 5B08h

5D00h, 5D01h

5D02h

----------------------------------------------------------------

Cross Reference Tables                                        198

Appendix B

TESTING FOR THE PRESENCE OF THE EXPANDED MEMORY MANAGER

Before an application program can use the Expanded Memory

Manager, it must determine whether DOS has loaded the

manager.  This appendix describes two methods your program

can use to test for the presence of the memory manager and

how to choose the correct one for your situation.

The first method uses the DOS "open handle" technique; the

second method uses the DOS "get interrupt vector" technique.

Which method should your program use?

The majority of application programs can use either the

"open handle" or the "get interrupt vector" method.

However, if your program is a device driver or if it

interrupts DOS during file system operations, you must use

only the "get interrupt vector" method.

Device drivers execute from within DOS and can't access the

DOS file system; programs that interrupt DOS during file

system operations have a similar restriction.  During their

interrupt processing procedures, they can't access the DOS

file system because another program may be using the system.

Since the "get interrupt vector" method doesn't require the

DOS file system, you must use it for these types of pro-

grams.

The "open handle" technique

Most application programs can use the DOS "open handle"

technique to test for the presence of the memory manager.

This section describes how to use the technique and gives an

example.

Caution.........................................................

Don't use this technique if your program is a device driver

or if it interrupts DOS during file system operations.  Use

the "get interrupt vector" technique described later in this

appendix.

Using the "open handle" technique

This section describes how to use the DOS "open handle"

technique to test for the presence of the memory manager.

Follow these steps in order:

Testing For The Presence Of The EMM                           199

1.  Issue an "open handle" command (DOS function 3Dh) in

"read only" access mode (register AL = 0).  This

function requires your program to point to an ASCII

string which contains the path name of the file or

device in which you're interested (register set DS:DX

contains the pointer).  In this case the file is

actually the name of the memory manager.

You should format the ASCII string as follows:

ASCII_device_name  DB  'EMMXXXX0', 0

The ASCII codes for the capital letters EMMXXXX0 are

terminated by a byte containing a value of zero.

2.  If DOS returns no error status code, skip Steps 3 and 4

and go to Step 5.  If DOS returns a "Too many open

files" error status code, go to Step 3.  If DOS returns

a "File/Path not found" error status code, skip Step 3

and go to Step 4.

3.  If DOS returns a "Too many open files" (not enough

handles) status code, your program should invoke the

"open file" command before it opens any other files.

This will guarantee that at least one file handle will

be available to perform the function without causing

this error.

After the program performs the "open file" command, it

should perform the test described in Step 6 and close

the "file handle" (DOS function 3Eh).  Don't keep the

manager "open" after this status test is performed since

"manager" functions are not available through DOS.  Go

to Step 6.

4.  If DOS returns a "File/Path not found," the memory

manager is not installed.  If your application requires

the memory manager, the user will have to reboot the

system with a disk containing the memory manager and the

appropriate CONFIG.SYS file before proceeding.

5.  If DOS doesn't return an error status code you can

assume that either a device with the name EMMXXXX0 is

resident in the system, or a file with this name is on

disk in the current disk drive.  Go to Step 6.

6.  Issue an "I/O Control for Devices" command (DOS function

44h) with a "get device information" command (register

AL = 0).  DOS function 44h determines whether EMMXXXX0

is a device or a file.

Testing For The Presence Of The EMM                           200

You must use the file handle (register BX) which you

obtained in Step 1 to access the "EMM" device.

This function returns the "device information" in a word

(register DX).  Go to Step 7.

7.  If DOS returns any error status code, you should assume

that the memory manager device driver is not installed.

If your application requires the memory manager, the

user will have to reboot the system with a disk contain-

ing the memory manager and the appropriate CONFIG.SYS

file before proceeding.

8.  If DOS didn't return an error status, test the contents

of bit 7 (counting from 0) of the "device information"

word (register DX) the function returned.  Go to Step 9.

9.  If bit 7 of the "device information" word contains a

zero, then EMMXXXX0 is a file, and the memory manager

device driver is not present.  If your application

requires the memory manager, the user will have to

reboot the system with a disk containing the memory

manager and the appropriate CONFIG.SYS file before

proceeding.

If bit 7 contains a one, then EMMXXXX0 is a device.  Go

to Step 10.

10. Issue an "I/O Control for Devices" command (DOS function

44h) with a "get output status" command (register AL =

7).

You must use the file handle you obtained in Step 1 to

access the "EMM" device (register BX).  Go to Step 11.

11. If the expanded memory device driver is "ready," the

memory manager passes a status value of "FFh" in

register AL.  The status value is "00h" if the device

driver is "not ready."

If the memory manager device driver is "not ready" and

your application requires its presence, the user will

have to reboot the system with a disk containing the

memory manager and the appropriate CONFIG.SYS file

before proceeding.

If the memory manager device driver is "ready," go to

Step 12.

Testing For The Presence Of The EMM                           201

12. Issue a "Close File Handle" command (DOS function 3Eh)

to close the expanded memory device driver.  You must

use the file handle you obtained in Step 1 to close the

"EMM" device (register BX).

Testing For The Presence Of The EMM                           202

An example of the "open handle" technique

The following procedure is an example of the "open handle"

technique outlined in the previous section.

;--------------------------------------------------------------;

;    The following procedure tests for the presence of the     ;

;    EMM in the system.  It returns the CARRY FLAG SET if      ;

;    the EMM is present.  If the EMM is not present, this      ;

;    procedure returns the CARRY FLAG CLEAR.                   ;

;--------------------------------------------------------------;

first_test_for_EMM  PROC  NEAR

PUSH  DS

PUSH  CS

POP   DS

MOV   AX,3D00h                       ; issue "device open" in

LEA   DX,ASCII_device_name           ; "read only" mode

INT   21h

JC    first_test_for_EMM_error_exit  ; test for error

; during "device open"

MOV   BX,AX                          ; get the "file

; handle" returned by DOS

MOV   AX,4400h                       ; issue "IOCTL

INT   21h                            ; get device info"

JC    first_test_for_EMM_error_exit  ; test for error

; during "get device info"

TEST  DX,0080h                       ; test to determine

JZ    first_test_for_EMM_error_exit  ; ASCII_device_name

; is a device or a file

MOV   AX,4407h                       ; issue "IOCTL"

INT   21h

JC    first_test_for_EMM_error_exit  ; test for error

; during "IOCTL"

PUSH  AX                             ; save "IOCTL" status

MOV   AH,3Eh                         ; issue "close

INT   21h                            ; file handle"

POP   AX                             ; restore "IOCTL" status

CMP   AL,0FFh                        ; test for "device

JNE   first_test_for_EMM_error_exit  ; ready" status

; returned by the driver

first_test_for_EMM_exit:

POP   DS                             ; EMM is present

STC                                  ; in the system

RET

first_test_for_EMM_error_exit:

POP   DS                             ; EMM is NOT present

CLC                                  ; in the system

RET

ASCII_device_name   DB  'EMMXXXX0', 0

first_test_for_EMM  ENDP

Testing For The Presence Of The EMM                           203

The "get interrupt vector" technique

Any type of program can use the DOS "get interrupt vector"

technique to test for the presence of the memory manager.

This section describes how to use the technique and gives an

example.

Caution.........................................................

Be sure to use this technique (and not the "open handle"

technique) if your program is a device driver or if it

interrupts DOS during file system operations.

Using the "get interrupt vector" technique

This section describes how to use the DOS "get interrupt

vector" technique to test for the presence of the memory

manager.  Follow these steps in order:

1.  Issue a "get vector" command (DOS function 35h) to

obtain the contents of interrupt vector array entry

number 67h (addresses 0000:019Ch thru 0000:019Fh).

The memory manager uses this interrupt vector to perform

all manager functions.  The offset portion of this

interrupt service routine address is stored in the word

located at address 0000:019Ch; the segment portion is

stored in the word located at address 0000:019Eh.

2.  Compare the "device name field" with the contents of the

ASCII string which starts at the address specified by

the segment portion of the contents of interrupt vector

address 67h and a fixed offset of 000Ah.  If DOS loaded

the memory manager at boot time this name field will

have the name of the device in it.

Since the memory manager is implemented as a character

device driver, its program origin is 0000h.  Device

drivers are required to have a "device header" located

at the program origin.  Within the "device header" is an

8 byte "device name field."  For a character mode device

driver this name field is always located at offset 000Ah

within the device header.  The device name field

contains the name of the device which DOS uses when it

references the device.

If the result of the "string compare" in this technique

is positive, the memory manager is present.

Testing For The Presence Of The EMM                           204

An example of the "get interrupt vector" technique

The following procedure is an example of the "get interrupt

vector" technique outlined in the previous section.

;--------------------------------------------------------------;

;    The following procedure tests for the presence of the     ;

;    EMM in the system.  It returns the CARRY FLAG SET if      ;

;    the EMM is present.  If the EMM is not present, this      ;

;    procedure returns the CARRY FLAG CLEAR.                   ;

;--------------------------------------------------------------;

second_test_for_EMM  PROC  NEAR

PUSH  DS

PUSH  CS

POP   DS

MOV   AX,3567h                       ; issue "get interrupt

; vector"

INT   21h

MOV   DI,000Ah                       ; use the segment in ES

; returned by DOS, place

; the "device name field"

; OFFSET in DI

LEA   SI,ASCII_device_name           ; place the OFFSET of the

; device name string in SI,

; the SEGMENT is already

; in DS

MOV   CX,8                           ; compare the name strings

CLD

REPE  CMPSB

JNE   second_test_for_EMM_error_exit

second_test_for_EMM_exit:

POP   DS                             ; EMM is present in

STC                                  ; the system

RET

second_test_for_EMM_error_exit:

POP   DS                             ; EMM is NOT present

CLC                                  ; in the system

RET

ASCII_device_name   DB  'EMMXXXX0'

second_test_for_EMM ENDP

Testing For The Presence Of The EMM                           205

Appendix C

EXPANDED MEMORY MANAGER IMPLEMENTATION GUIDELINES

In addition to the functional specification, the expanded

memory manager should provide certain resources.  The

following guidelines are provided so required resources are

present in expanded memory managers which comply with this

version of the LIM specification.

o   The amount of expanded memory supported:

Up to a maximum of 32M bytes of expanded memory should

be supported.

o   The number of handles supported:

The maximum number of expanded memory handles provided

should be 255, the minimum should be 64.

o   Handle Numbering:

Although a handle is a word quantity, there is a maximum

of 255 handles, including the operating system handle.

This specification defines the handle word as follows:

the low byte of the word is the actual handle value, the

high byte of the handle is set to 00h by the memory

manager.  Previous versions of this specification did

not specify the value of handles.

o   New handle type:  Handles versus Raw Handles:

The difference between a raw handle and a regular handle

is slight.  If you use Function 27 to "Allocate raw

pages to a handle," what is returned in DX is termed a

raw handle.  The raw handle does not necessarily refer

to 16K-byte pages.  Instead it refers to the "raw" page

size, which depends on the expanded memory hardware.

An application program can use Function 26 to find the

raw page size, and by using the raw handle Function 27

returns, it can access them with the finer resolution

that a particular expanded memory board may allow.

On the other hand, applications which use Function 4 to

"allocate pages to handle" receive a handle which always

refers to 16K-byte pages.  On expanded memory boards

with smaller raw pages, the EMM driver will allocate and

maintain the number of raw pages it takes to create a

single composite 16K-byte page.  The difference between

the expanded memory boards' raw page size and the 16K-

byte LIM page size is transparent to the application

when it is using a handle obtained with Function 4.

EMM Implementation Guidelines                                 206

The memory manager must differentiate between pages

allocated to handles and pages allocated to raw handles.

The meaning of a call to the driver changes depending on

whether a handle or a raw handle is passed to the memory

manager.  If, for example, a handle is passed to

Function 18 (Reallocate), the memory manager will

increase or decrease the number of 16K-byte pages

allocated to the handle.  If Function 18 is passed a raw

handle, the memory manager will increase or decrease the

number of raw (non-16K-byte) pages allocated to the raw

handle.  For LIM standard boards, there is no difference

between pages and raw pages.

o   The system Raw Handle (Raw Handle = 0000h):

For expanded memory boards that can remap the memory in

the lower 640K-byte address space, managing the pages of

memory which are used to fill in the lower 640K can be a

problem.  To solve this problem, the memory manager will

create a raw handle with a value of 0000h when DOS loads

the manager.  This raw handle is called the system

handle.

At power up, the memory manager will allocate all of the

pages that are mapped into the lower 640K bytes to the

system handle.  These pages should be mapped in their

logical order.  For example, if the system board

supplies 256K bytes of RAM, and the 384K bytes above it

is mappable, the system handle should have its logical

page zero mapped into the first physical page at 256K,

its logical page one mapped into the next physical page,

and so on.

The system handle should deal with raw pages.  To

release some of these pages so application programs can

use them, an operating system could decrease the number

of pages allocated to the system handle with the

"Reallocate" function.  Invoking the "Deallocate"

function would decrease the system handle to zero size,

but it must not deallocate the raw handle itself.  The

"Deallocate" function treats the system handle dif-

ferently than it treats other raw handles.  If the

operating system can ever be "exited" (for example, the

way Windows can be exited), it must increase the size of

the system handle back to what is needed to fill 640K

and map these logical pages back into physical memory

before returning to DOS.

EMM Implementation Guidelines                                 207

There are two functional special cases for this handle:

-   The first special case deals with Function 4

(Allocate Pages).  This function must never return

zero as a handle value.  Applications must always

invoke Function 4 to allocate pages and obtain a

handle which identifies its pages.  Since Function 4

will never return a handle value of zero, an

application will never gain access to this special

handle.

-   The second special case deals with Function 6

(Deallocate Pages).  If the operating system uses

Function 6 to deallocate the pages which are

allocated to the system handle, the pages will be

returned to the manager for use, but the handle will

not be available for reassignment.  The manager

should treat a "deallocate pages" function request

for this handle the same as a "reallocate pages"

function request, where the number of pages to

reallocate to this handle is zero.

o   Terminate and Stay Resident (TSR) Program Cooperation:

In order for TSR's to cooperate with each other and with

other applications, TSR's must follow this rule:  a

program may only remap the DOS partition it lives in.

This rule applies at all times, even when no expanded

memory is present.

o   Accelerator Cards:

To support generic accelerator cards, the support of

Function 34, as defined by AST, is encouraged.

EMM Implementation Guidelines                                 208

Appendix D

OPERATING SYSTEM/ENVIRONMENT USE OF FUNCTION 28

All expanded memory boards have a set of registers that

"remember" the logical to physical page mappings.  Some

boards have extra (or alternate) sets of these mapping

registers.  Because no expanded memory board can supply an

infinite number of alternate map register sets, this

specification provides a way to simulate them using Function

28 (Alternate Map Register Set).

Examples

For the examples in this section, assume the hardware

supports alternate map register sets.  First Windows is

brought up, then "Reversi" is started.  Then control is

switched back to the MS-DOS Executive.  For this procedure,

here are the calls to the expanded memory manager:

Example 1

Allocate alt reg set           ; Start up the MS-DOS

(for MS-DOS Executive)         ; Executive

Set alt reg set

(for MS-DOS Executive)

Allocate alt reg set           ; Start up Reversi

(for Reversi)

Set alt reg set

(for Reversi)

Map pages

(for Reversi)

Set alt ret set                ; Switch back to MS-DOS

(for MS-DOS Executive)         ; Executive

Operating System Use Of Function 28                           209

Notice this procedure needed no "get" calls because the

register set contained all the information needed to save a

context.  However, using "Get" calls would have no ill

effects.

Example 2

Allocate alt reg set           ; Start up MS-DOS

(for MS-DOS Executive)         ; Executive

Set alt reg set

(for MS-DOS Executive)

Get alt reg set

(for MS-DOS Executive)

Allocate alt reg set           ; Start up Reversi

(for Reversi)

Set alt reg set

(for Reversi)

Map pages

(for Reversi)

Get alt reg set

(for Reversi)

Set alt reg set                ; Switch back to MS-DOS

(for MS-DOS Executive)         ; Executive

The important point to follow is that a Set must always

precede a Get.  The model of Set then Get is the inverse of

what interrupt handlers use, which is Get then Set (Get the

old map context and Set the new one).  Another crucial point

is that an alternate map register set must have the current

mapping when allocated; otherwise, the Set will create

chaos.

What happens if this is simulated in software?  The same Set

and Get model applies.  The main difference is where the

context is saved.

Operating System Use Of Function 28                           210

Since the allocate call is dynamic and there is no limit on

the number of sets allocated, the OS/E must supply the space

required.  Device drivers cannot allocate space dynamically,

since the request would fail.  If the Allocate register set

call returns a status indicating the alternate map register

sets aren't supported, the OS/E must allocate space for the

context.  It must also initialize the context using Function

15.  At that point it can do the Set, passing a pointer to

the map context space.  On the Get call, the EMM driver is

to return a pointer to the same context space.

Example 3

Allocate alt reg set           ; Start up MS-DOS

(for MS-DOS Executive)         ; Executive

Get Page Map

(for MS-DOS Executive)

Set alt reg set

(for MS-DOS Executive)

Allocate alt reg set           ; Start up Reversi

(for Reversi)

Set alt reg set

(for Reversi)

Map pages

(for Reversi)

Get Page Map

(for Reversi)

Set alt ret set                ; Switch back to MS-DOS

(for MS-DOS Executive)         ; Executive

Operating System Use Of Function 28                           211

GLOSSARY

The following terms are used frequently in this specifica-

tion:

Allocate                 To reserve a specified amount of

expanded memory pages.

Application Program      An application program is the program

you write and your customer uses.  Some

categories of application software are

word processors, database managers,

spreadsheet managers, and project

managers.

Conventional memory      The memory between 0 and 640K bytes,

address range 00000h thru 9FFFFh.

Deallocate               To return previously allocated expanded

memory to the memory manager.

EMM                      See Expanded Memory Manager.

Expanded memory          Expanded memory is memory outside DOS's

640K-byte limit (usually in the range of

C0000h thru EFFFFh).

Expanded Memory          A device driver that controls the

Manager (EMM)          interface between DOS application

programs and expanded memory.

Extended memory          The 15M-byte address range between

100000h thru FFFFFFh available on an

80286 processor when it is operating in

protected virtual address mode.

Handle                   A value that the EMM assigns and uses to

identify a block of memory requested by

an application program.  All allocated

logical pages are associated with a

particular handle.

Logical Page             The EMM allocates expanded memory in

units (typically 16K bytes) called

logical pages.

Mappable Segment         A 16K-byte region of memory which can

have a logical page mapped at it.

Glossary                                                      212

Map Registers            The set of registers containing the

current mapping context of the EMM

hardware.

Mapping                  The process of making a logical page of

memory appear at a physical page.

Mapping Context          The contents of the mapping registers at

a specific instant.  This context

represents a map state.

Page Frame               A collection of 16K-byte contiguous

physical pages from which an application

program accesses expanded memory.

Page Frame               A page frame base address is the

Base Address           location (in segment format) of the

first byte of the page frame.

Physical Page            A physical page is the range of memory

addresses occupied by a single 16K-byte

page.

Raw Page                 The smallest unit of mappable memory

that an expanded memory board can

supply.

Resident Application     A resident application program is loaded

Program                by DOS, executes, and remains resident

in the system after it returns control

to DOS.  This type of program occupies

memory and is usually invoked by the

operating system, an application

program, or the hardware.  Some example

of resident application programs are RAM

disks, print spoolers, and "pop-up"

desktop programs.

Status code              A code that an EMM function returns

which indicates something about the

result of running the function.  Some

status codes indicate whether the

function worked correctly and others may

tell you something about the expanded

memory hardware or software.

Glossary                                                      213

Transient Application    A transient application program is

Program                loaded by DOS, executes, and doesn't

remain in the system after it returns

control to DOS.  After a transient

application program returns control to

DOS, the memory it used is available for

other programs.

Unmap                    To make a logical page inaccessible for

reading or writing.

Glossary                                                      214

INDEX

Allocate Alternate Map Register Set  36, 163

Allocate DMA Register Set  36, 168, 190

Allocate Pages  5, 14, 23, 30, 34, 42, 43, 47, 49, 144,

147, 148, 193, 206, 208

Allocate Raw Pages  36, 46, 80, 89, 147-149, 190, 193,

206

Allocate Standard Pages  36, 42, 46, 80, 89, 144, 145,

147, 190, 193

Alter Page Map & Call  7, 10, 35, 113, 118, 189

Alter Page Map & Jump  7, 10, 35, 109, 189

Alternate Map  10, 36, 151, 153-155, 157-159, 161, 163,

164, 165-168, 170, 173, 175, 179, 182, 190, 197,

209, 210, 211

Alternate Map Register Set  10, 36, 151, 153-155, 157,

158, 159, 161, 163-168, 170, 173, 175, 190, 197,

209, 210

Alternate Mapping and Unmapping Methods  81

Alternate Register  139, 166, 173, 177

Data Aliasing  12

Deallocate Alternate Map Register Set  36, 166

Deallocate DMA Register Set  36, 175, 190

Deallocate Pages  5, 14, 25, 31, 34, 43, 49, 88, 145,

148, 208

Design Considerations  91, 151

Device Driver  1, 15, 43, 53, 55, 144, 148, 199, 201,

202, 204, 212

Disable DMA on Alternate Map Register Set  173

Disable OS/E Function Set  36, 179, 180, 182, 185

DMA  36, 138-140, 151, 152, 168-176, 190, 197

DMA Channels  139, 171, 173, 174, 197

DMA Register  36, 139, 140, 151, 152, 168-171, 173-176,

190, 197

DOS  1, 12, 14, 15, 19, 21, 30, 31, 49, 53, 88, 199-205,

207-214

Enable DMA on Alternate Map Register Set  170

Enable OS/E Function Set  36, 179, 180

Enable/Disable OS/E Function Set  179, 180, 182, 185

Exchange Memory Region  7, 10, 35, 120, 126, 127, 189

Expanded Memory Support of DMA  151

Expanded Memory Support of DMA Register Sets  151

Extended Memory  91

Function 1  37

Function 10  57

Function 11  58

Function 12  59

Function 13  61

Function 14  7, 63

Index                                                         215

Function 15  13, 53, 55, 65, 67, 69, 71, 73, 76, 139,

153, 154, 155, 158, 211

Function 16  13, 73, 76, 78

Function 17  6, 80, 82, 85

Function 18  6, 43, 88, 144, 148, 207

Function 19  7, 91, 92, 94, 96

Function 2  4, 38

Function 20  7, 98, 100

Function 21  7, 42, 102, 105, 107

Function 22  109

Function 23  113, 118

Function 24  7, 120, 126

Function 25  6, 8, 46, 74, 85, 132, 136

Function 26  138, 142, 179, 182, 206

Function 27  42, 46, 80, 89, 144, 145, 147-149, 193, 206

Function 28  140, 151, 153, 157, 161, 163, 164, 166,

168, 170, 173, 175, 179, 182, 209

Function 29  177

Function 3  4, 40, 142

Function 30  138, 151, 153, 157, 161, 163, 166, 168,

170, 173, 175, 179, 182, 185

Function 4  4, 42, 43, 46, 47, 49, 80, 89, 144, 145,

147, 149, 193, 206, 208

Function 5  4, 46, 81

Function 6  4, 43, 49, 88, 145, 148, 208

Function 7  5, 51

Function 8  46, 50, 53, 55

Function 9  46, 50, 53, 55

Get & Set Page Map  35, 69

Get All Handle Pages  9, 34, 63

Get Alternate Map Register Set  36, 153, 154, 157, 190

Get Alternate Map Save Array Size  36, 161, 190

Get Attribute Capability  7, 96

Get Expanded Memory Hardware Information  10, 138, 142,

179, 182

Get Handle Attribute  35, 92

Get Handle Count  9, 34, 59

Get Handle Directory  10, 35, 102, 105, 107

Get Handle Name  35, 98

Get Handle Pages  7, 9, 34, 61

Get Hardware Configuration Array  36, 138

Get Interrupt Vector  15, 21, 30, 199, 204, 205

Get Mappable Physical Address Array  6, 8, 10, 35, 46,

85, 132, 136

Get Mappable Physical Address Array Entries  8, 136

Get Page Frame Address  5, 34, 38

Get Page Map  35, 65, 118, 153-155, 158, 211

Get Page Map Stack Space Size  35, 118

Get Partial Page Map  35, 73, 78

Get Size of Page Map Save Array  35, 65, 67, 71, 139

Get Size of Partial Page Map Save Array  74, 76, 78

Get Status  5, 34, 37

Index                                                         216

Get Total Handles  35, 107

Get Unallocated Page Count  5, 22, 34, 40, 142

Get Unallocated Raw Page Count  36, 142, 189

Get Version  5, 34, 51

Get/Set Handle Attribute  9, 91, 92, 94, 96

Get/Set Handle Name  10, 98, 100

Get/Set Page Map  9, 13, 65, 67, 69, 71

Get/Set Partial Page Map  9, 13, 73, 76, 78

Handle Attribute  9, 35, 91-94, 96, 188

Handle Name  6, 7, 10, 35, 98, 100, 105, 106, 188, 198

Intel  i, ii, 1, 5, 57, 58

Interrupt Vector  12, 15, 21, 30, 199, 204, 205

LIM  1, 7, 13, 19, 27, 53, 55, 138, 140, 206, 207

Logical Page  1, 5, 12, 16, 19, 23, 28, 31, 32, 46-48,

80-83, 85, 86, 88, 110, 111, 115, 116, 120, 122,

123, 125, 126, 128, 129, 131, 147, 194, 196, 207,

212-214

Logical Page/Physical Page Method  82

Logical Page/Segment Address Method  85

Lotus  i, ii, 1, 5, 57, 58

Map Register  10, 13, 36, 53, 55, 151, 153-155, 157-159,

161, 163-168, 170, 173, 175, 179, 182, 190, 197,

209-211

Map/Unmap Handle Pages  46

Map/Unmap Multiple Handle Pages  9, 35, 80, 82, 85

Mapping and Unmapping Multiple Pages Simultaneously  80

Mapping Multiple Pages  6, 80

Microsoft  i, ii, 1, 5, 14, 30, 42, 57, 58, 144, 148

Move Memory Region  35, 120, 121, 189

Move/Exchange Memory Region  7, 10, 120, 126

Open Handle  64, 102, 199, 200, 203, 204

Operating System  3, 8, 10-12, 42, 43, 59, 63, 107, 138,

139, 141, 142, 144-151, 153-159, 161-163, 165-171,

173-177, 179-183, 185, 186, 190, 191, 198, 206,

207-209, 213

Page Frame  1-6, 14, 17-19, 24, 28, 31, 34, 38, 39, 47,

53, 55, 121, 128, 133, 187, 213

Page Map  7, 9, 10, 13, 34, 35, 50, 53, 55, 65, 67, 69,

71, 73-76, 78, 109, 113, 118, 139, 153-155, 158,

187, 188, 189, 211

Page Mapping Register I/O Array  57

Page Translation Array  58

Physical Page  1, 5, 6, 8, 10, 12, 16, 23, 28, 31, 35,

46, 47, 48, 80-83, 85, 109-112, 114-117, 132-134,

136, 138, 139, 142, 147, 188, 194, 207, 209, 213

Prepare Expanded Memory Hardware For Warm Boot  10, 177

Raw Handle  147, 149, 150, 206, 207

Raw Page  36, 142, 143, 147, 189, 206

Reallocate Pages  9, 35, 43, 88, 144, 145, 148, 208

Restore Page Map  9, 13, 34, 50, 53, 55

Return Access Key  185

Save Page Map  9, 13, 34, 50, 53, 55

Index                                                         217

Search For Named Handle  7, 35, 105

Set Alternate Map Register Set  36, 153-155, 157, 158,

163, 190

Set Handle Attribute  9, 35, 91, 92, 94, 96

Set Handle Name  7, 10, 35, 98, 100

Set Page Map  9, 13, 35, 65, 67, 69, 71, 188

Set Partial Page Map  9, 13, 35, 73, 76, 78

Standard Handle  146

Standard Page  147

System DMA Capabilities  151

TSR  12, 13, 208

Unmapping Multiple Pages  6, 80

Index                                                         218