TOSHIBA CORPORATION   YANAGICHO WORKS

TECHNICAL SUPPORT ROM DRIVES

PRODUCT PLANNING AND TECHNICAL SUPPORT DEPARTMENT

PPTSY-901

1996/04/11

-------------------------------------------------------------------------------------------------

CD-ROM Device Driver Install Program Control Manual

-------------------------------------------------------------------------------------------------

*Please do not copy this manual.

*Please do not distribute this manual to the final user.

Contents of Table

Chapter 1.Introduction

Chapter 2.Control Procedure of Install Program (INSTALL.EXE).

2.1 Startup of Install Program

2.2 Confirmation of IDE Port Connection Environment

2.3 Designation of Install Destination

2.4 Confirmation of Device Driver Overwrite

2.5 Designation of Parameters

2.6 Confirmation of CONFIG. SYS Additional Line

2.7 Designation of MSCDEX.EXE Directory

2.8. Confirmation of AUTOEXEC.BAT Additional Line

2.9 During Install Work

2.10 End of Install

Chapter 3.Change of CONFIG.SYS and AUTOEXEC.BAT

3.1Change of CONFIG.SYS

3. 2 Change of AUTOEXEC.BAT

Chapter 4. Precautions

# Chapter 1.Introduction

This document describes the control procedure of program

(INSTALL.EXE) for installing the CD-ROM device driver

(CDROMDRV.SYS)into the system disc.

# Chapter 2.Control Procedure of Install Program (INSTALL.EXE).

Hereunder shown is the control procedure of install program

(INSTALL.EXE) of CD-ROM device driver. The install work

is to be done as per the procedures as follows.

1.Confirmation of IDE port connection state.

2.Designation of directory for installing the CD-ROM device

driver.

3.Designation of directory where the MSCDEX.EXE is placed.

4.Designation of CD-ROM device driver and MSCDEX.EXE parameters.

5.Confirmation of change of CONFIG.SYS and AUTOEXEC.BAT.

6.Copy of CD-ROM device driver.

7.Change of CONFIG.SYS and AUTOEXEC.BAT.

Precaution : The install program rewrites the contents of

existing CONFIG.SYS and AUTOEXEC.BAT. The CONFIG.SYS and

AUTOEXEC.BAT before rewriting are stored in the CONFIG.CDO and

AUTOEXEC.CDO respectively.

## 2.1 Startup of Install Program

1.Insert the install disc of CD-ROM device driver into the

FDD.

2.The install program is executed with reference to the

below mentioned example.

[Example] In case that the FDD where the install program

has been inserted is the A-drive.

A:\INSTALL

3.When the install program has been executed,the following

picture will be displayed.

CD-ROM Install Program

Thank you very much for purchasing the TOSHIBA CD-ROM

drive unit.

This software installs the CD-ROM device driver onto your

system and sets the system configuration in order to take

effect.

Use the Enter key to start the installation, and use the

Esc key to let you exit form want you are doing, and to

return to the MS-DOS prompt.

4.Press the [Enter] key for starting the install work in

accordance with the message,and press the [Esc] key when

stopping the work.

## 2.2 Confirmation of IDE Port Connection Environment

The IDE port connection environment of computer is to be

confirmed. Here,what are connected to the Master and the

Slave in respects of the respective ports of Primary,Secondary,

Tertiary and Quaternary are displayed in the tree style

concerning the connection state of present IDE port.

CD-ROM Install Program

Computer

Master         Slave

Hard Disk       Not Used          Primary    (Port:0x1FX, IRQ:14)

CD-ROM         Not Used          Secondary  (Port:0x17X, IRQ:15)

Not Used        Not Used          Tertiary   (Port:0x1EX, IRQ:11)

Not Used        Not Used          Quaternary (Port:0x16X, IRQ:10)

Confirm the connection environment of the computer. Are you sure? (Y/N)

*Hard Disk : This indicates that the hard disk drive is

connected.

*CD-ROM : This indicates that the CD-ROM drive is connected.

*Not Used : This indicates that nothing is connected.

After confirming that the CD-ROM drive is connected to

the IDE port,press the [Y] key.The said example indicates

that the CD-ROM drive is connected to the Secondary Master.

If  the  CD-ROM  drive has not been connected,the install

work can be interrupted by pressing the [N] key. For information,

even if the CD-ROM is not connected,the install work

can be continued.

## 2.3 Designation of Install Destination

The directory of install destination of device driver is

to be designated. Input the directory of install destination

of device driver and press the [Enter] key.

Specify the directory you want to place the CD-ROM driver in,

and push Enter key.

[C:\CDROM............................................]

If the designated directory is unavailable,it will be

automatically prepared.The install destination directory of

initial setting is [\CDROM] of startup drive. Press the [Enter]key,

as it is,when installing the directory of initial setting.

If the directory has not been designated,the install

destination directory will the current directory.

If the [Esc] key is pressed,the install work will be interrupted.

## 2.4 Confirmation of Device Driver Overwrite

If the CD-ROM device drive has already been installed at

the directory of designation install destination,you should

confirm overwriting the existing device driver.

Your computer already has the CD-ROM driver installed on it.

Do you replace the currently installed driver? (Y/N)

If you overwrite the existing device driver, press the [Y] key.

For interrupting the install of device driver,press the [N] key.

## 2.5 Designation of Parameters

Designate the parameters to be used in the MSCDEX.EXE. In

the initial setting,the [/D] for designating the driver name

is effective,and the [CDROM001] has been set for the driver name.

CD-ROM Install Program

Specify the parameter to use.

Move a cursor to the parameter to specify and push a space key.

The specification of the default is “/D:CDROM001”.

( )/E            :Specifies that the CD-ROM driver be allowed to use

expanded memory, if available, to store sector buffer.

( )/K            :Specifies that MS-DOS should recognize CD-ROM volumes

encoded in Kanji.

( )/S            :Enable sharing of CD-ROM drivers on MS-NET or Windows

for Workgroups servers.

( )/V            :Directs MSCDEX to display memory statistics when it

start.

(*)/D [CDROM001] :Specifies the driver signature of the CD-ROM device

driver.

( )/L [ ]        :Specifies the drive letter to assign to the CD-ROM drive.

( )/M [        ] :Specifies the number of sector buffer.

Use the Enter key to continue.

Designation Method

1.Move the cursor up to the parameters which you want to make

effective,using the arrow keys (up arrow,down arrow).

2.If the space key is pressed, the [ *] is displayed to the

left extreme side of [( )] ,and the parameter becomes effective.

3.When the space key is pressed again,the [*] of [( )]

disappears, and the parameter can be made ineffective. (However,

the designation of [/D] parameter can not be made ineffective).

When the designation of parameter is over, press the [Enter] key.

When interrupting the install work, press the [Esc] key.

For the details of MSCDEX parameters, see the manual of MSCDEX.EXE.

## 2.6 Confirmation of CONFIG. SYS Additional Line

Confirm the line added to the CONFIG. SYS.

The following line will be added to CONFIG.SYS Are you sure ?(Y/N)

DEVICE=C:\CDROM\CDROMDRV.SYS /D:CDROM001

If the contents to be added to the CONFIG.SYS are correct, press

the [Y] key. when restarting the designation of parameters, press

the [N] key.

Caution : In this install program, the parameters other than

the  [/D]  of device driver can not be designated.If you want to

designate the parameters other than [/D],  you are requested to

refer to [README.TXT] and rewrite the contents of CONFIG.SYS.

## 2.7 Designation of MSCDEX.EXE Directory

If the MSCDEX.EXE isn't available in the MS-DOS directory

( \DOS of startup drive) , designate the directory having the

MSCDEX.EXE.

There is not C:\DOS\MSCDEX.EXE.

Specify the directory where there is MSCDEX.EXE, and push Enter key.

[C:\DOS..............................................]

When you have input the directory, press the [Enter] key.

When interrupting the install work, press the  [Esc] key.

There is not C:\DOS\MSCDEX.EXE.

To use CD-ROM drive, MSCDEX.EXE is necessary.

Copy the installation of CD-ROM device driver in the following

directory with MSCDEX.EXE preparing an included disk after ending.

C:\DOS

Does it continue the installation of CD-ROM device driver? (Y/N)

If the MSCDEX.EXE doesn't exist in the designated directory,

the said message will be displayed. Press the [Y]key

when continuing the installation as it is, and press the [N]

key when interrupting the install work.

## 2.8. Confirmation of AUTOEXEC.BAT Additional Line

Confirm the line to be added to the AUTOEXEC.BAT. The

line to be added varies with the designated parameter.

The following line will be added to AUTOEXEC.BAT. Are you sure? (Y/N)

C:\DOS\MSCDEX.EXE /D:CDROM001 /M:12

If the line to be added to the AUTOEXEC.BAT is correct,

press the [Y] key. When restarting the designation of parameters,

press the [N] key.

## 2.9 During Install Work

The CD-ROM device driver is copied to the  directory  of

install destination to change the system files ( CONFIG.SYS and

AUTOEXEC.BAT). The following message will be displayed while

the install work is being done.

Please wait....

## 2.10 End of Install

When the install of CD-ROM device driver has been ended,

the following picture informing that the installation is ended

will be displayed, so press the [Enter] key.

CD-ROM Install Program

Installation is complete.

To make CD-ROM drive effective, it corrected CONFIG.SYS and

AUTOEXEC.BAT in the part. CONFIG.SYS and AUTOEXEC.BAT before

correction have been preserved respectively in CONFIG.CD0

and AUTOEXEC.CD0.

To make CD-ROM drive effective, re-start a computer.

If the MSCDEX.EXE can not be discovered during the installation

work, the message as follows will be displayed　after the

foregoing message.

Incidentally, prepare the disk which MSCDEX.EXE was stored

in before re-starting a computer and copy MSCDEX.EXE in the

following directory.

C:\DOS

Restart the computer following the messages.

# Chapter 3.Change of CONFIG.SYS and AUTOEXEC.BAT

The install program automatically add the description

for making the CD-ROM drive effective to the existing CONFIG.SYS

and AUTOEXEC.BAT.

Here explained is the change for the install program to

make the change to CONFIG.SYS and AUTOEXEC.BAT.

## 3.1Change of CONFIG.SYS

The change against the CONFIG.SYS adds the description

for incorporating the CD-ROM device driver and also corrects

the designation of [LASTDRIVE]

The [LASTDRIVE] designation shall be fixed to the [Z] and

added to the head of file.The [LASTDRIVE] appearing therefater

will all be deleted.

The line to be added varies with the designation of

parameter.Hereunder,　an example of additional line is indicated.

*An example of CONFIG.SYS additional line

REM=============APPEND CD-ROM DRIVE=============

DEVICE=C:\CDROM\CDROMDRV.SYS/D:CDROM001

REM=============APPEND CD-ROM DRIVE=============

If there is already the description of [CDROMDRV.SYS]

incorporation,the said description is deleted,and the line

is added at the same position.

If  there is not the description of incorporation,the

additional position varies with the single configuration and

the multi configuration.

In case of the single configuration,the line is added to

the head of file (the next line of [ LASTDRIVE ] ).

In  case of the multi configuration, the line is added to

the last of common block ([COMMON]).The line is added just

before the ( [MENU] ) if there is no common block.

## 3. 2 Change of AUTOEXEC.BAT

Add the execution of MSCDEX.EXE to the AUTOEXEC.BAT.

The line to be added varies with the designation of

parameter. Hereunder, an example of added line is shown.

*An example of added line of AUTOEXEC.BAT.

REM=============APPEND CD-ROM DRIVE=============

C:\DOS\MSCDEX.EXE/D:CDROM001 /M:12

REM=============APPEND CD-ROM DRIVE=============

The line is basically added to the last of file, but if

there is the description (WIN, WIN.COM) for starting up the

Windows, the line is added just before that line.

If there is already the description of MSCDEX.EXE, that

line is made the comment line at the REM. However, if the

description of that MSCDEX.EXE execution is the line added by

this installer, that line is deleted. If nor not the description

of MSCDEX.EXE execution depends on this installer is to

be judged by the presence of comment line before or after the

line, the added comment line should not be deleted.

For information, the CONFIG.SYS and AUTOEXEC.BAT before

the change are stored in the names of CONFIG.CD0 and AUTOEXEC.CD0.

If there are no CONFIG.SYS and AUTOEXEC.BAT, they should be

prepared anew.

# Chapter 4. Precautions

Described hereunder are the precautions of install program

and CD-ROM device driver.

1.If this install program can not be installed normally、

rewrite the CONFIG.SYS and AUTOEXEC.BAT with reference to the

attached README.TXT and this manual.

2.The troubles of device driver when you desire this device

driver on the Windows95 and OS/2, there may be a case that the

troubles can not be avoided unless they should be tried so by

the respective makers.

3.If this device driver is used on the Windows95, the performance

falls down.

4.If the install work was interrupted in the midway, the CD-ROM

device driver will not be incorporated.