Special Note: 
The BIOS and DEVICE DRIVERS contained here are exclusively for FOXCONN  products only. FOXCONN assumes no responsibility for any damages resulting from improper use or lack of technical expertise.

WARNING!!!!!
DON'T FLASH FROM A FLOPPY DISK!!!!
DON'T FLASH WHEN YOUR SYSTEM IS RUNNING FINE!!!!
DON'T FLASH IF YOU DON'T KNOW WHAT YOU ARE DOING!!!!

Award BIOS Flash Procedure

(Special Thanks to Forum Moderator Bas for his valuable contribution)
1. Check your BIOS for BIOS FLASH DISABLED and ENABLE it, some boards have this function.

For Example:

NF4UK8AA  SuperBIOS features/ superBIOS Protect/ Disabled
TBL jumper on Mothboard is Disabled

2. Know Your model number

3. Download the BIOS that matches your motherboard AND version number (some motherboards are named like NF4UK8AA , make sure you have the right version number).

4. Download the BIOS-archive to c:\test

5. Boot your system from a Win98 or WinME boot-floppy.

How to make boot floppy in case your don’t have it ready:

For Win9X, You can type [C:\ format a:/s] from the DOS prompt.
For WinMe, You can make a boot floppy from control panel--> add/remove program-->make boot floppy. Remove autoexec.bat & config.sys file if there's any.
For Win2000, there's no way to make boot floppy, so you have to either use Win9X or WinME boot floppy.

For WinXP, you can make a DOS boot disk. Go to Your Computer, right click drive A:, select Format, select copy system files. 

7. When you get the A:\ prompt, type the following sequence:
C: <enter>
cd\test <enter>
C:\test> Afu860h BIOS file  /cc/cd/wb

(to save or not to save old BIOS is the user’s decision)
8. Reboot your system when prompted