DigiWARE Release Notes 93000043K

for

DigiCHANNEL MC/X and PC/X OS/2 2.x Device Driver

DGX.SYS V 2.0.1

IBM OS/2 1.3, 2.0.0, 2.0.1

6/10/93

1. Enhancements from V. 2.0.0

2. Bug fixes

- Return Value from IOCtl GET_OUT_QUEUE_COUNT was incorrect. (201b)

- Byte order of value returned from GET_COMMERROR was LO-HI

reversed.(201c)

- Interim (not FINAL) Close requests were not clearing internal thread

status signals. (201d)

- Transmit FIFO on 16550 ports was not being fully explointed (201e).

- IOCtl 0x63 GET_EXTENDED_BITRATE was not filling in the MAX_BITRATE

and MIN_BITRATE fields (201f).

- Extremely heavy full duplex loads on individual channels with 16550

uarts could lead to ignored interrupts (201g).

3. Known limitations

- Though many channels can be active at 1200-4800 baud, only a few

channels can sustain the higher baud rates of 9600 or more without

data loss and serious system performance degradation.

3. Manual Errata

- The included manual has not yet been updated to incorporate the

MC/X.  See the 'INSTALL.DOC' file on the distribution diskette.

In general, the MC/X no requiring per-card hardware arguments is

the only substantive difference.

- Command line parameters have been much simplified and restricted

to the minimum necessary for successful driver initialization.

Consequently, the only remaining valid parameters are hardware

specifications (if PC/X) ,and logical device naming conventions.

The parameter and ability to exclude certain certain channels has

been eliminated due to potential device contention problems.

The parameters previously used to set default baud rates and data

types have been eliminated since applications that access the

devices immediately override those settings anyway.

Interrupt sharing is no longer an option: it is automatically

invoked on MCA architectures that permit it and prohibited on the

ISA bus that precludes it.