This README describes CD-ROM device driver and software parameters for

DOS and OS/2.

------------

| For DOS  |

------------

When the CD-ROM device driver is installed in your computer, the

installation program (UINSTALL.EXE) automatically modifies the

CONFIG.SYS file and AUTOEXEC.BAT file so that your computer can

access the CD-ROM drive.

1) Device Driver IBMTPCD.SYS Parameters

For the IBMTPCD.SYS device driver, the line in CONFIG.SYS is in the

following format.

DEVICE=�drive:��path�IBMTPCD.SYS /R �/C� �/S�

This is the CD-ROM device driver (IBMTPCD.SYS) statement. Make sure

this line is inserted after the EMM386 statement.

�drive:��path�IBMTPCD.SYS

Specifies the location (drive and directory) and name of the

device driver file.

/R

Enables the resident type device driver. Even when the CD-ROM

drive is not installed at power-on, the device driver can be

loaded in the memory. To disable this option, remove the syntax.

�/C�

Sets the cache size in the XMS memory. When this parameter

is set, the cache size in the XMS memory is 512 sectors.

When it is not set, the default value is 0.

�/S�

Sets the power-saving mode to OFF. When using DOS with another

operating system (for example, when using it with OS/2 in dual

boot), set the power-saving mode to OFF using this parameter.

2) MSCDEX (Microsoft CD-ROM Extensions) Parameters

Microsoft CD-ROM Extensions (MSCDEX.EXE) is an executable program

for MS-DOS (Version 3.1 or higher)  that works in conjunction with

the device driver (IBMTPCD.SYS) to allow your computer to access

CD-ROM discs.

The syntax of the MSCDEX.EXE entry in AUTOEXEC.BAT is:

�drive:��path�MSCDEX.EXE /D:TPCD001 �/M:n��/E��/V��/L:x�

�drive:��path�MSCDEX.EXE

Specifies the location (drive and directory) of the MSCDEX.EXE

file.

/D

Specifies the name of the CD-ROM drive, TPCD001.

�/M:n�

Specifies the number of sector buffers for temporary storage

of the most recent disc data. For example to specify four

sector buffers you would use:

C:\MSCDEX.EXE /D:TPCD001 /M:15

UINSTALL will set this value to 15. The default value is 4.

�/E�

Specifies that the computer will use expanded memory,

if available.

�/V�

Displays on the LCD a summary of RAM allocation and

expanded memory usage at power-on.

�/L:drive letter�

Specifies the drive letter to be assigned to the CD-ROM drive.

Do not assign a letter already used by an existing drive or the

computer will be unable to access the CD-ROM drive.

3) SMARTDRV cache for PhotoCD and CD-i movies

If you are using DOS SMARTDRV caches and planning to use a Photo CD

or multiple session discs, you have to add /U parameter to the SMARTDRV

line in the AUTOEXEC.BAT file. This is because Photo CDs or multi-session

discs are not compatible with the SMARTDRV caches.

To be able to play CD-i movies in Windows 3.11, do not cache the

CD-ROM drive by the SMARTDRV Version 5.0 or 5.1, which is supplied

with PC DOS Version 7.0.

------------

| For OS/2 |

------------

This section explains the parameters in the CONFIG.SYS file for

the OS/2 CD-ROM device drivers.

1) Device Driver OS2CDROM.DMD (Device)

DEVICE=OS2CDROM.DMD �Options�

This is the CD-ROM device class driver.

The following options apply:

�/V�

Displays messages on the LCD during the installation.

�/Q�

Does not display messages on the LCD during the installation.

2) Device Driver IBMIDECD.FLT

BASEDEV=IBMIDECD.FLT �Options�

This is the IDE adapter device driver for ATAPI devices.

The following options apply:

�/V�

Displays messages on the LCD during the installation.

3) CDFS (IFS)

IFS=�drive:��path�CDFS.IFS �Options�

This is the ISO9660/HSG file system for OS/2. All CD data

discs are recorded with this file format.

The following options apply:

�/C:n�

Indicates how many 64K segments to use for the sector cache.

Default is n=2, two 64K segments.

�/M:n�

Indicates the maximum number of file sectors to be read at a

time. Default is n=8, 8 sectors.

�/V�

Displays messages on the LCD during the installation.

�/Q�

Does not display messages on the LCD during the installation.