AGX014 OS/2 2.1 Display Drivers installation

1.	 Make sure the Volume Label of AGX OS/2 Display Driver Diskette  is "AGX_OS2".  This is extremely important for drivers downloaded from the BBS.

2.	If  you are going to install the OS/2 to the hard drive, please select the Primary Display as "VGA" during the installation. The AGX display drivers will be installed after the OS/2 itself is completely installed. The OS/2 may show Primay Display as "Extended Graphic Array (XGA)", you need to change it to "VGA".

3.	If the OS/2 is installed on the hard drive already, please configure the OS/2 Primary Display as "VGA" before installing AGX's OS/2 drivers. (Refer to the OS/2 user's guide, if you have not done that before.)

4.	Reboot the OS/2 after configuration. Make sure all features of OS/2 are running OK.

5.	Put the AGX OS/2 Display Driver Diskette into drive A. (or drive B)

6.	Open the OS/2 Window. Change the current drive to drive A (or drive B), then at the OS/2 Command Prompt  A>, type:

COPY  *.DSC   C:\OS2\INSTALL

or

COPY  *.DSC   D:\OS2\INSTALL

7.	Now, run the display driver installation program by typing:

DSPINSTL

This program is in the \OS2\INSTALL subdirectory.

8.	When the DSPINSTL starts, check "Primary Display" and click "OK" to change the display driver.

9.	Scroll down to "X Technology AGX014/AGX015...", Select the desired resolution coupled with the amount of memory on your graphics board and press "OK".

10.	Click "Install..." and the new drivers will be installed. If the AGX014 Display Driver diskette is in drive B, click "Change"  then click "Install"

11.	When the DSPINSTL finished, click "OK".

12.	Shut down the OS/2 and reboot the system. The OS/2 now will run with the AGX014 high resolution display driver you just installed.