QuickCam Device Driver for OS/2 - Version 1.0

=============================================

Features

~~~~~~~~

QCAMDD.SYS is a device driver for OS/2 systems that enables complete

access to the Connectix QuickCam digital camera.  Some of the features

include:

- Autodetection of the parallel port the camera is connected to.

- Simple access via DosOpen, DosRead, and DosClose functions.

- Full control of camera with DosDevIOCtl routines.

- Images captured in a format easily usable with GPI bitmaps.

- Support for 16-bit and 32-bit applications.

- Default camera options can be set via command line arguments.

- Developed using the Connectix QuickCam Interface Specifications

(but according to my agreement with them not officially endorsed

by them in any way).

System Requirements

~~~~~~~~~~~~~~~~~~~

QCAMDD.SYS will work with all versions of OS/2 (including OS/2 Warp)

starting with Version 2.11.  Versions 2.0 and 2.1 can also be made to

work if the PRINT01.SYS (parallel port device driver) is updated to

the OS/2 2.11 (or CSDXR06200) level.

Automatic Installation

~~~~~~~~~~~~~~~~~~~~~~

The QuickCam device driver can be installed using the OS/2 device

driver installation program.  Follow these steps:

- If you received the QuickCam driver as a ZIP file, extract it

into a temporary directory.

- Open your OS/2 System Setup folder and execute 'Device Driver

Install', or type 'DDINSTAL' at an OS/2 command prompt.

- Change the SOURCE DIRECTORY entry to match the temporary

directory in which the QuickCam driver is located.

- The DESTINATION DIRECTORY should match the root directory of your

OS/2 boot drive.

- When you click on the INSTALL button, the QuickCam device driver

should appear in a list box.

- Select the driver by clicking on its name, then click the OK

button.  This will copy the device driver files into a directory

named QCAMDD on the root of your boot drive and add the device

driver loading lines to your CONFIG.SYS

- You can now exit the Device Driver Install program and reboot

your OS/2 system to enable the driver.

Manual Installation

~~~~~~~~~~~~~~~~~~~

- Create a directory on the root of your boot drive named QCAMDD.

- If you received the QuickCam driver as a ZIP file, extract it

into the QCAMDD directory, otherwise, copy all the driver files

to that directory.

- Add the following line to your CONFIG.SYS file:

DEVICE=x:\qcamdd\qcamdd.sys

(where 'x' is the letter of your boot drive)

- Reboot your system to enable the driver.

Startup

~~~~~~~

When you restart your system after the QuickCam device driver has been

installed you should see the following message:

QuickCam Device Driver Installed.

(C) Copyright  Useful Software  1996.

All Rights Reserved.

Camera found on LPTn at address xxxh.

The port name (LPTn) will be one of LPT1, LPT2, or LPT3, and the port

address (xxxh) is typically 278h, 378h, or 3bch.

If the last line of the message is:

Camera not detected on any printer port.

be sure that the QuickCam is plugged in, or if you have a switch box,

that it is setup for camera use.  If for some reason the QuickCam

device driver fails to automatically detect the port the camera is on,

you can explicitly define either the Port Number or Port Address when

the device driver starts up.  See the section on Command Line

Arguments for more details.

About the Camera

~~~~~~~~~~~~~~~~

QuickCam is a black and white digital camera.  The heart of QuickCam

is a charge couple device (CCD) detector array.  Physically the CCD

consists of a two dimensional array of light-sensitive "pixels".  This

320x240 pixel detector receives light that has been focused by a lens

(about f1.9 - equivalent to a 38mm lens on a 35mm camera), achieving a

field-of-view of about 60 degrees.  Images can be captured at 4-bits

per pixel (16 grays) or 6-bits per pixel (64 grays).  This is often

referred to as the pixel "depth".

QuickCam captures images and sends them (on command) to the PC via the

parallel port.  Each image is called a "frame".  The QuickCam Device

Driver for OS/2 takes care of setting up the camera and transferring

images to your application (without having to learn the low-level

protocols to do this).  All of the capabilities of the QuickCam have

been implemented in this driver.

Parallel Port

~~~~~~~~~~~~~

Communication to the QuickCam is through the parallel or printer port.

While the camera is in "use", no other application (including one that

wants to use the QuickCam) can access the port.

Most parallel ports today will support what is called "bi-directional"

mode.  Older ports are uni-directional.  The QuickCam device driver

for OS/2 will automatically detect the port's capability.  This is

significant because bi-directional ports will operate up to 3 times

faster than uni-directional when accessing QuickCam.

Image Formats

~~~~~~~~~~~~~

Images are transferred to your application from the QuickCam in two

formats, packed or bitmap.

Packed images are obtained with the DosRead command, require the least

amount of setup, and are transferred from the camera quicker because

of their smaller size.  However they will require more work by the

application to interpret and display.

Images in bitmap format are somewhat larger, but are designed from the

start to integrate well with OS/2 bitmaps.  Bitmap images are obtained

with the IOCtl Get Frame command.

Packed Format

~~~~~~~~~~~~~

Packed images are placed into the user supplied buffer in the minimum

space required.  So 4-bit images (16 grays) will be "packed" 2 pixels

per byte, while 6-bit images (64 grays) will be "packed" 4 pixels for

every three bytes.  The maximum user buffer size required for this

format is defined in qcamdd.h as QCAM_READ_MAXBUF.

Images are placed into the buffer as consecutive pixels starting with

the upper left hand corner and proceeding down the frame from left to

right, top to bottom.  The size of the frame is dependent on the

values you have specified for left, top, right, and bottom corners and

the transfer mode (set via the IOCtl Set Camera call described below).

By default these values are left=1, top=1, right=320, and bottom=240,

with the transfer mode set to every pixel (in other words the full

size of the CCD array).

Because the pixels are being packed, sometimes the frame width that

you specify (calculated from the frame left and right variables) will

not fit into an even number of bytes.  (This is somewhat complicated

by the way that the camera firmware works but I won't go into detail

on that.)  In these cases, the device driver will "round-up" to the

nearest even "byte granularity" boundary.  The chart that follows will

help you calculate the "actual" width (in pixels) for the frame.

Port Capability         Frame Depth        Byte Granularity

===============         ===========        ================

uni-directional           4-bits                  1

uni-directional           6-bits                  3

bi-directional            4-bits                  3

bi-directional            6-bits                  3

So for example if you specified a frame left=10 and a frame right=160,

and the port was capable of birdirectional transfers, and you were

capturing 6-bit images, then:

frame width = 160-10+1 = 151            (pixel width requested)

granularity = 151/3 = 50.33 = 51        (granularity adjustment)

|           |

from the chart +--+           +--+ since the number of bytes does

|                not divide evenly round up

|

actual width = 51x3 = 153               (pixels per line)

The number of bytes you will receive per line is easily calculated by:

frame bytes = bytes read/frame height

The DosRead command returns an accurate "bytes read" value.  You will

need both the "actual width" and "frame bytes" values to be able to

interpret the image data.

Bitmap Format

~~~~~~~~~~~~~

The first step in obtaining bitmap images from the QuickCam is to

request a "buffer" from the device driver using the IOCtl Get Buffer

call (see below for details).  Get Buffer returns the address of a

32-bit, flat (linear) memory space.  This memory space is pre-defined

to be a maximal 320x240 pixel frame AND DOES NOT CHANGE.  Each pixel

is represented by one byte (8-bits).

Image data is laid out in a format that is compatible with OS/2

bitmaps.  To that end the upper left corner of the image is mapped to

the lower left corner of the frame buffer (ie. upside-down).  Using

the default frame size of 320x240 the entire buffer will be filled

with image data when an IOCtl Get Frame call is issued.  If the frame

size is set to anything other than the maximum, and/or the transfer

mode reduces the image size,  image data will fill only the bottom

left corner of the frame buffer.  The diagram that follows shows the

whole (maximal) buffer, and how an image would be positioned within

this buffer if the frame width were set to 160 pixels, and the frame

height to 120 lines.

(320 pixels/bytes)

+-----------------------------------------------+

|                                               |

|                                               |

|                                               |

|                                               |

|                         FULL FRAME            |

|                                               |

|                                               |

|      (160 pixels)                             |

+-----------------------+                       |  (240 lines)

|                       |                       |

|                       |                       |

|      PARTIAL FRAME    |                       |

|                       |                       |

|                       | (120 lines)           |

|                       |                       |

|                       |                       |

|                       |                       |

+-----------------------+-----------------------+

(top of images)

Because bitmap format uses 1 byte per pixel, the problems with the

"byte granularity" of packed format go away.  You can specify any

frame size that you want.

Each pixel/byte in the image buffer will have a value between 0 and 63

where 0 represents white, 63 represents black, and numbers in-between

are evenly distributed shades of gray (from lightest to darkest).  For

6-bit frame depths, values will extend across the full range (all 64

shades of gray).  4-bit images will still use the same range (0-63)

but only every 4th value will be represented (0,4,8,12,...60 for 16

evenly distributed grays).  By mapping both frame depths to the same

range of values, the application never has to change the palette

associated with the frame when the depth resolution changes.

Using the Camera

~~~~~~~~~~~~~~~~

You control the QuickCam with standard DosOpen, DosDevIOCtl, DosRead,

and DosClose function calls.  You should include the supplied QCAMDD.H

header file in your application to access the pre-defined constants.

DosOpen

~~~~~~~

Before you can interact with the QuickCam you must first establish a

connection to it with the DosOpen command.  DosOpen will return a

"handle" to the camera that must be used in subsequent IOCtl, Read,

and Close calls.

Usage:

#define         INCL_DOSFILEMGR

#include        <os2.h>

#include        "qcamdd.h"

HFILE           CamHandle;              // OS "handle" to QuickCam

ULONG           OpenAction;             // Action taken on open

//  (not used)

APIRET          RC;                     // Return code

RC=DosOpen(QCAM_OPEN_NAME,              // Device driver name

&CamHandle,                  // Pointer to handle

&OpenAction,                 // Pointer to open action

QCAM_OPEN_SIZE,              // Not required

QCAM_OPEN_ATTRIBUTE,         // Not Required

QCAM_OPEN_FLAG,              // Open device

QCAM_OPEN_MODE,              // Fail on error

QCAM_OPEN_EABUF);            // No extended attributes

Returns:

Zero if open was successful.  Will return non-zero if the QuickCam

device driver is not loaded, or if the parallel port that the

camera is connected to is in use by another process.

DosRead

~~~~~~~

You can use the DosRead command to get images from the QuickCam in

"packed" format (as described above).  You must ensure that the buffer

supplied is large enough to hold the whole image at the current frame

size and transfer mode.

Usage:

#define         INCL_DOSFILEMGR

#include        <os2.h>

#include        "qcamdd.h"

HFILE           CamHandle;              // OS "handle" to QuickCam

//  (obtained from DosOpen)

UCHAR           Frame[QCAM_READ_MAXBUF];// Buffer to store image

ULONG           BytesRead;              // Returned by Read

APIRET          RC;                     // Return code

RC=DosRead(CamHandle,                   // Camera handle

Frame,                       // Frame buffer pointer

QCAM_READ_MAXBUF,            // Maximum size of buffer

&BytesRead);                 // Actual bytes read

Returns:

Zero if read was successful.  Number of bytes copied into the image

buffer will be returned in BytesRead.  Non-zero return indicates

read failure.

DosDevIOCtl

~~~~~~~~~~~

The DosDevIOCtl calls are used to control QuickCam settings and to get

images from the camera in "bitmap" format.

IOCtl Get Buffer

~~~~~~~~~~~~~~~~

Before you can use the IOCtl Read function to get a bitmap image, you

must first obtain a buffer from the QuickCam device driver.

Usage:

#define         INCL_DOSDEVICES

#include        <os2.h>

#include        "qcamdd.h"

HFILE           CamHandle;              // OS "handle" to QuickCam

//  (obtained from DosOpen)

ULONG           BufLength=0L;           // Size of buffer obtained

ULONG           BufRetLen=0L;           // Size of length var.

UCHAR           *BufAddress=NULL;       // Address buffer obtained

ULONG           AddRetLen=0L;           // Size of address var.

APIRET          RC;                     // Return code

RC=DosDevIOCtl(CamHandle,

QCAM_IOCTL_CATEGORY,     // Category used for Qcam

QCAM_IOCTL_ALLOC_BUF,    // Allocate buffer function

&BufLength,              // Return buffer size here

sizeof(ULONG),           // Size of BufLength var.

&BufRetLen,              // Not used

&BufAddress,             // Return buffer address

sizeof(UCHAR *),         // Size of BufAddress ptr.

&AddRetLen);             // Not used

Returns:

Zero if get buffer was successful.  The linear 32-bit address of

the buffer obtained will be passed back in BufAddress and the size

of this buffer will be passed back in BufLength.

IOCtl Get Frame

~~~~~~~~~~~~~~~

Use the IOCtl Get Frame function to copy the next frame from the

QuickCam as a bitmap image into the buffer obtained by the IOCtl Get

Buffer call.

Usage:

#define         INCL_DOSDEVICES

#include        <os2.h>

#include        "qcamdd.h"

HFILE           CamHandle;              // OS "handle" to QuickCam

//  (obtained from DosOpen)

ULONG           FrameLength=0L;         // Size of image obtained

ULONG           FrameRetLen=0L;         // Size of length var.

UCHAR           *BufAddress=NULL;       // Address buffer obtained

ULONG           AddRetLen=0L;           // Size of address var.

APIRET          RC;                     // Return code

RC=DosDevIOCtl(CamHandle,

QCAM_IOCTL_CATEGORY,     // Category used for Qcam

QCAM_IOCTL_GET_FRAME,    // Get frame function

&FrameLength,            // Return frame size here

sizeof(ULONG),           // Size of FrameLength var.

&FrameRetLen,            // Not used

&BufAddress,             // Buffer pointer address

sizeof(UCHAR *),         // Size of BufAddress ptr.

&AddRetLen);             // Not used

Returns:

Zero if get frame was successful.  The number of bytes copied into

the image buffer will bet returned in FrameLength.

IOCtl Free Buffer

~~~~~~~~~~~~~~~~~

When you are done accessing the QuickCam device driver, and prior to

doing a DosClose, you should free the buffer obtained by the IOCtl Get

Buffer call.

#define         INCL_DOSDEVICES

#include        <os2.h>

#include        "qcamdd.h"

HFILE           CamHandle;              // OS "handle" to QuickCam

//  (obtained from DosOpen)

ULONG           BufLength=0L;           // Size of buffer obtained

ULONG           BufRetLen=0L;           // Size of length var.

UCHAR           *BufAddress=NULL;       // Address buffer obtained

ULONG           AddRetLen=0L;           // Size of address var.

APIRET          RC;                     // Return code

RC=DosDevIOCtl(CamHandle,

QCAM_IOCTL_CATEGORY,     // Category used for Qcam

QCAM_IOCTL_FREE_BUF,     // Free buffer function

&BufLength,              // Return frame size here

sizeof(ULONG),           // Size of FrameLength var.

&BufRetLen,              // Not used

&BufAddress,             // Buffer address

sizeof(UCHAR *),         // Size of BufAddress ptr.

&AddRetLen);             // Not used

Returns:

Zero if free buffer was successful.

IOCtl Set Camera

~~~~~~~~~~~~~~~~

Use the IOCtl Setup Camera function to control the camera settings.

#define         INCL_DOSDEVICES

#include        <os2.h>

#include        "qcamdd.h"

HFILE           CamHandle;              // OS "handle" to QuickCam

//  (obtained from DosOpen)

USHORT          QcamRequest;            // Request the setups to do

ULONG           ReqRetLen=0L;           // Size of request var.

QCAM_SETTINGS   QcamSetup;              // Camera setup structure

ULONG           SetRetLen=0L;           // Size of setup var.

APIRET          RC;                     // Return code

QcamRequest=QCAM_GET_SETTINGS;

RC=DosDevIOCtl(CamHandle,

QCAM_IOCTL_CATEGORY,     // Category used for Qcam

QCAM_IOCTL_SET_CAMERA,   // Set camera function

&QcamRequest,            // Operations to do

sizeof(USHORT),          // Size of the request var.

&ReqRetLen,              // Not used

&QcamSetup,              // Settings struct. address

sizeof(QCAM_SETTINGS),   // Size of settings struct.

&SetRetLen);             // Not used

Returns:

Zero if set camera was successful.  Non-zero if there was a

problem.  All request calls including the QCAM_GET_SETTINGS

illustrated here return the current camera settings in the

QcamSetup structure.

Set Camera Requests

~~~~~~~~~~~~~~~~~~~

This call supports a number of different operations (requests). As in

the example above, request operation(s) are copied into a variable the

address of which is passed to the IOCtl Set Camera call.  Requests can

be combined by adding the parameter values.  For example:

(...)

USHORT          QcamRequest;            // Request the setups to do

QCAM_SETTINGS   QcamSetup;              // Camera setup structure

QcamSetup.contrast=120;

QcamSetup.exposure=200;

QcamRequest=SET_CONTRAST+SET_EXPOSURE;

(...)

Also note that input values for many of these operations are passed in

a QCAM_SETTINGS structure who's address is also passed to the IOCtl

Set Camera call.

No matter what operation is performed, all of the current QuickCam

settings are returned in the QCAM_SETTING structure.

QCAM_GET_SETTINGS

~~~~~~~~~~~~~~~~~

Does nothing except return the current settings into the QCAM_SETTINGS

structure who's address is passed.

QCAM_INITIALIZE

~~~~~~~~~~~~~~~

Resets the camera to a known state. The version number of the QuickCam

firmware is obtained.  Contrast and exposure values are set to their

starting defaults.  White balance is automatically calculated.  Frame

size is set to maximum (320x240).

QCAM_AUTO_CALIBRATE

~~~~~~~~~~~~~~~~~~~

Determines the white balance. Since there are variations between CCDs,

QuickCam firmware can automatically calibrate white balance (sometimes

called black level) to compensate.

QCAM_SET_CONTRAST

~~~~~~~~~~~~~~~~~

Sets the contrast level.  This operation expects the new contrast

value to be in the "contrast" element of the QCAM_SETTINGS structure.

The default value for the contrast parameter is 104.  In most cases

this will be appropriate.  Note that "auto calibrate" of the while

balance assumes that contrast is set properly.  Valid values are 1 to

255.

QCAM_SET_EXPOSURE

~~~~~~~~~~~~~~~~~

Set the exposure time.  This operation expects the new exposure value

to be in the "exposure" element of the QCAM_SETTINGS structure.  The

valid range of values are 1 to 254 which correspond to exposure times

of 1/1000 of a second to 1 second respectively.  Using a value of 255

will put the camera into "time exposure" mode.  The only operations

that the QuickCam will respond to while in this mode are DosRead,

IOCtlRead, and QCAM_SET_EXPOSURE (a value of 1 is suggested to stop

the time exposure).

QCAM_SET_DEPTH

~~~~~~~~~~~~~~

Set the pixel depth.  Valid values are 4 for 4-bit (16 grays) pixels,

and 6 for 6-bit (64 grays) pixels.  This operation expects the new

depth value to be in the "depth" element of the QCAM_SETTINGS

structure.

QCAM_SET_TRANS_MODE

~~~~~~~~~~~~~~~~~~~

For any given frame size, you can ask the camera to send every pixel

and line, every 2nd pixel/line or every 4th pixel/line of the frame.

This allows you to get the full image at reduced size but much higher

frame rates (because less image data needs to be moved).  Valid

values are 1 (every), 2 (every 2nd), and 4 (every 4th).  This

operation expects the new transfer mode value to be in the

"transfer_mode" element of the QCAM_SETTINGS structure.

QCAM_SET_FRAME

~~~~~~~~~~~~~~

Any rectangular area within the 320x240 pixel CCD window can be

selected for imaging.  This "frame size" is selected by placing the

left, top, right, and bottom values in the QCAM_SETTINGS structure

then calling Set Camera with a QCAM_SET_FRAME request.  The frame

width can be calculated as right-left+1, and similarly frame height

is bottom-top+1.  (NOTE: for DosRead calls the frame width might be

slightly larger as described in the Packed Format section.)

QCAM_SET_BALANCE

~~~~~~~~~~~~~~~~

Explicitly set the white balance.  This operation expects the new

white balance value to be in the "balance" element of the

QCAM_SETTINGS structure.  The value for the while balance is normally

calculated automatically (see QCAM_AUTO_CALIBRATE above).  In most

cases the calculated value will be most appropriate.

QCAM_SET_PORT

~~~~~~~~~~~~~

Normally the QuickCam device driver will auto-detect the printer port

that the camera is attached to.  You can however explicitly set the

port to use with this call.  Setting the port_number value in the

QCAM_SETTINGS structure to 1, 2, or 3 will select LPT1, LPT2, or LPT3

respectively.  In addition if you specify a port_address that value

will be used by the device driver, otherwise the port address is

determined from the system BIOS depending on the port number you set.

If you set the port_number to 0, you can have the device driver

re-try the auto-detect routines.

QCAM_SET_DELAY

~~~~~~~~~~~~~~

When outputting values through the printer port, the device driver

must pause for a short period of time (usually about 500 nanoseconds)

for the port to "settle" before continuing.  For the parallel ports

tested, a single delay of 500ns seemed sufficient.  However older

hardware might need a longer delay.  Setting the delay_factor in the

QCAM_SETTINGS structure can be used to accomplish this.  The valued

entered will cause that many 500ns delays (ie a value of 4 will delay

for 500x4=2000ns).

DosClose

~~~~~~~~

When you have finished accessing the QuickCam you should close the

connection to it.  No other process will have access to the parallel

port until you do.  This is especially important if your camera shares

the same port as you printer (via a switch box).

Usage:

#define         INCL_DOSFILEMGR

#include        <os2.h>

#include        "qcamdd.h"

HFILE           CamHandle;              // OS "handle" to QuickCam

//  (obtained from DosOpen)

APIRET          RC;                     // Return code

RC=DosClose(CamHandle);                 // Camera handle

Returns:

Zero if close was successful.

QuickCam Device Driver Test Program

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

To help put all of the above in perspective, have a look at the sample

code included in this archive.  The source QCAMTEST.C was used to

generate the test program QCAMTEST.EXE.  When run the program sould

display a 320x240 6-bit image from the camera.  You can change the

brightness setting on the camera by pressing the '+' and '-' keys.

Though quite simple, QCAMTEST.C demonstrates most of what you need to

know to write your own QuickCam enabled programs.

Command Line Arguments

~~~~~~~~~~~~~~~~~~~~~~

The following command line arguments can be added to the DEVICE= line

for the QuickCam device driver:

Argument        Command Name        Valid Values    Default

========        ============        ============    =======

/CB=nnn         Camera Balance      1-254           (varies)

/CC=nnn         Camera Contrast     1-255           104

/CE=nnn         Camera Exposure     1-254           210

/CT=n           Camera Transfer     1,2,4           1

/FD=n           Frame Depth         4,6             6

/FH=nnn         Frame Height        1-240           240

/FL=nnn         Frame Left          1-320           1

/FT=nnn         Frame Top           1-240           1

/FW=nnn         Frame Width         1-320           320

/PA=nnn         Port Address        278,378,3bc     (varies) [*]

/PN=n           Port Number         1,2,3           (varies)

/QM             Quiet Mode          (none)          (none)

[*] The driver is expecting a 3-digit hexadecimal number for the port

address.  All other values appearing after the '=' are treated as

decimals.

You can have as many arguments on the command line as you wish.  For

example:

DEVICE=C:\QCAMDD\QCAMDD.SYS /PN=2 /PA=3bc /CC=150 /QM

would set the parallel port to LPT2 with a port address of 3bc, change

the contrast value of the camera to 150 (from the default of 104), and

prevent the copyright notice from being displayed to the screen when

the driver starts up.

Don't leave any spaces between the argument name (case does not

matter), the '=', and the value that follows.

Licensing

~~~~~~~~~

The programs and documentation in this archive are

Copyright (c) 1996 by Useful Software

The licensing fee for using the QuickCam Device Driver for OS/2 varies

according to application.

Hobbyists  - For personal use on a single system by an individual

there is NO charge.

Developers - If you are using the QuickCam Device Driver with the

intent of developing a product which requires it, for a

registration fee of $25.00 you will be able to obtain

on-line support via E-mail and access to latest versions

of the driver from the Useful Software BBS.

Shareware  - You can bundle the QuickCam Device Driver with your

shareware product for a distribution fee of $250.00.  We

require that you sign a Distribution Agreement prior to

shipping your product, but are prepared to be quite

flexible with the payment schedule.

Commercial - The fee for the unlimited use of the QuickCam Device

Driver with any commercial product is $2500.00.  You must

sign a Distribution Agreement prior to shipping your

product.  This fee includes up to 10 hours programming

effort to customize the device driver to your specific

needs.

No guarantee is made as to the functionality of this software; however

it has been tested quite thoroughly by a large number of people.  If

it doesn't do what you want, don't keep it and send no money.

Any questions, suggestions, or bug reports, can be mailed to:

Useful Software

49 McCarron Cr.

Waterloo, Ontario

Canada, N2L 5M9

or send via electronic mail to:

InterNet:   megardi@wat.hookup.net

Enjoy,

Useful Software