Functions in the B8LIB library

------------------------------

#include <b8lib3>

USHORT EXPENTRY B8Open(phfB008)

PHFILE phfB008;  /* pointer to device driver file handle */

The B8Open functions opens the Transputer device driver (B008$) for subse-

quent use. The file handle returned by this function has to be used in all

other B8LIB functions.

Parameters Description

---------------------------------------------------------------------------

phfB008    Points to the device driver file handle.

Return Values

The return value is ERROR_B8_NO_OPEN, if the device driver can't be opened.

The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8Close(hfB008)

HFILE hfB008;   /* device driver file handle */

The B8Close functions closes the Transputer device driver (B008$).

Parameters Description

---------------------------------------------------------------------------

hfB008     The device driver file handle created previously by using the

B8Open\vB8Open\v function.

Return Values

The return value is ERROR_B8_NO_CLOSE, if the device driver can't be closed.

The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8Read(hfB008, pvBuf, usTimeOut, cbBuf, pcbBytesRead)

HFILE hfB00;          /* device driver file handle */

PBYTE pvBuf;          /* pointer to buffer receiving data */

USHORT usTimeOut;     /* timeout value */

ULONG cbBuf;          /* number of bytes in buffer */

PULONG pcbBytesRead;  /* pointer to variable for number of bytes read */

The B8Read function reads up to a specified number of bytes of data from the

transputer link into a buffer. The function may read fewer than the speci-

fied number of bytes in case of a timeout.

Parameters    Description

---------------------------------------------------------------------------

hfB008        The device driver file handle. This handle must have been

created previously by using the B8Open\vB8Open\v function.

pvBuf         Points to the buffer that receives the data.

usTimeOut     Timeout value. This value specifies the maximum

allowed time between two byte read from the link.

The TimeOut value has to be specified in 1/10-second units.

cbBuf         Specifies the number of bytes to read from the link.

pcbBytesRead  Points to the variable that receives the number of bytes read

from the link. After a timeout this value indicates the number

of bytes read before the timeout occurred.

Return Values

Possible return values are:

ERROR_B8_RD_ERR, if a general error occurred

ERROR_B8_RD_T_OUT, if a timeout occurred during transmission

ERROR_B8_RD_N_RDY, if a timeout occurred before the transmisssion started

zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8DMARead(hfB008, pvBuf, usTimeOut, cbBuf, pcbBytesRead)

HFILE hfB00;          /* device driver file handle */

PBYTE pvBuf;          /* pointer to buffer receiving data */

USHORT usTimeOut;     /* timeout value */

ULONG cbBuf;          /* number of bytes in buffer */

PULONG pcbBytesRead;  /* pointer to variable for number of bytes read */

The B8Read function reads up to a specified number of bytes of data from the

transputer link into a buffer. The function may read fewer than the speci-

fied number of bytes in case of a timeout.

This function is similar to the B8Read\vB8Read\v function except that it uses the DMA

if more than 256 bytes have to transferred.

Parameters    Description

---------------------------------------------------------------------------

hfB008        The device driver file handle. This handle must have been

created previously by using the B8Open\vB8Open\v function.

pvBuf         Points to the buffer that receives the data.

usTimeOut     Timeout value. This value specifies the maximum

allowed time between two byte read from the link.

The TimeOut value has to be specified in 1/10-second units.

cbBuf         Specifies the number of bytes to read from the link.

pcbBytesRead  Points to the variable that receives the number of bytes read

from the link. After a timeout this value indicates the number

of bytes read before the timeout occurred.

Return Values

Possible return values are:

ERROR_B8_RD_ERR, if a general error occurred

ERROR_B8_RD_T_OUT, if a timeout occurred during transmission

ERROR_B8_RD_N_RDY, if a timeout occurred before the transmisssion started

zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8Write(hfB008, pvBuf, usTimeOut, cbBuf, pcbBytesWritten)

HFILE hfB00;             /* device driver file handle */

PBYTE pvBuf;             /* pointer to buffer with transmission data */

USHORT psTimeOut;        /* timeout value */

ULONG cbBuf;             /* number of bytes in buffer */

PULONG pcbBytesWritten;  /* pointer to variable for number of bytes

written */

The B8Write function writes up to a specified number of bytes of data from

the buffer to the transputer link. The function may write fewer than the

specified number of bytes in case of a timeout.

Parameters    Description

---------------------------------------------------------------------------

hfB008           The device driver file handle. This handle must have been

created previously by using the B8Open\vB8Open\v function.

pvBuf            Points to the buffer that receives the data.

usTimeOut        Timeout value. This value specifies the maximum

allowed time between two bytes written to the link.

The TimeOut value has to be specified in 1/10-second units.

cbBuf            Specifies the number of bytes to write to the link.

pcbBytesWritten  Points to the variable that receives the number of bytes

written to the link. After a timeout this value indicates

the number of bytes written before the timeout occurred.

Return Values

Possible return values are:

ERROR_B8_WR_ERR, if a general error occurred

ERROR_B8_WR_T_OUT, if a timeout occurred during transmission

ERROR_B8_WR_N_RDY, if a timeout occurred before the transmisssion started

zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8DMAWrite(hfB008, pvBuf, usTimeOut, cbBuf, pcbBytesWritten)

HFILE hfB00;             /* device driver file handle */

PBYTE pvBuf;             /* pointer to buffer with transmission data */

USHORT psTimeOut;        /* timeout value */

ULONG cbBuf;             /* number of bytes in buffer */

PULONG pcbBytesWritten;  /* pointer to variable for number of bytes

written */

The B8Write function writes up to a specified number of bytes of data from

the buffer to the transputer link. The function may write fewer than the

specified number of bytes in case of a timeout.

This function is similar to the B8Write\vB8Write\v function except that it uses the

DMA if more than 256 bytes have to transferred.

Parameters    Description

---------------------------------------------------------------------------

hfB008           The device driver file handle. This handle must have been

created previously by using the B8Open\vB8Open\v function.

pvBuf            Points to the buffer that receives the data.

usTimeOut        Timeout value. This value specifies the maximum

allowed time between two bytes written to the link.

The TimeOut value has to be specified in 1/10-second units.

cbBuf            Specifies the number of bytes to write to the link.

pcbBytesWritten  Points to the variable that receives the number of bytes

written to the link. After a timeout this value indicates

the number of bytes written before the timeout occurred.

Return Values

Possible return values are:

ERROR_B8_WR_ERR, if a general error occurred

ERROR_B8_WR_T_OUT, if a timeout occurred during transmission

ERROR_B8_WR_N_RDY, if a timeout occurred before the transmisssion started

zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8Reset(hfB008)

HFILE hfB008;   /* device driver file handle */

The B8Reset functions resets the root-transputer.

Parameters Description

---------------------------------------------------------------------------

hfB008     The device driver file handle created previously by using the

B8Open\vB8Open\v function.

Return Values

The return value is ERROR_B8_NO_RESET, if the transputer can't be reset.

The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8Analyse(hfB008)

HFILE hfB008;   /* device driver file handle */

The B8Analyse functions resets the root-transputer into the analyse mode.

Parameters Description

---------------------------------------------------------------------------

hfB008     The device driver file handle created previously by using the

B8Open\vB8Open\v function.

Return Values

The return value is ERROR_B8_NO_ANALYSE, if the transputer can't be ana-

lysed. The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8TestError(hfB008, pfError)

HFILE hfB008;   /* device driver file handle */

PBOOL pfError;  /* pointer to error flag */

The B8TestError functions reads the root-transputer error flag.

Parameters Description

---------------------------------------------------------------------------

hfB008     The device driver file handle created previously by using the

B8Open\vB8Open\v function.

pfError    Points to the flag that receives the result.

Return Values

The return value is ERROR_B8_GET_E_FLAG, if the flag can't be read.

The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8TestRead(hfB008, pfRead)

HFILE hfB008;   /* device driver file handle */

PBOOL pfRead;   /* pointer to the ready-for-read flag */

The B8TestRead functions reads the link ready-for-read flag.

Parameters Description

---------------------------------------------------------------------------

hfB008     The device driver file handle created previously by using the

B8Open\vB8Open\v function.

pfRead     Points to the flag that receives the result.

Return Values

The return value is ERROR_B8_GET_R_FLAG, if the flag can't be read.

The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8TestWrite(hfB008, pfWrite)

HFILE hfB008;   /* device driver file handle */

PBOOL pfWrite;  /* pointer to the ready-for-write flag */

The B8TestWrite functions reads the link ready-for-write flag.

Parameters Description

---------------------------------------------------------------------------

hfB008     The device driver file handle created previously by using the

B8Open\vB8Open\v function.

pfWrite    Points to the flag that receives the result.

Return Values

The return value is ERROR_B8_GET_W_FLAG, if the flag can't be read.

The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8BootFile(hfB008, hfBootFile)

HFILE hfB008;      /* device driver file handle */

HFILE hfBootFile;  /* boot file handle */

The B8BootFile function resets the root transputer, then boots it from the

specified file.

Parameters        Description

---------------------------------------------------------------------------

hfB008             The device driver file handle created previously by

using the B8Open\vB8Open\v function.

hfBootFile         The handle of the boot file previously open with

the DosOpen\vDosOpen\v function.

Return Values

Possible return values are:

ERROR_B8_BOOT_FHANDLE, if an invalid bootfile handle has been specified

ERROR_B8_BOOT_NULL,    if the file has zero size

ERROR_B8_BOOT_NOMEM,   if not enough memory is available

ERROR_B8_BOOT_FILE,    if the bootfile cannot be read

ERROR_B8_BOOT_RESET,   if the board cannot be reset

ERROR_B8_BOOT_NOBOARD, if no board was detected

ERROR_B8_BOOT_TRANSM,  if a transmission error occurred during boot

The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8DMABootFile(hfB008, hfBootFile)

HFILE hfB008;      /* device driver file handle */

HFILE hfBootFile;  /* boot file handle */

The B8BootFile function resets the root transputer, then boots it from the

specified file.

This function is similar to the B8Boot\vB8Boot\v function except that it uses the

DMA if more than 256 bytes have to transferred.

Parameters        Description

---------------------------------------------------------------------------

hfB008             The device driver file handle created previously by

using the B8Open\vB8Open\v function.

hfBootFile         The handle of the boot file previously open with

the DosOpen\vDosOpen\v function.

Return Values

Possible return values are:

ERROR_B8_BOOT_FHANDLE, if an invalid bootfile handle has been specified

ERROR_B8_BOOT_NULL,    if the file has zero size

ERROR_B8_BOOT_NOMEM,   if not enough memory is available

ERROR_B8_BOOT_FILE,    if the bootfile cannot be read

ERROR_B8_BOOT_RESET,   if the board cannot be reset

ERROR_B8_BOOT_NOBOARD, if no board was detected

ERROR_B8_BOOT_TRANSM,  if a transmission error occurred during boot

The return value is zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8Peek(hfB008, pvBuf, usTimeOut, cbBuf, pcbBytesRead)

HFILE hfB00;          /* device driver file handle */

PBYTE pvBuf;          /* pointer to buffer receiving data */

USHORT usTimeOut;     /* timeout value */

ULONG cbBuf;          /* number of bytes in buffer */

PULONG pcbBytesRead;  /* pointer to variable for number of bytes read */

The B8Peek function peeks up to a specified number of bytes from the root

transputer into a buffer. The function may read fewer than the speci-

fied number of bytes in case of a timeout.

Parameters    Description

---------------------------------------------------------------------------

hfB008        The device driver file handle. This handle must have been

created previously by using the B8Open\vB8Open\v function.

pvBuf         Points to the buffer that receives the data.

usTimeOut     Timeout value. This value specifies the maximum

allowed time between two byte read from the link.

The TimeOut value has to be specified in 1/10-second units.

cbBuf         Specifies the number of bytes to read from the link.

pcbBytesRead  Points to the variable that receives the number of bytes

peeked. After a timeout this value indicates the number of

bytes read before the timeout occurred.

Return Values

Possible return values are:

ERROR_B8_PK_ERR, if a general error occurred

ERROR_B8_PK_T_OUT, if a timeout occurred during transmission

ERROR_B8_PK_N_RDY, if a timeout occurred before the transmisssion started

zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8GetInt(hfB008, psData, pbReady)

HFILE hfB00;       /* device driver file handle */

PUSHORT psData;    /* pointer to buffer receiving data */

PBOOL pbReady;     /* pointer to ready flag */

The B8GetInt function tries to read an INT16 value from the transputer.

If no data is available, the function returns immediately.

Parameters    Description

---------------------------------------------------------------------------

hfB008        The device driver file handle. This handle must have been

created previously by using the B8Open\vB8Open\v function.

psData        Points to the buffer that receives the data.

pbReady       Pointer to the ready flag which indicates whether the

function was successful in reading the INT16. If TRUE, the

function was successful.

Return Values

Possible return values are:

ERROR_B8_GET_INT, if a general error occurred

zero, if the function was successful.

#include <b8lib3>

USHORT EXPENTRY B8Get8Bytes(hfB008, pbData, pbReady)

HFILE hfB00;       /* device driver file handle */

PBYTE pbData;      /* pointer to buffer receiving data */

PBOOL pbReady;     /* pointer to ready flag */

The B8Get8Bytes function tries to read 8 bytes from the transputer.

If no data is available, the function returns immediately.

Parameters    Description

---------------------------------------------------------------------------

hfB008        The device driver file handle. This handle must have been

created previously by using the B8Open\vB8Open\v function.

pbData        Points to the buffer that receives the data.

pbReady       Pointer to the ready flag which indicates whether the

function was successful in reading the data. If TRUE, the

function was successful.

Return Values

Possible return values are:

ERROR_B8_GET_8BYTES, if a general error occurred

zero, if the function was successful.