����������������������������������������ͻ

�                                        �

�   MODEM Device Initialization Strings  �

�                                        �

����������������������������������������ͼ

DCAF Change Team

September 27, 1994

IBM Internal Use Only

ABSTRACT

----------

This is a first draft of a document aimed to provide some guidelines

on how to set up an asynch DCAF connection.

���������������������������������������������������������������������������Ŀ

�           General criteria for setting up an init string for DCAF.        �

�����������������������������������������������������������������������������

Commands for the 9600bps speed:

1) Reset to the factory: this command resets to the default value

the modem settings. Usually it is &F.

2) Track status of the Carrier Detect Signal. This is usually default.

Very common is &C1.

3) Monitor the Data Set Ready signal. When the DTR goes off, the modem

enters the command state. It is usually default, very common is &D2.

4) Have the modem to answer on the first ring; this can be generally done

by setting the S0 register to 0 (no rings before answering).

5) Have the modem to wait 60 seconds for a carrier; this can be generally

done by setting the S7 register to 60.

6) Disable any kind (SW and HW) of flow control; generally &K0.

7) Set the communication mode to asynch.; a common command is &Q0.

Commands for speeds beyond 9600bps:

general considerations:

- the connection will be generally more instable, expecially when

a huge file transfer is performed;

- a connection going at 19.2 between computer and modem, and at a

lower speed on the phone line is always possible, but will be greatly

depending on the modem in use. A huge file transfer is the best way

to test the session stability.

- some kind of error control and/or compression is strongly recommended

at speed greater than 9600bps.

1) Reset to the factory: this command resets to the default value

the modem settings. Usually it is &F.

2) Track status of the Carrier Detect Signal. This is usually default.

Very common is &C1.

3) Monitor the Data Set Ready signal. When the DTR goes off, the modem

enters the command state. It is usually default, very common is &D2.

4) Have the modem to answer on the first ring; this can be generally done

by setting the S0 register to 0 (no rings before answering).

5) Have the modem to wait 60 seconds for a carrier; this can be generally

done by setting the S7 register to 60.

6) Disable any kind (SW and HW) of flow control; generally &K0.

7) If possible, set the phone link rate. This command is not always available,

for example &Nxx for the Zyxel.

8) Set the communication mode to asynch.; a common command is &Q0.

9) Try compression V42bis and/or MNP5; if this does not work, try disabling

one out the two, then both;

- Sometimes 8) and 9) exclude each other: in these cases, an experimental

approach is the only possibility.

- When connecting two modems from different brands, ensure that both

of them support the same compression. If they don't, disable compression

on both and...keep the fingers crossed.

���������������������������������Ŀ

�   MODEMs tested from DCAF C/T   �

�����������������������������������

IBM 7855 up to 9600 bps

-------------------------

====================================

AT&FS7=60&C1&D2S0=0&Q0\Q0\R0|

====================================

Here is an explanation of the string for the IBM 7855:

1. &F       Reset to Factory;

2. S7=60    Handshaking time-out;

3. &C1      RLSD control;

4. &D2      DTR control (2=OFF)

5. S0=0     Rings before answering;

6. &Q0      Communication mode (0=ASYNC)

7. \Q0      Flow control (0=NONE)

8. \R0      V.24 Flow control (0=NONE)

IBM 7855 at 19200 bps

-----------------------

This is the Device Initialization String provided by David Young in the

DCAF ANSWERS file to run DCAF with a DTE-to-modem speed of 19,200 bps and

a modem-to modem speed of 12,000 bps with IBM 7855 Modems:

=======================================

AT&F0&C1S0=0\Q2\N3&AP8\M14&I1\A3%C0|

=======================================

Explanation:

&F0 : Reset to Factory Default settings

&C1 : Carrier Detect when signal detected

S0=0: Modem will not Auto-Answer

\Q2 : needed to enable RFS flow control for the modem to tell

the system to stop sending data.  The default for ACDI is

that it responds to RFS flow control.

\N3 : calling modem will initiate an ECL connection.  You must

have an ECL connection to achieve a modem-to-modem speed

of 12000 bps.

&AP8: sets modems preferred telephone line speed to 12000 bps.

\M14: sets modem to use a fixed DTE interface speed of 19,200 bps

(or whatever it senses the DTE interface speed to be)

regardless of what the modem-to-modem speed is.

&I1 : needed so modem will return a "CONNECT 19200" message to the

PS/2.  ACDI reads the connect message from the modem and it

will modify the system UART to the speed in the connect message.

If you configure the modem to use a fixed speed of 19200 bps

regardless of what the modem-to-modem speed is, you don't

want the system to reconfigure the system UART to a different

speed.

\A3 : ECL Buffer size set to 256, use if you have clean phone lines.

%C0 : Disable modem data compression (%C1 = MNP5, the default).

Moreover in the link record on the control specify 19200 line speed

On the Gateway specify EQNGWASY 1 19200

HAYES V_SERIES 9600 MODEM

---------------------------

====================================

AT&FS7=60&C1&D2S0=0&Q0&T5&K0|

====================================

Here is an explanation of the string for the HAYES V-SERIES 9600:

1. &F       Reset to Factory

2. S7=60    Handshaking timeout

3. &C1      RLSD control

4. &D2      DTR control (2=OFF)

5. S0=0     Rings before answering

6. &Q0      Communication mode (0=ASYNC)

7. &T5      No loopback

8. &K0      Disable local flow control

HAYES SMARTMODEM OPTIMA 9600 MODEM

------------------------------------

====================================

AT&FS7=60&C1&D2S0=0&Q0&T5&K0|

====================================

Here is an explanation of the string for the HAYES Smart Modem 9600

at 9600 bps:

1. &F       Reset to Factory

2. S7=60    Handshaking timeout

3. &C1      RLSD control

4. &D2      DTR control (2=OFF)

5. S0=0     Rings before answering

6. &Q0      Communication mode (0=ASYNC)

7. &T5      No loopback

8. &K0      Disable local flow control

HAYES SMARTMODEM OPTIMA 14400 MODEM

-------------------------------------

======================================

AT&FS7=60S11=55&C1&D2S0=0&Q5&P5&K1|

======================================

Here is an explanation of the string for the HAYES Smart Modem 14400

at 19200 bps:

This string was tested at Dean Witter (refer to PMR 4X680)

1. &F       Reset to Factory

2. S7=60    Handshaking timeout

. S11=55

3. &C1      RLSD control

4. &D2      DTR control (2=OFF)

5. S0=0     Rings before answering

6. &Q5      Error check enabled

7. &P5

8. &K1

US-ROBOTICS COURIER HIGH SPEED MODEM

--------------------------------------

for 9600bps:

====================================

ATZS7=60&C1&D2S0=0&H0|

====================================

for 19200bpp:

====================================

ATZS7=60&C1&D2S0=0&H1&K1&M0|

====================================

Here is an explanation of the string for the USROBOTICS COURIER HIGH SPEED:

1. Z        Reset to Factory

2. S7=60    Handshaking timeout

3. &C1      Normal CD operations

4. &D2      Normal DTR operations

5  &B0      Variable port speed  ???????

6. S0=0     Rings before answering

7. &H0      Transmit Data flow control disabled

8. &R1      The modem ignores RTS.

For this modem is also important the switch setting, we used the following:

1  OFF

2  OFF

3  ON

4  OFF

5  OFF

6  OFF

7  OFF

8  ON

9  OFF

10  OFF

ZYXEL U-1496 series at 9600bps:

---------------------------------

====================================

AT&FS7=60&C1&D2S0=0&M0&H0|

====================================

Here is an explanation of the string for the ZYXEL U-1496 AT 9600bps:

1. &F       Reset to Factory

2. S7=60    Handshaking timeout

3. &C1      Normal CD operations

4. &D2      Normal DTR operations

6. S0=0     Rings before answering

7. &M0      Asynch. mode

8. &H0      Transmit Data flow control disabled

ZYXEL U-1496 series at 19200bps:

---------------------------------

====================================

AT&FS7=60&C1&D2S0=0&M0&N36&H3|

====================================

Here is an explanation of the string for the ZYXEL U-1496 AT 19200bps:

1. &F       Reset to Factory

2. S7=60    Handshaking timeout

3. &C1      Normal CD operations

4. &D2      Normal DTR operations

6. S0=0     Rings before answering

7. &M0      Asynch. mode

8. &N36     Reserved for 19.2 link rate

9. &H3      Hardware CTS/RTS flow control enabled

HAYES ACCURA 144 + FAX 144:

---------------------------------

====================================

AT&FS7=60&C1&D2S0=0&Q9&K0|

====================================

Here is an explanation of the string for the

1. &F       Reset to Factory

2. S7=60    Handshaking timeout

3. &C1      Normal CD operations

4. &D2      Normal DTR operations

6. S0=0     Rings before answering

7. &M0      Asynch. mode

8. &Q9      Compression V42 and MN5

9. &K0      Hardware and Software flow control disabled

HAYES ACCURA 144 + FAX 144 at 19200bps:

---------------------------------

====================================

AT&FS7=60&C1&D2S0=0&Q9&K0|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. S7=60    Handshaking timeout

3. &C1      Normal CD operations

4. &D2      Normal DTR operations

6. S0=0     Rings before answering

8. &Q9      Compression V42 and MN5

9. &K0      Hardware and Software flow control disabled

US ROBOTICS 14400 Sportster modem at 19200bps:

-----------------------------------------------

====================================

AT&FB0Q0X4&C1&D2&H0&K3&A3&N0&S1S0=1|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. B0       ITU-T answer seq.

3. Q0       Display rc

4. X4       result code set

5. &C1      Normal CD operations

6. &D2      Normal DTR operations

7. &H0      Flow control off

8. &K3      MNP5 off

9. &A3      LAPM/MNP/NONE error control and V42BIS/MNP5

10. &N0      Variable rate

11. &S1      Modem controls DSR

12. S0=1     Rings before answering

US ROBOTICS 14400 Sportster modem at 9600bps:

-----------------------------------------------

====================================

AT&F&C1&D2S0=0S7=60&H0&N6&K0&M0|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. B0       ITU-T answer seq.

3. Q0       Display rc

4. X4       result code set

5. &C1      Normal CD operations

6. &D2      Normal DTR operations

7. &H0      Flow control off

8. &N6      Data rate at 9600

9. &K0      Compression disabled

10. &M0      Error control disabled

INTEL FAXMODEM 14.4 at 9600bps:

-----------------------------------------------

====================================

AT&F&C1&D2S0=0S7=60\N\Q%C"H|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. &C1      Normal CD operations

3. &D2      Normal DTR operations

4. \N       Normal connection

5. \Q       No flow control

6. %C       Disable MNP5 compr.

7. "H       Disable v42.bis compr.

INTEL FAXMODEM 14.4 at 19200bps:

-----------------------------------------------

====================================

AT&F&C1&D2S0=0S7=60\N3\Q"H3|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. &C1      Normal CD operations

3. &D2      Normal DTR operations

4. \N3      Auto-reliable connection (compr.)

5. \Q       No flow control

6. "H3      Enable v42.bis compr.

ATI 9600 ETC-E at 9600bps:

-----------------------------------------------

====================================

AT&F&C1&D2S0=0S7=60&K0&Q0|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. &C1      Normal CD operations

3. &D2      Normal DTR operations

4. &K0      Flow control off

5. &Q0      No compression

DIGICOM 9624LE at 9600bps:

-----------------------------------------------

====================================

AT&F&C1&D2&M0*E0*F0*H0*N3S0=1S7=60|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. &C1      Normal CD operations

3. &D2      Normal DTR operations

4. &M0      Async. connection

5. *E0      No Error control

6. *F0      No flow control

7. *H0      Turn off Enq/Ack flow control

8. *N3      Modem connect rate at 9600

DIGICOM 9624LE at 19200bps:

-----------------------------------------------

====================================

AT&F&C1&D2&M0*E9*F0*H0*N6S0=1S7=60|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. &C1      Normal CD operations

3. &D2      Normal DTR operations

4. &M0      Async. connection

5. *E9      Auto-reliable V42BIS error control

6. *F0      No flow control

7. *H0      Turn off Enq/Ack flow control

8. *N6      Modem connect rate at 14400

PROMETHEUS  PROMODEM 144E at 9600bps:

-----------------------------------------------

====================================

AT&F&C1&D2&K0&M0\G0\N0%C0S0=1S7=60|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. &C1      Normal CD operations

3. &D2      Normal DTR operations

4. &K0      No DTE flow control

5. &M0      Async. connection

6. \G0      No port flow control

7. \N0      non MNP operation

8. %C0      Disable class 5 operation

9. S0=1     Rings before answering

10. S7=60    Handshaking time-out

PROMETHEUS  PROMODEM 144E at 19200bps:

-----------------------------------------------

====================================

AT&F&C1&D2&K0&M0&Q5\G0\N6S0=1S7=60|

====================================

Here is an explanation of the string:

1. &F       Reset to Factory

2. &C1      Normal CD operations

3. &D2      Normal DTR operations

4. &K0      No DTE flow control

5. &M0      Async. connection

6. &Q5      V42BIS standard operation

7. \G0      No port flow control

8. \N6      Auto Reliable LAPM, MNP & Buffered mode

9. S0=1     Rings before answering

10. S7=60    Handshaking time-out

������������������������������Ŀ

�        CROSS LINKS           �

��������������������������������

We tried the following connections between modems of different firms:

CTRL Hayes Optima 9600 -------------------->  Target IBM 7855

CTRL Hayes Optima 9600 -------------------->  Gateway IBM 7855

CTRL IBM 7855  ---------------------------->  Target Hayes Optima 9600

CTRL IBM 7855  ---------------------------->  Gateway Hayes Optima 9600

The strings used are the ones listed above.

Moreover:

US Robotics Courier ----------------------->  Zyxel U-1496

using for 9600bps the strings listed above, and for 19.2 the following strings:

ZYXEL U-1496 series at 19200bps:

====================================

AT&FS7=60&C1&D2S0=0&M0&N36&H3|

====================================

US-ROBOTICS COURIER high speed at 19.2:

====================================

AT&F0S7=60&C1&D2S0=0&H1&N0&K3&M4&R0&S1|

where:

&H1 = Hardware flow control enabled

&N0 = normal link speed (to be negotiated)

&K3 = V42 bis only data compression

&M4 = normal/ARQ mode

&R0 = RTS flow control on

&S1 = modem controls DSR

====================================

The important thing is that since Zyxel uses data compression and error check

by default, and it works really fine (the session does never drop, even if

huge files are transferred), these kind of features must be also enabled on

the partner modem. If the error check is not performed, the DCAF session

uses to drop especially during the file transfer of huge files at 19.2.

������������������������������Ŀ

�  MODEMs tested by customers  �

��������������������������������

List of modems and related initialization strings successfully used

by customers to run DCAF: these settings haven't been tested over here!

============================================================================

===> Siemens 9632-M Modem Settings: ATZ|AT*FL2|AT*SM1|

============================================================================

===> Elsa 2440-TR Modem Settings: ATZ|AT\Q3\N0|

============================================================================

===> Practical Peripherals PM14400FXMT: AT&FS7=60&C1&D0S0=0N1&K0&Q0&B0|

============================================================================

===> Thinkpad 700C high speed datafax (internal) modem:

===> AT &FS7=60&C1&D2S0=0Q5&T5&K3%C3|

============================================================================

===> PRESTICON 9600 modem: AT &FE1V1X4S7=60&C1&D2S0=0\N6&S1\Q3%T0|

============================================================================

===> US ROBOTICS 14400 Sportster modem:

===> AT&FB0Q0X4&C1&D2&H0&K3&A3&N0&S1S0=1|

============================================================================

===> DIGICOM EAGLE (at 9600): AT&FS7=60&C1&D2S0=0*F0*Q2E0|

============================================================================

===> Telebit: AT&F&C1&D2S58=0S7=60S68=0S69=0S180=0S190=0|

============================================================================

===> Intel Satisfaxtion 400e: AT&F&C1&D2S7=60S0=0&T5\G0\N0\Q0%E1|

============================================================================

===> Motorola UDS FastTalk II: AT&FS7=60&C1&D2S0=1&M0%C\C0\G0\J1\N1\Q0\X0|

where:

&M0 = Async operation

%C  = Disable V.42 bis or MNP level 5 compression

\C0 = Disable auto.reliable buffer

\G0 = Disable modem port flow control

\J1 = Disable slaved DTE/DCE speed

\N1 = Direct mode

\Q0 = Disable flow control

\X0 = No XON/XOFF characters to remote DCE

============================================================================

===> Hayes Optima 288 V.FC: AT&FN5S7=60S11=55S37=12&C1&D2S0=1&Q5&K1B81| (19.200)

============================================================================

===> Motorola UDS FasTalk V.32/42b: AT&FS7=60&C1&D2&M%C%E\C\G\J1\M\N1\Q\X|

============================================================================

===> ZOOM FAXMODEM: AT&FE1Q0V1X4S7=60&C1&D2S0=0&T5|

============================================================================

===> IBM PS/2 Internal 14.4 DATA/FAX modem:

===> AT&F&C1&D2S0=0&K0&Q0|

============================================================================

===> IBM 7851 up to 9600 bps

===> AT&F~~~E1V1X4&C1&D2%E1&K0

============================================================================

=============================================

===> US-Robotics 144 Sportster v32  at 9600

===>    AT&F&C1&D2S0=0S7=60&H0&N6&K0&M0|

===> Intel

===>    AT&F&C1&D2S0=0S7=60-N0"H0%C0-J0|

=============================================

Technical Support Telephone List:

-----------------------------------

US ROBOTICS:  708-982-5151

HAYES: 404-441-1617

ZYXEL: 886-35-783942   (TAIWAN)

INTEL: 800-524-3388

614-457-0802