�����������������������������������͸

�  COMMUNICATION PORTS TEST CASES   �

�����������������������������������;

Objective:   To execute different applications which will use the

����������   communication driver to test the video device drivers.

This test runs on two machines 386/486 with communication ports and modems

attached.  Various communication applications are run on these machines.

Different files are uploaded and downloaded in this test.

Applications to be tested are:

������������������������������

Procomm** Plus

DynaComm**

There are two machines necessary for this test. A "host" machine, and the

"remote" machine. The remote machine will be calling the host machine to

receive data.

For this test, a clean OS/2* install should be done for every video device

driver tested (or press Ctrl-Alt-F1 during boot-up to reset the desktop).

Make sure to press "Ctrl+Esc" to switch between the desktop and the

applications while running the test. Repeat the process at different stages

of the application.

Verify that both machines are connected to analog telephone lines.

PROCOMM PLUS

������������

1.  Open an DOS full screen session (VDM) to run Procomm Plus. If you do

not have Procomm Plus installed, please refer to your Procomm Plus

Installation Manual for instructions.

2.  At the Procomm Plus screen, press Enter to go to terminal mode.

3.  Press Alt-D for dial mode.

4.  Enter the number and start the call.

5.  When the connection has been established perform login.

6.  Enter a "U"  to upload a file or a "D" to download a file.

8.  Enter modem protocol type. The most common protocols are Kermit, Xmodem

and Ymodem. Use these protocols along with a mixture of the others.

9.  Enter the file name.

10. To begin the file transfer, press Page Down to download files or

Page Up to upload files.

11. Verify that the file transfer has completed.

12. Send or receive at least three files changing modem types.

�����Ŀ

�  !  �  Verify that video corruption does not occur in all operations.

�������

DYNACOMM

��������

1.  Open an OS/2 full screen session to run DynaComm. If you do not have

DynaComm installed, please refer to your DynaComm Installation

Manual for instructions.

2.  Select serial port.

3.  Click on Settings, select Communication and Phone Number.

4.  Select for Baud Rate and click on OK.

5.  Click on Setting and select Binary Transfer.

6.  Select the type of transfer to be done.

7.  Click on Phone and select Dial.

8.  When the connection has been established perform login.

9.  Enter a "U" to upload a file or a "D" to download a file.

10. Enter modem protocol type. The most common protocols are Kermit, Xmodem

and Ymodem. Use these protocols along with a mixture of the others.

11. Enter the file name.

12. To begin the file transfer, click on Transfers. Select Send Binary

Files for uploads or Receive Binary Files for downloads.

13. Verify that the file transfer has completed.

14. From the OS/2 session of the receiving machine, verify that the

date/time stamp of the file just received is current and the size

of the file has not changed.

15. Send and receive at least three files changing modem types.

�����Ŀ

�  !  �  Verify that video corruption does not occur in all operations.

�������

____________________________________________________________________________

TRADEMARKS AND SERVICE MARKS

____________________________

The following terms, denoted by an asterisk (*)

are trademarks or registered trademarks of the

IBM Corporation in the United States or other

countries:

IBM

------------------------------------------

Presenation Manager

------------------------------------------

OS/2

------------------------------------------

WIN-OS/2

------------------------------------------

The following terms, denoted by a double

asterisk (**) in this README, are trademarks of

other companies as follows:

Trademark:            Owner:

--------------------------------------------------------------

DynaComm              Future Soft Engineering, Inc.

--------------------------------------------------------------

IBM DISCLAIMS ALL WARRANTIES, WHETHER EXPRESSED

OR IMPLIED, INCLUDING WITHOUT LIMITATION,

WARRANTIES OF FITNESS AND MERCHANTABILITY WITH

RESPECT TO THE INFORMATION IN THIS DOCUMENT.

BY FURNISHING THIS DOCUMENT, IBM GRANTS NO

LICENSES TO ANY RELATED PATENTS OR COPYRIGHTS.

Copyright IBM Corporation, 1994, all rights

reserved.