B

RADIO REMOTE CONTROL PROGRAM FOR OS/2

By Lydia Rosa

Copyright 1995,1996  by Lydia Rosa

RADIO REMOTE CONTROL PROGRAM FOR OS/2

CONTENTS

WHAT'S NEW IN THIS RELEASE ....................................... 1

DISCLAIMERS ...................................................... 3

INTRODUCTION ..................................................... 4

REQUIREMENTS ..................................................... 5

STARTUP .......................................................... 7

General Description .............................................. 7

THE DISPLAY ...................................................... 9

VFO Frequency.  .................................................. 9

Modulation.  ..................................................... 9

Channel #.  ...................................................... 9

VFO step value.  ................................................ 10

Priority Indicator .............................................. 10

Digit Scrolling ................................................. 11

Freq/Memory Scroll Buttons ...................................... 11

Signal Meter .................................................... 11

Attenuator ...................................................... 12

Scan Mode Buttons.  ............................................. 12

Manual Mode ................................................... 12

Scan Mode ..................................................... 12

Search Mode ................................................... 14

Load and Set Buttons ............................................ 15

Load ALL ...................................................... 15

Set  ALL ...................................................... 16

Mem->VFO ...................................................... 16

VFO->Mem ...................................................... 16

Operation Modification Buttons .................................. 17

COMPUTER/RADIO Button ......................................... 17

VFO Frequency Button .......................................... 17

Priority ...................................................... 18

Banks On ...................................................... 18

Resume ........................................................ 18

Ack ........................................................... 18

Logging ....................................................... 19

Radio Selection ............................................... 19

Speed Control ................................................. 19

MEMORY  CHANNEL DATA FIELDS ..................................... 19

CHANNEL number ................................................ 20

VALID (Vld) ................................................... 20

DELAY (Dly) ................................................... 20

ATTENUATION (Att) ............................................. 21

RESUME (Rsm) .................................................. 21

FREQUENCY ..................................................... 21

MODE .......................................................... 21

COUNT ......................................................... 22

TIME .......................................................... 22

SIGNAL ........................................................ 22

SERVICE ....................................................... 22

Contents  ii

RADIO REMOTE CONTROL PROGRAM FOR OS/2

TIME STAMP .................................................... 22

SCAN BANK  DATA FIELDS .......................................... 23

CHANNEL number ................................................ 23

Valid ......................................................... 23

Start ......................................................... 23

STOP .......................................................... 24

SERVICE ....................................................... 24

SEARCH BANK DATA FIELDS ......................................... 25

CHANNEL number ................................................ 25

VALID ......................................................... 25

Start-Freq .................................................... 26

Stop-Freq ..................................................... 26

Incr-Freq ..................................................... 26

Mode .......................................................... 26

Service ....................................................... 27

Status line ..................................................... 27

SPECTRUM DISPLAY ................................................ 27

The Menu ........................................................ 28

File menu ....................................................... 28

Database menu ................................................... 28

Common Parms panel .............................................. 29

Priority ...................................................... 29

Delay ........................................................ 29

Resume ....................................................... 29

Spectrum Background .......................................... 30

Tone Decode Enable ........................................... 30

Radio  Parms panel .............................................. 30

Radio Name .................................................... 30

Radio Type .................................................... 30

TXADR ......................................................... 31

Max Channel ................................................... 31

Origin .......................................................... 31

Max Frequency ................................................... 31

Min Frequency ................................................... 31

Baud Rate ....................................................... 32

Com Port ........................................................ 32

Data Bits ....................................................... 32

Stop Bits ....................................................... 32

Flow Control .................................................... 32

Signal Detect ................................................... 33

Send Delay ...................................................... 33

SPECIAL KEYS .................................................... 34

ENTER ......................................................... 34

ESC ........................................................... 34

tab keys ...................................................... 34

PgDn/PgUp ..................................................... 34

ctl-PgDn/ctl-PgUp ............................................. 34

alt-PgDn/ctl-PgUp ............................................. 35

DATA FILE FORMATS ............................................... 36

LOGFILE format ................................................ 36

FREQLIST.DBF .................................................. 36

data file format .............................................. 37

Contents  iii

RADIO REMOTE CONTROL PROGRAM FOR OS/2

RS232 INTERFACE ................................................. 46

PROGRAM LIMITS AND RANGES ....................................... 47

Contents  iv

RADIO REMOTE CONTROL PROGRAM FOR OS/2

WHAT'S NEW IN THIS RELEASE

Version 0.1

Beta Test Release

Version 0.2

Bug Fix release.

Fixed problem with entering SERVICE data  with signal present.

Fixed several hang problems.

Added VFO attenuation button

Added active HELP display

Added support for AOR8000 (untested)

Version 0.3

Fixed WARP problem with signal strength meter not showing.

Some radio parameters were not being saved in .INI file

Fixed various problems with the KENWOOD protocol.

Increased number of radios to 8.

Added Modem Status Reg (MSR) display.

Added scroll bars for FREQUENCY and MEMORY CHANNEL

Added frequency display sync selection

Fixed spectrum display problems.

Version 0.4

Fixed problem with certain errors not displaying.

Fixed problem with DBASE selection.

Added leading blank removal on Service string load.

Added save/recall of strength values.

enhanced error handling of file read.

added CHAIN command (in TEXT files) for file chaining.

added support for certain commands in TEXT files.

Added support for AOR2500 (untested)

Version 0.5

Added resume for individual memory channels.

Added time stamp for memory channel log.

Modified spectrum display routines.

Fixed sort bug that left displayed channel unsorted.

Version 0.6

Fixed bug -service not being read on SCAN bank.

Fixed bug -SCAN bank info not being saved correctly.

Fixed bug -comments not being rewritten to file.

Fixed bug -KENWOOD modulation not being correctly read.

Fixed bug -Two copies of Radioctl could not run as same time.

bank number now shows current bank.

Added buffer for service info. DBASE file now closed after read.

Added pop-up window on exit for data save.

Added command line option for loading a file.

Version 0.7

Removed UP/DN buttons. Replaced with direct mouse clicks.

Rearranged screen to make room for longer SERVICE strings.

WHAT'S NEW IN THIS RELEASE  1

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Added file logging.

Fixed problem that kept program from working with normal speed UART.

Added support for OptoScan protocol.

Added hardware signal detect for AOR radios.

Added additional PL parms in save file (for future enhancement)

Fixed a few AOR8000 bugs.

Version 0.8

added several pop-up status messages.

Fixed a few file read and write bugs.

Fixed search bug: existing frequency not found in database when

blank channels were present.

Fixed a problem that caused corrupted SERVICE data

when DBASE file had no service information on frequency record.

Version 0.9

Added VFO selection for KENWOOD radios.

Fixed a problem with some KENWOOD radios not being able to SET

channels.

Added support for KENWOOD radios that have signal strength logging.

Fixed memory channel scroll bar so that radio channel changes

when MEMORY tracking mode is selected.

WHAT'S NEW IN THIS RELEASE  2

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DISCLAIMERS

IBM is a trademark of IBM corporation.

ICOM is a trademark of Icom incorporated.

KENWOOD is a trademark of Kenwood Corporation.

AOR     is a trademark of AOR inc.

DBASE III is a trademark of Borland International.

RADIO SHACK is a trademark of Tandy Corporation.

The author  takes no  responsibility for  any damages that  may

occur  to your radio or  computer due to  the operation  of  this

program.

This program is being distributed as SHAREWARE. This means that

voluntary  contributions  are  the  only  source of  remuneration

obtained for this work. A contribution of $25.00  gives  you  the

following:

1.  Fixes for any bugs.

2.  Schematics and instructions for the interfaces.

3.  A clear conscience.

Please mail  all contributions, problem reports  and  suggestions

to

Lydia Rosa

P.O. Box 414

Marlboro, N.Y.

12542

The author (and  the  latest versions of  RADIOCTL) can also be

contacted on the following Bulletin Board System:

Rosa's Folly BBS

(914) 236-7607

300/1200/2400/9600/14400 baud

Thank you for your support.

DISCLAIMERS  3

RADIO REMOTE CONTROL PROGRAM FOR OS/2

INTRODUCTION

The  RADIO REMOTE CONTROL PROGRAM for  OS/2  will allow  you to

control selected radios from  any  computer  serial  port (RS232)

using  OS/2.  This program expands the capabilities of your radio

to 10000 channels and  100 programmable scan or search banks.  In

addition, scanning, searching, and logging  facilities  are  also

provided.  Some radios  will  need  an  additional  interface  to

utilize  the  signal  detection  facilities  of  this program.  A

circuit  diagram  and  build instructions  are  provided free  of

charge to all registered users.

The program allows you to select various baud rates, COM ports,

and other parameters  associated with individual  radios.  Up  to

eight different radio configurations  can  be  defined  for  easy

switching.  A  LOCAL mode is also  provided  to  run  the program

without the need for a radio connection.

Frequency information can be loaded  or saved from a text file.

An interface to a DBASE type  file is also provided for frequency

to service lookup. Most setup information is saved in a .INI file

for ease in restarting the program.

INTRODUCTION  4

RADIO REMOTE CONTROL PROGRAM FOR OS/2

REQUIREMENTS

In order to run this program you will need the following:

*    A 386,486 or Pentium based computer.

*    At least 4mb of memory (8mb is recommended).

*    about 1mb of free disk space (for the program and document).

*    OS/2 2.11, WARP, or higher level of OS/2.

*    one of the following models of radio:

-   ICOM R-7000*

-   KENWOOD R-5000*

-   AOR3000a

-   AOR8000

-   AOR2500

-   RadioShack  radio equipped with OPTOSCAN  interface (from

OptoElectronics).

*NOTE:Although not  tested,  this program  should work with other

models of this manufacturer equipped with a serial port.

For Kenwood and Icom Radios, a modified interface  is  required

for  full program functionally  (signal detection). For all other

radios, the  manufacturer's interface will work.  However, signal

detection can be speeded  up by use of the modified interface for

all of the radios. Plans for  this interface are  made  available

free of charge for all registered users.

It is also recommended  that the COM  driver included with OS/2

be replaced with  the  SIO driver by  Ray Guinn.  This driver has

proved to be more reliable than the default  COM.SYS driver. This

driver  package is  available  in shareware, and can be found  on

most bulletin boards.

If you are controlling the R7000, you will need either the ICOM

CI-V interface adapter, or the  modified RS232 interface.  If you

wish to  use  the signal  detection feature  of this program, you

will need to use the modified interface.

For  the  R-5000,  you  will  need  the  IC-10  option  kit.  A

modification must  be made to the R-5000 if you  wish to  use the

signal detection feature of this program.

REQUIREMENTS  5

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The AOR3000, AOR2500, AOR8000 and OPTO equipped radios  already

have  a RS232  interface. No  modification  is  needed  for these

radios, as signal detection is done in software.  However, use of

the modified signal detection will allow an increase in the speed

of  scanning.  In addition, the modification instructions include

a  change to the AOR3000  to allow 9600  baud operation (from the

default 4800 baud) .

It is  highly recommended that  the  modified interface be used

for  ICOM  and  KENWOOD radios.  Most of  the  features  of  this

program  make  use  of  the  detection  of  an  available  signal

(SQUELCH).  These radios do NOT  currently have squelch detection

as part of the standard interface.  Although  this  program  will

work fine without this  signal detection, operations such as SCAN

and SEARCH will have no usability.

The interface circuit is  a inexpensive, easy to build project.

All the  parts are readily  available  at  any  electronic store.

Estimated cost for each interface is $15. Plans  and instructions

for  these interfaces  are  provided to  you  when  you  become a

registered user.

REQUIREMENTS  6

RADIO REMOTE CONTROL PROGRAM FOR OS/2

STARTUP

It is  recommended  (but  not  required)  that  this program be

installed  in  it's  own  directory.  You  should  also  add  the

directory to your PATH  and DPATH  statements  in CONFIG.SYS. The

FREQLIST.DBF  file  should be placed in the same  subdirectory as

the  RADIOCTL.EXE file. Start the program by entering RADIOCTL on

the command line.  If you wish, you can  also  specify  a file to

load on the  command  line. This  file MUST be a  RADIOCTL format

file  (see  APPENDIX  for  the format of  this  file).  Once  the

program loads, the main control  panel will be displayed. You can

maximize  the window to see all  of the display, or you can  size

the window as you like.

GENERAL DESCRIPTION

RADIOCTL provides the user with a flexible way of controlling a

radio. It  also provides a uniform interface for  different types

of radios. This is a high-level block diagram of the program:

��������Ŀ

|search  |---�

|banks   |   |

����������   |

|

|

|      ������Ŀ

��������Ŀ   �----->|      |   serial  ��������Ŀ

|memory  |<---------|VFO   |---------->|  radio |

|channels|--------->|      |<----------|        |

����������          ��������  �------->|        |

^  ^                      | port   ����������

|  |                      |

|  |    ���������Ŀ       |

|  �----|channel #|-------�

|       �����������

������Ŀ

|scan  |

|banks |

��������

Memory channels  store  information  on frequency,  modulation,

squelch  count,  duration, and  other  important  parameters. The

program  provides  for  a  maximum   of  10000  memory  channels.

Information  can  be  directly entered  from  the  control panel,

loaded  from a  text file, loaded  from  the  radio's  memory, or

transferred from the program's VFO.

The  VFO represents  a  single  frequency and  modulation. This

setting can be sent to the radio, or  stored in a memory channel.

STARTUP  7

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The  VFO can  be set from the control panel direct input,  loaded

from a memory channel, loaded from a search bank, or set from the

radio.

Scan banks provide the ability to group blocks of channels  for

scanning.  Scan  banks  allow  the  user  to  define  up  to  100

different blocks of memory channels for selection.

Search banks provide groups of frequencies to search. Up to 100

different ranges of  frequency, modulation and  step value can be

defined.

The Channel # provides selection of the memory channels. It can

also be used to address the memory channels of the current radio.

Radio  control information is  sent  to and  received  from the

radio  by  way  of the  serial  port. The  program  provides  for

selection of up to eight different radios, each with a unique set

of  operation  characteristics.  The  program  can  also  provide

simulation of  a  radio.  The  actual  commands  to the radio are

defined  by a  protocol  setting.  The  program  current supports

protocols for KENWOOD,  ICOM, AOR8000, AOR3000, AOR2500  and OPTO

equipped radios. A LOCAL protocol is also provided which does not

require a radio to be attached.

STARTUP  8

RADIO REMOTE CONTROL PROGRAM FOR OS/2

THE DISPLAY

The control panel shows you  the status  of your radio, as well

as the status  of the program.  There  are numerous controls, and

various status fields. Controls are toggled by moving  the  mouse

cursor  over the  control and  clicking.  Entry  fields require a

ENTER or TAB key to complete the entry.

VFO FREQUENCY.

This display  shows the current frequency of the program.  This

frequency  may or may  not reflect the VFO  state of the attached

radio.  Any frequency information sent  to the radio is displayed

in this field.  The value of this  field can be changed using the

scroll buttons to the left of the display  or by using the  mouse

buttons when the mouse pointer is over the  digit  displayed. The

left mouse  button  will increase  a  digit, and the right button

will decrease.  A frequency  value  can  also be  entered  in the

entry  field under the  display.  If  a value is entered, changed

with mouse buttons,  or scrolled with the VFO scroll bar,  it  is

also sent to the radio.

MODULATION.

This  is a combination display and selection box.  It shows the

current  modulation used by  the  program. This may  or  may  not

reflect  the state of  the  radio.  A  modulation can  be  set by

pressing the  drop-down  control  on  the  right of  the box, and

double-clicking on  the  desired modulation. When  selected,  the

radio's modulation will be set.

CHANNEL #.

This display shows the current channel number being used by the

program. Depending on the state of the COMPUTER/RADIO  switch, it

will represent the  internal memory channels  in the program,  or

the radio's memory channels. The memory channel  can  be  changed

using the scroll buttons to the left  of the display,  using left

(increment) or right (decrement)  mouse  buttons, or  entering  a

channel number  in  the  entry field under  the display.  When  a

memory channel number is changed,  and the  COMPUTER/RADIO switch

is set to  COMPUTER,  the corresponding  computer memory  channel

information  is  set  to  the  VFO,  and  to  the  radio. If  the

THE DISPLAY  9

RADIO REMOTE CONTROL PROGRAM FOR OS/2

COMPUTER/RADIO  switch  is  in the  RADIO  position, the  radio's

memory channel is changed.  If the program is in MANUAL mode, and

the MEM SYNC switch is selected, changing the channel number will

display the corresponding Memory Channel information.

NOTE:The AOR8000 is organized into 20 banks of 50 channels.  Each

bank is identified as a lower or upper case letter. RADIOCTL maps

these channels as follows:

��������������������������������������������������������Ŀ

� radio   �   computer       �    radio    �   computer  �

��������������������������������������������������������Ĵ

�A00-A49  � 000 - 049        �    a00-a49  � 050 - 099   �

��������������������������������������������������������Ĵ

�B00-B49  � 100 - 149        �    b00-b49  � 150 - 199   �

��������������������������������������������������������Ĵ

�C00-C49  � 200 - 249        �    c00-c49  � 250 - 299   �

��������������������������������������������������������Ĵ

�D00-D49  � 300 - 349        �    d00-d49  � 350 - 399   �

��������������������������������������������������������Ĵ

�E00-E49  � 400 - 449        �    e00-e49  � 450 - 499   �

��������������������������������������������������������Ĵ

�F00-F49  � 500 - 549        �    f00-f49  � 550 - 599   �

��������������������������������������������������������Ĵ

�G00-G49  � 600 - 649        �    g00-g49  � 650 - 699   �

��������������������������������������������������������Ĵ

�H00-H49  � 700 - 749        �    h00-h49  � 750 - 799   �

��������������������������������������������������������Ĵ

�I00-I49  � 800 - 849        �    i00-i49  � 850 - 899   �

��������������������������������������������������������Ĵ

�J00-J49  � 900 - 849        �    j00-j49  � 950 - 999   �

����������������������������������������������������������

VFO STEP VALUE.

This display  shows  the  current step value that is  used for

Search mode. This value will be added to or subtracted  from  the

VFO frequency to obtain the next VFO frequency. This value can be

set  using  the  left  (increment)  or  right  (decrement)  mouse

buttons, or direct entry in the field under the display. During a

bank search, this field will be loaded from the selected bank.

PRIORITY INDICATOR

When  the  unit  is  scanning,  and  the  PRIORITY  button  is

selected, a specific channel is  defined as the priority channel.

When  this channel is  being scanned,  the Priority Indicator  is

lit. This is a red "P" situated next to the Channel # display.

THE DISPLAY  10

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DIGIT SCROLLING

The VFO Step  Value, VFO frequency, and Channel #  displays can

be changed  using the left and  right  mouse buttons to increment

and decrement the digit. The mouse pointer  selects the  digit to

change, and the left button  will increment, and the right button

will decrement the digit.  This  allows  quick changing of any of

the information in the display.

FREQ/MEMORY SCROLL BUTTONS

Both frequency  and memory channel number can be incremented or

decremented  using the  scroll  buttons  next  the  the  affected

display.  For the  Frequency,  the  increment  or decrement value

will be  the  current  VFO  STEP  setting. For the memory channel

number, increment or decrement value is always 1.

SIGNAL METER

The signal is a 12 step indicator corresponding to  the  signal

strength being detected. For ICOM and KENWOOD  radios, the signal

indicated will  always  be 9. This is  due  to the  fact that the

signal detection is a  binary (On or  OFF)  line,  with no way to

read  the  actual  strength. For  all  other  radios,  the actual

strength detected by the radio will  be  displayed. For  a  LOCAL

radio, clicking any  of the  signal  display buttons will turn on

the signal at that strength. For all radios, the OFF  button will

temporarily turn off the signal. If scanning  or searching,  this

will  cause  the  radio  to  resume scan  or  search.  For  LOCAL

operation, the OFF button is the only way to turn  off the signal

detection.

When not running in LOCAL mode, the signal  buttons can be used

to define  a  minimum squelch trigger. Clicking on a button  will

cause the  program to  ignore any signal at or below that  signal

level. When the squelch minimum is active, the signal limit level

buttons are displayed in green.

NOTE:This feature is not useful for the ICOM and KENWOOD radios,

as  these radios do not  have  the  ability  to determine  signal

strength values.

THE DISPLAY  11

RADIO REMOTE CONTROL PROGRAM FOR OS/2

ATTENUATOR

The AOR3000 and AOR8000 have a built in attenuator. This can be

switched on to reduce overload  from  a  strong  signal. Pressing

this button will toggle the current  attenuator  setting  in  the

radio. When  in  SCAN  mode,  this  button  will  be  set by  the

currently scanning memory channel's attenuator setting.

SCAN MODE BUTTONS.

These buttons select the scanning mode. The program runs in one

of three scanning modes. MANUAL, SCAN  and SEARCH modes determine

the primary operation of the program. MANUAL mode is the default.

Manual Mode

Of three scanning modes.  MANUAL mode  is the default. In  this

mode, the  program will continuously monitor the signal  strength

from the radio. For the  ICOM, changes in  the radio frequency or

modulation will be logged in the VFO displays.

The program will switch to manual mode if one  of the following

conditions is met:

*   The MANUAL button is pressed.

*   SCAN mode detects no valid channels or banks.

*   SEARCH mode detects no valid  banks

Scan Mode

The SCAN  button  starts  the program scanning through selected

memory  channels. Memory channels that are flagged with the VALID

field will be scanned. Channels with the VALID  flag  off will be

bypassed.  When a signal is detected,  scanning  will  stop,  the

signal  strength  will be  displayed in  the  Signal  Meter,  and

information (duration, squelch  count, etc) will be logged in the

memory channel.  If  the MEM SYNC switch  is selected, the Memory

Channel display will change  to the Memory Channel number that is

being logged.  When  the  signal  stops, scanning  will continue.

Memory channels  are normally incremented.  The direction of  the

scan can be changed by pressing the CHANNEL  NUMBER scroll button

THE DISPLAY  12

RADIO REMOTE CONTROL PROGRAM FOR OS/2

to the side of the channel display.

Several parameters  and  memory  channel  settings  affect  the

operation of SCAN. Each channel has  a VALID flag associated with

it. If  this  flag is set off,  the  corresponding memory channel

will NOT be scanned.  If  the  DELAY  field of  the corresponding

memory  channel is set,  the program will delay for  a number  of

seconds after the  signal  stops before resuming  scan. The DELAY

value can be set from the PARMS panel.

The SPEED CONTROL setting determines how fast each channel will

be scanned. In  the event that  a scan is  too fast to respond to

certain frequencies,  it can  be slowed by  adjusting  the  SPEED

CONTROL.  Setting the SPEED CONTROL  to  a lower value causes the

computer to wait on each channel when there is no signal present.

This gives the radio time for the squelch to detect a signal.

The  RESUME  switch  determines  the  action  after the squelch

breaks. If RESUME  is  off, scanning will not continue  until the

signal  ends. If RESUME is  on, scanning will  resume  after  the

number  of seconds specified by the  RESUME value. This value can

be set from the PARMS panel.

NOTE:The RESUME  switch  overrides  any  RESUME  setting for  an

individual memory channels.

If  a  channel is entered  in the  PRIORITY channel, it will be

scanned every  interval specified by  the  priority value if  the

PRIORITY switch is set.  If a signal is detected  on the priority

channel, normal scanning stops,  and the program switches  to the

priority channel. Normal scanning will resume when the  signal is

no  longer  present on the PRIORITY  channel.  The  interval  for

priority checking is set on the PARMS panel.

Note:The RESUME  counter does  not  function when stopped on the

priority  channel. Scan or  Search will resume  from  a  priority

channel ONLY when the signal disappears.

If BANKS  ON button is  on,  the order  of channel scan will be

determined by  the START and END of the current bank. The current

bank  is scanned from 0 to  99.  When  BANKS ON is selected, each

valid bank will determine  starting and ending channels for scan.

Channel numbers will be  incremented  for a selected bank  if the

START  channel  number is less  than  the  STOP  channel  number.

Otherwise, channel numbers will be  decremented.  Failure to find

a valid bank,  or  a valid channel  for  the specified banks will

force MANUAL mode.

The MEM SYNC button allows the Memory Channel display to follow

active channels.  If  this button is selected  for SCAN or SEARCH

mode, the displayed Memory Channel information will change to any

channel  where  a  signal  is  detected.   This  allows  you   to

automatically  display   the  channel  information  whenever  the

squelch breaks.  If this button is off, then the  memory  channel

display will NOT change when a signal is detected.

The  COMPUTER/RADIO  switch  determines   the   source  of  the

frequency  information  for the  radio. If COMPUTER is  selected,

frequency and modulation  information  is sent to the  radio from

THE DISPLAY  13

RADIO REMOTE CONTROL PROGRAM FOR OS/2

the computer's memory channels. If RADIO is set, then the program

will change the  radio's  memory  channel  to  correspond  to the

channel being  sent.  This  allows  use  of  the  radio's  memory

instead of the information in the computer. If RADIO is selected,

only the channel numbers supported by the  selected radio will be

allowed. The  maximum  channel  number can be  changed  using the

PARMS panel.

NOTE.The  OPTO  protocol  does  not provide  for  memory channel

control. When using the OPTO protocol, this switch remains in the

COMPUTER position, and cannot be changed.

Search Mode

Search  Mode  allows  you  to scan a range  of  frequencies for

action.  Up to 100 different ranges can be programmed. Each range

consists of  a  starting frequency, stopping frequency, increment

value and  modulation.  In  addition, searching can  start at the

current VFO frequency and be incremented or  decremented by a set

amount.  For  non-bank  processing,  the  program  will  normally

increment the frequency.  Pressing  VFO STEP  UP/DN  buttons will

change the search direction.

When a  range is searched,  the  radio  is  set to the  initial

frequency and modulation. If a signal is  detected, the frequency

and  and modulation are recorded in the next free log record, and

the VALID flag is  set  for  this  record.  If this frequency  is

already  recorded in the log, then COUNT, TIME  and SIGNAL values

are updated. If the log entry has  the VALID flag off, the signal

is ignored.  If there are no more free  channels  (frequency = 0)

then the message  "SEARCH FULL" is displayed, and the information

is NOT saved.

Several  parameters and  memory  channel  settings  affect  the

operation  of  SEARCH. Each channel has  a VALID flag  associated

with it.  If this  flag  is set off, the  corresponding frequency

will NOT respond to a squelch break.  If the DELAY field of the a

memory channel is set, and the corresponding frequency breaks the

squelch, the program will delay for a number of seconds after the

signal stops before resuming  scan. The DELAY  value  can  be set

from the PARMS panel.

The  SPEED  CONTROL setting  determines how fast each frequency

will be monitored.  In  the  event that a search  is  too fast to

respond to certain frequencies, it can be slowed by adjusting the

SPEED CONTROL.  Setting the SPEED CONTROL to a lower value causes

the computer to wait  on each frequency when there is  no  signal

present. This gives  the radio time for  the squelch to detect  a

signal.

The RESUME switch  determines  the  action  after  the  squelch

breaks.  If RESUME  is off, searching will not continue until the

signal ends. If RESUME is on,  scanning  will  resume  after  the

number of seconds specified by the RESUME value.  This  value can

THE DISPLAY  14

RADIO REMOTE CONTROL PROGRAM FOR OS/2

be set from the PARMS panel.

NOTE:The  RESUME switch overrides  any  RESUME  setting  for  an

individual memory channels.

If  BANKS  ON button is on, the order of frequency scan will be

determined by the START and END of the current bank.  When  BANKS

ON  is  selected, each  valid  bank will determine  starting  and

ending  frequency  ,  increment  and  modulation.  If  the  START

frequency  is  less  than the STOP frequency, then the  frequency

will   be  incremented.   Otherwise,   the   frequency  will   be

decremented.  Failure  to  find  a  valid bank will  force MANUAL

mode.

The MEM SYNC button allows the Memory Channel display to follow

active channels. If this button is selected, the displayed Memory

Channel information will  change to any channel where a signal is

detected.  This allows  you  to automatically display the channel

information whenever the squelch  breaks.  If this button is off,

then the memory channel display will NOT change when a signal  is

detected.

The PRIORITY  button does NOT  have  any effect on Search Mode.

Priority  is not supported during a  search operation.  Likewise,

the COMPUTER/RADIO switch  is  also  ignored. Search  Mode always

uses the computer's VFO.

LOAD AND SET BUTTONS

Four  buttons provide load and set operations. These operations

transfer information  between  the  VFO,  memory channels and the

computer. When one of these buttons is pressed, the current  scan

mode  is  suspended,  and the operation is started.  Pressing the

button  a  second  time  before  the operation is  complete  will

terminate  the  operation and  resume  the suspended  scan  mode.

Selecting the MANUAL scan button will also terminate  any load or

set operation.

Load ALL

When pressed,  all  of the radio's memory  channels are  loaded

into the computer's memory channels. The load starts at the first

channel  in  the radio  (0  or 1)  and continues  to  the maximum

channel.  The maximum channel  can be set on the PARMS panel. The

contents of computer memory channels are overwritten with the new

information.  Any  memory  channel with  a non-zero  frequency is

marked as  valid.  The count,  duration and strength  fields  are

reset  to 0.  The service  string is also  cleared.  For a  LOCAL

radio, a  set of random frequency and modulations is set for each

channel.

THE DISPLAY  15

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Note: The OPTO  protocol  does  not  provide  for memory  channel

control. When using the OPTO protocol, this switch is disabled.

Set  ALL

When  pressed, the contents of the computer memory channels are

stored  into the radio's memory  channels.  The set starts at the

first channel in the radio (0  or 1) and continues to the maximum

channel. The maximum  channel can be set  on the PARMS panel. The

contents of  radio  memory  channels are overwritten with the new

information. For ICOM and KENWOOD  radios, a zero  frequency will

cause the memory channel to be cleared. For AOR3000 radios,  zero

frequency channels will NOT be  set.  For LOCAL mode, each memory

channel will be transferred to the VFO in turn.

NOTE.The  OPTO  protocol does not  provide  for  memory  channel

control. When using the OPTO protocol, this switch is disabled.

Mem->VFO

This button will transfer the contents  of  the Memory  Channel

indicated  by  the  MEMORY  #   display,  to  the   VFO.  If  the

COMPUTER/RADIO switch is in the COMPUTER position, the Computer's

memory  channel  will  be  transferred.  If the RADIO position is

selected,  the  information will be transferred from the  radio's

memory. For a LOCAL radio, the input will be a  random  frequency

and modulation. This operation is the same as a Load All for  one

channel.

VFO->Mem

This button will transfer the contents of the VFO frequency and

modulation  to the Memory  Channel  indicated  by  the  MEMORY  #

display.  If  the  COMPUTER/RADIO  switch  is   in  the  COMPUTER

position, the Computer's memory channel will be  updated.  Count,

Strength, and  Duration fields are reset.  The Service string  is

also cleared.  If the RADIO position is selected, the information

will be transferred to the radio's memory. For a LOCAL radio,  no

data is transferred.  This  operation is  the same as  a Save All

for one channel.

THE DISPLAY  16

RADIO REMOTE CONTROL PROGRAM FOR OS/2

OPERATION MODIFICATION BUTTONS

Next to the  Channel #  display  is  a  column  of buttons that

affect various operations. These buttons determine what memory is

used, priority, resume, bank  selection,  acknowledgment  and the

radio being used. Each of these buttons is a  toggle switch. Move

the mouse cursor over the button and  right click to toggle on or

off.  The  Priority  switch  has  an entry field  next to it. The

others have a status field.

COMPUTER/RADIO Button

For various Scan modes  and  operations, the program  needs  to

determine  who's  memory is  the  source  or  destination  of the

operation. When  in  the COMPUTER position, the  program is using

the  internal Memory  Channels for  the operation.  If the  RADIO

position  is  selected,  the  operation  will  affect the  memory

channels in the selected radio.

Note:  The  OPTO  protocol does not provide  for  memory  channel

control. When using the OPTO protocol, this switch is disabled.

VFO Frequency Button

Above the Frequency display is a button used  for VFO selection

on  KENWOOD  radios. These radios have  two selectable  frequency

memories (VFO). This button determines which VFO will be selected

when setting the frequency on the radio.

The MEM SYNC button allows the Memory Channel display to follow

active  channels. If this  button is selected  for SCAN or SEARCH

mode, the displayed Memory Channel information will change to any

channel  where  a  signal  is  detected.  This   allows   you  to

automatically  display  the  channel  information  whenever   the

squelch breaks.  If  the  program  is  in  MANUAL  mode, then the

memory  channel display  will change  in  sync  with  the  memory

channel number.

THE DISPLAY  17

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Priority

Priority scanning allows a selected channel to be tested for  a

signal  once  every interval. The channel number to select  as  a

Priority channel  is  entered  in  the  entry field  next  to the

Priority switch. If the Priority  switch is ON, then the priority

channel will be used.  The priority checking interval can  be set

using the PARMS panel. Priority only works during SCAN mode.

Banks On

When in SCAN or SEARCH mode, the program can  make use of banks

of scan or search information. These banks  allow the grouping of

channels or frequencies  for  the SCAN mode. The  Banks On switch

allows the scan mode  to  use the bank associated  with the  SCAN

mode. When  selected,  the  indicator  next  to  the switch  will

display what bank number (00-99) is currently being used.

Resume

The  Resume  switch  will allow the Scan or Search  to continue

when a  signal  is present. This  is useful when a Scan or Search

encounters a  constant  signal.  With  Resume  on,  the signal is

ignored after  a  set amount of time. This time  interval  is set

using the Parms  panel. The  indicator next to the  Resume switch

shows  approximately  how   many  seconds  are  left   until  the

SCAN/SEARCH resumes.

Note:This switch  is a global setting. If on, the RESUME  switch

for each memory channel is ignored.

Ack

Ack sets radio acknowledgement  on.  When  a command is sent to

the  radio, the program will normally wait for acknowledgement of

that  command  to  come from the radio.  In the  event  that  the

command is lost or garbled, the program will  resend the command.

Ack forces  the  program  to  wait until this  acknowledgement is

received  before continuing  to the next channel  during  SCAN or

SEARCH mode. If Ack  is OFF, then the  program  will not wait for

acknowledgment.  This will speed up  some Scan/Search operations,

but may cause incorrect data  to  be  exchanged between the radio

and program.

NOTE:  If Ack is ON, the radio will retry a  command up to three

times if acknowledgement is  set on. After the third failure, the

THE DISPLAY  18

RADIO REMOTE CONTROL PROGRAM FOR OS/2

program will continue to the next command.

Logging

When this switch is selected, all actions are logged to  a data

file. This includes mode changes and squelch breaks. A time stamp

is  also logged with each event. This  file is cumulative, and is

not erased by this  program. The name of the logging file  can be

changed with the FILE menu selection.

Radio Selection

Up to eight  different radios can be defined  to  the  program.

These radios can have different BAUD rates, Com ports, protocols,

etc. Each radio  is assigned a name. You can change the name (and

other  parameters) for a given radio  from the Parms panel. Radio

Selection  is  a  drop-down selection  field.  Click on the right

button  to  display the radio  list.  Double click  on a radio to

select it.  The Baud and Com port used will be displayed  next to

the Radio Selection control.  NOTE: When a new radio is selected,

Manual mode is forced.

Speed Control

For SCAN or SEARCH mode, the speed control determines the time

between each channel or frequency.  When the  control  is all the

way to the left, the time between each setting is minimal. If the

slider  is moved  to  the right,  the  program  will  delay for a

certain time  before changing the  setting on  the radio.  During

this time, the program will  monitor the radio  for a signal. The

SPEED control  allows  you  to reduce the SCAN or SEARCH speed to

give the radio more time to respond to a signal.

MEMORY  CHANNEL DATA FIELDS

Program  Memory channels  contain  information for  a  specific

radio  setting.  Each channel contains  information on Frequency,

Modulation and attenuation  to be sent  to (or received from) the

radio.  In  addition,  information on signal  strength, number of

squelch breaks and the frequency assignment are also displayed.

THE DISPLAY  19

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Each  program  channel displayed has several  fields associated

with  it. Some of these fields  are entry fields that information

can be  entered.  Other  fields are toggle switches  that can  be

turned  on  and  off.  Note  that entry field  information is NOT

accepted  until ENTER  is pressed,  a TAB key is  pressed,  a new

entry field is selected with the mouse, or the channel display is

changed with the scroll bars.

CHANNEL number

This is the first field  on  the  display.  It determines which

memory  channel  is  being  displayed  or  changed.   The  memory

channel can be changed  by direct entry in the channel  field, or

using  the  scroll bars to the left of the  channel  number.  The

PgUp and PgDn keys will  also scroll the  memory  channel  up and

down.  If entering the channel number, keep in mind that the data

will NOT be entered unless you press ENTER or a TAB key.

VALID (Vld)

The VALID field determines whether or not SCAN and  SEARCH will

use the frequency currently  entered into that channel.  For SCAN

mode, the program will not scan the memory channel if VLD is OFF.

For the SEARCH operation, a frequency  entered in a memory record

that has the VALID flag off will NOT cause the search to stop and

log.

DELAY (Dly)

The DELAY  field is use during the  SCAN and SEARCH operations.

If delay is set ON,  scanning  will continue DELAY  seconds after

the  signal  ends on that  channel.  If  delay  is set OFF,  then

scanning  will  continue immediately after the  signal ends.  The

DELAY time value can be set on the PARMS panel.

THE DISPLAY  20

RADIO REMOTE CONTROL PROGRAM FOR OS/2

ATTENUATION (Att)

The  ATTENUATION  field  selects  the  attenuator of the radio.

This  is a circuit that reduces the signal  strength so that high

level signals do not overload  the radio. The AOR3000 and AOR8000

radios support  this  feature.  This  option  has  no  effect for

KENWOOD and ICOM radios.

RESUME (Rsm)

The RESUME field selects the RESUME function for the individual

memory channel.  If this switch is  on, and a  signal is detected

for this memory channel in SEARCH or  SCAN  mode,  SEARCH or SCAN

will continue after a set number  of second if the signal has not

gone away. This works the same as the global RESUME switch except

that it only affects the single channel.

FREQUENCY

The FREQUENCY field is used to record or set the current memory

channel  frequency.  Frequency information  is  entered  in  Mhz.

Note that INS and DEL keys do not work when editing the FREQUENCY

field.

MODE

The  MODE  field  represents the  modulation  being used for  a

specific  frequency.  This field is a drop down selection box. To

select a modulation, click on the button to the right of the box.

Double  click on  the  desired  modulation.  Note  that  not  all

modulations will be accepted by your particular radio.

THE DISPLAY  21

RADIO REMOTE CONTROL PROGRAM FOR OS/2

COUNT

This  field  displays the number of times a signal was detected

on  a specific channel.  This  field cannot be  edited.  However,

left-clicking on this field with the mouse cursor will  reset the

field to 0.

TIME

This field displays the average number of seconds that a signal

was detected on  a specific channel. This field cannot be edited.

However, left-clicking on this field  with  the mouse cursor will

reset the field to 0.

SIGNAL

The  signal  strength of the last recorded signal  is  reported

here. This field cannot  be  edited.  However,  left-clicking  on

this field with the mouse cursor will reset the field to 0.

SERVICE

This  field  is  used  to  record  the  service for  a specific

channel.  This  is a scratch  pad location to  record information

about  a specific  frequency.  If  the  squelch  breaks  on  this

channel during  SCAN or  SEARCH mode, the program will attempt to

lookup the service for the frequency entered. If the frequency is

found in the current frequency database, and the SERVICE field is

blank, then  it will be filled in  with the service  found in the

database.

TIME STAMP

When ever  a  squelch break occurs for  this  channel, the time

stamp  is updated with the time and date of the occurrence.  This

field  cannot be edited.  However,  left-clicking  on this  field

with the mouse cursor will reset the field to 0.

THE DISPLAY  22

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SCAN BANK  DATA FIELDS

Scan  Bank  entry  allows you  to  define banks of channels for

scanning.  RADIOCTL allows you to assign any range of channels to

various banks.  Up to 100 banks (0-99) can be assigned. Banks can

overlap, and can  be  turned on  or off selectively. In order for

SCAN mode to recognize banks, the Banks On button must be active.

The bank displayed has several fields associated with it.

CHANNEL number

This is the first  field  on  the display.  It determines which

scan bank  is  being  displayed  or  changed.  The range for this

channel number  is 0 to 99.  The Channel number can be changed by

direct entry in the channel  field, or using  the scroll bars  to

the left of the channel number.  The  shift-PgUp  and  shift-PgDn

keys  will  also  scroll  the  memory  channel  up and  down.  If

entering  the channel number, keep in mind that the data will NOT

be entered unless you press ENTER or a TAB key.

Valid

The VALID  field determines whether or  not SCAN  will use this

bank.  Scanning banks without the  VALID flag will be ignored  at

all times.

Start

START  identifies the first channel  to start scanning with for

the given bank. When this bank is active, START will be the first

channel scanned  if  the  channel's  VALID  flag  is set.  If the

channel's VALID flag  is  NOT set, then the  first channel  to be

scanned will be the next sequential channel with a VALID flag.

THE DISPLAY  23

RADIO REMOTE CONTROL PROGRAM FOR OS/2

STOP

STOP identifies  the  last  channel  to scan when the  bank  is

active.  After this  channel  is scanned,  scanning will continue

with the START channel of the next VALID scan bank.

If the STOP  channel is  set lower  than the START channel, the

SCAN  will  decrement channel  numbers. If  STOP is greater  than

START,  channel numbers will increment.  If STOP  and  START  are

equal, the  single  channel  number  in START  and  END  will  be

scanned.

SERVICE

This field is provided to enter comments about  a  given  bank.

It is not used or modified by any program function.

THE DISPLAY  24

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SEARCH BANK DATA FIELDS

The search  bank  panel allows you to define banks of  channels

for  searching.  RADIOCTL  allows  you  to  assign  any  range of

frequencies,  along  with  a  modulation   and  step  value   for

searching.  Up  to  100  search banks  (0-99) can be defined.  In

order  for SEARCH mode to  recognize banks,  the Banks  On button

must be active.  The bank displayed has several fields associated

with it.

When searching, the program will start  with  the  first search

channel marked as  valid, set the  radio VFO to  the frequency in

the START field and the modulation in the MODE  field, and  check

for  a  signal. If  no  signal  is  detected,  the  program  will

increment or  decrement the frequency by  the  amount  in the INC

field, and repeat  the process.  When the frequency  in  the STOP

field is reached, the program will select the next  valid  search

bank  and repeat the process. When all valid search channels have

been processed, the program returns to the first search bank. For

more details on the SEARCH  operation, see the description of the

SEARCH mode.

CHANNEL number

This  is the first field  on the display.  It determines  which

search bank  is  being  displayed or changed.  The range for this

channel number is 0 to 99.  The  Channel number can be changed by

direct entry in the  channel  field,  or using the scroll bars to

the left of the  channel number.  The alt-PgUp and alt-PgDn  keys

will also scroll the memory channel up and down.  If entering the

channel number, keep in  mind  that the data will NOT be accepted

unless you press ENTER or a TAB key.

VALID

The VALID  field determines whether or not SEARCH will use this

bank.  Search banks without the VALID flag will be ignored at all

times.

SEARCH BANK DATA FIELDS  25

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Start-Freq

Start-Freq  identifies  the  first  frequency to  set  for  the

search.  Please note that the program does NOT check the range of

this value. If it is  outside  the range of the  radio, the radio

may  ignore the setting.

Stop-Freq

Stop-freq  identifies the last frequency to set for the search.

The search bank ends when the vfo frequency reaches the Stop-freq

value.  If the Stop-freq value is less than the Start-freq value,

the frequency is decremented. If the  Stop-freq value is  greater

than the Start-freq value, the  frequency is incremented.  Please

note that the program does NOT check the range of this  value. If

it is outside  the range of  the radio, the radio may  ignore the

setting.

Incr-Freq

Inc-Freq  determines the steps for  the  given search bank. The

Inc-Freq value is added to (or subtracted from) the VFO frequency

for each  step.  If an Inc-Freq value of 0 is entered, the SEARCH

mode will automatically reset the VLD flag for that bank.

Mode

The MODE field represents  the modulation  being  used for  the

search  range. The radio will  be set to  this  modulation at the

beginning  of  the  search  range.  This  field  is  a  drop down

selection box. To select a modulation, click on the button to the

right of the box.  Double click on the desired  modulation.  Note

that not all modulations  will  be  accepted  by your  particular

radio.

SEARCH BANK DATA FIELDS  26

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Service

This field  is  provided to enter  comments about a given bank.

It is not used or modified by any program function.

STATUS LINE

The status line displays messages from the program. This may be

error messages, or normal operation messages.  The time  and date

are included with all messages as an indication of when the error

occurred.

SPECTRUM DISPLAY

The spectrum display shows a  graph  of  frequency  vs.  signal

strength.  An active channel will be shown as a red  line (if the

graph is  in  the range  of  the  active  frequency). Frequencies

representing past activity will be displayed in green.

In  the lower  left hand corner, the starting frequency will be

displayed. The ending frequency of the spectrum will be displayed

in  the  lower  right  hand  corner of the  display.  The  center

frequency  of the  spectrum  is changed  by moving the horizontal

scroll bar. The minimum start frequency is 0  mhz and the maximum

stop  frequency  is   9999.9mhz.   The  horz  step  increment  is

determined by the spectrum resolution.  The resolution is changed

by moving  the vertical  scroll bar. Resolution goes from  100khz

per pixel to 1khz per pixel in 1khz steps.

Tic marks are  displayed for reference. Tic marks are displayed

every 10 mhz when the resolution is above  10khz/pixel.  When the

resolution is 10khz or below, the tic marks  are  displayed every

1mhz.

SEARCH BANK DATA FIELDS  27

RADIO REMOTE CONTROL PROGRAM FOR OS/2

THE MENU

The menu on the top of the panel provides additional functions.

these functions provide the ability  to load and save information

to  disk,  customize  your radio setup, and manipulate the memory

channel information. Menu  items  can be  selected by placing the

mouse  cursor over the item and pressing  the right mouse button.

indicated  by an  underline under the  character  in the specific

menu  item.  For  the primary  menu,  the  ALT  key must  also be

pressed.  For  drop-down menus,  only  the key itself  is need to

select an item.

FILE MENU

The File menu provides the ability to  load or save the current

memory channel,  scan banks,  and search banks to  or from a text

file.  The file format  for  this file is  described  in the DATA

FORMAT  section.  The  scan information can be saved to or loaded

from many different files.  When  a  file is  loaded,  only those

memory channels, scan bank channels, or search channels that  are

included  in  the  file  are  changed. For  a  save, only  memory

channels with a frequency greater than zero , and scan and search

banks that  are marked as  valid  are  written.  If  the  program

detects a file format problem when loading, the type of error and

the line number causing the error will be displayed, and the load

will stop.

A  new  DBASE file  can also be selected from this panel.  This

file is used for service lookup operations.  The  format  of this

file is also discussed in the DATA FORMAT section.

The  name of the logging file is also set from this menu.  This

is the file that is written to when the LOGGING switch is on.

The File menu also  contains the Quit for the program. Function

key 3 will also terminate the program.

DATABASE MENU

The Database menu allows  you to manipulate various  aspects of

the memory channels. A sort of memory channel by frequency can be

done from  this panel. With a sorted  frequency  list,  the radio

being  controlled  requires less settling time for each frequency

change, allowing for faster signal detection.

The  Service  search will  update all Service  fields  for  any

memory channel with a non-zero frequency. The service information

The Menu  28

RADIO REMOTE CONTROL PROGRAM FOR OS/2

is obtained from the current DBASE file.

The  Clear option allows  you to clear all memory channels, all

service  fields  from all memory channels, or all count, time and

signal logs from all memory channels.

COMMON PARMS PANEL

Various parameters determine the operation of the program.  For

the most  part,  these  parameters  are set once  when  you first

install the program. The first page of the  Parms Notebook allows

you to set program operation parameters.

Priority

This common panel  parm  sets  the  time  (in seconds)  between

priority checks.

Delay

The  number of seconds that the program will wait on a  channel

after a  signal disappears. This  value only  has  affect if  the

DELAY flag is set on the current memory channel.

Resume

If RESUME is selected, this is the number  of seconds that SCAN

or SEARCH will continue after a signal is detected.

The Menu  29

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Spectrum Background

This is the color of  the  SPECTRUM  DISPLAY window background.

The default is BLACK. However, it can be changed to suit personal

preference.

Tone Decode Enable

This selection is for a future  enhancement.  It  is  currently

disabled, and cannot be selected.

RADIO  PARMS PANEL

The program  provides  settings  for  up to eight radios.  Each

radio can be setup with different operation characteristics.  For

the  most part,  these  parameters are set once  when  you  first

install the  program.  However,  you  may  need to  update  these

parameters if you connect different radios.  Except for Baud rate

and Com port, the defaults for the particular radio types  should

only be changed if a non-standard radio is being supported.

Radio Name

This  is  the  name  that you  give the  radio.  There  are  no

restrictions  on the  name,  other  than  is  is  limited  to  30

alpha-numeric  characters.  This  name  is  used   in  the  Radio

Selection control on the main panel.

Radio Type

This  field  defines  the type of radio  (protocol).  There are

currently  seven  types  of  radios   supported.  ICOM,  KENWOOD,

AOR3000, AOR8000, AOR2500, OPTO and LOCAL.

The Menu  30

RADIO REMOTE CONTROL PROGRAM FOR OS/2

TXADR

ICOM radios  require an address. For the R-7000, the address is

normally  8.  If you connect a different ICOM radio, or you  have

more than one ICOM radio connected,  you can change this  address

to match. This parameter has no affect for other radio types.

Max Channel

This  field  defines  the maximum memory channel in the  radio.

For the ICOM R-7000, and KENWOOD R-5000, this  would  normally be

99. For the AOR3000 this value should be 399. Changing this field

allows the program to support other  radios with  more  (or less)

memory.  This parm has  no  effect for the OPTO protocol, as  the

OPTO protocol does not support memory channels.

ORIGIN

Origin is the first memory  channel.  ICOM  radios  start  with

memory  channel  1. The AOR3000 and  KENWOOD  start  with  memory

channel 0.  This parm has no effect for the OPTO protocol, as the

OPTO protocol does not support memory channels.

MAX FREQUENCY

This is the maximum frequency that the radio will accept.

MIN FREQUENCY

This is the minimum frequency that the radio will accept.

The Menu  31

RADIO REMOTE CONTROL PROGRAM FOR OS/2

BAUD RATE

The baud setting for this  radio. Most radios can be configured

to accept baud rates from 1200 to 9600  baud. The default setting

for ICOM and  AOR3000 is 9600.  For the  KENWOOD, the  default is

4800 baud.  Adjust this setting to  match the baud rate that your

radio is set for.

COM PORT

This  is  the com port  that  your  radio is  attached  to. The

program allows you to select  com  1 thru com 4 or LOCAL. A LOCAL

selection will bypass all serial port output.

DATA BITS

Most of the radios supported work with an 8 bit data format. If

you have a radio that requires 7 bits, this can be changed.

STOP BITS

Most of the  radios supported use  2 stop bits. If needed,  you

can change this to 1 stop bit.

FLOW CONTROL

Flow   Control   provides  the  ability  to   stop   and  start

communications based on the  ability of the radio and computer to

accept data.  Flow control requires the use of additional control

lines (RTS/DTS).  These lines are  used as signals to indicate to

the radio and the computer to start and stop sending information.

The  ICOM  interface  that this program  is designed to work with

does NOT  provide  for RTS/CTS control lines (flow  control). The

KENWOOD and AOR3000 interfaces  will use RTS/CTS flow control. It

is recommended  that flow control be left on for these radios, as

they may not work properly with it off.

The Menu  32

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SIGNAL DETECT

There are three ways to detect  a  squelch break.  The first is

using a spare signal on the RS-232 port. This signal detection is

part  of  the modified  interface, and  also provided  with  OPTO

equipped radios. This is referred to as HARDWARE detection.

The  second detection method involves  using a command  to  the

radio  to detect squelch  break. This is referred to as  SOFTWARE

detection.  This  method  also uses a command  to  the  radio  to

determine   the  signal  strength.  SOFTWARE  detection  is   NOT

available for ICOM or KENWOOD radios.

The  third method  is a combination of  HARDWARE  and SOFTWARE.

With  this  technique, the signal  line  is  checked  for squelch

break,  and  a  signal  strength  command  is issued ONLY  if the

squelch  break signal has  been detected. This method is the most

efficient for scanning speed. However, it cannot be used for ICOM

and  KENWOOD  radios, as  there is  no  signal  strength commands

available.

Please note that to make use of the HARDWARE, or  HARDWARE  and

SOFTWARE detect methods,  either  the  modified interface or  the

OPTO interface must be present.

SEND DELAY

Some  radios  require a delay between each  character sent.  If

this value is set greater than 0, the computer will pause for the

number of milliseconds entered between each character sent to the

radio.  Although  this  delay  is not  needed  on  the  currently

supported radios, this option is provided for  future  support of

other radios. The maximum  value that the program will allow  for

this value is 50. If a greater value is  set, it  will be reduced

when  the  PARMS  panel is  exited.  Be  aware that setting  this

parameter will  slow  down  the  SCAN  and  SEARCH  rates  of the

program.

NOTE: The delay time  generated  is approximate. This  is due to

the   unpredictability   of    the    multi-tasking   dispatching

environment. If this option is required, some experimentation may

be needed to determine the correct value.

The Menu  33

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SPECIAL KEYS

ENTER

The  ENTER key finalizes input to the program. For  data  entry

fields, the information is stored into the computer's memory, and

the cursor advances to the next field.

ESC

Pressing ESC terminates the current operation. It is equivalent

to pressing the MANUAL button.

tab keys

The tab keys will move the cursor to  the next field. A forward

or reverse tab key will complete data entry of the current field.

PgDn/PgUp

These keys increment or decrement  the  current memory  channel

being displayed.

ctl-PgDn/ctl-PgUp

These keys increment or decrement the current scan bank channel

being displayed.

SPECIAL KEYS  34

RADIO REMOTE CONTROL PROGRAM FOR OS/2

alt-PgDn/ctl-PgUp

These  keys  increment  or  decrement the current  search  bank

channel being displayed.

SPECIAL KEYS  35

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DATA FILE FORMATS

There are several files that  this  program makes use of. These

files are used to store and retrieve data used by the program.

LOGFILE format

When LOGGING is enabled, certain  transactions are logged to a

file.  The  name  of this file is  set by the  user.  The initial

setting for this filename is RADIOCTL.LOG.

For a given transaction, the following information is logged:

*   Timestamp- The Date and Time of the transaction.

*   The scanning mode (SCAN/SEARCH/MANUAL).

*   The VFO frequency.

*   The VFO modulation.

*   The current memory channel.

*   The signal strength at the time of transaction.

*   The length of the transaction.

*   The service information for the current memory channel.

FREQLIST.DBF

This  is  a standard  DBASE  III+ type  data  file. It contains

frequencies  and allocations  for the radio  spectrum.  There are

three fields that are used by this program.

field name     allocation       description

-----------------------------------------------------------

FREQUENCY      decimal 10.5     holds the frequency in Mhz

DESCRIPT       character 50     usage of the frequency

ACTION         logical          flags the frequency in use

If the ACTION field is  not present, the program  will  use the

DATA FILE FORMATS  36

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DESCRIPT  field  to  determine  ACTION.  A  blank DESCRIPT  field

signals an inactive channel.

The length of the DESCRIPT field can be changed, if so desired.

This program only uses the first 80 chars of this field.

Any other fields can be added or deleted from this file without

affecting the  operation of the program.  However, if  either the

FREQUENCY  or DESCRIPT field is  not present in the database, the

DBASE function will not operate.

data file format

The  SAVE command creates a file with  the  current settings of

memory channels, and various scan parameters. The file created is

an ASCII  readable  file.  Each  record  is terminated by a CR/LF

sequence.  One record represents one frequency channel, one  scan

bank channel, one search bank channel, one command, or a comment.

Which  type  if  record  is  determined by  the  first  non-blank

character on the  line.  The following table shows the identifier

for the various types of records:

��������������������������������������������Ŀ

� identifier   �  type of record             �

��������������������������������������������Ĵ

� number       � frequency channel info      �

��������������������������������������������Ĵ

� @            � scan bank information       �

��������������������������������������������Ĵ

� #            � search bank information     �

��������������������������������������������Ĵ

� keyword      � command to program          �

��������������������������������������������Ĵ

� *            � comment                     �

����������������������������������������������

Each record contains one or more fields delimited by blanks.  The

file can be altered  (or created) using any standard text editor.

The maximum  record length  that the program  can  handle is  200

characters.  If  the  comment  record  flag  is   detected,   all

information following the '*'  is ignored on that line.  When the

program reads the file, any missing required parameter, parameter

range error, or unrecognized keyword will terminate the file read

at that line.

The format for a frequency channel entry is as follows:

DATA FILE FORMATS  37

RADIO REMOTE CONTROL PROGRAM FOR OS/2

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � CHANNEL   � 0-9999         �channel number to set       �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   2 � FREQUENCY � 0-999.9999     �frequency in Mhz            �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   3 � MODE      � 3 chars        �valid modulation for radio  �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   4*� COUNT     � 0-999999       �count of squelch breaks     �

���������������������������������������������������������������Ĵ

�   5*� DURATION  � 0-999999       �total duration of squelch   �

���������������������������������������������������������������Ĵ

�   6*� SIGNAL    � Sxx            �Signal strength value (xx)  �

�     �           � xx=00 to 15    � optional                   �

���������������������������������������������������������������Ĵ

�   7*� VALID     � VALID/NOTVALID �indicator of channel valid  �

���������������������������������������������������������������Ĵ

�   8*� ANTEN     � ANTEN/NOANTEN  �indicator of attenuation    �

���������������������������������������������������������������Ĵ

�   9*� RESUME    � RESUME/NORESUME�indicator of resume         �

���������������������������������������������������������������Ĵ

�  10*� TSTAMP    � TSTAMP value   �squelch break time stamp    �

�     �           � TSTAMP 0       �time value is UNIX time     �

�     �           �                �(number of seconds since    �

�     �           �                � 1970) 0 = no stamp.        �

���������������������������������������������������������������Ĵ

�  11*� TONE      � PL 0 0 0       �future enhancement.         �

�     �           �                �If included, all three      �

�     �           �                �numbers must be specified.  �

�     �           �                �                            �

���������������������������������������������������������������Ĵ

�  12*� DELAY     � DELAY/NODELAY  �indicator of channel delay  �

���������������������������������������������������������������Ĵ

�  13*� SERVICE   � 80 chars       �service for channel         �

�����������������������������������������������������������������

*optional fields

The first  three fields MUST be  specified and be in the proper

order. Optional fields do NOT have to be specified. However,

Optional  fields do  not have  to  be specified.  However,  the

following positional restrictions apply:,

*   If COUNT is specified, it MUST follow the MODE field.  If not

specified, the COUNT value for the channel is set to 0.

*   If  DURATION is specified,  COUNT must also be specified, and

DURATION  must  follow  the  COUNT.  If  not  specified,  the

DURATION value for the channel is set to 0.

*   Duration  value MUST be specified if  any  of  the  following

DATA FILE FORMATS  38

RADIO REMOTE CONTROL PROGRAM FOR OS/2

fields are specified:

SIGNAL, VALID, ANTEN, RESUME, TSTAMP, TONE, DELAY, SERVICE.

*   If SIGNAL  value is not  specified, then SIGNAL value for the

channel is set to 0.

*   If VALID  is not specified, the channel will be marked VALID.

*   If ANTEN, RESUME  or  DELAY is not specified,  the respective

flags will be reset for the channel.

*   If DELAY is specified, it MUST precede the service field.

*   If SERVICE is specified,  DELAY must  also be specified.  All

characters (up to 80) following the DELAY value and up to the

end of the  line (CR/LF)  are  taken  as  a  SERVICE  string.

Leading blanks and any special ASCII characters are removed.

DATA FILE FORMATS  39

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The format for a scan bank entry is as follows:

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � BANKID    � @              �@ identifies a scan bank    �

���������������������������������������������������������������Ĵ

�   2 � BANK      � 1-100          �bank number                 �

���������������������������������������������������������������Ĵ

�   3 � START     � 0- MAX         �starting channel for bank   �

���������������������������������������������������������������Ĵ

�   4 � END       � 0- MAX         �ending   channel for bank   �

���������������������������������������������������������������Ĵ

�   5*� SCAN      � SCAN/NOSCAN    �valid bank                  �

���������������������������������������������������������������Ĵ

�   6*� DESCRIPT  � 80 chars       �description of bank         �

�����������������������������������������������������������������

*optional fields

If NOSCAN is not specified, the bank is marked as valid.

DATA FILE FORMATS  40

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The format for a search bank entry is as follows:

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � BANKID    � #              �@ identifies a search bank  �

���������������������������������������������������������������Ĵ

�   2 � BANK      � 1-100          �bank number                 �

���������������������������������������������������������������Ĵ

�   3 � START     � 0- 9999.99999  �starting frequency for bank �

���������������������������������������������������������������Ĵ

�   4 � END       � 0- 9999.99999  �ending   frequency for bank �

���������������������������������������������������������������Ĵ

�   5 � INC       � 0- 9999.99999  �frequency increment         �

���������������������������������������������������������������Ĵ

�   5 � MODE      �AM,FM,FMN,etc.  �modulation for bank         �

���������������������������������������������������������������Ĵ

�   5*� SEARCH    � SEARCH/NOSEARCH�valid bank                  �

���������������������������������������������������������������Ĵ

�   6*� DESCRIPT  � 80 chars       �description of bank         �

�����������������������������������������������������������������

*optional fields

If NOSEARCH is not specified, the bank is marked as valid.

DATA FILE FORMATS  41

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Certain commands can be  entered in a text file  for execution.

These commands provide the  ability to execute certain operations

automatically, without  having to  select  them from the  various

panel controls.

COMMANDS RECOGNIZED

��������������������������������������������������������������Ŀ

�command         � option(s)      � description                �

��������������������������������������������������������������Ĵ

�ACK             �    -           �turn ON acknowledgement     �

��������������������������������������������������������������Ĵ

�BANK            � ON|OFF         �BANK mode on or off.        �

�                �                �Equivalent to pressing BANK �

�                �                �button.                     �

��������������������������������������������������������������Ĵ

�CHAIN           � filespec       �start reading another text  �

�                �                �file. Maximum CHAIN depth   �

�                �                �is 3 files.                 �

��������������������������������������������������������������Ĵ

�CHANNEL         � 0 to 9999      �set channel number.         �

�                �                �Equivalent to setting       �

�                �                �CHANNEL display value.      �

��������������������������������������������������������������Ĵ

�CLEAR           �    -           �Clear all memory channels   �

�                �    ALL         �Clear all memory channels   �

�                �    COUNTS      �Clear all squelch counts    �

�                �    SERVICE     �Clear all service strings   �

�                �    SIGNAL      �Clear all strength readings �

�                �    TIME        �Clear all squelch times     �

��������������������������������������������������������������Ĵ

�DBNAME          � filespec       �change DBASE filename for   �

�                �                �service lookups.            �

��������������������������������������������������������������Ĵ

�DELAY           �    xxxx        �Set DELAY value to xxxx.    �

��������������������������������������������������������������Ĵ

�F               �  frequency     �Set VFO frequency           �

��������������������������������������������������������������Ĵ

�HALT            �     -          �Force MANUAL mode           �

��������������������������������������������������������������Ĵ

�LOAD            �     -          �load memory from radio      �

��������������������������������������������������������������Ĵ

�M               �   modulation   �set VFO modulation          �

����������������������������������������������������������������

DATA FILE FORMATS  42

RADIO REMOTE CONTROL PROGRAM FOR OS/2

COMMANDS RECOGNIZED (cont)

��������������������������������������������������������������Ŀ

�NOACK           �     -          �turn OFF acknowledgement    �

��������������������������������������������������������������Ĵ

�MANUAL          �     -          �Force MANUAL mode           �

��������������������������������������������������������������Ĵ

�MEMORY          �     -          �use radio's memory          �

��������������������������������������������������������������Ĵ

�PDELAY          �   0 to 9999    �set priority sample delay   �

��������������������������������������������������������������Ĵ

�QUIT            �     -          �exit RADIOCTL               �

��������������������������������������������������������������Ĵ

�PRIORITY        �     ON         �set priority switch on      �

�                �     OFF        �set priority switch off     �

�                �   0 to 99999   �set priority channel        �

��������������������������������������������������������������Ĵ

�RESUME          �     ON         �set resume   switch on      �

�                �     OFF        �set resume   switch off     �

�                �   0 to 9999    �set resume value            �

��������������������������������������������������������������Ĵ

�SCAN            �     -          �Start SCAN mode             �

��������������������������������������������������������������Ĵ

�SEARCH          �     -          �Start SEARCH mode           �

��������������������������������������������������������������Ĵ

�SORT            �     -          �Sort memory channels        �

��������������������������������������������������������������Ĵ

�VFO             �     -          �use computer's memory       �

����������������������������������������������������������������

For  compatibility  with  the  DOS  version  of  Radioctl,  the

following  command  keywords  are  recognized,  but ignored  when

reading:

*   BANK {bank_number}

*   BAUD

*   CD

*   COM1

*   COM2

*   DBASE

*   DIR

*   DMAX

*   DMIN

DATA FILE FORMATS  43

RADIO REMOTE CONTROL PROGRAM FOR OS/2

*   FILENAME

*   FKn

*   GOTO

*   HELP

*   INC

*   LOG

*   LOGNAME

*   LPT1

*   LPT2

*   MAX

*   MAXMEM

*   MODE

*   NOSIGNAL

*   PRINT

*   PRTLENGTH

*   PRTWIDTH

*   QFK

*   QSET

*   RECALL

*   RESET

*   SAVE

*   SET

*   SHELL

*   SIGNAL

*   SPEED

*   START

*   STOP

*   TXADR

DATA FILE FORMATS  44

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Any  record  that  is not recognized as a Frequency, Scan  bank,

Search  bank,  command  or comment  will be flagged as a line  in

error.

DATA FILE FORMATS  45

RADIO REMOTE CONTROL PROGRAM FOR OS/2

RS232 INTERFACE

This  program is designed to use the serial communications port

(RS232) of the IBM computer.  In order  to interface  a radio  to

this  port,  an interface adapter  is  needed  designed  for  the

specific radio. Each radio  manufacturer  makes such an  adapter.

However,  none  of  the  adapters supplied  from the manufacturer

provide for squelch detection. Without this detection, the signal

sensing functions of this program will not work.

For  each  radio  supported,  a  circuit and  instructions  are

available.  This circuit provides the squelch detection needed by

this  program.  The  interface circuits are easy  to  build,  and

require few  parts.  All of the parts can be  obtained cheaply at

any electronics supply store. The schematics and instructions are

provided to all registered users.

RS232 INTERFACE  46

RADIO REMOTE CONTROL PROGRAM FOR OS/2

PROGRAM LIMITS AND RANGES

MEMORY REQUIRED:       4 meg (OS/2 minimum). 8 Meg+ suggested

PROCESSOR REQUIRED:    80386 or Higher

OS/2 LEVEL REQUIRED:   2.1  or higher

MAXIMUM MEMORY CHANNELS    10000

MAXIMUM SCAN BANKS          100

MAXIMUM SEARCH BANKS        100

FREQUENCY RANGE           .01khz to 9999.99999mhz*

MINIMUM FREQUENCY STEP     10hz*

MAXIMUM FREQUENCY STEP     9999.99999mhz

*may be limited by radio model.

PROGRAM LIMITS AND RANGES  47

RADIO REMOTE CONTROL PROGRAM FOR OS/2

INDEX

A                                  H

Ack 18                             Hot Keys

AOR3000 3                             Alt PgDn 25

attenuation 20                        Alt PgUp 25

Attenuator 11                         PgDn 20

PgUp 20

Shift PgDn 23

Shift PgUp 23

B

Banks On 18

I

ICOM 3

C                            interface 5

Channel # 9

channel RESUME 21

COMPUTER/RADIO Button 17                 K

COUNT 21

KENWOOD 3

D

L

DBASE 3

delay 20                           Load ALL 15

Digit Scrolling 11                 Load/Set Buttons 15

Logging 19

E

M

ENTER key 34

ESC key 34                         Manual Mode 12

Mem Sync 17

Mem->VFO 16

Memory Channel

F                               CHANNEL number 20

Memory Channels 19

Freq/Memory Scroll Buttons 11      Menu 27

frequency 21                          Database 28

FREQUENCY field 21                    File 28

Parms 29

MODE field 21

modulation 21

Index  48

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Scan Mode

O                               BANKS 13

COMPUTER 13

Operation Modification Buttons 16     DELAY 13

MEM SYNC 13

PRIORITY 13

RADIO 13

P                               RESUME 13

SPEED 13

Parms                                 VALID 13

Baud Rate 31                    Search Bank 24

Com Port 32                        Channel number 25

Data Bits 32                       Incr-freq 26

Delay 29                           Mode 26

Flow Control 32                    Service 26

Max Channel 31                     Start-freq 25

Max Frequency 31                   Stop-freq 26

Min Frequency 31                   Valid 25

Origin 31                       Search Mode 14

Priority 29                        BANKS 15

Radio Name 30                      DELAY 14

Radio Type 30                      MEM SYNC 15

Resume 29                          RESUME 14

Send Delay 33                      SPEED 14

Signal Detect 32                   VALID 14

Spectrum Background 29          SERVICE 22

Stop Bits 32                    Set ALL 16

Tone Decode Enable 30           shareware 3

TXADR 30                        SIGNAL 21

PgDn key 34                        Signal Meter 11

Pgup key 34                        Spectrum Display 27

Priority 17

Priority indicator 10

T

R                            tab key 34

TIME  21

Radio Selection 19                 TIME STAMP 21

Resume 18

RS232 5

V

S                            valid 20

VFO Frequency 9

Scan Bank                          VFO Frequency Button 17

Channel number 23               VFO Modulation 9

Service 24                      VFO step value 10

Start 23                        VFO->Mem 16

Stop 23

Valid 23

Scan Mode 12

Scan Mode Buttons 12

MANUAL 12

SCAN 12

SEARCH 14

Index  49