When IBM introduced OS/2 2.1 back in June, I was wondering high- performance video support just like Microsoft Windows. 3.1. Fast video, 24-bit colors. That didn't happen so far, and worth yet many video cards are not even compatible with OS/2 2.1.

So last 2 weeks I spent some time for searching good video cards that really support IBM OS/2 2.1.  I start with video cards that using one of the listed in User Guide page 327-328.

I set the 2 computers with identical model and configuration except the video cards.

Intel 486DX-50Mhz and VESA local bus, Maxtor 245MB harddrive, HPFS formated,

16MB RAM,ProAudio Spectrum 16 Sound Cards, Toshiba 3401B SCSI CD-ROM Drive,

TECH Combo Disk drive, Microsoft Mouse II Bus version and set to IRQ 2.

Then I start with Trident TVGA 8900C Model, very popular cheap video card.  Slow performance, but solid display. It display any mode 640x480-256, 800x600-256, 1024x800-256

Very slow at large size mode when it's over 640x480-256. Street price under $60.00

Second was Hercules Dinamite TSENG ET4000 model, it is graphic accelerator for Windows 3.1.  Unfortunately this card didn't work at first time. System freezed during the OS/2 installation.  Problem was, display card and mouse bus memory IRQ interrupt.

After jumper change to IRQ 5 in mouse card, it was installable. But problem wasn't over yet!

This time there was no prompt for DOS-Windows, Full screen DOS. Street price $199.95

Third one was IBM Easy option, Cirrus SVGA card with 16 million color model.  This card will let you install, Not good as Trident TVGA 8900C model. Even it was more expensive.

Street price $149.95

Fourth one was Diamond Stealth 24 VLB "S3" graphic accelerator.  This card was not a compatible when OS/2 2.1 was out.  Device driver was developed August 1993

by S3 & IBM.  This card only displays as VGA mode at the installation, but you can upgrade by using Display Driver from IBM only.  If you like to download files from Diamond or other vendors, you will not get any 640x480-256 device file. Bad news is all the files from either IBM or Diamond will not work at all.  Problem is often system hangs off and sometimes system freeze.

Street price $199.95

Fifth one was Western Digital Pradise graphic accelerator. Also problem with installation.

Street price $99.00

Finally, ATI Graphic Ultra + with 1MB model.  This one was the most expansive card, but you will like to use this card. Since ATI Graphic models support 8514/A mode, you can run from native OS/2-8514/A. Great news is you can choose any mode from 640x480 to 1024x800 with 256 color. Not only these 4 modes, you can also choose IBM PS/2 mode.

ATI announced they will support 65K color mode for 2MB models.  These drivers are now under development.  But there is display driver Beta version for OS/2 2.1 Beta & OS/2 2.1 GA Version.

These files are avaliable from ATI Forum.

If you have any questions about this test or if you like to know more, please send mail to Chong Kim. ID#72672,2220 and I will happy to help you.  For people who like to know more any IBM OS/2 support also send mail to same address above. I will find answer for you.