MAD16.ADD DOCUMENTATION

Device driver to initialize the MAD16 Pro integrated digital

audio controller.  The MAD16 Pro is the OPTI / MediaCHIPs 82C928/82C929.

It is on the Media Magic ISP-16 and many other multimedia boards.

The reference design has the following primary components:

82C928 - (MAD16) Responsible for ISA bus interface,

CD-ROM interface for Sony, Panasonic, Mitsumi and IDE.

ISA bus interface.

CS4231 - Crystal Semiconductor audio CODEC or Analog Devices AD1848.

Microsoft Sound System compatible wave audio.

YMF262 - Yamaha (OPL-3) FM synthesis.

Used for FM MIDI synthesis - sound effects in games

With these parts, the board can perform CD-ROM functions, audio play/record

for both Windows Sound System audio and Sound Blaster as well as

FM synthesis.  All activity is coordinated through the MAD-16 and the

MAD-16 is responsible for selecting modes and configuring base I/O

address, IRQ and DMA for all parts on the board.

This device driver serves to select and enable the various hardware options.

Once the device is brought to life, device drivers specifc to the on-board

components can operate without knowledge of the installed board.

To install MAD16.ADD on OS/2 Warp.

1) Copy MAD16.ADD to \OS2\BOOT

2) Modify config.sys to include the follow line

BASEDEV=MAD16.ADD [...Options...]

Place this driver before CD-ROM .ADD drivers in config.sys.

3) With this wakeup code installed, existing drivers for the CD-ROM

and audio can be used.

The following is an example config.sys line which brings the device to

life in Windows Sound System mode at base address 530 with a Sony31A CD-ROM.

BASEDEV=MAD16.ADD MODE:wss BASE:530 CDTYPE:S

The Options to MAD16.ADD are:

MODE:     wss, sb            (default Windows Sound System)

BASE:     220, 240           (for Sound Blaster mode, default 220)

530, E80, F40, 604 (for Windows Sound System mode, default 530)

Choosing 530 or E80 for WSS gives 220 for Sound Blaster.

Choosing 604 or F40 for WSS gives 240 for Sound Blaster.

E80 and F40 are valid only with version 929 of the OPTi part.

CDTYPE:   None, Sony31A, Panasonic, Mitsumi, IDE (default None)

One character is adequate (e.g. CDTYPE:S)

CDBASE:   None, 320, 330, 340, 360 (default based on CDTYPE)

Sony31A   - 320, 330, 340, 360 (default 320)

Panasonic - 330, 340 (default 330)

Mitsumi   - 320, 340 (default, 320)

IDE       - 170      (fixed)

CD-ROM device drivers accept "P:xxx" to specify a base port

CDIRQ:    None, 3, 5, 7, 9, 10, 11 (default based on CDTYPE)

Sony31A, Mitsumi (All above choices are valid)

Panasonic - None (polled device, no interrupts)

IDE       - 15 (fixed)

CD-ROM device drivers accept "I:x" to specify an interrupt.

SBIRQ:    5, 7, 10, 11 (default 7)

SBDMA:    1, 2, 3 (default 1)

WSSIRQ:   7, 9, 10, 11

WSSDMA:   0, 1, 3 (default 1, avoid 0)

NOTE: The WSS IRQ and DMA are specified here, but can be

reset by the audio PDD when it loads.  Optimally, the

audio PDD will query MAD16.add to learn of the device settings.

As of this writing, this communication does not exist.

JOYSTICK: 0, 1  (default 1, joystick enabled)

OPL:      3, 4  (default OPL:3, must match installed OPL chip)

OS/2 AUDIO and CD-ROM Device Driver Installation:

These instructions assume OS/2 base operating system is already installed

and that the MAD16.ADD is already installed per the instructions above.

1) Run OS/2 selective install (install.exe)

2) Your CD-ROM should be auto-detected.

If not, click the icon to manually select.

3) To select audio driver, Click on ICON and select (Thinkpad CS4231)

For Analog Devices based MAD-16 boards, select "Business Audio"

4) Run the install to completion

5) Modify config.sys to:

a) Insure the MAD16.ADD is listed in config.sys before the CD-ROM driver.

b) Add parms to the CS4231.sys or BUSAUDIO.SYS device driver to inform

it of the BaseIOAddress, IRQ and DMA selections.

(e.g. device=c:\mmos2\cs4231 P:530 I:7 D:1 N:BSAUD1$)

Where P: is BasePort,

I: is Interrupt

D: is DMA channel (1 or 3)

Notes on hardware revisions:

The MAD-16 comes in multiple forms.  This driver (MAD16.add) knows how to

program the 82C928 revisions B and C and the 82C929.  No user action is

required, the driver auto-detects the hardware level.

Boards based on the MAD-16 can utilize different audio CODECs.

MAD16.add knows how to initialize the CS4231 and CS4238 CODECS from

Crystal Semiconductor and the AD1848 CODEC from Analog Devices.

You may encouter times where a value listed as valid in the documentation

is rejected by the device driver.  Depending on the hardware version,

some of the options have different valid sets.

For example, Sound Blaster IRQ can be configured for any of 5, 7, 10, 11.

Wave audio IRQ-10 will only work on 82C929 versions of the MAD-16.

Similarly, IRQ-11 requires the 82C928.

Where a value is not supported, the driver will display a message

recommending alternate values that *will* work with your hardware.