.*	SCCSID = @(#)COM.DOC	   11.13 89/03/14

.im ..\..\equate.sci

.im ..\..\macros.sci

.fo left

.*.us 1

.*

.* PLEASE:

.*

.* DON'T USE the word "baud" in this document (unless you know

.* what it means).  Use "bit rate" or "bits per second" to

.* indicate bit rate.

.*

.*.ce &vcom. Component

:h3.&vcom. Architecture

:h4.Problem Description/Objectives

:p.The &vcom. component consists of virtual support for the

serial communication I/O ports and for the serial channel-related

&cb. entry points.  It provides support in each &vdm. for up to

two communication channels on the &pcat., and up to three on the &m80.

and compatible &ps2. systems.

:p.This component allows extant &dosapps. which access the

communication channels, either by direct access to the I/O ports, or

by normal &cb.  calls, to operate without change.

:p.The &vcom. supports only access to communication channels which are

physically present on a given system.  This does not include support

for accessing communication devices which may be redirected by network

software.

:histref refid=COMHS02.&gml.

:p.Throughput performance is not as high as a real &box3., due to

the overhead associated with system tasking and I/O port

virtualization.  The performance objective for the &vcom. component is

to support, on an otherwise unburdened &m80., data transfer on one

full-duplex communication channel at a transfer rate of 240

characters per second in both directions simultaneously, with no

receive hardware overrun errors.

:h4.Solutions/Justification

:p.From the perspective of the &vdm., the &vcom. component supports

access via:

:ul

:li.&cb.

:li.I/O address space

:eul

:h5.Support Summary

:p.The &vcom. component directly supports invocations of &cb.  INT

14h, rather than allowing the &cb. to access the ports directly.

:p.Direct access to the virtual I/O ports is also supported.  Direct

access to I/O ports results in emulation of a communication

controller with the INS8250 UART.  I/O port emulation of a

controller with the NS16550 UART is not provided.

:p.When referenced via the virtual I/O ports, the &vcom.

component also provides all support for virtual interrupt

indication.

:p.The hardware appears to the &vdm. as a serial interface as

supported by standard expansion bus serial ports on a &pcat. or

&ps2..

:h5.Information Flow

:p.Both data and control/status information flow bidirectionally

to and from the &vdm.  through the serial communication channels.

Data is transferred directly to and from the &pdd.  via

special entry points in the &pdd..  As data arrives and/or events

occur, the &pdd. notifies the &vdd. and passes the

event status.

:h6.Receive

:p.When the &pdd. has informed the &vdd. that received data has been

queued, the &vcom. calls the COM &pdd. for the received characters

until all the data has been passed to the &dosapp..  Line status for

each character received is also passed from the &pdd. to the &vcom..

:h6.Transmit

:p.As data is passed from the &dosapp. to the &vcom. to transmit, the

&vdd. calls the COM &pdd. to write the character to the serial port.

:h5.Interrupt Service

:p.The &vcom. component provides full support for the

virtualization of communication channel interrupts, following the

same logic as the actual hardware.

:h4.Architectural Review

:h3.&vcom. Design

:h4.Design Overview

:p.The execution control of the &vcom. is divided into eight

components:

:ol

:li.&vcom. Initialization

:li.&vdm. Creation

:li.Channel Open

:li.&cb. Access

:li.Virtual I/O Port Access

:li.Virtual Interrupt Indication

:li.&vdm. Termination

:li.MVDM Shutdown

:eol

:h5.&vcom. Initializaton

:p.Upon &mvdm. initialization, after the &pdd.s have been

initialized, the &vcom. component is invoked at it's initialization

entry point.  This routine involves setting up a private interface

with the COM &pdd. (VDHOpenPDD).

:p.Then, for each of the supported COM ports (COM1 and COM2 on a

&pcat., COM1 - COM3 on a &ps2.), the &vcom. searches for that serial

device's I/O port base address in the system &cb. data area.  If the

address is zero (indicating that the device is not present or is owned

by a &pdd. other than the COM PDD), the &vcom. will not attempt to

support that serial device.

:p.If the COM port's base address is found in the &cb. area, the

&vcom. will verify that the COM &pdd. has installed for that

particular serial device.  If the COM &pdd. indicates it has not

installed for that port, the &vcom. will not attempt to support that

serial device.

:p.For the serial devices it can support, the &vcom. proceeds to

request the &vpic. component to obtain an IRQ handle, for use to

support virtual interrupt indication.  In addition, the &vcom.

registers its VDM Creation, VDM Termination, and MVDM Shutdown event

handlers with the VDM Manager component.

:h5.&vdm. Creation

:p.Upon creation of the &vdm., the &vcom. initializes the internal

states of the virtual serial ports.  The &vdm.'s &cb. data area,

initialized by the &vbios. component, is referenced

:p.To allow monitoring of I/O activity, the &vcom. instructs the

&v86mgr. to notify it when a &vdm. program accesses a virtual serial

port.  It also calls the &v86mgr. to set the &cb. serial channel

service vector to invoke the &vdd. when calls to &cb. INT 14h are

issued.

:h5.Channel Open

:p.After &vdm.  creation, the &cb. data area provides a logical link

between the virtual communication channels in the &vdm. and the

physical serial channel hardware.  Since the &vcom. will not support

a COM port if its address is not in the &cb. area, then if the

&dosapp.  does not find the device address, it can handle its own

errors and/or terminate in its usual fashion.

:p.If the &cb. area indicates the device is present, however, then

when the &dosapp. makes its first access to that device, the

&vcom. determines if the device is already in use in another &vdm..

If so, the &vcom. will not attempt to send the OPEN command to the COM

&pdd..  If it is not already in use, the &vcom. calls the

COM &pdd. with the OPEN command.  This request will indicate success

if the COM port is not in use at that time (by a protected mode &os2.

process).

:p.If the COM port is already in use (either by another &vdm. or by an

&os2. process), the &vdd. will issue a popup message that informs

the user of the resource contention and allows for a user selected action.

As a result of the user action the conflict is resolved or that &vdm.

may be terminated.

:p.Once the device is opened, the &vdd. communicates directly with

the &pdd. to perform all virtual hardware operations.  This includes

sending and receiving data, detection and simulation of hardware

interrupts, and setting or querying the status and control registers.

:h5.&cb. Access

:p.The &vcom. component also supports &cb.  access through

software interrupt 14h (20).  Rather than allowing the &cb.  to

perform the functions by accessing the virtual I/O ports, this

component directly emulates the &cb. functions.  This support is

similar to the virtual I/O port access support, although simpler.

The &cb. access emulation supports six functions:

:ul

:li.Initialize

:lp.The initialize function establishes the communication speed

and framing options for the channel.  The modem status and line

status are returned.  To specify bit rates of greater than 9600

&bps., the extended initialize function (below) must be used.

:li.Send Character

:lp.The send character function sends a character out the

communication channel.

:li.Receive Character

:lp.The receive character function waits for a character from the

communication channel and returns the character.

:li.Read Status

:lp.The read status function returns the modem status and line

status.

:li.Extended Initialize

:lp.The extended initialize function, like the initialize

function, establishes the communication speed and framing options

for the channel.  The modem status and line status are returned.

This function is used if a bit rate of 19,200 bits per second is

desired or space or mark parity selection is desired.

:li.Extended Port Control

:lp.The extended port control function sets or reads the modem

control register.

:eul

:h5.Virtual I/O Port Access

:p.A serial port controller provides eight I/O ports:

:ol

:li.Receive/Transmit Buffer and Divisor Latch

:li.Interrupt Enable and Divisor Latch

:li.Interrupt Identification

:li.Line Control

:li.Modem Control

:li.Line Status

:li.Modem Status

:li.Scratch

:eol

:p.For a more detailed description of the layout of these ports,

see :hdref id=COMHD01..

:p.The divisor latch is a 16-bit latch which holds a counter

value that specifies the serial bit rate.

:p.The receive and transmit buffer registers are accessed to

receive and transmit data from and to the communication line.

:p.The interrupt enable register is used to specify what types of

status changes are to cause interrupts to the system.

:p.The interrupt identification register indicates what type or

types of status changes caused an interrupt to the system.

:p.The line control register is used to specify character framing

parameters and to multiplex the divisor latch register with the

receive and transmit data buffers onto a single I/O port.

:p.The modem control register is used to specify the state of

certain RS-232 control lines.

:p.The line status register indicates the status of received data

and the receive and transmit buffers.

:p.The modem status register indicates the state of certain

RS-232 status signals.

:p.The scratch register is a read-write register which provides

no function other than a single-byte storage.  In the virtual

environment, this register always returns 0FFh when read, to

indicate that an INS8250 is being emulated.

:h5.Virtual Interrupt Indication

:p.The following INS8250 interrupt types are virtualized by the

&vcom..  The &dosapp. must properly enable interrupts and the

specific interrupt type before the interrupt will be simulated to the

&vdm.:

:ul

:li.Line Status Interrupt

:li.Receive Data Available Interrupt

:li.Transmitter Holding Register Empty Interrupt

:li.Modem Status Interrupt

:eul

:p.When the &pdd. has notified &vcom. of an interrupt condition,

the &vcom.  passes indication of a virtual interrupt to the &vpic.

component which, in turn, simulates the interrupt to the &vdm..

:p.To maintain high performance on the physical serial channel, the

COM &pdd. does not notify the &vcom. on every interrupt.  Rather, the

&vcom. receives notification of certain COM PDD events, and determines

whether to begin or continue simulating interrupts to the &vdm..

:h5.&vdm. Termination

:p.When the &vdm.  terminates, the &vcom. component is called in its

VDM Termination handler entry point to perform any cleaning up that it

may require.  It releases any I/O port hooks that have been active in

that &vdm., and deinstalls that VDM's software interrupt handlers for

INT 14h.  Additionally, the &vcom. calls the &pdd. command interface

with the CLOSE request to deactivate the physical communications

channel.

:h5.MVDM Shutdown

:p.During &vcom. initialization, the &vcom. sets an event hook

(VDHInstallUserHook) with VDM Manager to be notified if the VDM

Manager invokes an MVDM shutdown.  Since all VDMs have already been

terminated prior to this event, there are no open virtual serial

channels at this time.

:p.Any global resources, such as semaphores and memory, that have been

explicitly allocated by the COM VDD and remain allocated at this time,

must be released by the COM VDD at this time.

:p.Finally, the &vcom. unhooks the hardware interrupt(s) owned

(VDHCloseVIRQ).

:h4.Data Structures Description

:h5 ID=COMHD01.Virtual I/O Ports

:p.The INS8250 serial port controller provides eight I/O ports which

access eleven eight-bit I/O registers:

:ul

:li.RBR - Receiver Buffer	    (Input)

:li.THR - Transmit Buffer	    (Output)

:li.DLL - Divisor Latch - Low Byte  (Input/Output)

:li.DLM - Divisor Latch - High Byte (Input/Output)

:li.IER - Interrupt Enable	    (Input/Output)

:li.IIR - Interrupt Identification  (Input)

:li.LCR - Line Control		    (Input/Output)

:li.MCR - Modem Control		    (Input/Output)

:li.LSR - Line Status		    (Input)

:li.MSR - Modem Status		    (Input)

:li.SCR - Scratch		    (Input/Output)

:eul

:p.The I/O address for each register depends upon the controller

number (COM1, COM2, or COM3) and the system configuration.  The I/O

addresses supported for each COM port on a typical (default) &ps2.

configuration is shown in

:figref refid=COMFG01 page=no..

:note.Although some &ps2. systems may have serial devices installed

which provide additional registers than those listed above, I/O to

such registers will be ignored by the &cr. &vcom..

:enote.

:fig id=COMFG01 frame=none place=inline

.ti > 09

.tb   17 26 37 47

.bx 16 25 36 46 60

>      >Register>     >I/O

>      >	>     >Address

>Device>Name	>R/W  >(varies)

.bx

>COM1  >RBR	> R   >03F8h

>      >THR	> W   >03F8h

>      >DLL	>R/W  >03F8h

>      >DLM	>R/W  >03F9h

>      >IER	>R/W  >03F9h

>      >IIR	>R    >03FAh

>      >LCR	>R/W  >03FBh

>      >MCR	>R/W  >03FCh

>      >LSR	>R    >03FDh

>      >MSR	>R    >03FEh

>      >SCR	>R/W  >03FFh

.bx

>COM2  >RBR	> R   >02F8h

>      >THR	> W   >02F8h

>      >DLL	>R/W  >02F8h

>      >DLM	>R/W  >02F9h

>      >IER	>R/W  >02F9h

>      >IIR	> R   >02FAh

>      >LCR	>R/W  >02FBh

>      >MCR	>R/W  >02FCh

>      >LSR	> R   >02FDh

>      >MSR	> R   >02FEh

>      >SCR	>R/W  >02FFh

.bx

>COM3  >RBR	> R   >3220h

>(PS/2 >THR	> W   >3220h

>only) >DLL	>R/W  >3220h

>      >DLM	>R/W  >3221h

>      >IER	>R/W  >3221h

>      >IIR	> R   >3222h

>      >LCR	>R/W  >3223h

>      >MCR	>R/W  >3224h

>      >LSR	> R   >3225h

>      >MSR	> R   >3226h

>      >SCR	>R/W  >3227h

.bx 16 25 60

>COM4  >Not Supported

.bx off

:figcap.Serial Controller Port Addresses (Example)

.ti 3E 3E

:efig

:note.Direct I/O to the COM3 port addresses is supported only on a

&ps2. system.  &dosapp. I/O to these addresses on a &pcat. system

will be ignored.  The I/O addresses for a specific COM port may vary

depending on the hardware configuration.  For example, on a &ps2., the

user may configure the COM port base addresses to appear different

from those shown in :figref refid=COMFG01 page=no..  &dosapp.s are

typically not programmed with dependencies on specific I/O addresses,

but (usually) rely on the &cb. data area to determine what the port's

base address is.

See :figref refid=COMFG02. for more information.

:enote.

:h6.RBR - Receive Buffer Register

:p.The RBR register provides data which was received from the

communication line.  It returns the most recently received

character.  The LSR may be read to determine when a new character

has arrived.  See the description of the LCR to see how I/O

accesses to this register's ports differentiate between this

register and the DLL register.

:h6.THR - Transmitter Holding Register

:p.The THR register is written to indicate the next character

that is to be transmitted over the communication line.  The LSR

may be read to determine when this register may be written

without overwriting the value that was previosly written.  See

the description of the LCR to see how I/O accesses to this

register's ports differentiate between this register and the DLM

register.

:h6.DLL - Low Divisor Latch

:p.The DLL register is written to specify the least significant

eight bits of the divisor latch.  The divisor latch contains a

counter which specifies the bit rate of the communication line.

This counter counts down a 1,843,200 hertz clock, which is again

divided by sixteen to achieve the bit rate.  Thus the bit rate,

BR, is:

:fig place=inline

BR = 1,843,200 / 16 / DL

:efig

:pc.where DL is the 16-bit divisor latch value.  The divisor

latch value can be determined from the desire bit rate as:

:fig place=inline

DL = 1,843,200 / 16 / BR

:efig

:p.The divisor latch value for 1200 bits per second is, for

example, 96.  Using this example, to program the UART to 1200

bits per second, the DLL register would be loaded with 96 (60h)

and the DLM register with zero (00h).

:p.See the description of the LCR to see how I/O accesses to this

register's ports differentiate between this register and the RBR.

:h6.DLM - High Divisor Latch

:p.The DLM register is written to specify the most significant

eight bits of the divisor latch.

:p.See the description of the LCR to see how I/O accesses to this

register's ports differentiate between this register and the THB.

:h6.IER - Interrupt Enable Register

:p.The IER register is written to specify which UART events will

signal an interrupt.  The low order four bits of the register

select the four events.  For each bit which is set, the

corresponding event will signal an interrupt.  For an interrupt

to actually occur, however, the interrupt line must be enabled by

the OUT2 control signal, explained in the description of the MCR,

below.  The bits and their associated events are:

:ul

:li.Bit 0 - Rx Data Available

:lp.This event is indicated when a character has been received.

This interrupt is cleared when the RBR is read.

:li.Bit 1 - Tx Register Empty

:lp.This event is indicated when the transmitter holding register

has been placed into the serialization buffer.  It is cleared

when the IIR is read or the THR is written.

:li.Bit 2 - Rx Line Status

:lp.This event is indicated when a break signal has been received

or a framing, parity, or overrun error was detected in an

incoming character.  It is cleared when the LSR is read.

:li.Bit 3 - Modem Status

:lp.This event is indicated when one of four modem status signals

have changed (CTS, DSR, RI, DCD).  It is cleared by reading the

MSR.

:li.Bits 4 through 7

:lp.These bits should be zero.  In virtual mode, these bits are

ignored when written.

:eul

:h6.IIR - Interrupt Identification Register

:p.This register identifies the current interrupt event, if any.

The low order bit of this register indicates if an interrupt is

pending.  If this bit is set (1), then an interrupt is not

pending.  If it is reset, the next two bits form a number, from 0

to 3, indicating the event which caused the interrupt:

:ul

:li.0 --> MODEM Status Change

:lp.One of the four RS232 input signals changed.  This interrupt

is cleared by reading the MSR.

:li.1 --> THR Empty

:lp.The THR has been emptied into the shift register.  This

interrupt is cleared by reading the IIR or writing the THR.

:li.2 --> Rx Data Available

:lp.The RBR has data which has not been read yet.  This interrupt

is cleared by reading the RBR.

:li.3 --> Rx Break or Error

:lp.A break signal was received or a parity, framing, or overrun

error was detected in an incoming character.  This interrupt is

cleared by reading the LSR.

:eul

:pc.The remaining bits in the register are always zero.  This

register should not be written.  In virtual mode, if this

register is written, the operation is ignored.

:h6.LCR - Line Control Register

:p.The LCR is written to specify character framing parameters, to

send a break signal, and/or to enable/disable access to the divisor

latch.  The function of the bits in this register are:

:ul

:li.Bits 0 and 1 - Word Length Select

:lp.These two bits form a number, from 0 to 3, which added to

five, specifies the number of data bits in a character.  Thus

the word length may be from 5 to 8 bits.

:li.Bit 2 - Number of Stop Bits

:lp.This bit specifies the number of stop bits in transmitted

character frames.  It does not affect the interpretation of

received frames.  If this bit is zero, it specifies one stop bit.

If it is one, it specifies two stop bits if the word length is 6,

7, or 8; it specifies one and one-half stop bits if the word

length is 5.

:li.Bit 3 - Parity Enable

:lp.This bit specifies that a parity bit is to be added to

transmitted frames and a parity bit is to be expected and checked

on received frames.  A "1" in this position enables this

function.  It is disabled if "0" is written to this bit.

:li.Bit 4 - Even/Mark Parity

:lp.This bit selects even/odd or space/mark parity.  If bit 5 is

reset (0), then this bit selects even parity if set (1), or odd

parity if it is reset (0).  If bit 5 is set (1), then this bit

selects space parity if set (1) or mark parity if reset (0).

:lp.Even parity in a character frame is indicated by an even

number of "1" bits in the data and parity bits.  Odd parity is

indicated by an odd number.  Space parity is indicated by a "0"

in the parity bit, and mark parity is indicated by a "1".

:li.Bit 5 - Space/Mark Parity Enable

:lp.This bit selects space/mark parity instead of even/odd parity.

The description of bit 4, above, elaborates on this selection.

:li.Bit 6 - Break Control

:lp.This bit is used to transmit a break signal.  When this bit

is set, the transmitted data signal is forced to spaces (all

zeros).  When it is reset, the transmitted data signal follows

the transmitter shift register.  This bit is logically OR'd with

the output of the transmitter shift register.

:li.Bit 7 - Divisor Latch Access Bit

:lp.This bit enables access to the divisor latch.  When set (1), I/O

address BASE+0 accesses the DLL and I/O address BASE+1 accesses the

DLM.  When reset (0), I/O address BASE+0 accesses the RBR and THR and

address BASE+1 accesses the IER.  Note that the BASE address may vary

depending on the configuration.

See :figref refid=COMFG02. for more information.

:eul

:h6.MCR - Modem Control Register

:p.The MCR is written to control certain external signals and to

select internal loopback mode.  The bits are assigned as follows:

:ul

:li.Bit 0 - Data Terminal Ready (DTR)

:lp.This bit controls the RS-232 DTR signal.  When set (1) DTR is

asserted; when reset, DTR is negated.

:li.Bit 1 - Request to Send (RTS)

:lp.This bit controls the RS-232 RTS signal.  When set (1) RTS is

asserted; when reset, RTS is negated.

:li.Bit 2 - OUT1

:lp.This bit controls the OUT1- signal which is used for

diagnostic purposes only.  In the virtual environment, this bit

is ignored.

:li.Bit 3 - OUT2

:lp.This bit controls the OUT2- signal which is connected to the

interrupt signal enable logic.  When this bit is set (1), the

interrupt signal is passed to the interrupt controller.  When

reset (0), the interrupt signal is not forwarded.  Note that this

function controls the logical output of the interrupt signal that

may be generated from the events described in the sections above

explaining the IER and IIR.

:li.Bit 4 - Loopback

:lp.This bit enables a loopback test mode in the hardware.  In

the virtual environment, this bit is ignored when written and

returns zero (0) when read.

:li.Bits 5, 6, 7 - Zero

:lp.These bits should be written as zeros.  In the virtual

environment, these bits are ignored when written and return zero

(0) when read.

:eul

:h6.LSR - Line Status Register

:p.The LSR indicates receiver buffer and transmitter holding

register status.  Its I/O port is read-only.  If its port is

written in the virtual environment, the operation is ignored.

The bits are:

:ul

:li.Bit 0 - Rx Data Available

:lp.This bit indicates that a character has been loaded into the

RBR but has not yet been read by the &dosapp..  This bit is cleared

by reading the RBR.  In the virtual environment, this bit is not

cleared until all pending characters have been read by the

&dosapp..

:li.Bit 1 - Overrun Error

:lp.This bit indicates that the character in the RBR was

overwritten by the next received character before the &dosapp. read

the RBR.  This bit is cleared by reading the LSR.

:lp.In the virtual environment, the overrun error will be indicated

when the &vcom. gets a character from the COM &pdd. that had this

error associated with it.

:li.Bit 2 - Parity Error

:lp.This bit indicates that a parity error was detected in the

received character.  This bit is cleared by reading the LSR.

:lp.In the virtual environment, the parity error will be indicated

when the &vcom. gets a character from the COM &pdd. that had this

error associated with it.

:li.Bit 3 - Framing Error

:lp.This bit indicates that a framing error was detected in the

received character.  In particular, it indicates that the

received character frame did not contain a valid stop bit.  This

bit is cleared by reading the LSR.

:lp.In the virtual environment, the framing error will be indicated

when the &vcom. gets a character from the COM &pdd. that had this

error associated with it.

:li.Bit 4 - Break Indication

:lp.This bit indicates that a break signal was detected.  This

bit is reset by reading the LSR.  In the virtual environment,

when this bit is set the &vcom. RBR is loaded with a zero (00h)

character (received from the COM &pdd.).

:li.Bit 5 - Transmitter Holding Register Empty (THRE)

:lp.This bit indicates that the THR is empty and that the &dosapp.

may write another character to the THR.  In the virtual

environment, this bit is set until the COM &pdd. transmit queue

becomes full.  It is reset when the transmit queue again becomes

not full.

:li.Bit 6 - Transmitter Empty

:lp.This bit is set when the last bit of the last character frame has

been transmitted.  It is reset (zero) as long as there is a character

in the THR or in the shift register.

:lp.In the virtual environment, it is set when the THRE bit is set.

:li.Bit 7 - Reserved

:lp.This bit is always returned as 0.

:eul

:h6.MSR - Modem Status Register

:p.The MSR returns the state of four RS-232 status signals and

indicates if they have changed since the last time the MSR was

read.

:ul.

:li.Bit 0 - CTS Changed

:p.When set, this bit indicates that the CTS signal has changed

since the last time the MSR was read.  This bit is cleared when

the MSR is read.

:li.Bit 1 - DSR Changed

:p.When set, this bit indicates that the DSR signal has changed

since the last time the MSR was read.  This bit is cleared when

the MSR is read.

:li.Bit 2 - Trailing Edge of RI Detected

:p.When set, this bit indicates that the RI signal has changed

from a logic 1 to a logic zero since the last time the MSR was

read.  This bit is cleared when the MSR is read.

:li.Bit 3 - DCD Changed

:p.When set, this bit indicates that the DCD signal has changed

since the last time the MSR was read.  This bit is cleared when

the MSR is read.

:li.Bit 4 - Clear to Send (CTS)

:p.This bit reflects the state of the RS-232 CTS signal.

:li.Bit 5 - Data Set Ready (DSR)

:p.This bit reflects the state of the RS-232 DSR signal.

:li.Bit 6 - Ring Indication

:p.This bit reflects the state of the RS-232 RI signal.

:li.Bit 7 - Data Carrier Detect

:p.This bit reflects the state of the RS-232 DCD signal.

:eul

:h6.SCR - Scratchpad Register

:p.In later replacement components for the INS8250, this byte is

implemented as a one-byte read/write register.  In the virtual

environment, this register may be written, but when read always

returns 0FFh.  This is to indicate to the &dosapp. that the &vdd.

is emulating an INS8250 component.

:h5.&cb. Data Area

:p.The system &cb. Data Area is referenced during &vcom.

initialization, and the VDM instance of &cb. Data Area is initialized

at VDM creation time.  The system &cb. area is not modified, however,

by the &vcom..

:figref REFID=COMFG02 page=no.  lists each &cb. data item

associated with the &vcom. component.  The offsets shown are

relative to the &cb. data segment (0040:0000):

:fig id=COMFG02 frame=none place=inline

.ti > 09

.tb 2 11 23

.bx 1 10 22 63

>      >Initial

>Offset>Value	 >Description

.bx

.in 23

.ir 5

.un 21

.*	      |23456789x123456789x123456789x123456789x1|

0000 >Varies   >COM1 Base.  Normally initialized to

.un 21

>	       >the base I/O address of the first

.un 21

>	       >serial port found. (WORD)

.bx

.un 21

0002 >Varies   >COM2 Base. (WORD)

.bx

.un 21

0004 >Varies   >COM3 Base. (WORD)

.bx

.un 21

0006 >Varies   >COM4 Base - Not Supported (WORD)

.bx

.un 21

0010 >Varies   >Installed hardware.  Bits 9-11

.un 21

>	       >indicate the number of installed

.un 21

>	       >serial controllers. (WORD)

.bx

.un 21

007C >Varies   >COM1 Timeout Counter.  This value

.un 21

>	       >reflects the amount of time the CBIOS

.un 21

>	       >will wait if the device is busy. (BYTE)

.bx

.un 21

007D >Varies   >COM2 Timeout Counter. (BYTE)

.bx

.un 21

007E >Varies   >COM3 Timeout Counter. (BYTE)

.bx

.un 21

007F >Varies   >COM4 Timeout Counter. (BYTE)

.bx off

.in

.ir

:figcap.&vcom. Component &cb. Data Items

.ti 3E 3E

:efig

:note.If the I/O address for a COM port is zero (0) in the &cb. data

area, then I/O to that COM port is not supported.  On the

other hand, if the I/O address for a COM port is NON-zero, the

serial device may be physically present but not available in the

&vdm..  For example, the &cb. data area may contain a valid base

address for the COM4 device, but this device is not supported by the

&cr. &vcom..

:histref refid=COMHS01.&gml.

:enote

:h5.Coexistence with other serial Device Drivers

:p.Since there is always the potential for other device drivers to own

a serial port, the &vcom. will not assume ownership of any devices

for which the COM &pdd. has not installed.  For example, a serial

mouse device driver may be installed and may own the COM1 serial port.

The COM &pdd. will not install for this port, so the &vcom. may

support only the higher numbered serial ports (if installed).

:note.If another &pdd. installs and zeros the COM port base addresses

in the &cb. data area, the &vcom. will not attempt to initialize for

that COM port and will not assume any responsibility to virtualize

that serial device hardware.  This may result in problems for certain

&dosapp.s which rely on the &cb. data area in order to access multiple

serial ports.

:histref refid=COMHS01.&gml.

:enote.

:h4.Exported Interfaces

None.

:h4.Imported Interfaces

.im comimp.sci

:h4.Design Constraints

:ol.

:li.Timing Is Different

:lp.No attempt has been made to insure that real-time behavior

matches that of the real environment, beyond gross performance

objectives.

:eol

:h4.Design Review

Held 2/21/89.

:h3.&vcom. Implementation

:h4.Internal Interfaces

:h4.Implementation

:h4.Implementation Review

:h3.&vcom. Appendix

:h4.Glossary

:h4.Size and Performance Considerations

:h4.Implementation Estimates

:h4.Design History

:history id=COMHS01. &cb. Data Area Initialization

:p.There are five possible &cb. initialization configurations for a

physical COM port:

:ol

:li.Port is not physically present in system

:li.Port was installed exclusively by the &cr. COM &pdd.

:lp.This is the default configuration for a COM port, where only the

&cr. COM &pdd. has installed for that port.

:li.Port was installed by the &cr. COM &pdd. AND by an alternate &pdd.

:lp.This configuration may arise when an alternate &pdd. installs and

properly shares access to the serial device hardware.  This alternate

&pdd. must not ZERO the &cb. data area for that COM port.

:li.Port was allocated to an alternate &pdd. for entire IPL

:lp.In order to own the port for an entire IPL, the alternate &pdd.

must install prior to installation of the &cr. COM PDD or it may

not be able to support a COM port for which the &cr. COM PDD installs.

:hp4.An alternate serial &pdd. should NOT ZERO OUT the &cb. data area for

the COM port(s) that it supports:ehp4..  An example of this type of &pdd is

the &cr. MOUSE PDD.  Some pre-&cr. &os2. &pdd.s may ZERO out the &cb.

data area for a supported COM port.  However, in &cr., this is not

considered compatible behavior for an alternate serial &pdd..

:li.Port is physically present but not supported by &cr. COM &pdd.

:lp.This is a possible configuration for COM3 and COM4 on a &pcat.,

and for COM4 on a &ps2., if the appropriate adapters are installed.

:eol

:p.Under configurations 2 and 3 (above), the COM &pdd.

will be installed and will support commands from the &vcom. for those

ports.

:p.If configuration 1, 4, or 5 applies, the &cr. &vcom. will not support

the COM port.  If no other &vdd. supports that serial port, it will not

be accessible from a &vdm. during that IPL.

:ehistory

:history id=COMHS02. Network Redirected Serial Channel Support

:p.Although support for serial channels redirected from a network

server would be desireable, this support was dropped from the &vcom.

design due to the following reasons:

:ul

:li.Performance

:lp.The overhead of employing a Helper Process layer to interface to

a remote network server COM &pdd. would cause an unacceptably low

performance level on those devices.

:li.Down Level Servers

:lp.It would not be possible to derive the necessary enhanced function

from a remote network server whenever that server had a pre-&cr.

version of &os2. installed, since the COM &pdd. on that server would

not have the required IOCTL enhancements.

:li.System Complexity

:lp.The presence of Helper Processes for remote serial port I/O

required a significant number of new API.  VDM Manager would have

required complex logic to support posting and reporting events to and

from these API.  Also, the &vcom. device driver would have been

significantly more complex in order to support both local and remote

COM port I/O.

:eul.

:ehistory

.fo