Compact Flash Creation and Management using IDFBuilder and SDFCreator

---------------------------------------------------------------------

Last updated: August 21, 2000 by Christopher Glendenning

This document explains the use of IDFBuilder and SDFCreator

to create a flash image and update the flash from the client

during the client-side boot process.

The document formats we use are known as an Image Description File,

or IDF as we will refer to it, and a Software Description File, or SDF.

The IDF is a collection of lines, with each line representing one SDF

file, it's size, CRC value, the date it was created, and where the file

can be found on the server. The IDF files themselves are constructed

using IDFBuilder. An SDF is the same as an IDF with the exception that

each line in an SDF is a file, such as libraries, binaries, text files,

etc.

----------------

Using IDFBuilder

----------------

IDFBuilder is a GUI tool which allows for easy creation of the IDF

file which will be used to create the image for your flash.

The GUI lists all packages you have available to include on a flash

on the left hand side of the window. These packages are all SDF

files which have been created by IBM or custom SDF files which

you have created using SDFCreator.

You select a package from the list on the left and the information

regarding this package will appear on the right hand side of the

window. There will be a description of the package itself, and below

that are the size of the package and a checkbox to indicate whether

the item has been included within the package or not.

To add or remove a package from being included in the IDF file is as

simple as checking or unchecking the checkbox on the bottom right.

You can gather information about the image you currently have in

progress by going to the Options menu and choosing Image Info.

If you will be adding a large number of packages from among the available

list of SDF files, you may wish to go to the Options menu and choose

Add All Packages, and then simply unchecking the few you do not wish

to include in the IDF. Another choice under the Options menu is the

Refresh Directory Listing which is useful for when you have used

SDFCreator to create a new SDF file and wish to refresh the IDFBuilder

listing on the left of available packages (see SDFCreator usage below).

When you wish to create the IDF file, simply going to the File menu

and choosing Save will prompt the person for a filename if this is a new

IDF or simply save it under the current name if you are editing a previously

created IDF file. The Save As will of course allow you to modify the name

the file is saved to.

The second major phase of creating packages and the image is the use of

SDFCreator to build custom software packages. This program can be invoked

from the command line by typing "./SDFCreator" or it can also be spawned

as a separate process from within IDFBuilder using the Options menu command

titled Create Custom SDF.

----------------

Using SDFCreator

----------------

To begin creating a new SDF, choose New under the file menu. A dialog box

will appear asking for the desired SDF package name, the version number,

and the description you wish to give to the package. This is the same

description which will be displayed within IDFBuilder when the package is

selected. The name chosen here and the version are also what you see

on the left side of IDFBuilder for the list of packages.

The view for SDFCreator consists of 4 panes. The top-left pane is an explorer-like

tree interface, allowing you to choose which diretory to examine quickly

and easily. The top-right pane is used to display the contents of the directory

selected. You navigate to the directories where the files to be included

in the SDF are located, and when the files to be added are shown on the sorted

list on the right you will select the file(s) by either clicking one file

at a time and right clicking and selecting the Add option, or by multiple

selections using control or shift-click styles and then adding with the right

click method.

The files you have added will appear in the bottom two panes. The bottom-left

pane is for the Source of the file, which will be used by the server when

updating a client or building the image by looking in this location for the

file specified. The bottom-right pane is for the destination, which is where

you would like the file to be copied to. If their flash cards are to use

a unique or different directory structure for files or programs than what is

on the server, then changing the destination would be where this is specified.

To modify the destination you select the file they wish to edit and then

right clicking and choosing Edit. This brings up a small window allowing

you to edit the path name and either accept or abort the changes by clicking

OK or Cancel respectively. Under the right click menu when done on an item

in the bottom two panes is also an option for remove. Choosing to remove an

entry will remove the listing from both panes and therefore the SDF.

The Settings menu provides many useful commands for you to manipulate the

working SDF. The first command is Default Protocol, through which you can

set a default protocol to be NFS, FTP, or HTTP for all files you add to the SDF.

If you choose to have FTP or HTTP you will be prompted to set the server's

information, which consists of the desired username, the password and the

server's IP. If you elect to not set the server up at this time, you may do

so later by using the Server Settings command. You may also change the protocol

information on a per-file basis by right-clicking on the file in the bottom-right

pane and setting it to whatever you desire for that file. You can change the directory

where you store the SDFs you create by using the Working Directory command.

You may also set the client mount point, which is the directory that the client

machine using this SDF will be booting from on the server. A final option

under the Settings menu is the ability to Save Settings. This will create,

or overwrite, a configuration file with your settings stored for use the

next time the program is launched.

The Actions menu provides a Refresh option which will gather the latest

information about each file in the list, such as permissions, size, crc, and so

forth. This command is very useful for when you have modified a file which is part

of an SDF and you wish for the newest incarnation of the file to be in the SDF

without having to find that file in the file panes on the bottom half of the

program window, remove it manually and then re-add the file to have the changes

acknowledged. Simply invoke the Refresh command and all files in the SDF will

be verified and updated. You can change a package's name, version and description

by invoking the Package Header command under the Edit menu.

--------------------------

Updating the Compact Flash

--------------------------

The program which handles the updates of the client machine is called LCMA,

or Linux Client Management Agent, and it can be utilized in a few different manners.

The methods for updating the flash are through an automatic invocation of the

update program at the client boot time, a manual invocation on the command line,

or through remote administration using SNMP. This program will compare

the IDF which exists on the flash and the one which is of the same name on the

server. If there are no differences between the packages there will be no update

performed and the update program will exit. In the case that the server has

an IDF which matches the name of the one on the client, but the contents of the

IDF are more recent on the server than the client, then an update will occur.

At this time the client will grab the information from the server about the

additional packages to install and begin the copying process to the flash.

Once this is complete the program exits normally. There is no support for

deletion of items from the flash at this time if the server IDF no longer

includes a package that it had used in the past, however this

feature is currently in the works.

If someone on a server has just modified the IDF file which the clients

happen to be booting off of at the time, perhaps to add a new package,

or modify a package which was already included in the case of a bug

fix or adding functionality, they use the IDFBuilder to modify their existing

IDF (and SDFCreator if changes are needed on the package at the SDF level).

Once the changes are saved, the client can issue an update command through one

of the methods mentioned above and the upgrade will be performed.

The lcma program also supports a method to force an update down to the client

which will ignore whatever IDF exists, if any, on the client and the entire

IDF will be copied down to the server without any comparison checking of

any kind.

Lcma supports another option which can be used to send down commands to the client,

such as those within a shell script and any supporting files. The command script

and any files needed for the command are embedded in an SDF file and the lcma

will automatically send down the files and run whatever script is specified.

The command and any resulting files will NOT be added to the IDF, because this would

cause major inconsistencies with the management system where the master IDF files

on the server would be outdated and out-of-synch with the client files.

If you wish to add the command to the IDF, build the script into an IDF file since

the command is actually an SDF, and then the post-download script of the command

line in the SDF will run the command itself.

For more usage information, as well as the syntax for the commands, you can run

./lcma --help

This displays the syntax and usage of the commands available, as well as example

invocations for all events possible.

You may also use SNMP to invoke the lcma program to perform updates. The

information necessary to do this can be found in the separate SNMP documentation.