Intel Confidential

12/11/2001

SR460AC4 Post Platform Release BIOS, Server Management Firmware Update Instructions

Valid BIOS and Firmware combinations are listed in the following table. The table only applies to SR460AC4 qual systems or higher.

Grey : Indicates the version that you are trying to upgrade to.

| BIOS # | FW BMC Ver# | FW HSC Ver # | FW CBC Ver # | SDR Ver # |
| --- | --- | --- | --- | --- |
| B105 | BMC27 | HSC 7 | CBC 02 | SDR 15 |
| B104 | BMC26 | HSC 7 | CBC 02 | SDR 14 |
| B100 | BMC26 | HSC 7 | CBC 02 | SDR 14 |
| B99 | BMC24 | HSC 7 | CBC 02 | SDR 14 |
| B97 | BMC 24 | HSC 7 | CBC 02 | SDR 14 |
| B83 | BMC 22 | HSC 7 | CBC 02 | SDR 13 |
|  | BMC – Baseboard Management Controller. | HSC – Hot Swap controller | CBC – Chassis Bridge Controller | SDR – Sensor Data Records |

To update the SR460AC4 platforms to BIOS B105 and the server management FW BMC 27 and SDR 15 releases, follow the procedure outlined below:

Package Contents of BIOS105_SMFW_pkg:

- BIOS105.zip (contains BIOS Release B105- oemfw105.bin, Iflash64 Ver 1.0 utility, release notes for Bios and Iflash64 utility)

- BMC27.zip (contain BMC Release 27, release notes)

- FRUSDR_v1_02_15.zip (contains the FRUSDR v1.02.15 package and release notes)

- Update_Instructions_Read_Me_First.doc (this file)

- SR460AC4_Platform_Release_FWSW_Components.doc – gives a complete list of BIOS, Server Management Firmware, utilities and OS component list for platform release.

- CMOSRW1_02.zip (contains cmosrw.efi,  cmosrw_release _ notes_v1_02, and bin file cmos105.bin).

Getting Started:

- Unzip all of the above zip files to a blank LS-120 diskette from a Win32 system.

- Determine your BIOS version of the SR460AC4 system that you are trying to upgrade the BIOS and Server management Firmware.

- Boot to EFI shell.

- From the EFI prompt (shell>), type ‘ver’ to find the BIOS version.

- You will find the following string: S460AC4A.86B.XXXX.B.yymmddhhmm

- The BIOS version number is the ‘XXXX’ from the above string.

- Determine the BMC version using the FWUPDATE utility.

- Insert the LS-120 diskette created in step 1 on the SR460AC4 system.

- Map the diskette. Type: map -r

- Find the floppy drive and change to floppy. Type: fs0:

- load IPMI driver ver 1.2. Type: load ipmi.efi

- Using the Fwupdate ver 1.01, Issue the following command:  fwupdate /fwversion

- Read the ‘Op code’ numbers of BMC. They are the current versions of BMC on the system.

- Determine the SDR version by running the FRUSDR utility.

- Type the command:  FRUSDR –d sdr. This command will display the SDR records in your system. During the display hit ‘enter’ when the screen pauses.

- The last SDR record will display the SDR version loaded on the system.

Note: The assumption is that your system is configured to one of the valid configurations as shown in the matrix above.

Standard Upgrade Procedure:

- Insert the LS-120 diskette containing all of the unzipped files from the package in the SR460AC4 system(created in step 1 of ‘Getting Started’).

- Boot to EFI shell.

- Read release notes for IPMI driver ver 1.2.

- Map the diskette. Type: map -r

- Find the floppy drive and change to floppy. Type: fs0:

- load IPMI driver ver 1.2. Type: load ipmi.efi

- Load IPMI driver version 1.2. From the shell prompt, Type: load ipmi.efi

- Read release notes for Fwupdate ver 1.01 and BMC 27 (lnbmc27.txt).

To update BMC firmware release from 26 to 27, type:  fwupdate lnbmc27.hex /upload

- After the successful completion of the firmware update, it will return to the EFI prompt.

- Update the SDR (Sensor Data Records) release from 14 to 15.  SDR 14 & 15 should be updated only on qual system or higher. From the EFI prompt, execute the FRUSDR Ver 1.02.15 utility as follows. Type: frusdr –cfg srcfg09.cfg

- To update the SDR records, type:  1

- This will update the SDR records on the system to SDR 15. After the update, it will return to the EFI prompt.

- Read release notes for BIOS B105 and IFLASH64 Ver 1.0.

- To upgrade from BIOS B100 or B104 to B105, type: iflash64 oemfw105.bin  /update

- Upgrade the B105 CMOS image by typing: cmosrw  –wd   cmos105b.bin

- After the successful completion of the firmware update, it will return to the EFI prompt.

- Reset the system

- If necessary, enter CMOS setup, set the date and time to the current date\time. Save settings. System will automatically reboot to EFI.

Version Verification:

- Determine your BIOS version. Read the version in the BIOS banner string.

- Determine the BMC version using the FWUPDATE utility.

  - Insert the LS-120 diskette created in step 1 on the SR460AC4 system.

  - Map the diskette. Type: map -r

  - Find the floppy drive and change to floppy. Type: fs0:

  - load IPMI driver ver 1.2. Type: load ipmi.efi

  - Using the Fwupdate ver 1.01, Issue the following command:  fwupdate /fwversion

  - Read the ‘Op code’ numbers of BMC. This is the current versions of BMC on the system.

- Determine the SDR version by running the FRUSDR utility.

  - Type the command:  FRUSDR –d sdr. This command will display the SDR records in your system. During the display hit ‘enter’ when the screen pauses.

  - The last SDR record will display the SDR version loaded on the system

After the update, the following should be the final configuration of the system:

BIOS 105, BMC 27, and SDR15

End of update procedure.