XLT86™

8080 to 8086 Assembly Language Translator

USER'S GUIDE

Copyright © 1981

Digital Research, Inc.
P.O. Box 579
801 Lighthouse Avenue
Pacific Grove, CA 93950
(408) 649-3896
TWX 910 360 5001

All Rights Reserved

COPYRIGHT

Copyright © 1981 by Digital Research. All rights reserved. No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, chemical, manual or otherwise, without the prior written permission of Digital Research, Post Office Box 579, Pacific Grove, California, 93950.

This manual is, however, tutorial in nature. Thus, the reader is granted permission to include the example programs, either in whole or in part, in his own programs.

DISCLAIMER

Digital Research makes no representations or warranties with respect to the contents hereof and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose. Further, Digital Research reserves the right to revise this publication and to make changes from time to time in the content hereof without obligation of Digital Research to notify any person of such revision or changes.

TRADEMARKS

CP/M is a registered trademark of Digital Research.  CP/M-86, CP/NET, LINK-80, MP/M, MP/M-86, RMAC, XLT86 and PL/I-80 are trademarks of Digital Research. Z80 is a registered trademark of Zilog, Inc.

The XLT86 User's Guide was prepared using the Digital Research TEX-80 Text Formatter and was printed in the United States of America by Commercial Press/Monterey.

First Printing: September 1981

FOREWORD

XLT86™ is a Digital Research software product that aids in the translation of 8080 assembly language programs to equivalent 8086 programs. XLT86 takes the CP/M™ and MP/M™ environment into account, so that translated programs operate properly under both CP/M-86™ and MP/M-86™. XLT86 can also be used as a teaching tool by examining the output when XLT86 is applied to existing 8080 programs. Unlike other 8086 translators, XLT86 uses global data flow analysis techniques to determine 8080 register usage and reduce the number of generated 8086 instructions.

The XLT86 translator is available for operation under CP/M and MP/M for the 8080, 8085, and Z800 microprocessors with a minimum 40K Transient Program Area (TPA). XLT86 requires a 64K CP/M system to effectively translate any significant 8080 programs. Using a 4Mhz Z80 microprocessor, XLT86 translates programs at approximately 120 to 150 lines per minute, depending upon backup storage access speed. XLT86 is written in PL/I-80™ and thus can be adapted for use on computer systems that support Subset G. Specifically, XLT86 is available for cross-development on the Digital Equipment Corporation VAX 11/750 or 11/780 minicomputer, operating with the standard DEC VMS software. However, programs are supplied in machine code form, so it is not necessary to own PL/I-80 or any of its subsystems to operate XLT86.

The XLT86 system components, including the files XLT86.COM, XLTOO.OVL, and XLT01.OVL, are distributed in IBM-compatible single density disk form. Before operating XLT86, copy these system components to a working disk and save the distribution disk for archive purposes. If the working disk medium can be dismounted, it must be marked with the notice shown below to properly comply with the Software License Agreement:

Copyright © 1981

Digital Research, Inc.

This User's Guide presents the overall translation process, along with operator interface and command syntax. This manual also describes the format of the translated program, including the details of the 8080 to 8086 operation code translation.

TABLE OF CONTENTS

Section 1	THE TRANSLATION PROCESS	1

1.1	Input and Output Files	1

1.2	Translation Phases	1

Section 2	TRANSLATION PARAMETERS	3

2.1	Parameter Syntax	3

2.2	The B (Block Trace) Parameter	3

2.3	The C (Compact) Parameter	4

2.4	The J (Jump) Parameter	5

2.5	The L (List) Parameter	5

2.6	The N (Number) Parameter	6

2.7	The R (Return) Parameter	6

2.8	The S (Segment) Parameter	6

2.9	The 80 Parameter	6

2.10	The NO Parameter	6

Section 3	TRANSLATED PROGRAM FORMAT AND CONTENT	9

3.1	Translated Program Format	9

3.2	Translated Program Content	10

Section 4	XLT86 ERROR MESSAGES	17

4.1	Pseudo-assembly Process Error Messages	17

4.2	Translate-86 Error Messages	17

4.3	Memory overflow	18

Appendix A	SAMPLE PROGRAM TRANSLATIONS	19

# THE TRANSLATION PROCESS

## Input and Output Files

XLT86 reads an 8080 program from a file with type ASM and produces a file of type A86 containing the equivalent translated 8086 assembly language program. The filename for the 8080 source program, as well as filenames for all output files from XLT86, is taken from the command line typed by the operator. For example, the console command:

XLT86 DUMP

executes the XLT86 program using the file "DUMP.ASM" as input. The translation produces the output file "DUMP.A86".

The 8080 source program must be in a form acceptable to the standard Digital Research assembly language translators ASM, MAC, or RMAC. XLT86 processes conditional assembly statements, and produces an output program that results from evaluation of the particular conditions included in the 8080 program. However, macro definitions, macro invocations, and repeat loops are not altered in the translation. To properly translate programs that include macros or repeat loops, first assemble the programs under MAC or RMAC to produce a printer listing file of type PRN. Rename this PRN file to type ASM and edit the file to remove the beginning column positions, resulting in a file acceptable as input to XLT86. The A86 output file is now in a form acceptable to the Digital Research ASM86 assembler, requiring little or no modification for execution under CP/M-86 and MP/M-86.

XLT86 produces two additional files: a PRN file and a $$$ file. A file of type PRN contains error lines and messages along with optional listing and trace information. The PRN file is in a form suitable for listing on the system printer and contains embedded form-feed and tab characters. A temporary file of type $$$ is also created during translation. This temporary file is automatically deleted upon normal completion of XLT86.

The XLT86 program consists of a "root module" called XLT86.COM, which is loaded and executed when you enter the XLT86 command line shown above. There are two additional "overlays" called XLTOO.OVL and XLT01.OVL that must be present on your default disk drive. These two overlays are automatically loaded and executed at the appropriate time during the translation.

## Translation Phases

The translation itself takes place in five phases. Each phase has a specific name that appears at the console during translation so that the operator can monitor the progress of XLT86. Table 1-1 lists the phase names.

| Phase | Meaning |
| --- | --- |
| Symbol Setup | Determines the location of each symbol in the 8080 source program. |
| Setup Blocks | Determine the "Basic Blocks" necessary for the data flow analysis. |
| Join Blocks | Construct a "Directed Graph" connecting each basic block, corresponding to program flow of control. |
| List Blocks | Produce an optional list of Basic Blocks following flow analysis showing register and flag usage for each 8080 instruction. |
| Translate-86 | Translates the 8080 instructions to 8086 form, using the information gathered by the flow analysis. |

Table 1-1. XLT86 Translation Phases

The command line:

XLT86 DUMP

activates the XLT86 translator using the DUMP.ASM program as input. The default action of XLT86 prints the name of each phase at the console as the translation proceeds, as shown below.

Symbol Setup

Setup Blocks

Join Blocks

List Blocks

Translate-86

The files processed by the "XLT86 DUMP" command are shown in Figure 1-1, below.

DUMP.ASM

Source

DUMP.$$$

Temp

XLT 86

DUMP.PRN

Print

DUMP.A86

Output

Figure 1-1. Processed Files

All files are placed on the drive specified by the operator as the prefix on the source filename. In the above example, all files are placed on the current default drive, which must also contain the XLT86 program along with its overlays. An alternative form:

XLT86 B:DUMP

overrides the default drive and obtains the source file from drive B. XLT86 creates the output, temporary, and print files on drive B as well. When several drives are available, it may be advantageous to place the various files on separate disks. In this case, you must use XLT86 "parameters," described in the following section, to override the default values.

# TRANSLATION PARAMETERS

## Parameter Syntax

Several XLT86 parameters can be included in the command line by the operator or embedded within the 8080 source program to control the translation process. Parameters are grouped together into a parameter list enclosed within square brackets:

[p1 p2 … pn]

where p1 through pn denote one or more parameters optionally separated by blanks. When included on the command line, the XLT86 invocation appears as follows:

XLT86 filename [p1 p2 … pn]

When included within the source program, the opening bracket of the parameter list must begin in the first column position. The parameters denoted by p1 through pn are one or two character sequences in upper- or lower-case, with optional intervening blanks, as listed in Table 2-1, below.

| Parameter | Meaning |
| --- | --- |
| Ax | Place the A86 file on drive x where x = A, B, P. |
| B | Produce a list of Basic Blocks in the PRN file. |
| C | Assume the 8086 "compact model" for execution. |
| J | Translate conditional jumps to short conditionals. |
| L | Send the PRN file directly to the system printer. |
| N | Show the line and statement number being processed. |
| P | Place the PRN file on drive x where x = A, B, P. |
| R | Assume all flags active at subroutine returns. |
| S | Assume non-overlapping 8086 code and data segments. |
| T | Place the $$$ file on drive x where x = A, B, … P. |
| 80 | Create an 8080 assembly listing in the PRN file. |
| 86 | Create an 8086 line and statement listing. |

Table 2-1. Translation Parameters

## The B (Block Trace) Parameter

The A (A86), P (PRN), and T (TMP) parameters allow you to select alternate disk drives for use during the translation process when only limited disk space is available on each drive. Otherwise, disk drives are selected as described above.

The B (Block Trace) parameter provides a trace in the PRN file showing register usage information collected by the data flow analyzer. This parameter is not normally selected since the trace information is of no particular value unless you are interested in detailed register usage. The B parameter trace consists of a sequence of register usage tables for each Basic Block in the form shown below.

| Block At 011E (subr), A86 = 083F |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Entry Active: B-D-HL-AOZSPI Exit Active: BCDEHL------- |  |  |  |  |  |  |
| stmt# | opcode uses | op | v1 | v2 | opcode kills | live regs |
| 23 | -------AOZSPI | PUSH | PSW |  | ------------- | B-D-HL-A----- |
| 24 | -------A----- | MOV | E | A | ---E--------- | B-DEHL------- |
| 25 | ------------- | MVI | C | 05 | -C----------- | BCDEHL------- |
| 26 | ------------- | CALL | 0005 |  | ------------- | BCDEHL------- |

The Basic Block address in the original 8080 program is listed and the type of block is identified. The block type is "subr" for subroutines, "code" for main-line code, and "data" for data blocks. The A86 address is an approximation of the corresponding 8086 address used to determine short and long branch jump ranges. The remaining information shows register and flag use at block entry and at each instruction within the block. The registers and flags are displayed as a vector of letters and hyphens, where each letter represents the presence of a register or flag in the display, and each hyphen signifies that the corresponding register or flag is absent in the vector. Given that all registers and flags are present, the display appears as follows:

BCDEHLMAZOSPI

Table 2-2 lists the letter denotations of the above display.

| Letter | Meaning |
| --- | --- |
| B | Register B, or high(BC) |
| C | Register C, or low (BC) |
| D | Register D, or high(DE) |
| E | Register E, or low (DE) |
| H | Register H, or high(HL) |
| L | Register L, or low (HL) |
| M | Register M, memory operand |
| A | Register A, 8-bit Accumulator |
| 0 | overflow Flag, carry or borrow |
| Z | Zero Flag |
| S | Sign Flag |
| P | Even Parity Flag |
| I | Interdigit Carry Flag |

Table 2-2. Letter Denotations for Registers and Flags

The registers active upon entry are listed first. In the example shown above, the data flow analysis has determined that the B, D, and HL registers, along with all flag registers, are in use upon entry to the block. The active registers following this block are then listed, consisting of the BC, DE, and HL register pairs. Then each instruction in the Basic Block is given, with a preceding statement number that can be cross-referenced with the 8080 source program. The instruction itself is listed with the hexadecimal values of its two optional parameters.

The "opcode uses" field shows the register set used by the operation code, while the "opcode kills" field lists the registers destroyed by the operation. The "live registers" field provides the information used by the Translate-86 phase to minimize the generated code. This field lists the registers and flags that are referenced following the instruction and is derived by examining the Directed Graph corresponding to the 8080 source program. Again, the information collected by the flow analyzer is optionally displayed using the B parameter. This display is not required for normal operation of the translator.

## The C (Compact) Parameter

The C (Compact) parameter causes XLT86 to generate 8086 machine code using the "Compact Memory Model" described in the CP/M-86 System Guide. Under normal circumstances, XLT86 assumes the "8080 Memory Model" where code and data segments overlap. To accomplish this overlap of segments, the program is analyzed to determine Basic Blocks that contain code and data. The program is assumed to begin with a code segment and, if a data segment is encountered as defined by a sequence of DS, DB, or DW statements, XLT86 produces the following statements that provide the proper transition:

L@n	EQU	$
DSEG
ORG	Offset L@n

Similarly, the transition from a data segment back to a code sequence is marked by the generated statements:

L@n	EQU 	$
CSEG
ORG	Offset L@n

where L@n is a sequentially generated label. The labels are generated as required by XLT86, taking the form:

L@1	L@2	L@3	L@4	…	L@32767

Enabling the "C" parameter prevents the code and data segments from overlapping. In this case, the transition from code to data and data to code is marked by either DSEG or CSEG respectively. See also the description of the S (Segments) parameter. When enabled, the S parameter completely overrides the C parameter.

## The J (Jump) Parameter

The J (Jump) parameter enables the short jump analysis option of XLT86. When enabled, XLT86 translates 8080 conditional jumps to either short conditional jumps or negated short conditional jumps followed by short unconditional jumps, depending upon the byte count to the target of the jump. That is, a "JZ x" instruction becomes either

JZ x

X.

or

JNZ	L@n

JMPS	x

L@n:

The first case results if the label "x" is within the range of a short jump, while the second form results from a target label beyond the range of a short jump. The J parameter is enabled by default, and should be disabled using the NOJ form described below only if you want to manually edit your conditional jumps following program translation.

## The L (List) Parameter

The L (List) parameter sends the listing file directly to the system printer, thus avoiding the intermediate PRN file. The system printer, or printer driver, must handle form-feeds (ctl-L) and tabs (ctl-I) to every eighth column position. If your printer does not properly support these characters, you can leave the L parameter disabled and use the CP/M PIP utility command form:

PIP LST:=filename.PRN[T8F]

where the PIP parameter "TV expands tabs to blanks at every eighth column, and the "F" parameter deletes the form-feed character on transmission.

## The N (Number) Parameter

The N (Number) parameter displays the current line and statement number on CRT-type console devices as the translation proceeds. Each line and statement number is displayed with an intervening carriage return, without a line feed, so that each successive display overwrites the previous value. In this way, you can easily monitor the progress of XLT86 as it proceeds through the source program during the translation.

## The R (Return) Parameter

The R (Return) parameter overrides the default assumptions about register usage at the end of a subroutine. XLT86, by default, assumes that all registers are in use at the end of a subroutine in the absence of additional information. This is a safe, but possibly restrictive, assumption that might cause more 8086 code to be generated near the return statements of each subroutine. If you know that the entire 8080 program being translated contains subroutines that do not return flag registers, then you should include the R parameter in the command line to reduce the amount of generated code.

Alternatively, you can precede the return statements of various subroutines with "[R]" parameters when they do not return flag registers, as long as balancing "[NOR]" parameters, described below, are included to return to the default assumptions, where necessary.

## The S (Segment) Parameter

The S (Segment) parameter informs XLT86 that the original source program contains embedded CSEG and DSEG directives that delimit the code and data segments. In this case, XLT86 makes no attempt to de-rive the code and data segment information and, instead, assumes that the CSEG and DSEG directives passed through to the 8086 program correctly define the appropriate segments. The S parameter is automatically set when the source program contains ASEG, CSEG, or DSEG directives, and completely overrides the effect of the C (Compact) parameter.

## The 80 Parameter

The 80 parameter causes XLT86 to produce a pseudo-assembly listing of the original 8080 source program, giving the source line and statement number along with the assembled machine code location. If the B parameter is simultaneously enabled, additional Basic Block information precedes each straight-line code segment. When both 80 and B are enabled, the trace appears as shown below:

| --------- Basic Block (2) 011E |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Predecessors: |  | 0119 0111 0105 0100 |  |  |  |
|  |  | Successors : |  | 0125 |  |  |  |
|  |  | Reg's Killed: |  | -C-E -------- |  |  |  |
|  |  | Reg's Used : |  | ------- AOZSPI |  |  |  |
| 22 | 22 |  | 011E |  | pr: |  |  |
| 23 | 23 |  | 011E |  |  | push | psw |
| 24 | 24 |  | 011F |  |  | mov | e,a |
| 25 | 25 |  | 0120 |  |  | mvi | c,lst |
| 26 | 26 |  | 0122 |  |  | call | bdos |

Each Basic Block of the listing is preceded by the Basic Block Header consisting of the location (011E in the example above) , a set of predecessor blocks where the program flow of control comes from (0119, 0111, 0105, and 0100), and a set of successor blocks where program flow could continue (0125, above). The set of registers killed are listed, along with the set of registers used by the operation codes within the block. No global data flow information is displayed in this trace (see the B parameter described earlier).

## The NO Parameter

The two character sequence "NO" preceding the B, C, J, L, N, R, S, 80, and 86 parameters negates the effect of the parameter once it has been set. Further, the A, P, and T parameters are ignored when they occur within the source file and are effective only on the command line. The parameters B, C, J, L, N, R, S, 80, and 86 parameters, along with their negated forms, can occur in the command line or within the source program. When they occur within the program, they apply to the segment of code following their occurrence. Assuming that the default drive is d, where d is a valid drive code A, B, … or P, the default values assumed for each parameter are identical to the complete, but redundant, command line shown below:

XLT86 d:filename [Ad NOB NOC J NOL NON Pd NOS NOR Td N080 N086]

# TRANSLATED PROGRAM FORMAT AND CONTENT

## Translated Program Format

XLT86 constructs the 8086 program from the original 8080 program by first analyzing the program register usage. Then, using the collected information, XLT86 translates each label, operation code, and operand expression into an equivalent 8086 program segment. In performing the translation, XLT86 uses as many program fragments from the original 8080 source program as possible. These program fragments include labels, expressions, and comment fields. Due to differences in assembly language formats, however, labels and expressions might be altered somewhat to maintain their original meaning.

The translation occurs line-by-line, where each 8080 source line may contain several statements delimited by exclamation symbols. XLT86, however, always generates a single statement per output line. The output line includes an optional label in column one, followed by a single tab character. The translated operation code field is placed immediately following the tab character. If the operation code has one or two operand fields, another tab character is included and the operand fields are inserted. The operand fields themselves are constructed by either translating 8080 registers to their 8086 equivalents, or through the construction of an expression that is the translation of the original form. If a comment field is present in the source program, it is copied to the 8086 program intact with sufficient leading tabs to position the comment to column forty, if that position has not already been reached. Comments beginning in column one are reproduced without leading tab characters. Further, comments that begin in column one with the character "*" are started, instead, by the two character sequence ";*" to maintain compatibility with ASM-86.

For pseudo-assembly purposes, the assumed origin of the 8080 program is 0100H, corresponding to the base of the TPA under CP/M. This assumed origin resolves label addresses during pseudo-assembly and does not normally affect the translation process. However, if an ORG statement is encountered at the beginning of the program before any code or data is encountered, the program origin is set to the value given in the operand field of the ORG statement.

Program-relative operand references, along with absolute addresses, are allowed in the source program. In this case, XLT86 generates a label of the form "L@n" at the target location. For example below, the 8080 instruction sequence shown to the left results in the 8086 program shown to the right:

NOP	L@l:	NOP

NOP 		NOP

JMP $-2	JMPS L@l

Similarly, the absolute 8080 assembly language shown to the left below results in the program shown to the right:

|  | ORG | 300H |  |  | ORG | 300H |
| --- | --- | --- | --- | --- | --- | --- |
|  | NOP |  |  | L@l: | NOP |  |
|  | NOP |  |  |  | NOP |  |
|  | JMP | 300H |  |  | JMP | L@l |

In this case, the ORG statement is necessary to override the default assumption.

From this last example, it appears that XLT86 is capable of translating 8080 programs produced through disassembly. Unfortunately, disassemblers cannot generally distinguish between code and data areas. If the code and data sections can be separated into distinct areas, where the code is disassembled with absolute address operands and the data areas consist of DS, DB, and DW operations, then XLT86 performs the translation.

Operand fields are translated according to their context and, for notational purposes, we make the following definitions.

| Abbreviation | Definition | Example |
| --- | --- | --- |
| ib | immediate byte operand | (MVI A,ib) |
| iw | immediate word operand | (LXI H,iw) |
| mb | byte in memory | (STA x) |
| mw | word in memory | (LHLD x) |
| mn | near memory | (CALL x) |
| rb | byte in register | (ADD B) |
| rw | word in register | (DAD B) |

Table 3-1. Operand Field Abbreviations

The translation of an expression is denoted by a prime following the expression type. Thus, ib is translated to ib' , iw to iw', and so forth. Register translation takes place according to the following table.

| 8080 Register (rb) | 8086 Register (rb') |
| --- | --- |
| A | AL |
| B | CH |
| C | CL |
| D | DH |
| E | DL |
| H | BH |
| L | BL |

Table 3-2. Register Translation

The M (Memory) register has no direct equivalent in the 8086 environment, so XLT86 produces an "equate" statement in the following form at the beginning of each program.

M EQU Byte Ptr 0[BX]

Thus, the M register remains unchanged in the translation with the assumption that the BX register contains the offset to the proper memory location.

The 16-bit register pair translation occurs as shown in Table 3-3, below.

| 8080 Register (rw) | 1 8086 Register (rw') |
| --- | --- |
| PSW | AX |
| B | CX |
| D | DX |
| H | BX |
| SP | SP |

Table 3-3. 16-Bit Register Translation

The 8080 PSW and 8086 AX register have a loose correspondence depending upon register usage at the time of translation. The exact correspondence is defined below under the PUSH and POP operators.

## Translated Program Content

Expressions are normally composed of literal constants, data variable references, program label references, and register references. XLT86 computes the type of each expression as the translation proceeds, resulting in one of the following expressions.

| Expression | Meaning |
| --- | --- |
| constant | consists only of literal constants |
| variable | consists of zero or more constants and one or more variable references |
| label | consists of zero or more constants or variables, and one or more labels |
| register | consists of zero or more constants, variables, or labels and one or more register references |

Table 3-4. Expressions

The translation of ib, iw, mb, mw, and mn is described in Table 3-5, below. This translation takes place after XLT86 scans the expression to determine its type, as described above.

| Operand Field | Translation |
| --- | --- |
| ib and iw | ib' and iw' are constructed from the original ib and iw by first determining the expression type. If the type is "constant," the expression ib or iw remains unchanged in the translation. Otherwise, for each variable, label, dollar sign ($) , or register reference in the expression, XLT86 changes the reference, denoted by x, to " (offset x) " so that the resulting expression ib' or iw' represents a CS or DS relative offset computation. |
| mb | The resulting expression mb' is constructed from the original expression mb according to the type of mb. If mb is "constant" then mb' becomes "Byte Ptr mb" denoting a single byte operand located at a literal constant address relative to DS or CS. Otherwise, the expression mb' becomes "Byte Ptr mb" denoting a byte variable or label address. |
| mw | Similar to mb, mw1 becomes "Byte Ptr .mw" if mw is "constant" and "Byte Ptr mw" otherwise. |
| mn | The expression mn' is the same as the original mn unless there is no literal label at the target address. In this latter case, a label of the form "L@n" is created at the target address, which becomes the value of mn |

Table 3-5. Operand Field Translation

Due to differences in 8080 and 8086 program formulation requirements, not all valid 8080 expressions can be successfully converted to valid 8086 expressions. Thus, you must be aware that additional editing is required if your translated program produces errors during assembly with ASM-86. In particular, expressions that use arbitrary operations upon constants, variables, labels, and registers are unlikely to assemble correctly under ASM-86, or any other assembler that uses the Intel conventions.

In the translation table given below, the 8080 operation code is shown to the left, with the translated 8086 code sequence shown to the right. In many cases, the registers that are live at the point where the 8080 operation code occurs determine the exact sequence of code that is generated. In these cases, the alternative forms are given separately. Conditional assembly notation specifies the alternative forms, with the introduction of the following two pseudo-functions:

live(rl,r2, …, rn)

and

short(mn')

The "live" function takes a variable number of register arguments and results in a TRUE value if one or more of these registers is live at the point of translation. Otherwise, the "live" function results in a FALSE value. In the Section 2 example for the B parameter, statement 24 (MOV E,A) has the live register set given by the vector:

B-DEHL -------

so that

live(B,C,D) = TRUE	and	live(A,O) = FALSE

The "short" function is used in the translation of conditional jump instructions where the value of short(mn') is TRUE if the target of the translated jump address mn' is within the range of a conditional jump, or if the "J" parameter is enabled. otherwise, short(mn') results in a FALSE value. XLT86 also uses the notation in Section 2 for label generation. The form "L@n" represents labels produced sequentially, starting at n = 1, used in the translation of conditional calls, returns, and conditional jumps outside the range of an 8086 conditional transfer. The CC (Call if Carry) operator, for example, translates to a jump conditional to a generated label followed by a direct call. The generated label is then inserted, as shown in the expansion of the 8080 instruction CC SUBR:

JNB L@1

CALL SUBR

L@l:

Table 3-6 gives the translation of each operation code. Note in particular that the following BDOS entry operations:

CALL 0	CALL 5	imp 0	JMP 5

are treated as special cases that are translated to Interrupt 224, reserved by Intel Corporation for entry to CP/M-86 and MP/M-86.

| Operation Code | Translation |
| --- | --- |
| ACI ib | ADC AL,ibl |
| ADC rb | ADC AL,rb' |
| ADD rb | ADD AL,rb' |
| ADI ib | ADD AL,ib |
| ANA rb | AND AL,rb' |
| ANI rb | AND AL,rb' |
| CALL 0 | MOV CL,O MOV DL,O INT 224 |
| CALL 5 | INT 224 |
| CALL mn | CALL mn' |
| CC mn | JNB L@n CALL mn' L@n: |
| CM mn | JNS L@n CALL mn' L@n: |
| CMA | NOT AL |
| CMC | CMC |
| CMP rb | CMP AL,rb' |
| CNC mn | JNAE L@n CALL mn' L@n: |
| CNZ mn | JZ L@n CALL mn' L@n: |
| CP mn | JS L@n CALL mn' L@n: |
| CPE mn | JNP L@n CALL mn' L@n: |
| CPI ib | CMP AL,ib |
| CPO mn | JP L@n CALL mn' L@n: |
| CZ mn | JNZ L@n CALL mn' L@n: |
| DAA | DAA |
| DAD rw | IF rw = H SHL BX,1 ELSE IF live(O) AND NOT live(Z,S,P,I) ADD BX,rw1 ELSE IF NOT live(O) AND live(Z,S,P,I) LAHF ADD BX,rw' SAHF ELSE LAHF ADD BX,rw' RCR SI,l SAHF RCL SI,l ENDIF ENDIF ENDIF |
| DEC rb | DEC rb1 |
| DCX rw | DEC rwl |
| DI | CLI |
| EI | STI |
| HLT | HLT |
| IN ib | IN AL,ibl |
| INR rb | INC rb' |
| INX rw | IF NOT live(Z,S,P,I) INC rw' ELSE LAHF INC rw' SAHF ENDIF |
| JC mn | IF short(mn') JB mn' ELSE JNB L@n JMPS mn' L@n: ENDIF |
| JM mn | IF short(mn') JS mn' ELSE JNS L@n JMPS mn' L@n: ENDIF |
| imp 0 | MOV CL,O MOV DL,O INT 224 RET |
| imp 5 | INT 224 RET |
| JMP mn | JMPS mn' |
| JNC mn | IF short(mn') JNB mn' ELSE JNAE L@n JMPS mn' L@n: ENDIF |
| JNZ mn | IF short(mn') JNZ mn' ELSE JZ L@n JMPS mn' L@n: ENDIF |
| JP mn | IF short(mn') JNS mn' ELSE JS L@n JMPS mn' L@n: ENDIF |
| JPE mn | IF short(mn') JPE mn' ELSE JNP L@n JMPS mn' L@n: ENDIF |
| JPO mn | IF short(mn') JPO mn' ELSE JP L@n JMPS mn' L@n: ENDIF |
| JZ mn | IF short(mn') JZ mn' ELSE JNZ L@n JMPS mn' L@n: ENDIF |
| LDA mb | MOV AL,mb' |
| LDAX rw | MOV SI,rw1 MOV AL,[SI] |
| LHLD mw | MOV BX,mw' |
| LXI rw,iw | MOV rwl,iwl |
| MOV rbl,rb2 | MOV rbl',rb2l |
| MVI rb,ib | MOV rb',ib' |
| NOP | NOP |
| ORA rb | OR AL,rb' |
| ORI ib | OR AL,rb' |
| OUT ib | OUT ib',AL |
| PCHL | JMP BX |
| POP rw | POP rwl IF rw = PSW AND live(OIZIS'Pli) XCHG AL,AH SAHF ELSE IF rw = PSW AND live(A) XCHG AL,AH ENDIF ENDIF |
| PUSH rw | IF rw = PSW AND live(A) LAHF XCHG AL,AH PUSH AX XCHG AL,AH ELSE if rw = PSW LAHF XCHG AL,AH PUSH AX ELSE PUSH rwl ENDIF ENDIF |
| RAL | RCL AL,l |
| RAR | RCR AL,l |
| RC | JNB L@n RET L@n: |
| RET | RET |
| RLC | ROL AL,l |
| RM | JNS L@n RET L@n: |
| RNC | JNAE L@n RET L@n: |
| RNZ | JZ L@n RET L@n: |
| RP | JS L@n RET L@n: |
| RPE | JNP L@n RET L@n: |
| RPO | JP L@n RET L@n: |
| RRC | ROR AL,l |
| RST ib | INT ibl |
| RZ | JNZ L@n RET L@n: |
| SBB rb | SBB AL,rb' |
| SBI ib | SBB AL,ibl |
| SHLD mw | MOV mw',BX |
| SPHL | MOV SP,BX |
| STA mb | MOV mb',AL |
| STAX rw | MOV DI,rw1 MOV [DI1,AL |
| STC | STC |
| SUB rb | SUB AL,rb' |
| SUI ib | SUB AL,ibl |
| XCHG | XCHG BX,DX |
| XRA rb | XOR AL,rb' |
| XRI ib | XOR AL,ib |
| XTHL | MOV BP,SP XCHG BX,[BPI |

Table 3-6. Translation Table

# XLT86 ERROR MESSAGES

## Pseudo-assembly Process Error Messages

XLT86 issues error messages that fall into two categories: those produced by the pseudo-assembly process, and those produced during translation. Errors in the first category are not considered fatal, but are simply annotated in the source listing file following the line in which the error occurs. If errors are present, the message:

Number of Errors: n

is displayed at the console following the pseudo-assembly. Examine the PRN file to determine if the errors are significant. Error messages take the form:

** Error: e **, Near t

where e is one of the error codes, and t is a program element near the position where the error occurred. Table 4-1 lists the error codes.

| Error Code | Meaning |
| --- | --- |
| Bad Flag | invalid parameter list [pl pn] |
| Balance | Unmatched right parenthesis or missing trailing string quote. |
| Boundary | Invalid program boundary, usually results from a branch to the middle of an instruction. |
| Convert | Cannot convert an operand to internal form. |
| End-Line | The end of a program line contains extraneous characters. |
| Exp Ovfl | Expression stack overflow; the expression is nested too deeply. |
| Gtr 7 | An expression produced a value greater than 7, where a value from 0-7 is required. |
| Gtr 255 | An expression produced a value greater than 255, where a value from 0-255 is required. |
| Mov M,M? | The source line contains the invalid instruction MOV M,M. |
| No Comma | Missing comma where comma is required. |
| No Value | A label or variable was encountered that does not have an assigned value. |
| Not Impl | The instruction or directive is not implemented in XLT86. |
| Phase | A label or variable has a different value on two passes through the source program. |
| Str Len | A string was encountered that exceeds the capacity of XLT86, check for missing right quote mark. |
| Value | The value produced by an expression is not compatible with the context in which it occurs. |

Table 4-1. XLT86 Error Codes

## Translate-86 Error Messages

The Translate-86 phase also produces a limited number of error messages. All errors produced by this phase are fatal, and cause immediate termination of XLT86. Table 4-2 lists these error messages.

| Error Message | Meaning |
| --- | --- |
| Bad Oper | Invalid 8080 operation code was encountered during translation; probably due to bad disk 1/0 operation. Check for hardware controller faults. |
| Not BDOS | A CALL or JMP occurred below the base origin of the program where the target is not OOOOH (warm boot) or0005H (BDOS entry). |
| Phase (B) | The Directed Graph does not correspond to the source program at the Basic Block level; usually due to a hardware malfunction. |
| Phase (S) | The Directed Graph does not correspond to the source program at the statement level; usually due to a hardware malfunction |

Table 4-2. Translate-86 Error Messages

An error produced by Translate-86 is accompanied by the console error message:

Fatal Error (See PRN file)

to indicate that such an error occurred.

## Memory overflow

The XLT86 program occupies approximately 30K bytes of main memory. The remainder of memory, up to the base of CP/M, stores the program graph that represents the 8086 program being translated. The error message:

ERROR (7) "Free Space Exhausted"

is issued if the program graph exceeds available memory. A 64K CP/M system allows translation of 8080 programs of up to approximately 6K.

The above error causes XLT86 to terminate. To continue, you must divide your source program into smaller modules and retry the translation.

# SAMPLE PROGRAM TRANSLATIONS

The DUMP.ASM program presented here and normally included as a sample assembly language program with CP/M illustrates the translation process. The XLT86 command line:

XLT86 DUMP [80 86]

produces the first example shown below. The "80" parameter selects the 8080 program listing option, while the "86" parameter selects the 8086 listing option. XLT86 places full lines of dashes (" ---- 11) between the Basic Blocks in the 8080 listing. This translation of the DUMP program, however, requires modification to run under CP/M 86. In particular, the DUMP.ASM program contains initialization code that saves the entry SP (statements 34 to 37) and resets the SP to a local stack (statement 39). The return statement following the FINIS label (statement 95) returns control to the CCP.

To perform an exactly equivalent sequence of operations, you must also save the stack segment register (SS) upon entry to the DUMP program, and restore this value before executing the return. Further, the simple RET operation must be replaced by a Far Return (RETF) to balance the original Far Call from the CCP. A simpler solution is to eliminate the initialization code (statements 33 through 39) and use the CCP's built-in 96 byte stack. Control returns to the CCP by executing a RETF at statement 95. If you want to use a local stack, set the SS register to the value of DS upon entry, and set SP to the Offset of STKTOP. Control returns to the CCP through execution of function call #0 in place of the RET in statement 95, as follows:

MOV CL,O

MOV DL,O

INT 224

The second listing shows the Basic Block information collected by the flow analyzer, and produced by the command line:

XLT86 DUMP [B]

where the "B" parameter selects the Basic Block trace. Under normal circumstances, either of the commands shown below are sufficient and reduce the amount of trace information:

XLT86 DUMP [N]

or

XLT86 DUMP

The first command is used only with a CRT-type device where the carriage-return character does not cause an automatic line-feed (see the description of the "N" parameter).

-------------------------------------------------------------------

1       1 0100   ;	FILE DUMP PROGRAM, READS AN INPUT FILE AND PRINTS IN HEX

2       2 0100   ;

3       3 0100   ;	COPYRIGHT (C) 1975, 1976, 1977, 1978

4       4 0100   ;	DIGITAL RESEARCH

5       5 0100   ;	BOX 579, PACIFIC GROVE

6       6 0100   ;	CALIFORNIA, 93950

7       7 0100   ;

8       8 0100   	ORG	100H

9       9 0100   BDOS	EQU	0005H	;DOS ENTRY POINT

10      10 0100   CONS	EQU	1	;READ CONSOLE

11      11 0100   TYPEF	EQU	2	;TYPE FUNCTION

12      12 0100   PRINTF	EQU	9	;BUFFER PRINT ENTRY

13      13 0100   BRKF	EQU	11	;BREAK KEY FUNCTION (TRUE IF CHAR READY)

14      14 0100   OPENF	EQU	15	;FILE OPEN

15      15 0100   READF	EQU	20	;READ FUNCTION

16      16 0100   ;

17      17 0100   FCB	EQU	5CH	;FILE CONTROL BLOCK ADDRESS

18      18 0100   BUFF	EQU	80H	;INPUT DISK BUFFER ADDRESS

19      19 0100   ;

20      20 0100   ;	NON GRAPHIC CHARACTERS

21      21 0100   CR	EQU	0DH	;CARRIAGE RETURN

22      22 0100   LF	EQU	0AH	;LINE FEED

23      23 0100   ;

24      24 0100   ;	FILE CONTROL BLOCK DEFINITIONS

25      25 0100   FCBDN	EQU	FCB+0	;DISK NAME

26      26 0100   FCBFN	EQU	FCB+1	;FILE NAME

27      27 0100   FCBFT	EQU	FCB+9	;DISK FILE TYPE (3 CHARACTERS)

28      28 0100   FCBRL	EQU	FCB+12	;FILE'S CURRENT REEL NUMBER

29      29 0100   FCBRC	EQU	FCB+15	;FILE'S RECORD COUNT (0 TO 128)

30      30 0100   FCBCR	EQU	FCB+32	;CURRENT (NEXT) RECORD NUMBER (0 TO 127)

31      31 0100   FCBLN	EQU	FCB+33	;FCB LENGTH

32      32 0100   ;

33      33 0100   ;	SET UP STACK

34      34 0100   	LXI	H,0

35      35 0103   	DAD	SP

36      36 0104   ;	ENTRY STACK POINTER IN HL FROM THE CCP

37      37 0104   	SHLD	OLDSP

38      38 0107   ;	SET SP TO LOCAL STACK AREA (RESTORED AT FINIS)

39      39 0107   	LXI	SP,STKTOP

40      40 010A   ;	READ AND PRINT SUCCESSIVE BUFFERS

41      41 010A   	CALL	SETUP	;SET UP INPUT FILE

-------------------------------------------------------------------

42      42 010D   	CPI	255	;255 IF FILE NOT PRESENT

43      43 010F   	JNZ	OPENOK	;SKIP IF OPEN IS OK

-------------------------------------------------------------------

44      44 0112   ;

45      45 0112   ;	FILE NOT THERE, GIVE ERROR MESSAGE AND RETURN

46      46 0112   	LXI	D,OPNMSG

47      47 0115   	CALL	ERR

-------------------------------------------------------------------

48      48 0118   	JMP	FINIS	;TO RETURN

-------------------------------------------------------------------

49      49 011B   ;

50      50 011B   OPENOK:	;OPEN OPERATION OK, SET BUFFER INDEX TO END

51      51 011B   	MVI	A,80H

52      52 011D   	STA	IBP	;SET BUFFER POINTER TO 80H

53      53 0120   ;	HL CONTAINS NEXT ADDRESS TO PRINT

54      54 0120   	LXI	H,0	;START WITH 0000

-------------------------------------------------------------------

55      55 0123   ;

56      56 0123   GLOOP:

57      57 0123   	PUSH	H	;SAVE LINE POSITION

58      58 0124   	CALL	GNB

-------------------------------------------------------------------

59      59 0127   	POP	H	;RECALL LINE POSITION

60      60 0128   	JC	FINIS	;CARRY SET BY GNB IF END FILE

-------------------------------------------------------------------

61      61 012B   	MOV	B,A

62      62 012C   ;	PRINT HEX VALUES

63      63 012C   ;	CHECK FOR LINE FOLD

64      64 012C   	MOV	A,L

65      65 012D   	ANI	0FH	;CHECK LOW 4 BITS

66      66 012F   	JNZ	NONUM

-------------------------------------------------------------------

67      67 0132   ;	PRINT LINE NUMBER

68      68 0132   	CALL	CRLF

-------------------------------------------------------------------

69      69 0135   ;

70      70 0135   ;	CHECK FOR BREAK KEY

71      71 0135   	CALL	BREAK

-------------------------------------------------------------------

72      72 0138   ;	ACCUM LSB = 1 IF CHARACTER READY

73      73 0138   	RRC		;INTO CARRY

74      74 0139   	JC	FINIS	;DON'T PRINT ANY MORE

-------------------------------------------------------------------

75      75 013C   ;

76      76 013C   	MOV	A,H

77      77 013D   	CALL	PHEX

-------------------------------------------------------------------

78      78 0140   	MOV	A,L

79      79 0141   	CALL	PHEX

-------------------------------------------------------------------

80      80 0144   NONUM:

81      81 0144   	INX	H	;TO NEXT LINE NUMBER

82      82 0145   	MVI	A,' '

83      83 0147   	CALL	PCHAR

-------------------------------------------------------------------

84      84 014A   	MOV	A,B

85      85 014B   	CALL	PHEX

-------------------------------------------------------------------

86      86 014E   	JMP	GLOOP

-------------------------------------------------------------------

87      87 0151   ;

88      88 0151   FINIS:

89      89 0151   ;	END OF DUMP, RETURN TO CCP

90      90 0151   ;	(NOTE THAT A JMP TO 0000H REBOOTS)

91      91 0151   	CALL	CRLF

-------------------------------------------------------------------

92      92 0154   	LHLD	OLDSP

93      93 0157   	SPHL

94      94 0158   ;	STACK POINTER CONTAINS CCP'S STACK LOCATION

95      95 0158   	RET		;TO THE CCP

-------------------------------------------------------------------

96      96 0159   ;

97      97 0159   ;

98      98 0159   ;	SUBROUTINES

99      99 0159   ;

100     100 0159   BREAK:	;CHECK BREAK KEY (ACTUALLY ANY KEY WILL DO)

101     101 0159   	PUSH H! PUSH D! PUSH B; ENVIRONMENT SAVED

102     104 015C   	MVI	C,BRKF

103     105 015E   	CALL	BDOS

-------------------------------------------------------------------

104     106 0161   	POP B! POP D! POP H; ENVIRONMENT RESTORED

105     109 0164   	RET

-------------------------------------------------------------------

106     110 0165   ;

107     111 0165   PCHAR:	;PRINT A CHARACTER

108     112 0165   	PUSH H! PUSH D! PUSH B; SAVED

109     115 0168   	MVI	C,TYPEF

110     116 016A   	MOV	E,A

111     117 016B   	CALL	BDOS

-------------------------------------------------------------------

112     118 016E   	POP B! POP D! POP H; RESTORED

113     121 0171   	RET

-------------------------------------------------------------------

114     122 0172   ;

115     123 0172   CRLF:

116     124 0172   	MVI	A,CR

117     125 0174   	CALL	PCHAR

-------------------------------------------------------------------

118     126 0177   	MVI	A,LF

119     127 0179   	CALL	PCHAR

-------------------------------------------------------------------

120     128 017C   	RET

-------------------------------------------------------------------

121     129 017D   ;

122     130 017D   ;

123     131 017D   PNIB:	;PRINT NIBBLE IN REG A

124     132 017D   	ANI	0FH	;LOW 4 BITS

125     133 017F   	CPI	10

126     134 0181   	JNC	P10

-------------------------------------------------------------------

127     135 0184   ;	LESS THAN OR EQUAL TO 9

128     136 0184   	ADI	'0'

129     137 0186   	JMP	PRN

-------------------------------------------------------------------

130     138 0189   ;

131     139 0189   ;	GREATER OR EQUAL TO 10

132     140 0189   P10:	ADI	'A' - 10

-------------------------------------------------------------------

133     141 018B   PRN:	CALL	PCHAR

-------------------------------------------------------------------

134     142 018E   	RET

-------------------------------------------------------------------

135     143 018F   ;

136     144 018F   PHEX:	;PRINT HEX CHAR IN REG A

137     145 018F   	PUSH	PSW

138     146 0190   	RRC

139     147 0191   	RRC

140     148 0192   	RRC

141     149 0193   	RRC

142     150 0194   	CALL	PNIB	;PRINT NIBBLE

-------------------------------------------------------------------

143     151 0197   	POP	PSW

144     152 0198   	CALL	PNIB

-------------------------------------------------------------------

145     153 019B   	RET

-------------------------------------------------------------------

146     154 019C   ;

147     155 019C   ERR:	;PRINT ERROR MESSAGE

148     156 019C   ;	D,E ADDRESSES MESSAGE ENDING WITH "$"

149     157 019C   	MVI	C,PRINTF	;PRINT BUFFER FUNCTION

150     158 019E   	CALL	BDOS

-------------------------------------------------------------------

151     159 01A1   	RET

-------------------------------------------------------------------

152     160 01A2   ;

153     161 01A2   ;

154     162 01A2   GNB:	;GET NEXT BYTE

155     163 01A2   	LDA	IBP

156     164 01A5   	CPI	80H

157     165 01A7   	JNZ	G0

-------------------------------------------------------------------

158     166 01AA   ;	READ ANOTHER BUFFER

159     167 01AA   ;

160     168 01AA   ;

161     169 01AA   	CALL	DISKR

-------------------------------------------------------------------

162     170 01AD   	ORA	A	;ZERO VALUE IF READ OK

163     171 01AE   	JZ	G0	;FOR ANOTHER BYTE

-------------------------------------------------------------------

164     172 01B1   ;	END OF DATA, RETURN WITH CARRY SET FOR EOF

165     173 01B1   	STC

166     174 01B2   	RET

-------------------------------------------------------------------

167     175 01B3   ;

168     176 01B3   G0:	;READ THE BYTE AT BUFF+REG A

169     177 01B3   	MOV	E,A	;LS BYTE OF BUFFER INDEX

170     178 01B4   	MVI	D,0	;DOUBLE PRECISION INDEX TO DE

171     179 01B6   	INR	A	;INDEX=INDEX+1

172     180 01B7   	STA	IBP	;BACK TO MEMORY

173     181 01BA   ;	POINTER IS INCREMENTED

174     182 01BA   ;	SAVE THE CURRENT FILE ADDRESS

175     183 01BA   	LXI	H,BUFF

176     184 01BD   	DAD	D

177     185 01BE   ;	ABSOLUTE CHARACTER ADDRESS IS IN HL

178     186 01BE   	MOV	A,M

179     187 01BF   ;	BYTE IS IN THE ACCUMULATOR

180     188 01BF   	ORA	A	;RESET CARRY BIT

181     189 01C0   	RET

-------------------------------------------------------------------

182     190 01C1   ;

183     191 01C1   SETUP:	;SET UP FILE

184     192 01C1   ;	OPEN THE FILE FOR INPUT

185     193 01C1   	XRA	A	;ZERO TO ACCUM

186     194 01C2   	STA	FCBCR	;CLEAR CURRENT RECORD

187     195 01C5   ;

188     196 01C5   	LXI	D,FCB

189     197 01C8   	MVI	C,OPENF

190     198 01CA   	CALL	BDOS

-------------------------------------------------------------------

191     199 01CD   ;	255 IN ACCUM IF OPEN ERROR

192     200 01CD   	RET

-------------------------------------------------------------------

193     201 01CE   ;

194     202 01CE   DISKR:	;READ DISK FILE RECORD

195     203 01CE   	PUSH H! PUSH D! PUSH B

196     206 01D1   	LXI	D,FCB

197     207 01D4   	MVI	C,READF

198     208 01D6   	CALL	BDOS

-------------------------------------------------------------------

199     209 01D9   	POP B! POP D! POP H

200     212 01DC   	RET

-------------------------------------------------------------------

201     213 01DD   ;

202     214 01DD   ;	FIXED MESSAGE AREA

203     215 01DD   SIGNON:	DB	'FILE DUMP VERSION 1.4$'

204     216 01F3   OPNMSG:	DB	CR,LF

205     217 01F5   ;

206     218 01F5   ;	VARIABLE AREA

207     219 01F5   IBP:	DS	2	;INPUT BUFFER POINTER

208     220 01F7   OLDSP	DS	2	;ENTRY STACK POINTER VALUE

209     221 01F9   ;

210     222 01F9   ;	STACK AREA

211     223 01F9   	DS	64	;RESERVE 32 LEVEL STACK

-------------------------------------------------------------------

212     224 0239   STKTOP:

213     225 0239   ;

214     226 0239   	END

215     227 0239

1      0  M	EQU	Byte Ptr 0[BX]

1      1  ;	FILE DUMP PROGRAM, READS AN INPUT FILE AND PRINTS IN HEX

2      2  ;

3      3  ;	COPYRIGHT (C) 1975, 1976, 1977, 1978

4      4  ;	DIGITAL RESEARCH

5      5  ;	BOX 579, PACIFIC GROVE

6      6  ;	CALIFORNIA, 93950

7      7  ;

8      8  	ORG	100H

9      9  BDOS	EQU	0005H		;DOS ENTRY POINT

10     10  CONS	EQU	1		;READ CONSOLE

11     11  TYPEF	EQU	2		;TYPE FUNCTION

12     12  PRINTF	EQU	9		;BUFFER PRINT ENTRY

13     13  BRKF	EQU	11		;BREAK KEY FUNCTION (TRUE IF CHAR READY)

14     14  OPENF	EQU	15		;FILE OPEN

15     15  READF	EQU	20		;READ FUNCTION

16     16  ;

17     17  FCB	EQU	5CH		;FILE CONTROL BLOCK ADDRESS

18     18  BUFF	EQU	80H		;INPUT DISK BUFFER ADDRESS

19     19  ;

20     20  ;	NON GRAPHIC CHARACTERS

21     21  CR	EQU	0DH		;CARRIAGE RETURN

22     22  LF	EQU	0AH		;LINE FEED

23     23  ;

24     24  ;	FILE CONTROL BLOCK DEFINITIONS

25     25  FCBDN	EQU	FCB+0		;DISK NAME

26     26  FCBFN	EQU	FCB+1		;FILE NAME

27     27  FCBFT	EQU	FCB+9		;DISK FILE TYPE (3 CHARACTERS)

28     28  FCBRL	EQU	FCB+12		;FILE'S CURRENT REEL NUMBER

29     29  FCBRC	EQU	FCB+15		;FILE'S RECORD COUNT (0 TO 128)

30     30  FCBCR	EQU	FCB+32		;CURRENT (NEXT) RECORD NUMBER (0 TO 127)

31     31  FCBLN	EQU	FCB+33		;FCB LENGTH

32     32  ;

33     33  ;	SET UP STACK

34     34  	MOV	BX,0

35     35  	ADD	BX,SP

36     36  ;	ENTRY STACK POINTER IN HL FROM THE CCP

37     37  	MOV	Word Ptr OLDSP,BX

38     38  ;	SET SP TO LOCAL STACK AREA (RESTORED AT FINIS)

39     39  	MOV	SP,(Offset STKTOP)

40     40  ;	READ AND PRINT SUCCESSIVE BUFFERS

41     41  	CALL	SETUP			;SET UP INPUT FILE

42     42  	CMP	AL,255			;255 IF FILE NOT PRESENT

43     43  	JNZ	OPENOK			;SKIP IF OPEN IS OK

44     44  ;

45     45  ;	FILE NOT THERE, GIVE ERROR MESSAGE AND RETURN

46     46  	MOV	DX,(Offset OPNMSG)

47     47  	CALL	ERR

48     48  	JMPS	FINIS			;TO RETURN

49     49  ;

50     50  OPENOK:			;OPEN OPERATION OK, SET BUFFER INDEX TO END

51     51  	MOV	AL,80H

52     52  	MOV	Byte Ptr IBP,AL		;SET BUFFER POINTER TO 80H

53     53  ;	HL CONTAINS NEXT ADDRESS TO PRINT

54     54  	MOV	BX,0			;START WITH 0000

55     55  ;

56     56  GLOOP:

57     57  	PUSH	BX			;SAVE LINE POSITION

58     58  	CALL	GNB

59     59  	POP	BX			;RECALL LINE POSITION

60     60  	JB	FINIS			;CARRY SET BY GNB IF END FILE

61     61  	MOV	CH,AL

62     62  ;	PRINT HEX VALUES

63     63  ;	CHECK FOR LINE FOLD

64     64  	MOV	AL,BL

65     65  	AND	AL,0FH			;CHECK LOW 4 BITS

66     66  	JNZ	NONUM

67     67  ;	PRINT LINE NUMBER

68     68  	CALL	CRLF

69     69  ;

70     70  ;	CHECK FOR BREAK KEY

71     71  	CALL	BREAK

72     72  ;	ACCUM LSB = 1 IF CHARACTER READY

73     73  	ROR	AL,1			;INTO CARRY

74     74  	JB	FINIS			;DON'T PRINT ANY MORE

75     75  ;

76     76  	MOV	AL,BH

77     77  	CALL	PHEX

78     78  	MOV	AL,BL

79     79  	CALL	PHEX

80     80  NONUM:

81     81  	LAHF				;TO NEXT LINE NUMBER

81     81  	INC	BX

81     81  	SAHF

82     82  	MOV	AL,' '

83     83  	CALL	PCHAR

84     84  	MOV	AL,CH

85     85  	CALL	PHEX

86     86  	JMPS	GLOOP

87     87  ;

88     88  FINIS:

89     89  ;	END OF DUMP, RETURN TO CCP

90     90  ;	(NOTE THAT A JMP TO 0000H REBOOTS)

91     91  	CALL	CRLF

92     92  	MOV	BX,Word Ptr OLDSP

93     93  	MOV	SP,BX

94     94  ;	STACK POINTER CONTAINS CCP'S STACK LOCATION

95     95  	RET				;TO THE CCP

96     96  ;

97     97  ;

98     98  ;	SUBROUTINES

99     99  ;

100    100  BREAK:				;CHECK BREAK KEY (ACTUALLY ANY KEY WILL DO)

101    101  	PUSH	BX

101    102  	PUSH	DX

101    103  	PUSH	CX			; ENVIRONMENT SAVED

102    104  	MOV	CL,BRKF

103    105  	INT	224

104    106  	POP	CX

104    107  	POP	DX

104    108  	POP	BX			; ENVIRONMENT RESTORED

105    109  	RET

106    110  ;

107    111  PCHAR:					;PRINT A CHARACTER

108    112  	PUSH	BX

108    113  	PUSH	DX

108    114  	PUSH	CX			; SAVED

109    115  	MOV	CL,TYPEF

110    116  	MOV	DL,AL

111    117  	INT	224

112    118  	POP	CX

112    119  	POP	DX

112    120  	POP	BX			; RESTORED

113    121  	RET

114    122  ;

115    123  CRLF:

116    124  	MOV	AL,CR

117    125  	CALL	PCHAR

118    126  	MOV	AL,LF

119    127  	CALL	PCHAR

120    128  	RET

121    129  ;

122    130  ;

123    131  PNIB:					;PRINT NIBBLE IN REG A

124    132  	AND	AL,0FH			;LOW 4 BITS

125    133  	CMP	AL,10

126    134  	JNB	P10

127    135  ;	LESS THAN OR EQUAL TO 9

128    136  	ADD	AL,'0'

129    137  	JMPS	PRN

130    138  ;

131    139  ;	GREATER OR EQUAL TO 10

132    140  P10:	ADD	AL,'A' - 10

133    141  PRN:	CALL	PCHAR

134    142  	RET

135    143  ;

136    144  PHEX:					;PRINT HEX CHAR IN REG A

137    145  	LAHF

137    145  	XCHG	AL,AH

137    145  	PUSH	AX

137    145  	XCHG	AL,AH

138    146  	ROR	AL,1

139    147  	ROR	AL,1

140    148  	ROR	AL,1

141    149  	ROR	AL,1

142    150  	CALL	PNIB			;PRINT NIBBLE

143    151  	POP	AX

143    151  	XCHG	AL,AH

143    151  	SAHF

144    152  	CALL	PNIB

145    153  	RET

146    154  ;

147    155  ERR:					;PRINT ERROR MESSAGE

148    156  ;	D,E ADDRESSES MESSAGE ENDING WITH "$"

149    157  	MOV	CL,PRINTF		;PRINT BUFFER FUNCTION

150    158  	INT	224

151    159  	RET

152    160  ;

153    161  ;

154    162  GNB:					;GET NEXT BYTE

155    163  	MOV	AL,Byte Ptr IBP

156    164  	CMP	AL,80H

157    165  	JNZ	G0

158    166  ;	READ ANOTHER BUFFER

159    167  ;

160    168  ;

161    169  	CALL	DISKR

162    170  	OR	AL,AL			;ZERO VALUE IF READ OK

163    171  	JZ	G0			;FOR ANOTHER BYTE

164    172  ;	END OF DATA, RETURN WITH CARRY SET FOR EOF

165    173  	STC

166    174  	RET

167    175  ;

168    176  G0:					;READ THE BYTE AT BUFF+REG A

169    177  	MOV	DL,AL			;LS BYTE OF BUFFER INDEX

170    178  	MOV	DH,0			;DOUBLE PRECISION INDEX TO DE

171    179  	INC	AL			;INDEX=INDEX+1

172    180  	MOV	Byte Ptr IBP,AL		;BACK TO MEMORY

173    181  ;	POINTER IS INCREMENTED

174    182  ;	SAVE THE CURRENT FILE ADDRESS

175    183  	MOV	BX,BUFF

176    184  	ADD	BX,DX

177    185  ;	ABSOLUTE CHARACTER ADDRESS IS IN HL

178    186  	MOV	AL,M

179    187  ;	BYTE IS IN THE ACCUMULATOR

180    188  	OR	AL,AL			;RESET CARRY BIT

181    189  	RET

182    190  ;

183    191  SETUP:					;SET UP FILE

184    192  ;	OPEN THE FILE FOR INPUT

185    193  	XOR	AL,AL			;ZERO TO ACCUM

186    194  	MOV	Byte Ptr .FCBCR,AL	;CLEAR CURRENT RECORD

187    195  ;

188    196  	MOV	DX,FCB

189    197  	MOV	CL,OPENF

190    198  	INT	224

191    199  ;	255 IN ACCUM IF OPEN ERROR

192    200  	RET

193    201  ;

194    202  DISKR:					;READ DISK FILE RECORD

195    203  	PUSH	BX

195    204  	PUSH	DX

195    205  	PUSH	CX

196    206  	MOV	DX,FCB

197    207  	MOV	CL,READF

198    208  	INT	224

199    209  	POP	CX

199    210  	POP	DX

199    211  	POP	BX

200    212  	RET

200    212  L_1	EQU	$

200    212  	DSEG

200    212  	ORG	Offset L_1

201    213  ;

202    214  ;	FIXED MESSAGE AREA

203    215  SIGNON	DB	'FILE DUMP VERSION 1.4$'

204    216  OPNMSG	DB	CR,LF

205    217  ;

206    218  ;	VARIABLE AREA

207    219  IBP	RS	2			;INPUT BUFFER POINTER

208    220  OLDSP	RS	2			;ENTRY STACK POINTER VALUE

209    221  ;

210    222  ;	STACK AREA

211    223  	RS	64			;RESERVE 32 LEVEL STACK

212    224  STKTOP:

213    225  ;

214    226  	END

L I S T   O F   B A S I C   B L O C K S

Block At 0005 (subr), A86 = 7A74

Entry Active: ------------- Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

-------------------------------------------------------------------

Block At 0100 (code), A86 = 0100

Entry Active: BCDE---A----- Exit Active: BCDEHL-A-----

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   34|------------- |LXI |   H|0000|----HL------- |BCDEHL-A----- |

|   35|----HL------- |DAD |  SP|    |----HL--O---- |BCDEHL-A----- |

|   37|----HL------- |SHLD|01F7|    |------------- |BCDEHL-A----- |

|   39|------------- |LXI |   M|0239|------------- |BCDEHL-A----- |

|   41|------------- |CALL|01C1|    |------------- |BCDEHL-A----- |

-------------------------------------------------------------------

Block At 010D (code), A86 = 0115

Entry Active: BCDEHL-A----- Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   42|-------A----- |CPI |  FF|    |--------OZSPI |BCDEHL-AOZSPI |

|   43|---------Z--- |JNZ |011B|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0112 (code), A86 = 011C

Entry Active: BC--HL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   46|------------- |LXI |   D|01F3|--DE--------- |BCDEHL-AOZSPI |

|   47|------------- |CALL|019C|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0118 (code), A86 = 0122

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   48|------------- |JMP |0151|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 011B (code), A86 = 0125

Entry Active: BCDE----OZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   51|------------- |MVI |   A|  80|-------A----- |BCDE---AOZSPI |

|   52|-------A----- |STA |01F5|    |------------- |BCDE---AOZSPI |

|   54|------------- |LXI |   H|0000|----HL------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0123 (code), A86 = 012D

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   57|----HL------- |PUSH|   H|    |------------- |BCDEHL-AOZSPI |

|   58|------------- |CALL|01A2|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0127 (code), A86 = 0136

Entry Active: BCDE---AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   59|------------- |POP |   H|    |----HL------- |BCDEHL-AOZSPI |

|   60|--------O---- |JC  |0151|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 012B (code), A86 = 0141

Entry Active: -CDEHL-A----- Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   61|-------A----- |MOV |   B|   A|B------------ |BCDEHL------- |

|   64|-----L------- |MOV |   A|   L|-------A----- |BCDEHL-A----- |

|   65|-------A----- |ANI |  0F|    |-------AOZSPI |BCDEHL-AOZSPI |

|   66|---------Z--- |JNZ |0144|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0132 (code), A86 = 014C

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   68|------------- |CALL|0172|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0135 (code), A86 = 014F

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   71|------------- |CALL|0159|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0138 (code), A86 = 0152

Entry Active: BCDEHL-A-ZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   73|-------A----- |RRC |    |    |-------AO---- |BCDEHL-AOZSPI |

|   74|--------O---- |JC  |0151|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 013C (code), A86 = 015A

Entry Active: BCDEHL--OZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   76|----H-------- |MOV |   A|   H|-------A----- |BCDEHL-AOZSPI |

|   77|------------- |CALL|018F|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0140 (code), A86 = 015F

Entry Active: BCDEHL--OZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   78|-----L------- |MOV |   A|   L|-------A----- |BCDEHL-AOZSPI |

|   79|------------- |CALL|018F|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0144 (code), A86 = 0164

Entry Active: BCDEHL--OZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   81|----HL------- |INX |   H|    |----HL------- |BCDEHL--OZSPI |

|   82|------------- |MVI |   A|  20|-------A----- |BCDEHL-AOZSPI |

|   83|------------- |CALL|0165|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 014A (code), A86 = 016C

Entry Active: BCDEHL--OZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   84|B------------ |MOV |   A|   B|-------A----- |BCDEHL-AOZSPI |

|   85|------------- |CALL|018F|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 014E (code), A86 = 0171

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   86|------------- |JMP |0123|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0151 (code), A86 = 0174

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   91|------------- |CALL|0172|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0154 (subr), A86 = 0177

Entry Active: BCDE---AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|   92|------------- |LHLD|01F7|    |----HL------- |BCDEHL-AOZSPI |

|   93|----HL------- |SPHL|    |    |------------- |BCDEHL-AOZSPI |

|   95|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 0159 (subr), A86 = 017E

Entry Active: BCDEHL-AOZSPI Exit Active: -------AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  101|----HL------- |PUSH|   H|    |------------- |BCDE---AOZSPI |

|  102|--DE--------- |PUSH|   D|    |------------- |BC-----AOZSPI |

|  103|BC----------- |PUSH|   B|    |------------- |-------AOZSPI |

|  104|------------- |MVI |   C|  0B|-C----------- |-------AOZSPI |

|  105|------------- |CALL|0005|    |------------- |-------AOZSPI |

-------------------------------------------------------------------

Block At 0161 (subr), A86 = 0195

Entry Active: -------AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  106|------------- |POP |   B|    |BC----------- |BC-----AOZSPI |

|  107|------------- |POP |   D|    |--DE--------- |BCDE---AOZSPI |

|  108|------------- |POP |   H|    |----HL------- |BCDEHL-AOZSPI |

|  109|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 0165 (subr), A86 = 01A8

Entry Active: BCDEHL-AOZSPI Exit Active: -------AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  112|----HL------- |PUSH|   H|    |------------- |BCDE---AOZSPI |

|  113|--DE--------- |PUSH|   D|    |------------- |BC-----AOZSPI |

|  114|BC----------- |PUSH|   B|    |------------- |-------AOZSPI |

|  115|------------- |MVI |   C|  02|-C----------- |-------AOZSPI |

|  116|-------A----- |MOV |   E|   A|---E--------- |-------AOZSPI |

|  117|------------- |CALL|0005|    |------------- |-------AOZSPI |

-------------------------------------------------------------------

Block At 016E (subr), A86 = 01C1

Entry Active: -------AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  118|------------- |POP |   B|    |BC----------- |BC-----AOZSPI |

|  119|------------- |POP |   D|    |--DE--------- |BCDE---AOZSPI |

|  120|------------- |POP |   H|    |----HL------- |BCDEHL-AOZSPI |

|  121|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 0172 (subr), A86 = 01D4

Entry Active: BCDEHL--OZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  124|------------- |MVI |   A|  0D|-------A----- |BCDEHL-AOZSPI |

|  125|------------- |CALL|0165|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0177 (code), A86 = 01D9

Entry Active: BCDEHL--OZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  126|------------- |MVI |   A|  0A|-------A----- |BCDEHL-AOZSPI |

|  127|------------- |CALL|0165|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 017C (subr), A86 = 01DE

Entry Active: BCDEHL-AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  128|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 017D (code), A86 = 01DF

Entry Active: BCDEHL-A----- Exit Active: BCDEHL-A-----

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  132|-------A----- |ANI |  0F|    |-------AOZSPI |BCDEHL-A----- |

|  133|-------A----- |CPI |  0A|    |--------OZSPI |BCDEHL-AO---- |

|  134|--------O---- |JNC |0189|    |------------- |BCDEHL-A----- |

-------------------------------------------------------------------

Block At 0184 (code), A86 = 01E8

Entry Active: BCDEHL-A----- Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  136|-------A----- |ADI |  30|    |-------AOZSPI |BCDEHL-AOZSPI |

|  137|------------- |JMP |018B|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 0189 (code), A86 = 01ED

Entry Active: BCDEHL-A----- Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  140|-------A----- |ADI |  37|    |-------AOZSPI |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 018B (code), A86 = 01EF

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  141|------------- |CALL|0165|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 018E (subr), A86 = 01F2

Entry Active: BCDEHL-AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  142|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 018F (subr), A86 = 01F3

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-A-----

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  145|-------AOZSPI |PUSH| PSW|    |------------- |BCDEHL-A----- |

|  146|-------A----- |RRC |    |    |-------AO---- |BCDEHL-A----- |

|  147|-------A----- |RRC |    |    |-------AO---- |BCDEHL-A----- |

|  148|-------A----- |RRC |    |    |-------AO---- |BCDEHL-A----- |

|  149|-------A----- |RRC |    |    |-------AO---- |BCDEHL-A----- |

|  150|------------- |CALL|017D|    |------------- |BCDEHL-A----- |

-------------------------------------------------------------------

Block At 0197 (code), A86 = 0208

Entry Active: BCDEHL------- Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  151|------------- |POP | PSW|    |-------AOZSPI |BCDEHL-AOZSPI |

|  152|------------- |CALL|017D|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 019B (subr), A86 = 0211

Entry Active: BCDEHL-AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  153|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 019C (subr), A86 = 0212

Entry Active: B-DEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  157|------------- |MVI |   C|  09|-C----------- |BCDEHL-AOZSPI |

|  158|------------- |CALL|0005|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 01A1 (subr), A86 = 0217

Entry Active: BCDEHL-AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  159|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 01A2 (subr), A86 = 0218

Entry Active: BCDEHL------- Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  163|------------- |LDA |01F5|    |-------A----- |BCDEHL-A----- |

|  164|-------A----- |CPI |  80|    |--------OZSPI |BCDEHL-AOZSPI |

|  165|---------Z--- |JNZ |01B3|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 01AA (code), A86 = 0222

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  169|------------- |CALL|01CE|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 01AD (code), A86 = 0225

Entry Active: BCDEHL-A----- Exit Active: BCDEHL-A-ZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  170|-------A----- |ORA |   A|    |-------AOZSPI |BCDEHL-A-ZSPI |

|  171|---------Z--- |JZ  |01B3|    |------------- |BCDEHL-A-ZSPI |

-------------------------------------------------------------------

Block At 01B1 (subr), A86 = 022C

Entry Active: BCDEHL-A-ZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  173|------------- |STC |    |    |--------O---- |BCDEHL-AOZSPI |

|  174|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 01B3 (subr), A86 = 022E

Entry Active: BC-----A----- Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  177|-------A----- |MOV |   E|   A|---E--------- |BC-E---A----- |

|  178|------------- |MVI |   D|  00|--D---------- |BCDE---A----- |

|  179|-------A----- |INR |   A|    |-------A-Z--- |BCDE---A----- |

|  180|-------A----- |STA |01F5|    |------------- |BCDE--------- |

|  183|------------- |LXI |   H|0080|----HL------- |BCDEHL------- |

|  184|--DEHL------- |DAD |   D|    |----HL--O---- |BCDEHL------- |

|  186|------------- |MOV |   A|   M|-------A----- |BCDEHL-A----- |

|  188|-------A----- |ORA |   A|    |-------AOZSPI |BCDEHL-AOZSPI |

|  189|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 01C1 (subr), A86 = 0247

Entry Active: B---HL-A----- Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  193|-------A----- |XRA |   A|    |-------AOZSPI |B---HL-AOZSPI |

|  194|-------A----- |STA |007C|    |------------- |B---HL-AOZSPI |

|  196|------------- |LXI |   D|005C|--DE--------- |B-DEHL-AOZSPI |

|  197|------------- |MVI |   C|  0F|-C----------- |BCDEHL-AOZSPI |

|  198|------------- |CALL|0005|    |------------- |BCDEHL-AOZSPI |

-------------------------------------------------------------------

Block At 01CD (subr), A86 = 0254

Entry Active: BCDEHL-AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  200|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 01CE (subr), A86 = 0255

Entry Active: BCDEHL-AOZSPI Exit Active: -------AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  203|----HL------- |PUSH|   H|    |------------- |BCDE---AOZSPI |

|  204|--DE--------- |PUSH|   D|    |------------- |BC-----AOZSPI |

|  205|BC----------- |PUSH|   B|    |------------- |-------AOZSPI |

|  206|------------- |LXI |   D|005C|--DE--------- |-------AOZSPI |

|  207|------------- |MVI |   C|  14|-C----------- |-------AOZSPI |

|  208|------------- |CALL|0005|    |------------- |-------AOZSPI |

-------------------------------------------------------------------

Block At 01D9 (subr), A86 = 026F

Entry Active: -------AOZSPI Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  209|------------- |POP |   B|    |BC----------- |BC-----AOZSPI |

|  210|------------- |POP |   D|    |--DE--------- |BCDE---AOZSPI |

|  211|------------- |POP |   H|    |----HL------- |BCDEHL-AOZSPI |

|  212|BCDEHL-AOZSPI |RET |    |    |------------- |------------- |

-------------------------------------------------------------------

Block At 01DD (data), A86 = 0282

Entry Active: ------------- Exit Active: -------------

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

|  215|------------- |DB  |0016|    |------------- |------------- |

|  216|------------- |DB  |0002|    |------------- |------------- |

|  219|------------- |DS  |0002|    |------------- |------------- |

|  220|------------- |DS  |0002|    |------------- |------------- |

|  223|------------- |DS  |0040|    |------------- |------------- |

-------------------------------------------------------------------

Block At 0239 (code), A86 = 02DE

Entry Active: BCDEHL-AOZSPI Exit Active: BCDEHL-AOZSPI

-------------------------------------------------------------------

|stmt#| opcode uses  | op | v1 | v2 | opcode kills |  live regs   |

-------------------------------------------------------------------

-------------------------------------------------------------------

INDEX

1

16-bit register translation, 10

8

80 parameter, 6

8080 operation code, 11

8080 program fragments, 9

8080 program origin, 9

8080 register usage, 9

8080 source program, 1

8086 program segment, 9

A

A (A86) parameter, 3

A86 output file, 1

ASM input file, 1

ASM-86, 9

B

B (Block Trace) parameter, 3, 6

Basic Block, 4, 6

Basic Block address, 4

Basic Block Header, 6

C

C (Compact) parameter, 4, 6

code areas, 9

code segments, 5

command line, 3

comment field, 9

CSEG directives, 6

D

data areas, 9

data flow analysis, iii, 1, 4

data segments, 5

differences in assembly language formats, 9

disassemblers, 9

DSEG directives, 6

E

equate statement, 10

error codes, 17

error messages, 17

expression translation, 10

expressions, 9, 10, 11

F

flags, 4

I

input file, 1

J

J (jump) parameter, 5

join Blocks phase, 1

L

L (List) parameter, 5

label generation, 5, 9, 12

letter denotations for registers and flags, 4

List Blocks phase, 1, 2

live function, 11

M

M (Memory) register, 10

macros, 1

memory overflow, 18

monitoring the translation, 1, 6

N

N (Number) parameter, 6

NO parameter, 6

O

operand field abbreviations, 10

operand field translation, 11

operand fields, 9

operation code, 9

ORG statement, 9

output files, 1

output line, 9

overlays, 1, 2

P

P (PRN) parameter, 3

parameter list, 3

parameter syntax, 3

parameters, 2

PIP, 5

PRN file, 1, 17

processed files, 2

program fragments, 9

program graph, 18

program segment, 9

Pseudo-assembly Process error messages, 17

R

R (Return) parameter, 6

register translation, 10

register usage, 9

registers, 4

repeat loops, 1

root module, 1

S

S (Segment) parameter, 5, 6

Setup Blocks phase, 1

short function, 12

short jump analysis, 5

Symbol Setup phase, 1

syntax, 3

T

T (TMP) parameter, 3

temporary ($$$) file, 1

Translate-86 error messages, 17

Translate-86 phase, 2

translated program format, 9

translation parameters, 3

translation phases, 1

translation table, 11