CHINFO.EXE

begin:

PRINT "          DESCATOC Program Data Sender V1.0          "

PRINT

PRINT "   Copyright JD Digital Electronics Ltd, Sheffield, UK   "

PRINT "Copyright 1997, 1998. Patent Pending. All Rights Reserved."

PRINT: PRINT: PRINT: PRINT

OPEN "DESC1.DAT" FOR INPUT AS #1                'get directory data

INPUT #1, WW$                                                         'ww$ now has this info

OPEN "IO.DAT" FOR INPUT AS #2                        'get address of card

INPUT #2, IO$

RESET

IF IO$ = "" THEN

PRINT TAB (10);" There is no DESCATOC Card on this system.”

PRINT

PRINT

PRINT TAB (10);" Aborting Data Send..Continuing Batch Job…..”

END

END IF

XC1$ = LEFT$(IO$, 3)                                              'get address value for P

XC2$ = "&H" + XC1$                                                'signal a HEX value is needed

P = VAL(XC2$)                                                           'P has the base 10 value now.

XC1$ = MID$(WW$, 6, 3)                                         'P2 value of I/O address in Hex.

XC2$ = "&H" + XC1$                                                ' signal this is a HEX value

P2 = VAL(XC2$)                                                         'P2 now has the seconf I/O value

begin2:

A = INP(P2)                                                                 '1     setting up to read

X = (A AND &H06)                                                    'BRDY and HALT to be checked

IF X<>&H02 THEN GOTO begin2                           'if Z80 not ready, wait.

OUT (P), &H00                                                            'ready, so send anything.

OUT (P2), &H01                                                          'release HALT state, read in 00h.

FOR T = 1 TO 10: NEXT T                                        'small delay

OUT (P2), &H03                                                          'signal BSTB inactive now.

back1:

A = INP(P2)                                                                   '2     setting up to read

X = (A AND &H05)                                                      'ARDY and HALT to be checked

IF X<>&H02 THEN GOTO back1                              'Z80 not ready go back

A = INP(P)                                                                     'fetch info on what channel

'Card has selected

OUT (P2), &H01                                                           'BSTB goes low, Z80 conts program

FOR D = 1 TO 10: NEXT D                                         'small delay

OUT (P2), &H03                                                            'BSTB sent inactive

IF A = &HF0 THEN CH=0: GOTO ch0

IF A = &H01 THEN CH=1: GOTO ch1                       'get disk data for appr. channel

IF A = &H02 THEN CH=2: GOTO ch2

ch3:

CH=3

GOSUB gitthis                                                                 'retreiving commands for rch *

OPEN WW$ + "\DES3ABI.DAT" FOR INPUT AS #1   'chan 3 p. up commands

INPUT #1, W2$                                                               'w2$ now has p. up commands

OPEN WW$ + "\DES3BBI.DAT" FOR INPUT AS #2   'chan 3 p. down commands

INPUT #2, W3$

RESET

GOSUB gitthis2                                                                   'sending commands to card...

GOSUB send

GOTO done                                                                          'done, continuing batch job..

ch0:

GOSUB gitthis

OPEN WW$ + "\DES0ABI.DAT" FOR INPUT AS #1

INPUT #1, W2$

OPEN WW$ + "\DES0BBI.DAT" FOR INPUT AS #2

INPUT #2, W3$

RESET

GOSUB gitthis2

GOSUB send

GOTO done

ch1:

GOSUB gitthis

OPEN WW$ + "\DES1ABI.DAT" FOR INPUT AS #1

INPUT #1, W2$

OPEN WW$ + "\DES1BBI.DAT" FOR INPUT AS #2

INPUT #2, W3$

RESET

GOSUB gitthis2

GOSUB send

GOTO done

ch2:

GOSUB gitthis

OPEN WW$ + "\DES2ABI.DAT" FOR INPUT AS #1

INPUT #1, W2$

OPEN WW$ + "\DES2BBI.DAT" FOR INPUT AS #2

INPUT #2, W3$

RESET

GOSUB gitthis2

GOSUB send

GOTO done

gitthis:

PRINT " Retrieving commands for Ring Channel ";CH

RETURN

gitthis2:

PRINT " sending commands to the DESCATOC RAM Memory..."

RETURN

send:

FOR L2 = 1 TO 2                                                                                         'do twice for p up / p down

IF L2 = 1 THEN W4$ = W2$                                                                      'power up goes first.

IF L2 = 2 THEN W4$ = W3$                                                                      'power down goes second.

L = LEN(W4$)                                                                                             'get length of the data string

HI = INT(L/256)                                                                                          'calculate hi byte value.

LOW = L-(HI * 256)                                                                                    'calculate low byte value.

send2:

A = INP(P2)                                                                                                   '3    setting up to read HALT/BRDY

X = (A AND &H06)                                                                                      'strip word only to see HALT/BRDY

IF X<>&H01 THEN GOTO send2                                                              'Z80 not ready, wait.

OUT (P), HI                                                                                                   'ready, so send the hi byte

OUT (P2), &H01                                                                                           'signal BSTB enabled, Z80 fetches data

FOR T = 1 TO 10: NEXT T                                                                         'small delay

OUT (P2), &H03                                                                                           'signal BSTB inactive.

send3:

A = INP(P2)                                                                                                    '4    setting up to read HALT/BRDY

X = (A AND &H06)                                                                                       'strip word only to see HALT/BRDY

IF X<>&H01 THEN GOTO send3                                                                'Z80 not ready so wait.

OUT (P), LOW                                                                                                'ready, so send the low value.

OUT (P2), &H01                                                                                             'signal BSTB enabled, Z80 fetches data

FOR T = 1 TO 10: NEXT T                                                                           'small delay

OUT (P2), &H03                                                                                             'signal BSTB inactive.

'AT THIS POINT THE BC REGISTER PAIR HAS THE LENGHT OF THE DATA STRING.

IF HI = &H01 THEN T1 = 1: T2 = 0: G = 255                                             'set up for 16 bit cou

IF HI = &H00 THEN T1 = 1: T2 = 0: G = LOW

send5:

FOR R = T1 TO G                                                                                            'set up loop to send all the chars.

send4:

A = INP(P2)                                                                                                       '5   setting up to read HALT/BRDY only.

X = (A AND &H06)                                                                                          'strip word only to see HALT/BRDY

IF X<>&H01 THEN GOTO send4                                                                   'Z80 not ready so wait a while.

A$ = MID$(W4$, R, 1)                                                                                     'get the character of the saved string

B = ASC(A$)                                                                                                      'get the characters numeric value

OUT (P), B                                                                                                          'send this value to the DESCATOC Card.

OUT (P2), &H01                                                                                                'get Z80 to read in this character.

FOR T = 1 TO 10: NEXT T                                                                               'do a small delay

OUT (P2), &H03                                                                                                'signal BSTB inactive, Z80 fetches next byte.

NEXT R                                                                                                               'do all most of the string.

IF HI = &H01 AND T2 = 0 THEN T1 = G + 1: T2 = 1: G = LEN(W4$): GOTO send5

'the above line means, do the remainder if length above 255 chars.

'this concludes the sending of the Power Up instructions.

'now do the Power down instructions...

NEXT L2                                                                                                             'after this, p.down done.

done:

PRINT: PRINT:

PRINT " Finished transferring data from the Hard Disk to DESCATOC RAM.

PRINT

PRINT "           >>>>>>>>>>>Continuing Batch Job........<<<<<<<<<<

CHINFO.EXE Concluded.